| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F709 |
| --- | --- |

| Title: | Crosscheck of Samsung’s proposal JCTVC-F598 by Huawei |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Qiu Shen Haitao Yang Haoping Yu Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | +86-755-28788991 shenqiu@huawei.com haitao.yang@huawei.com haoping.yu@huawei.com |
| Source: | Huawei Technologies Co. Ltd |  |  |

_____________________________

# Abstract

An adaptive context type selection method for significance map was proposed in JCTVC-F598 by Samsung. The best context model type is selected according to coefficient distributions, and signaled to decoder on a given condition. The software implementation is consistent with the algorithm description, and crosscheck results show that this method can provide 0.3% gain in HE All Intra configuration, which matches the results from proponent.

# Cross-verification

## Code-level check

The proposal was implemented on top of HM3.0 only for I_SLICE.

SAMSUNG_SIGCONTYPE macro is used for the proposed modification. Two types of context are used for significance map coding of large transforms, and alternative context is indicated to decoder only when last non-zero coefficient locates outside the predefined region.

SIMPLE_CONTEXT_SIG macro was turned off in the proposed method.

## Test condition

The test conditions specified in the JCTVC-E700 are used for the simulation. VS2005 x64 compiler is used to generate the executable file from the provided source code. Simulations were done on computer cluster running Windows Server 2008 x64 OS.

## Test results

Only three HE configurations are used for the simulation.

The executables generated from the provided source code can be used for the encoding and decoding test. The decoded sequences match the reconstructed sequences.

The RD data provided by Samsung are exactly the same as the results we obtained. The encoding and decoding time we obtained is similar to that from proponent. Detailed RD data can be found in the attached excel sheet.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.61 | -0.49 | -0.47 |
| Class B | -0.26 | -0.17 | -0.22 |
| Class C | -0.11 | -0.17 | -0.22 |
| Class D | -0.13 | -0.17 | -0.24 |
| Class E | -0.12 | -0.15 | 0.04 |
| All | -0.25 | -0.23 | -0.24 |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 100% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.16 | -0.42 | -0.84 |
| Class B | -0.06 | -0.11 | -0.03 |
| Class C | 0.00 | 0.04 | -0.18 |
| Class D | 0.02 | -0.09 | -0.27 |
| Class E |  |  |  |
| All | -0.05 | -0.14 | -0.31 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.04 | 0.45 | -0.01 |
| Class C | 0.06 | 0.12 | 0.11 |
| Class D | 0.09 | 0.60 | 1.23 |
| Class E | 0.15 | 1.75 | 0.35 |
| All | 0.08 | 0.65 | 0.40 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |