| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F710 |
| --- | --- |

| Title: | The result of SIMD implementation for Core Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chanyul Kim Jeong-Yong Choi Minseok Choi Kulbhushan Pachauri Rahul Nagpal Biplab Raut | Email: dionism@samsung.com eldragon.choi@samsung.com minseok48.choi@samsung.com kb.pachauri@samsung.com n.rahul@samsung.com biplab.raut@samsung.com |  |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution provides results on SIMD optimization of core inverse transform proposed for current HEVC and new proposal JCTVC-F251. The SSE SIMD assembly code for x86 64-bit CPU is hand-written carefully. The optimized source code is applied to HM3.1 and examined. The quantitative results obtained by Valgrind are provided both for partial butterfly and matrix multiplication. The quantitative results show that core inverse transforms of JCTVC-E243 and JCTVC-F251 execute the exact same amount of instructions.

# Introduction

In the 5th meeting of the JCT-VC, the transform design of [1] was adopted and implemented in current HM software in two forms: as partial butterfly and matrix multiplication. In [2], a new core transform is proposed, which keeps the same design as [1], but provides additional implementation form so-called full butterfly.

This contribution reports the complexity measures of aforementioned two transforms, which are optimized using SIMD operation. We implemented two 8x8 inverse transforms (i.e., JCTVC-E243 and JCTVC-F251) both for partial butterfly and matrix multiplication using SIMD operation on x86 64-bit CPU. The complexity of the two transforms was measured quantitatively using the Valgrind software (http://www.valgrind.org/).

In Section 2, SIMD implementation of partial butterfly and matrix multiplication is described, and the experimental results are followed in Section 3. Finally, conclusion is mentioned in Section 4.

# SIMD Implementation

## Matrix Multiplication

Following steps are performed to calculate 2D inverse transform using matrix multiplication:

- Transpose

- Horizontal transformation (HT)

- Vertical transformation (VT)

- Naive

Naive matrix multiplication is very simple and calculates single value of each transformation stage. In HT, transposed coefficient and transposed input are loaded to compute the single value. Intermediate result is written in column manner. Similarly in VT, transposed coefficient and intermediate result of HT are loaded to compute the single value. In each transformation stage, single value is calculated using multiplication and accumulation followed by horizontal addition.

- Fast Matrix Multiplication

Due to fixed ordering imposed by core transform in HM, method from [3] is adopted and modified to suit current core transform. In HT, 4 values are calculated by prearranging the transform coefficient matrix. Result of HT are computed such that no further additional operation (shuffling of input) are required for VT. In VT, 4 final values are calculated by multiplying prearranged transform coefficient matrix with intermediate result from HT. In each transformation stage, 4 values are calculated using multiplication and accumulation followed by vertical addition.

## Partial Butterfly

Following steps are performed to calculate 2D inverse transform using partial butterfly:

- Transpose

- Horizontal transformation (HT)

- Transpose

- Vertical transformation (VT)

Current partial butterfly SIMD implementation is based on complete unraveling, followed by multiplication of two vectors of 16-bit short values (the coefficient vector and the transposed input vector) and adding the result of multiplication of adjacent operand.

This implementation uses multiplication and accumulation (PMADDWD) followed by couple of horizontal addition (PHADDD) of multiplied intermediate 32-bit integer values.

# Experimental Results

SIMD-optimized schemes described in previous section are analyzed using Valgrind. Fig. 1 shows an example of graphical representation of function call relationships obtained by Valgrind.

Fig. 1 Graphical representation of function call relationship of xID() function.

Table 1 presents the quantitative results of each scheme in unit of Instruction Fetch Cost (IFC), which is same as cycle estimation in current experiment.

As shown in Table 1, all the schemes of JCTVC-E243 and JCTVC-F251 execute the exact same instruction cost, since JCTVC-E243 and JCTVC-F251 adopt the same schemes but different coefficient values.

Table 1. Comparison of Instruction Fetch Cost

|  |  | Total IFC | Called Count | Unit IFC |
| --- | --- | --- | --- | --- |
| C-Only Partial Butterfly | E243 | 389312560 | 282520 | 1378 |
|  | F251 | 390219284 | 283178 | 1378 |
| SIMD Partial Butterfly | E243 | 203979440 | 282520 | 722 |
|  | F251 | 204454516 | 283178 | 722 |
| SIMD Naive Matrix Multiplication | E243 | 299471200 | 282520 | 1060 |
|  | F251 | 300168680 | 283178 | 1060 |
| SIMD Fast Matrix Multiplication | E243 | 193243680 | 282520 | 684 |
|  | F251 | 193693752 | 283178 | 684 |

# Conclusion

We performed quantitative analysis for core transforms proposed in JCTVC-E243 and JCTVC-F251. The 8x8 IDCT functions of HM software are SIMD-optimized on x86 64-bit CPU. The SIMD-optimized partial butterfly and matrix multiplication schemes are analyzed using the Valgrind software. The analytical results verify that the two core transforms show the exact same complexity.

# References

- A. Fuldseth, G. Bjøntegaard, M. Sadafale, M. Budagavi, “Transform design for HEVC with 16 bit intermediate data representation,” JCTVC-E243, 5th. JCTVC, Meeting, Geneva, Mar. 2011.

- E.Alshina, A.Alshin, I.-K. Kim, P. Topiwala, “CE10: Full-factorized core transform proposal by Samsung/FastVDO,” JCTVC-F251, 6th. JCTVC, Meeting, Torino, July, 2011.

- X. Zhou, E. Q. Li, and Y.-K. Chen, “Implementation of H.264 Decoder on General-Purpose Processors with Media Instructions,” in SPIE Conf. on Image and Video Communications and Processing, pp.224-235, Jan. 2003.

# Patent rights declaration(s)