| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F713 |
| --- | --- |

| Title: | CE9: Result about the combination of experiments ROB02 and ROB04. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information/ Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche, Patrice Onno Tangi Poirier, Christophe Gisquet Rue de la Touche Lambert 35517 Cesson-Sévigné, France Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: Email: | {guillaume.laroche, patrice.onno christophe.gisquet}@crf.canon.fr {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Canon Research Centre France, MediaTek |  |  |

_____________________________

# Abstract

This contribution reports the results of the combination of two experiments ROB02 and ROB04 presented in the scope of CE9. It is reported that this combination leads to an average loss of 0.2% in RA-HE, 0.1% in RA-LC, 0.2% in LB-HE and 0.1% in LB-LC, respectively. Compared to ROB02 only, the combination brings an average coding gain of 0.4% in RA-HE, 0.2% in RA-LC, 0.6% in LB-HE and 0.3% in LB-LC, respectively. Compared to ROB04 only, the combination brings an average coding gain of 0.2% in RA-HE, 0.3% in RA-LC, 0.2% in LB-HE and 0.4% in LB-LC, respectively.

It is also reported that the encoding time is 102% in RA-HE, 103% in RA-LC, 103% in LB-HE and 104% in LB-LC, respectively. The decoding time is reportedly 96% in RA-HE, 97% in RA-LC, 96% in LB-HE and 97% in LB-LC, respectively.

# Introduction

In HM3.0, AMVP and Merge candidates consist not only of spatial MVP candidates but also of temporal MVP candidates from previous reference pictures. Before coding the index of the best MVP candidate, redundant MVPs are removed from the candidates set. This might cause parsing errors when previous reference pictures are lost.

In JCTVC-E219 [2] and in JCTVC-F474 [1] , Canon presented two tools to solve the parsing issue related to the temporal motion prediction for both Inter and Skip/Merge modes. The first tool (Tool 1) consists in forcing the availability of the temporal candidate for the Inter and Merge modes even if this predictor doesn’t exist. Thanks to this tool, the parsing issue is fully solved. The second tool (Tool 2) aims at improving the coding efficiency of the first tool. This second tool consists in replacing the duplicate candidates by candidates with different (non-redundant) values.

In JCTVC-F052 [3], a basic scheme with parsing robustness by skipping the process of removing redundant motion vector predictors (MVPs) and forcing the availability of the temporal MVP was firstly provided by MediaTek. To reduce the bit rate increases, two approaches are tested in ROB04. The first approach is replacing the redundant MVPs by other substitutes, and the second approach is reducing redundant MVPs without comparing motion vector values.

# Technical description of the combination

## ROB02 description (Canon)

### Tool 1 and Tool 2 of ROB02

In JCTVC-F474[1], the detailed technical description of the ROB02 technology has been performed. This section summarizes the main technical aspects of the 2 tools in ROB02:

- The determination of the number of candidates for the Merge/Skip and Inter modes is simply determined by only checking the availability of the candidates.

- If the bottom right temporal predictor and the collocated center are not available the temporal predictor is replaced by the zero value. The derivation process of predictor is the same as the HM3.0 anchor. So, the duplicate candidates are removed from the set but it is not taken into account for the index coding.

- In order to improve the coding efficiency, the duplicate candidates are replaced by candidates with different values (non-redundant). This offers the possibility to efficiently use the signaling bits which have to be transmitted to solve the parsing issue and which may cause coding efficiency loss.

- Adaptation of the ‘avoid imitating 2Nx2N’ concept for Merge modes as follows: left and right predictors of the Merge modes are removed from the set for the second 2NxN PU.

- Fast derivation at the decoder: on the decoder side, the index of predictors can be known before applying the predictor set derivation. In ROB02, a candidate is considered to be a duplicate candidate compared to the previous candidates already inserted in the list of candidates. This gives a significant reduction with 95% decoding runtime compared to HM3.0.

- For ROB02, we disabled the RD cost evaluation of non-duplicate candidate obtained with Tool 2 for Merge Mode 2Nx2N (and not for Merge Skip mode). This leads to a reduction of 5% for the encoding time with 0.1% BDR increase.

## ROB04 description (MediaTek)

In JCTVC-F052 [3], MediaTek presented a basic approach to solve the parsing problem caused by data loss of reference pictures. To achieve parsing robustness, two constraints are applied for the basic approach of parsing robustness:

- Avoid the process of removing redundant MVPs

- Force the availability of the temporal MVP as long as it is inside the picture boundaries

Based on the basic approach, two tools are tested to improve the coding efficiency of the parsing robustness solution. The first one (Tool 1) is replacing the redundant MVPs by other substitutes, and the second one (Tool 2) is reducing redundant MVPs by comparing motion IDs instead of MV values. This section summarizes the main technical aspects of ROB04:

- Tool 1 replaced redundant MVPs by searching other substitutes or simply using zero MV or the MVP plus a constant MV to represent the redundant MVPs

- Tool 2 is proposed to reduce redundant MVPs without using MV values before finding other substitutes.

- In Tool 2, a motion identification (ID) is introduced to describe the inheritance process of motion information passed from a coded PU to the current PU to follow the trail of the MV.

- A unique motion ID is assigned for each new MV created in the Inter mode or in the Merge and Merge-Skip modes merged with a new temporal MVP.

- The motion ID can be inherited through the Merge and Merge-Skip modes. For example, as shown in the Figure 1, the merged PU inherits the motion ID of the selected candidate MVP B.

- By comparing the motion ID, the MVP having the same motion ID as a previous MVP in the candidate set can thus be removed.

- For the ‘avoid imitating 2Nx2N’ concept, those MVPs whose motion ID are identical to that of first PU can be removed. For the fourth PU in NxN Merge mode, those MVPs which will make this NxN Merge as one of 2Nx2N, 2NxN or Nx2N Merge can also be removed using motion IDs.

Figure 1.  An example of motion ID inheritance through motion sharing.

# ROB06: combination of ROB02 and ROB04

## ROB06 description

The combination of both experiments has been done as follows:

- Merge mode:

- The concept of Motion ID is used to determine the list size of motion vector predictors

- If some candidates in the list are duplicates, they are replaced by non-duplicate candidates with the same process as ROB02.

- AVMP:

- The same process as described for ROB02 is used.

The use of the Motion ID concept reduces the number of predictors in the Merge list compared to the case where we only check the availability of spatial candidates. This reduces the amount of RD estimation.

## Simplification of ROB06

To simplify the complexity of ROB06, we avoid using the co-located MV as a substitute for the redundant spatial MVP. If a spatial predictor is a duplicate predictor, following predictors are tested until a non-duplicate value is found:

- The predictor without ¼-pel accuracy.

- The predictor without fractional pixel accuracy.

- The predictor plus a delta on horizontal component of the motion vector predictor.

- The predictor minus a delta on horizontal component of the motion vector predictor.

- The predictor plus a delta on vertical component of the motion vector predictor.

- The predictor minus a delta on vertical component of the motion vector predictor

# Results

The implementation was conducted in the HM3.0 with the inclusion of bug #146 as described in JCTVC-E709 [5]. Simulation results are based on sequences and common test configurations specified in the document JCTVC-E700 [4].

## Combination of ROB02 and ROB04

The following Table 1 gives the results when combining the two experiments ROB02 and ROB04. For this combination, only Tool 2 of ROB04 is used and the comparison of the Motion ID values are only performed for the Skip/Merge and Merge modes to eliminate duplicated candidates having the same motion ID. On average, this combination gives only 0.1% loss compared to the HM3.0 Anchor. The encoding time is 102% in RA-HE, 103% in RA-LC, 103% in LB-HE and 104% in LB-LC, respectively. The decoding times are similar to those of ROB02, where a decrease is observed compared to HM3.0 anchors: 96% in RA-HE, 97% in RA-LC, 96% in LB-HE and 97% in LB-LC, respectively.

|  |  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A | 0.1 | 0.1 | 0.0 | 0.1 | 0.2 | -0.1 |
|  | Class B | 0.1 | 0.2 | 0.1 | 0.0 | 0.1 | 0.1 |
|  | Class C | 0.2 | 0.3 | 0.3 | 0.3 | 0.2 | 0.3 |
|  | Class D | 0.2 | 0.1 | 0.2 | 0.2 | 0.1 | 0.1 |
|  | Class E |  |  |  |  |  |  |
|  | Overall | 0.2 | 0.1 | 0.2 | 0.1 | 0.1 | 0.1 |
|  | Enc Time[%] | 102% |  |  | 103% |  |  |
|  | Dec Time[%] | 96% |  |  | 97% |  |  |
|  |  | Low delay B HE |  |  | Low delay B LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | 0.0 | -0.1 | -0.3 | -0.3 | -0.2 | 0.1 |
|  | Class C | 0.1 | 0.2 | 0.0 | 0.1 | 0.0 | 0.0 |
|  | Class D | 0.1 | -0.4 | 0.5 | 0.1 | 0.1 | -0.1 |
|  | Class E | 0.5 | 1.0 | 0.6 | 0.8 | 0.8 | 1.0 |
|  | Overall | 0.2 | 0.1 | 0.1 | 0.1 | 0.1 | 0.2 |
|  | Enc Time[%] | 103% |  |  | 104% |  |  |
|  | Dec Time[%] | 96% |  |  | 97% |  |  |

Table 1: Simulation results for ROB02+ROB04 experiment

## Simplification of ROB06

Table 2 gives the results of the simplification of ROB06. For this simplification, co-located MV is not used as a replacement for the redundant spatial MVP in both AMVP and the Skip/Merge modes. The results show that the simplification reduces the complexity while achieving the same coding efficiency.

|  |  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
|  | Class B | 0.1 | 0.2 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | Class C | 0.2 | 0.3 | 0.4 | 0.3 | 0.3 | 0.3 |
|  | Class D | 0.2 | 0.3 | 0.2 | 0.1 | 0.0 | 0.0 |
|  | Class E |  |  |  |  |  |  |
|  | Overall | 0.2 | 0.2 | 0.2 | 0.1 | 0.1 | 0.1 |
|  | Enc Time[%] | 104% |  |  | 105% |  |  |
|  | Dec Time[%] | 101% |  |  | 102% |  |  |
|  |  | Low delay B HE |  |  | Low delay B LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | 0.1 | -0.2 | -0.3 | -0.3 | -0.2 | -0.1 |
|  | Class C | 0.2 | 0.2 | 0.1 | 0.0 | -0.1 | -0.1 |
|  | Class D | 0.1 | -0.1 | 0.7 | 0.2 | -0.1 | 0.0 |
|  | Class E | 0.6 | 0.7 | 0.1 | 0.7 | 0.4 | 1.4 |
|  | Overall | 0.2 | 0.1 | 0.1 | 0.1 | -0.1 | 0.2 |
|  | Enc Time[%] | 105% |  |  | 106% |  |  |
|  | Dec Time[%] | 103% |  |  | 102% |  |  |

Table 2: Simulation results for simplification method 1 on ROB02+ROB04 experiment

## Reminder: ROB02 results of JCTVC-F474

|  |  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A | 0.4 | 0.3 | 0.5 | 0.2 | 0.4 | 0.2 |
|  | Class B | 0.6 | 0.5 | 0.4 | 0.5 | 0.6 | 0.5 |
|  | Class C | 0.7 | 0.8 | 0.8 | 0.4 | 0.5 | 0.5 |
|  | Class D | 0.6 | 0.7 | 0.6 | 0.3 | 0.3 | 0.3 |
|  | Class E |  |  |  |  |  |  |
|  | Overall | 0.6 | 0.6 | 0.6 | 0.4 | 0.4 | 0.4 |
|  | Enc Time[%] | 102% |  |  | 104% |  |  |
|  | Dec Time[%] | 96% |  |  | 94% |  |  |
|  |  | Low delay B HE |  |  | Low delay B LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | 0.5 | 0.1 | -0.4 | 0.1 | 0.2 | 0.5 |
|  | Class C | 0.5 | 0.4 | 0.2 | 0.2 | 0.1 | 0.3 |
|  | Class D | 0.5 | 0.1 | 1.0 | 0.4 | -0.2 | 0.0 |
|  | Class E | 1.9 | 2.2 | 1.4 | 1.4 | 2.1 | 2.7 |
|  | Overall | 0.8 | 0.6 | 0.4 | 0.4 | 0.4 | 0.7 |
|  | Enc Time[%] | 102% |  |  | 104% |  |  |
|  | Dec Time[%] | 95% |  |  | 94% |  |  |

Table 3: Simulation results of ROB02.

Compared to ROB02 only (see Table Table 3), the combination brings an improvement  of 0.4% in RA-HE, 0.2% in RA-LC, 0.6% in LB-HE and 0.3% in LB-LC, respectively.

## Reminder: ROB04 results of JCTVC-F052

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.6 | 0.6 | 0.4 | 0.6 | 0.6 |
| Class B | 0.2 | 0.2 | 0.2 | 0.3 | 0.3 | 0.3 |
| Class C | 0.4 | 0.4 | 0.6 | 0.6 | 0.7 | 0.7 |
| Class D | 0.4 | 0.4 | 0.3 | 0.4 | 0.4 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.4 | 0.4 | 0.4 | 0.5 | 0.5 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | -0.1 | -0.2 | 0.3 | 0.1 | 0.1 |
| Class C | 0.3 | 0.1 | 0.1 | 0.4 | 0.4 | 0.3 |
| Class D | 0.2 | -0.1 | 0.8 | 0.3 | 0.3 | -0.2 |
| Class E | 0.8 | 1.0 | -0.2 | 1.3 | 1.2 | 1.2 |
| All | 0.3 | 0.2 | 0.1 | 0.5 | 0.4 | 0.3 |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

Table 4: Simulation results of ROB04.

Compared to ROB04 only (see Table Table 4), the combination brings an improvement of 0.2% in RA-HE, 0.3% in RA-LC, 0.2% in LB-HE and 0.4% in LB-LC, respectively.

# Parsing Check

In order to be sure that the combination of ROB02 and ROB04 experiments solves entirely the parsing issue, we have simulated losses at decoder side (as in JCTVC-E474 [2]). In this implementation, for the decoding of each frame of the sequence, all its reference frames are re-initialized. This means that all data for all CUs of the previous frames are initialized (CU partitioning, mode, reference frames indexes, directions, motion vectors, etc.) to the default values. Please note that the slice headers are also considered lost, except the POC number. Indeed, the same parsing issue when the POC of the reference frame is missing occurs in HEVC as in H.264/AVC. In an error resilient implementation, this issue can be solved easily. This functionality can be enabled/disabled by a macro in our CE9 software and easily tested.

For this experiment (ROB02 +ROB04) and each test case (RAHE, RALC, LDHE, LDLC), the decoder did not crash which demonstrates that the combined solutions fully solve the parsing issue of Inter and Skip/Merge modes.

# Conclusion

This contribution provides the results of ROB02 and ROB04 experiments of CE9. The combination experiments fully solve the parsing issue of temporal predictor. Compared to the individual contribution JCTVC-F474 [2] and JCTVC-F052 [3], a significant improvement in terms of BDR is obtained while keeping the encoding and decoding runtime at the same level of the initial proposals.

Based on the results provided in this contribution, we recommend adopting this combination that fully solves the parsing issue in HEVC WD 4.

# References

- G. Laroche, P. Onno, T. Poirier, C. Gisquet, “CE9: Description of experiments ROB01 and ROB02”, JCTVC-F474, 6th Meeting: Torino, IT, 14-22 July, 2011.

- G. Laroche, C. Gisquet, P. Onno, E. François, N. Ouedraogo, J. Ricard, “Robust solution for the AMVP parsing issue”, JCTVC-E219, 5th Meeting: Geneva, CH, 16-23 March, 2011.

- J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei, “CE9: Results of Experiment ROB04”, JCTVC-F052, 6th Meeting: Torino, IT, 14-22 July, 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011.

- Y.-W. Huang, B. Bross, M. Zhou, W.-J. Chien, I.-K. Kim, “Description of Core Experiment 9: MV Coding and Skip/Merge Operations”, JCTVC-E709, 5th Meeting: Geneva, CH, 16-23 March, 2011

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).