| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F714 |
| --- | --- |

| Title: | High-level syntax mismatches between WD and HM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Qiu Shen Huawei Technologies Co., Ltd. Bantian, Longgang District Shenzhen 518129, China Ye-Kui Wang Huawei Technologies 400 Crossing Blvd, 2nd Floor Bridgewater, NJ 08807, USA | Tel: Email: | +86 755 2878 8206 shenqiu@huawei.com +1 908 541 3518 yekui.wang@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides an analysis of high-level syntax related mismatches, including both syntax mismatches and behavior mismatches, between the HEVC WD (JCTVC-E603_d8) and the reference software HM-3.2.

# Syntax mismatches

The syntax elements in the NAL unit header, SPS, PPS and slice header in the WD and the reference software (HM-3.2) are compared, and mismatches are pointed out as follows:

- A blue highlight indicates that the syntax element is present in the WD but not in HM-3.2.

- A yellow highlight indicates that the syntax element is present in HM-3.2 but not in the WD.

- A pink highlight indicates that the syntax element is present in both the WD and HM-3.2, but not in an aligned manner (e.g., syntax element name or “semantics”).

Note that in the tables below, the order of syntax elements in the right column is not exactly the same as in HM-3.2, because some syntax elements have been moved to correspond to the syntax element in the WD, for easy comparison.

## NALU header syntax

There is no mismatch in this part.

## SPS syntax

| WD. profile_idc. reserved_zero_8bits /* equal to 0 */. level_idc. seq_parameter_set_id. max_temporal_layers_minus1. pic_width_in_luma_samples. pic_height_in_luma_samples. bit_depth_luma_minus8. bit_depth_chroma_minus8. pcm_bit_depth_luma_minus1. pcm_bit_depth_chroma_minus1. log2_max_frame_num_minus4. pic_order_cnt_type. if( pic_order_cnt_type = = 0 ). log2_max_pic_order_cnt_lsb_minus4. else if( pic_order_cnt_type = = 1 ) {. delta_pic_order_always_zero_flag. offset_for_non_ref_pic. num_ref_frames_in_pic_order_cnt_cycle. for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ). offset_for_ref_frame[ i ]. }. max_num_ref_frames. gaps_in_frame_num_value_allowed_flag. log2_min_coding_block_size_minus3. log2_diff_max_min_coding_block_size. log2_min_transform_block_size_minus2. log2_diff_max_min_transform_block_size. log2_min_pcm_coding_block_size_minus3. max_transform_hierarchy_depth_inter. max_transform_hierarchy_depth_intra. chroma_pred_from_luma_enabled_flag. loop_filter_across_slice_flag. sample_adaptive_offset_enabled_flag. adaptive_loop_filter_enabled_flag. pcm_loop_filter_disable_flag. cu_qp_delta_enabled_flag. temporal_id_nesting_flag. | HM-3.2. max_temporal_layers_minus1. pic_width_in_luma_samples. pic_height_in_luma_samples. pad_x. pad_y. use_ldc_flag. use_mrg_flag. use_lm_chroma_flag. bit_depth_increment. pcm_bit_depth_luma_minus1. pcm_bit_depth_chrom_minus1. max_cu_width. max_cu_depth. for(i=0; i<max_cu_depth; i++){. amvp_mode. }. lf_cross_slice_boundary_flag. log2_min_transform_block_size_minus2. log2_diff_max_min_transform_block_size. pcm_log2_min_size_minus3. max_transform_hierarchy_depth_inter_minus1. max_transform_hierarchy_depth_intra_minus1. use_sao_flag. use_alf_flag. pcm_filter_disable_flag. use_dqp_flag. temporal_id_nesting_flag. |
| --- | --- |

## PPS syntax

| WD. pic_parameter_set_id. seq_parameter_set_id. entropy_coding_mode_flag. num_temporal_layer_switching_point_flags. for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ). temporal_layer_switching_point_flag[ i ]. num_ref_idx_l0_default_active_minus1. num_ref_idx_l1_default_active_minus1. pic_init_qp_minus26 /* relative to 26 */. constrained_intra_pred_flag. slice_granularity. shared_pps_info_enabled_flag. if( shared_pps_info_enabled_flag ). if( adaptive_loop_filter_enabled_flag ). alf_param( ). if( cu_qp_delta_enabled_flag ). max_cu_qp_delta_depth. | HM-3.2. num_temporal_layer_switching_point_flags. for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ). temporal_layer_switching_point_flag[ i ]. constrained_intra_pred_flag. slice_granularity. shared_pps_info_enabled_flag. if( cu_qp_delta_enabled_flag ). max_cu_qp_delta_depth. |
| --- | --- |

PS：The parameter in alf_param( ) are written outside the function of encodePPS().

## Slice header syntax

| WD. lightweight_slice_flag. if( !lightweight_slice_flag ) {. slice_type. pic_parameter_set_id. frame_num. if( IdrPicFlag ). idr_pic_id. if( pic_order_cnt_type = = 0 ). pic_order_cnt_lsb /*. if( slice_type = = P ¦ ¦ slice_type = = B ) {. num_ref_idx_active_override_flag. if( num_ref_idx_active_override_flag ) {. num_ref_idx_l0_active_minus1. if( slice_type = = B ). num_ref_idx_l1_active_minus1. }. }. ref_pic_list_modification( ). ref_pic_list_combination( ). if( nal_ref_idc != 0 ). dec_ref_pic_marking( ). }. if( entropy_coding_mode_flag && slice_type != I). cabac_init_idc. first_slice_in_pic_flag. if(first_slice_in_pic_flag = = 0). slice_address. if( !lightweight_slice_flag ) {. slice_qp_delta. if( sample_adaptive_offset_enabled_flag ). sao_param(). if( deblocking_filter_control_present_flag ) {. disable_deblocking_filter_idc. if( disable_deblocking_filter_idc != 1 ) {. slice_alpha_c0_offset_div2. slice_beta_offset_div2. }. }. if( slice_type = = B ). collocated_from_l0_flag. if( adaptive_loop_filter_enabled_flag ) {. if( !shared_pps_info_enabled_flag ). alf_param( ). alf_cu_control_param( ). }. }. | HM-3.2. entropy_slice_flag. if( !entropy_slice_flag ) {. slice_type. pic_order_cnt. symbol_mode (pps:entropy_coding_mode_flag). if( slice_type = = P ¦ ¦ slice_type = = B ) {. is_referenced_flag. num_ref_idx_l0. }. if( slice_type = = B ) {. num_ref_idx_l1. }. ref_pic_list_combination( ). }. first_tb_is_zero_flag. if(!first_tb_is_zero_flag). first_tb_in_slice. slice_qp. loop_filter_disable_flag. drb_flag. if(!drb_flag). erb_index. if(slice_type= =B). col_dir_flag. }. |
| --- | --- |

PS：The parameter in sao_param(), alf_param( ) and alf_cu_control_param( ) are written outside the function of encodeSliceHeader().

# Behavior mismatches

The behaviors of the slice decoding process in the WD (JCTVC-E603_d7) are compared to that in reference software (HM-3.2).

## Decoding process for picture order count

WD: No specifications about POC decoding process.

SW: The value of POC is get from directly from SliceHeader. The syntax element is pic_order_cnt, and no more decoding process is invoked.

## Decoding process for reference picture lists construction

1）	Decoding process for picture numbers

WD: Specifies the decoding process for picture numbers.

SW: No frame_num. No process for picture number decoding.

2)	Initialization process for reference picture lists

WD: The reference picture list is ordered so that short-term reference pictures have lower indices than long-term reference pictures.

SW: The reference picture list is ordered like: 1 short-term + 1 long-term + n short-term.

3)	Modification process for reference picture lists

WD: Specifies the modification process of reference picture lists.

SW: No modification process of reference picture lists.

4)	Mapping process for reference picture lists combination in B slices

WD: Different mapping processes are invoked according to the value of ref_pic_list_modification_flag_lc (0 or 1).

SW: Only mapping process for ref_pic_list_modification_flag_lc equal to 0. No operation for ref_pic_list_modification_flag_lc equal to 1.

## Decoded reference picture marking process

WD: Specifies two tools for reference picture marking: sliding window and MMCO.

SW: Two marking processes:1) Refresh marking for IDR and CDR; 2) pop picture from buffer when it come the maximum size. The process is similar to sliding window, but it is according to POC not frame_num as specified in WD.

# Acknowledgement

We appreciate related inputs from Benjamin Bross, Frank Bossen, Rickard Sjöberg, and Ying Chen, which have been taken into consideration in the above analysis.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).