| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F716_r1 |
| --- | --- |

| Title: | Cross-check of Samsung's proposal F554 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports Sony’s cross-check of Samsung’s proposal (Experiment 2 and Experiment 5) on secondary transforms for intra prediction residual, JCTVC-F554. BD-rate results match with proponent’s results for both cases. The encoding and decoding time results are close to proponent’s results in both cases.

# Introduction

JCTVC-F554 [1] proposed Mode-Dependent Secondary Transform (MD-ST) for transforms in HM3.0. This cross-check verified the results of Experiment 2 and Experiment 5 in JCTVC-F554 [1].

# Cross-check Results

## Experiment 2

The anchor is HM3.0. To be tested is Samsung’s implementation by setting the macro to enable 4x4 secondary transform.

#define MD_SEC_TRANSFORM              1

All intra and random access under both HE and LC settings are tested.

### Simulation results

The summary of cross-check results of Experiment 2 in [1] is shown in Table 1. Complete cross-check results are available in “JCTVC-F716_Exp2.xls”.

Table 1. Summary of Experiment 2, anchor is HM3.0

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | -0.2 | -0.2 | -0.3 | -0.4 | -0.5 |
| Class B | -0.4 | -0.3 | -0.3 | -0.2 | -0.5 | -0.5 |
| Class C | -0.2 | -0.4 | -0.4 | -0.3 | -0.4 | -0.4 |
| Class D | -0.1 | -0.3 | -0.3 | -0.1 | -0.3 | -0.3 |
| Class E | -0.6 | -0.8 | -0.8 | -0.5 | -0.8 | -0.9 |
| Overall | -0.4 | -0.4 | -0.4 | -0.3 | -0.5 | -0.5 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
|  | Random access HE |  |  | Random access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | 0.0 | -0.6 | -0.2 | -0.3 | -0.1 |
| Class B | -0.3 | -0.2 | -0.1 | -0.2 | -0.2 | -0.3 |
| Class C | -0.2 | -0.1 | -0.4 | -0.2 | -0.3 | -0.3 |
| Class D | -0.2 | -0.3 | -0.4 | -0.2 | -0.4 | -0.4 |
| Class E |  |  |  |  |  |  |
| Overall | -0.3 | -0.2 | -0.4 | -0.2 | -0.3 | -0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

## Experiment 5

The anchor is HM3 SDIP branch + 4x4 DCT/DST harmonized on SDIP. To be tested is Samsung’s proposed DCT/4x4 MD-ST applied to square blocks of sizes 8x8, 16x16 and 32x32, as well as to rectangular blocks of size 16x1, 8x2, 16x4 and 32x8 in SDIP.

All intra under both HE and LC settings are tested.

### Simulation results

The summary of cross-check results for Experiment 5 [1] is shown in Table 2. Complete cross-check results are available in “JCTVC-F716_Exp5.xls”.

Table 2. Summary of 4x4 MD-ST + SDIP

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | -0.2 | -0.2 | -0.4 | -0.4 | -0.5 |
| Class B | -0.6 | -0.5 | -0.4 | -0.5 | -0.5 | -0.5 |
| Class C | -0.3 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 |
| Class D | -0.2 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 |
| Class E | -0.9 | -0.7 | -0.7 | -0.8 | -0.5 | -0.7 |
| Overall | -0.5 | -0.4 | -0.4 | -0.4 | -0.4 | -0.5 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Reference

- A. Saxena, and F. Fernandes,  “On secondary transforms for intra prediction residual”, JCTVC-F554, July. 2011

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).