| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F719 |
| --- | --- |

| Title: | Cross-check report for Qualcomm's JCTVC-F585 on Luma/chroma interpolation precision |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jianle Chen | Email: | jianle.chen@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This document reports cross-checking results of luma/chroma interpolation precision. The results confirm the BD-rates and encoding/decoding time from the proponents. The proponents’ code has also been analysed and found to match with the context in the proposal.

# Test Results

In F585, 1 additional bit (total 7 bits) right shifting is performed after first dimension interpolation to restrict the intermediate buffer with 16 bits accuracy. In this cross-checking report, the method is verified and the code is analysed.

The Software was compiled on 64-bit Linux and executed on Linux cluster. The following tables represent test results, more details can be found in the accompanying Excel sheets.

Experiment results shows there is almost performance change by limiting the intermediate buffer in 16 bits when input data is 10 bits.

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  | 0.0 | 0.0 | -0.3 |
| Class B |  |  |  | 0.0 | 0.0 | 0.0 |
| Class C |  |  |  | 0.0 | 0.0 | 0.0 |
| Class D |  |  |  | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  | 0.0 | 0.0 | 0.0 |
| Enc Time[%] |  |  |  | 101% |  |  |
| Dec Time[%] |  |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay P HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.3 | 0.2 | 0.0 | 0.1 | 0.1 |
| Class C | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.3 | 0.0 | -0.5 | -0.2 |
| Class E | 0.1 | 0.4 | 0.0 | 0.0 | 0.1 | -0.2 |
| Overall | 0.0 | 0.2 | 0.1 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

# Conclusion

In this document, luma/chroma interpolation precision in F585 was confirmed. The code has also been analysed and found to match to the context in the proposal. Experiments results shows that no performance degradation is found by limiting the intermediate data accuracy in 16 bits.

# References

- M. Coban, P. Chen, M. Karczewicz, “Luma/chroma interpolation precision”, JCTVC-F585, 6th JCT-VC Meeting, Torino, IT, July, 2011.