| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F727 |
| --- | --- |

| Title: | CE10: A Cross-check report for core transform proposal JCTVC-F363 High QP configuration |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiran Misra, Louis Kerofsky, Andrew Segall 5750 NW Pacific Rim Blvd., Camas, WA 98607 USA | Tel: Email: | +1-360-817-8451 misrak@sharplabs.com, lkerofsky@sharplabs.com, asegall@sharplabs.com (Sharp) |
| Source: | Sharp Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This document reports a cross-check result for JCTVC-F363, core transform proposed by FastVDO and Samsung. The coding performance was measured using HM-3.0 as anchor. The encoder configurations used for performance evaluation is based on the test conditions specified in JCTVC-E710.

The average BD bitrate changes measured for High QP are: 0.0% AI-HE; 0.0% AI-LC; 0.0% RA-HE; -0.1% RA-LC; 0.3% LD-HE; 0.0% LD-LC.

# Tested Proposal

The tested proposal was integrated by the proponents into HM-3.0 [4]. The source changes were labeled and enabled using macros:

#define   FULL_FACTORIZATION                    1

#define LIFTING				1

The source was studied and it is confirmed that:

- The 4-point and 8-point transforms are the same as the one proposed by CISCO/TI and are the default transform of HM-3.0.

- The 16-point and 32-point transforms were modified to variants of those proposed in JCTVC-E253.

The source changes were found to be consistent with the description PDF attached to proposal JCTVC-F363 [1].

# Simulation Environment and Results

The performance of the proposed core transform was tested on conditions specified in JCTVC-E710 [2].

The encoder and decoder were run on a cluster of machines. The computing platform is summarized in Table 1 below.

Table 1: Computing platforms

| OS | Linux (64bit) |
| --- | --- |
| Some CPUs | Intel(R) Xeon(R) CPU X3360 @ 2.83GHz cache size : 6144 KB |
| Some CPUs | Intel(R) Core(TM)2 Extreme CPU Q6800 @ 2.93GHz cache size : 4096 KB |
| Compiler | gcc 4.3.2 |

# Results

A summary of BD Bitrate [3] results for High QP configuration described in JCTVC-E710 [2] is presented below. The timing information is not provided due to large variability observed in timing data over the shared cluster resource. The detailed R-D results can be found in attached excel sheet.

High QP (QP=36,42,47,51)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.2 | 2.3 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | -0.2 | -0.3 | 0.0 | 0.0 | 0.1 |
| Class C | -0.1 | -0.5 | 0.0 | 0.0 | -0.2 | 0.1 |
| Class D | 0.0 | -5.2 | 0.4 | 0.0 | -0.3 | -0.3 |
| Class E | -0.1 | -0.2 | 0.0 | 0.0 | 0.1 | 0.0 |
| Overall | 0.0 | -1.3 | 0.5 | 0.0 | -0.1 | 0.0 |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.1 | -2.0 | 0.0 | 0.3 | -1.4 |
| Class B | 0.0 | -0.3 | -1.2 | 0.0 | 0.2 | -0.3 |
| Class C | 0.1 | -0.5 | 1.4 | -0.1 | -2.6 | -0.1 |
| Class D | -0.1 | 1.2 | -0.6 | -0.3 | -4.1 | 0.9 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.1 | -0.7 | -0.1 | -1.4 | -0.2 |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.4 | -3.2 | 0.0 | 0.1 | -0.9 |
| Class C | -0.2 | 0.3 | 0.8 | 0.0 | 0.2 | 0.3 |
| Class D | 0.6 | -24.3 | -0.4 | 0.0 | 2.0 | -1.0 |
| Class E | 1.0 | 0.6 | 0.2 | 0.0 | 2.0 | -0.4 |
| Overall | 0.3 | -6.0 | -0.9 | 0.0 | 1.0 | -0.5 |

# Conclusion

Cross-check and software verification for JCTVC-F363 core transform proposed by FastVDO and Samsung is reported. The experiments yield the exact same R-D results as provided by the proponent.

# References

[1] Pankaj Topiwala, Wei Dai, Ilkoo Kim, “CE10: core transform proposal 1,” JCTVC-F363 6th Meeting: Torino, IT, 14-22 July, 2011.

[2] P. Topiwala, M. Budagavi, A. Fuldseth, R. Joshi,  I.-K. Kim, “Description of Core Experiment 10 (CE10): Core Transform Design,” JCTVC-E710, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[3] Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33

[4] High efficiency test model (HM) version 3.0 URL: https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0/