| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC- F730 |
| --- | --- |

| Title: | Cross-check for Motorola’s Proposal (F601) on Sub-pixel Interpolation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Authors: | Zhi Zhou Shan Liu 2860 Junction Ave. San Jose, CA 95134, USA | Emails: | zhi.zhou@mediatek.com shan.liu@mediatek.com |
| Source: | MediaTek USA |  |  |

_____________________________

# Abstract

This document cross-checked Motorola’s proposal JCTVC-F601 “Joint Sub-pixel Interpolation for bi-predicted Motion Compensation” [1]. The verification was done successfully and the BD-rates match those provided by Motorola exactly. The code was studied and found to match the proposal.

# Introduction

Motorola provided the HM3.0-based source code of their proposed algorithm. The macros are set as:

#define MMI_IF                                        1

#define PS_IF                                         0

Two schemes were tested: 4-tap and 6-tap filter.

Four configurations (RA_HE, RA_LC, LD_HE and LD_LC) were tested for the provided code.

# Test Results

The BD-rates from our experimental results match those provided by Motorola exactly. The encoding and decoding times are within the variability of our machines. The detailed results are listed in JCTVC-F730_4tap.xls and JCTVC-F730_6tap.xls.

Table-1 4-tap filter

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.2 | 1.1 | 1.3 | 1.7 | 1.4 | 1.6 |
| Class B | 1.6 | 1.2 | 1.2 | 2.3 | 1.4 | 1.4 |
| Class C | 1.1 | 1.3 | 1.3 | 1.5 | 1.3 | 1.4 |
| Class D | 1.6 | 1.6 | 1.5 | 2.4 | 1.7 | 1.8 |
| Class E |  |  |  |  |  |  |
| All | 1.4 | 1.3 | 1.3 | 2.0 | 1.4 | 1.5 |
| Enc Time[%] | 106% |  |  | 109% |  |  |
| Dec Time[%] | 110% |  |  | 114% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.8 | 1.6 | 1.7 | 0.9 | 1.2 | 0.7 |
| Class C | 0.6 | 2.1 | 2.3 | 0.4 | 1.8 | 1.8 |
| Class D | 0.8 | 3.7 | 4.6 | 1.0 | 2.1 | 2.7 |
| Class E | 1.2 | 1.5 | 1.5 | 0.9 | -0.1 | 0.5 |
| All | 0.8 | 2.2 | 2.5 | 0.8 | 1.3 | 1.4 |
| Enc Time[%] | 106% |  |  | 108% |  |  |
| Dec Time[%] | 109% |  |  | 114% |  |  |

Table-2 6-tap filter

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.3 | 0.3 | 0.3 | 0.5 | 0.6 |
| Class B | 0.2 | 0.2 | 0.3 | 0.7 | 0.3 | 0.3 |
| Class C | -0.2 | 0.2 | 0.2 | -0.2 | 0.3 | 0.3 |
| Class D | -0.7 | -0.1 | -0.2 | -0.7 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | 0.2 | 0.1 | 0.1 | 0.2 | 0.3 |
| Enc Time[%] | 105% |  |  | 109% |  |  |
| Dec Time[%] | 110% |  |  | 113% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.8 | 0.4 | 0.6 | 0.4 | 0.1 |
| Class C | -0.2 | 1.0 | 1.1 | -0.3 | 1.0 | 0.8 |
| Class D | -0.7 | 2.5 | 3.1 | -0.4 | 0.9 | 1.5 |
| Class E | 0.5 | 0.8 | 0.7 | 0.6 | -1.0 | -0.4 |
| All | -0.1 | 1.3 | 1.3 | 0.1 | 0.4 | 0.5 |
| Enc Time[%] | 106% |  |  | 109% |  |  |
| Dec Time[%] | 111% |  |  | 115% |  |  |

# Conclusion

This report confirms the results provided by Motorola in JCTVC-F601.

# References

- Koohyar Minoo, Jian Lou, “Joint Sub-pixel Interpolation for bi-predicted Motion Compensation”, JCTVC-F601, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, IT, 14-22 July, 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, March. 2011.

- Gisle Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T Q.6/SG16 VCEG, VCEG-AI11, Berlin, Germany, July 2008.