| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F732 |
| --- | --- |

| Title: | CE4 Subtest2: Verification result of Qualcomm's Spatial QP prediction (JCTVC-F705) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima Canon Inc. 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the cross check result of JCTVC-F705, Spatial QP prediction proposed by Qualcomm.

# Cross Check Procedure

The source code corresponds to JCTVC-F705 was provided by Qualcomm, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.  The results of CE4 Subtest2 test 2.3.a were used as the reference data.  In contrast, there are two test points defined in JCTVC-F705.  One of them is referred as “2.3.g+2.3.b” which is the combination of tools 2.3.g&2.3.b defined in CE4 Subtest2 while the other one is referred as “2.3.g+2.3.c” which is the combination of tools 2.3.g&2.3.c.

For the “2.3.g+2.3.b” results, the tested data were generated by modifying the provided source code as follows:

#define INTER_REFERENCE_QP_TYPE           2

Meanwhile, for the “2.3.g+2.3.c” results, the tested data were generated by modifying the provided source code as follows:

#define INTER_REFERENCE_QP_TYPE           3

The test condition defined in JCTVC-E704 Subtest2 was used for the crosscheck.

# Results

## Simulation Result of 2.3.g+2.3.b

Cross check results for “2.3.g+2.3.b” are summarized in Table 1, where proposed “2.3.g+2.3.b” is compared with CE4 subtest2 anchor (test 2.3.a).

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.2 | -0.2 | -0.2 | -6.2% | -0.2 | -0.2 | -0.2 | -5.9% |
| Class B | -0.3 | -0.3 | -0.3 | -9.9% | -0.3 | -0.3 | -0.3 | -9.0% |
| Class C | -0.4 | -0.4 | -0.4 | -11.7% | -0.4 | -0.4 | -0.4 | -11.1% |
| Class D | -0.4 | -0.4 | -0.4 | -10.8% | -0.4 | -0.4 | -0.4 | -10.8% |
| Class E | -0.8 | -0.8 | -0.8 | -17.0% | -0.6 | -0.6 | -0.6 | -14.9% |
| All | -0.4 | -0.4 | -0.4 | -10.8% | -0.4 | -0.4 | -0.4 | -10.0% |
| Enc Time[%] | 95% |  |  |  | 97% |  |  |  |
| Dec Time[%] | 107% |  |  |  | 111% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.1 | -0.1 | -0.2 | -2.7% | -0.1 | 0.1 | 0.0 | -2.8% |
| Class B | -0.2 | -0.2 | -0.2 | -4.5% | -0.2 | -0.2 | -0.2 | -4.9% |
| Class C | -0.2 | -0.2 | -0.2 | -5.9% | -0.3 | -0.2 | -0.1 | -6.4% |
| Class D | -0.1 | 0.0 | -0.2 | -4.5% | -0.2 | -0.1 | -0.3 | -5.2% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -0.1 | -0.2 | -4.4% | -0.2 | -0.1 | -0.1 | -4.8% |
| Enc Time[%] | 109% |  |  |  | 107% |  |  |  |
| Dec Time[%] | 117% |  |  |  | 115% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -0.1 | -0.2 | -2.0% | -0.2 | -0.1 | -0.2 | -2.6% |
| Class C | -0.2 | 0.0 | 0.1 | -3.1% | -0.2 | -0.1 | -0.2 | -3.9% |
| Class D | -0.1 | 0.0 | -0.4 | -2.0% | -0.2 | 0.0 | -0.3 | -2.9% |
| Class E | -0.1 | 1.0 | 1.1 | -2.5% | -0.2 | -0.4 | 0.1 | -2.9% |
| All | -0.1 | 0.1 | 0.1 | -2.4% | -0.2 | -0.1 | -0.1 | -3.0% |
| Enc Time[%] | 105% |  |  |  | 108% |  |  |  |
| Dec Time[%] | 115% |  |  |  | 113% |  |  |  |

Table 1: BD-rate results for proposed 2.3.g+2.3.b vs. test 2.3.a anchor

## Simulation Result of 2.3.g+2.3.c

Cross check results for “2.3.g+2.3.c” are summarized in Table 2, where proposed “2.3.g+2.3.c” is compared with CE4 subtest2 anchor (test 2.3.a).

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.2 | -0.2 | -0.2 | -6.2% | -0.2 | -0.2 | -0.2 | -5.9% |
| Class B | -0.3 | -0.3 | -0.3 | -9.9% | -0.3 | -0.3 | -0.3 | -9.0% |
| Class C | -0.4 | -0.4 | -0.4 | -11.7% | -0.4 | -0.4 | -0.4 | -11.1% |
| Class D | -0.4 | -0.4 | -0.4 | -10.8% | -0.4 | -0.4 | -0.4 | -10.8% |
| Class E | -0.8 | -0.8 | -0.8 | -17.0% | -0.6 | -0.6 | -0.6 | -14.9% |
| All | -0.4 | -0.4 | -0.4 | -10.8% | -0.4 | -0.4 | -0.4 | -10.0% |
| Enc Time[%] | 95% |  |  |  | 97% |  |  |  |
| Dec Time[%] | 107% |  |  |  | 112% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.2 | -0.4 | 0.1 | -2.8% | 0.0 | 0.0 | 0.0 | -2.8% |
| Class B | -0.2 | -0.1 | -0.2 | -4.7% | -0.2 | -0.2 | -0.2 | -5.0% |
| Class C | -0.2 | -0.3 | -0.3 | -6.5% | -0.3 | -0.3 | -0.2 | -7.0% |
| Class D | -0.1 | -0.1 | 0.0 | -4.8% | -0.2 | -0.2 | -0.2 | -5.7% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -0.2 | -0.1 | -4.7% | -0.2 | -0.2 | -0.1 | -5.1% |
| Enc Time[%] | 106% |  |  |  | 109% |  |  |  |
| Dec Time[%] | 117% |  |  |  | 116% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -0.2 | -0.2 | -2.4% | -0.2 | -0.1 | -0.3 | -2.9% |
| Class C | -0.2 | -0.1 | -0.1 | -4.1% | -0.3 | 0.0 | -0.3 | -5.0% |
| Class D | -0.1 | -0.2 | 0.0 | -2.9% | -0.2 | -0.3 | -0.5 | -3.6% |
| Class E | -0.2 | 1.2 | 1.2 | -3.7% | -0.2 | -0.2 | -0.2 | -4.1% |
| All | -0.2 | 0.1 | 0.1 | -3.2% | -0.2 | -0.1 | -0.3 | -3.8% |
| Enc Time[%] | 105% |  |  |  | 108% |  |  |  |
| Dec Time[%] | 114% |  |  |  | 113% |  |  |  |

Table 2: BD-rate results for proposed 2.3.g+2.3.c vs. test 2.3.a anchor

The BD-rate results obtained in our simulation match the results provided by Qualcomm.  Accordingly, bitrate and PSNR results provided by Qualcomm were verified.  There are some Enc/Dec time gap observed, but they should not be coming from the proposed change made by JCTVC-F705.

# Conclusion

It has been verified that the bitrate and PSNR results provided by Qualcomm on JCTVC-F705 are accurate.