| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F733 |
| --- | --- |

| Title: | Modification to JCTVC-F537: 16-bit bi-prediction interpolation process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Peisong Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1 858 658 3973 mcoban@qualcomm.com peisongc@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

Further reduction to resolution of biprediction interpolation process in JCTVC-F537 is introduced enabling 16-bit interpolation process without drop in coding efficiency. Simulation results show 0.0%, 0.0%, and 0.0% BD rate change for RA_HE, LB_HE and LP_HE cases, respectively.

# Introduction

In JCTVC-F537 [2], 2D bipred interpolated sample values without use of non-normative offset requires 17-bit precision.  If the sample values after second stage of filtering are right shifted by 1 extra bit, then 16-bit precision for 2D bipred values would be guaranteed without use of offset. The changes to JCTVC-F537 could be summarized by the following table. All other cases remain the same

| 2D Bipred | JCTVC-F537 | Modification |
| --- | --- | --- |
| BitDepth = 8 | Hor.Filtering >> 0, Ver. Filtering >> 6 | Hor.Filtering >> 0, Ver. Filtering >> 7 |
| BitDepth = 10 | Hor. Filtering >> 2, Ver. Filtering >> 6 | Hor.Filtering >> 2, Ver. Filtering >> 7 |

Appropriate scaling in bipred sample averaging step to take care of this extra bit shift is necessary.

# Results

The proposed changes are implemented in JCTVC-F537 software based on HM3.2. The simulations are conducted based on the JCT-VC common test condition defined in [1].  Table 1 shows summary of simulation results. As can be from the results on average there is 0.0% BD rate loss.

Table 1: Summary of simulation results (HM3.2 anchor)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.1 | 0.2 | 0.0 | -0.1 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | -0.1 | -0.1 | 0.0 | 0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | 0.0 | 0.0 | -0.1 | 0.1 |
| Class C | 0.0 | -0.1 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class D | 0.1 | 0.1 | -0.2 | 0.0 | 0.3 | -0.1 |
| Class E | -0.1 | -0.1 | -0.9 | -0.1 | -0.4 | -0.6 |
| Overall | 0.0 | 0.1 | -0.3 | 0.0 | 0.0 | -0.1 |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.5 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | -0.1 | -0.7 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.1 | -0.1 | 0.0 | 0.0 | 0.0 |

# Conclusions

This contribution has presented a modification to JCTVC-F537 that guarantees 16-bit precision for intermediate interpolation values without the use of additional non-normative sample offset. The method guarantees 16-bit range for storage of intermediate values for bi prediction interpolation process without loss in coding efficiency.

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, March 2011.

- F. Bossen, “On the motion compensation process,” JCT-VC document JCTVC-F537, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 3 (from JCTVC-F537 version)

###### Luma sample interpolation process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL.

Output of this process is a predicted luma sample value predSampleLXL[ xL, yL ]

Figure 8-1 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation

In Figure 8-1, the positions labelled with upper-case letters Ai, j within shaded blocks represent luma samples at full-sample locations inside the given two-dimensional array refPicLXL of luma samples. These samples may be used for generating the predicted luma sample value predSampleLXL[ xL, yL ]. The locations ( xAi, j, yAi, j ) for each of the corresponding luma samples Ai, j inside the given array refPicLXL of luma samples are derived as follows:

xAi, j = Clip3( 0, PicWidthInSamplesL – 1, xIntL +i )		(8-1)
yAi, j = Clip3( 0, PicHeightInSamplesL – 1, yIntL +j )		(8-2)

Variables shift1, shift2 and shift3 are derived as follows.

–	The variable shift1 is set equal to BitDepthY – 8, the variable shift2 is set equal to 6, and the variable shift3 is set equal to 14 – BitDepthY.

Given the luma samples Ai, j at full-sample locations ( xAi, j, yAi, j ), the luma samples ‘a0,0’ to ‘r0,0’ at fractional sample positions are derived by the following rules.

–	The samples labelled a0,0, b0,0, c0,0, d0,0, h0,0, and n0,0 shall be derived by applying the 8-tap filter to the nearest integer position samples:

a0,0 = ( -A-3,0 + 4*A-2,0 - 10*A-1,0 + 57*A0,0 +	 19*A1,0 - 7*A2,0 + 3*A3,0 - A4,0 ) >> shift1	(8-3)

b0,0 = ( -A-3,0 + 4*A-2,0 - 11*A-1,0 + 40*A0,0 +	 40*A1,0 - 11*A2,0 + 4*A3,0 - A4,0 ) >> shift1	(8-4)

c0,0 = ( -A-3,0 + 3*A-2,0 - 7*A-1,0 + 19*A0,0 +	 57*A1,0 - 10*A2,0 + 4*A3,0 - A4,0 ) >> shift1	(8-5)

d0,0 = ( -A0,-3 + 4*A0,-2 - 10*A0,-1 + 57*A0,0 +	 19*A0,1 - 7*A0,2 + 3*A0,3 - A0,4 ) >> shift1	(8-6)

h0,0 = ( -A0,-3 + 4*A0,-2 - 11*A0,-1 + 40*A0,0 +	 40*A0,1 - 11*A0,2 + 4*A0,3 - A0,4 ) >> shift1	(8-7)

n0,0 = ( -A0,-3 + 3*A0,-2 - 7*A0,-1 + 19*A0,0 +	 57*A0,1 - 10*A0,2 + 4*A0,3 - A0,4 ) >> shift1	(8-8)

–	The samples labelled e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be derived by applying the 8-tap filter to the samples a0,i, b0,i and c0,i where i = -3..4 in the vertical direction:

e0,0 = ( -a0,-3 + 4*a0,-2 - 10*a0,-1 + 57*a0,0 +	 19*a0,1 - 7*a0,2 + 3*a0,3 - a0,4 ) >> shift2	(8-9)

i0,0 = ( -a0,-3 + 4*a0,-2 - 11*a0,-1 + 40*a0,0 +	 40*a0,1 - 11*a0,2 + 4*a0,3 - a0,4 ) >> shift2	(8-10)

p0,0 = ( -a0,-3 + 3*a0,-2 - 7*a0,-1 + 19*a0,0 +	 57*a0,1 - 10*a0,2 + 4*a0,3 - a0,4 ) >> shift2	(8-11)

f0,0 = ( -b0,-3 + 4*b0,-2 - 10*b0,-1 + 57*b0,0 +	 19*b0,1 - 7*b0,2 + 3*b0,3 - b0,4 ) >> shift2	(8-12)

j0,0 = ( -b0,-3 + 4*b0,-2 - 11*b0,-1 + 40*b0,0 +	 40*b0,1 - 11*b0,2 + 4*b0,3 - b0,4 ) >> shift2	(8-13)

q0,0 = ( -b0,-3 + 3*b0,-2 - 7*b0,-1 + 19*b0,0 +	 57*b0,1 - 10*b0,2 + 4*b0,3 - b0,4 ) >> shift2	(8-14)

g0,0 = ( -c0,-3 + 4*c0,-2 - 10*c0,-1 + 57*c0,0 +	 19*c0,1 - 7*c0,2 + 3*c0,3 - c0,4 ) >> shift2	(8-15)

k0,0 = ( -c0,-3 + 4*c0,-2 - 11*c0,-1 + 40*c0,0 +	 40*c0,1 - 11*c0,2 + 4*c0,3 - c0,4 ) >> shift2	(8-16)

r0,0 = ( -c0,-3 + 3*c0,-2 - 7*c0,-1 + 19*c0,0 +	 57*c0,1 - 10*c0,2 + 4*c0,3 - c0,4 ) >> shift2	(8-17)

If predFlagL0 + predFlagL1 is equal to 1,  the values of e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be set to e0,0&-2, f0,0&-2, g0,0&-2, i0,0&-2, j0,0&-2, k0,0&-2, p0,0&-2, q0,0&-2 and r0,0&-2, respectively.

The positions labelled with lower-case letters within un-shaded blocks represent luma samples at quarter-pel sample fractional locations. The luma location offset in fractional-sample units ( xFracL, yFracL ) specifies which of the generated luma samples at full-sample and fractional-sample locations is assigned to the predicted luma sample value predSampleLXL[ xL, yL ]. This assignment is done according to Table 8-2. The value of predSampleLXL[ xL, yL ] shall be the output.

Table 8-2 – Assignment of the luma prediction sample predSampleLXL[ xL, yL ]

| xFracL | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| predSampleLXL[ xL, yL ] | A << shift3 | d | h | n | a | e | i | p | b | f | j | q | c | g | k | r |

###### Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC.

Output of this process is a predicted chroma sample value predSampleLXC[ xC, yC ]

Figure 8-2 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for eighth sample chroma interpolation

In Figure 8-1, the positions labelled with upper-case letters Bi, j within shaded blocks represent chroma samples at full-sample locations inside the given two-dimensional array refPicLXC of chroma samples. These samples may be used for generating the predicted chroma sample value predSampleLXC[ xC, yC ]. The locations ( xBi, j, yBi, j ) for each of the corresponding chroma samples Bi, j inside the given array refPicLXC of chroma samples are derived as follows:

xBi, j = Clip3( 0, PicWidthInSamplesC – 1, xIntC +i )		(8-18)
yBi, j = Clip3( 0, PicHeightInSamplesC – 1, yIntC +j )		(8-19)

Variables shift1, shift2 and shift3 are derived as follows.

–	The variable shift1 is set equal to BitDepthC – 8, the variable shift2 is set equal to BitDepthC – 2, and the variable shift3 is set equal to 14 – BitDepthC.

Given the chroma samples Bi, j at full-sample locations ( xBi, j, yBi, j ), the chroma samples ‘ab0,0’ to ‘hh0,0’ at fractional sample positions are derived by the following rules.

–	The samples labelled ab0,0, ac0,0, ad0,0, ae0,0, af0,0, ag0,0, and ah0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ab0,0 = ( -3*B-1,0 + 60*B0,0 + 8*B1,0 – B2,0  ) >> shift1	(8-20)

ac0,0 = ( -4*B-1,0 + 54*B0,0 + 16*B1,0 – 2*B2,0 ) >> shift1	(8-21)

ad0,0 = ( -5*B-1,0 + 46*B0,0 + 27*B1,0 – 4*B2,0 ) >> shift1	(8-22)

ae0,0 = ( -4*B-1,0 + 36*B0,0 + 36*B1,0 – 4*B2,0 ) >> shift1	(8-23)

af0,0 = ( -4*B-1,0 + 27*B0,0 + 46*B1,0 – 5*B2,0 ) >> shift1	(8-24)

ag0,0 = ( -2*B-1,0 + 16*B0,0 + 54*B1,0 – 4*B2,0 ) >> shift1	(8-25)

ah0,0 = ( -B-1,0 + 8*B0,0 + 60*B1,0 – 3*B2,0 ) >> shift1	(8-26)

–	The samples labelled ba0,0, ca0,0, da0,0, ea0,0, fa0,0, ga0,0, and ha0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ba0,0 = ( -3*B0,-1 + 60*B0,0 + 8*B0,1 – B0,2 ) >> shift1	(8-27)

ca0,0 = ( -4*B0,-1 + 54*B0,0 + 16*B0,1 – 2*B0,2 ) >> shift1	(8-28)

da0,0 = ( -5*B0,-1 + 46*B0,0 + 27*B0,1 – 4*B0,2 ) >> shift1	(8-29)

ea0,0 = ( -4*B0,-1 + 36*B0,0 + 36*B0,1 – 4*B0,2 ) >> shift1	(8-30)

fa0,0 = ( -4*B0,-1 + 27*B0,0 + 46*B0,1 – 5*B0,2 ) >> shift1	(8-31)

ga0,0 = ( -2*B0,-1 + 16*B0,0 + 54*B0,1 – 4*B0,2 ) >> shift1	(8-32)

ha0,0 = ( -B0,-1 + 8*B0,0 + 60*B0,1 – 3*B0,2 ) >> shift1	(8-33)

–	The samples labelled bX0,0, cX0,0, dX0,0, eX0,0, fX0,0, gX0,0 and hX0,0 for X being replaced by b, c, d, e, f, g and h, respectively, shall be derived by applying the 4-tap filter to the values aXi,0 where i = -1..2 in the vertical direction:

bX0,0 = ( -3*aXa0,-1 + 60*aXa0,0 + 8*aX0,1 – aX0,2 ) >> shift2	(8-34)

cX0,0 = ( -4*aX-0,-1 + 54*aX0,0 + 16*aX0,1 – 2*aX0,2 ) >> shift2	(8-35)

dX0,0 = ( -5*aX-0,-1 + 46*aX0,0 + 27*aX0,1 – 4*aX0,2 ) >> shift2	(8-36)

eX0,0 = ( -4*aX-0,-1 + 36*aX0,0 + 36*aX0,1 – 4*aX0,2 ) >> shift2	(8-37)

fX0,0 = ( -4*aX-0,-1 + 27*aX0,0 + 46*aX0,1 – 5*aX0,2 ) >> shift2	(8-38)

gX0,0 = ( -2*aX-0,-1 + 16*aX0,0 + 54*aX0,1 – 4*aX0,2 ) >> shift2	(8-39)

hX0,0 = ( -aX-0,-1 + 8*aX0,0 + 60*aX0,1 – 3*aX0,2 ) >> shift2	(8-40)

If predFlagL0 + predFlagL1 is equal to 1,  the values of e0 bX0,0, cX0,0, dX0,0, eX0,0, fX0,0, gX0,0 and hX0,0 for X being replaced by b, c, d, e, f, g and h, respectively, shall be set to bX0,0&-2, cX0,0&-2, dX0,0&-2, eX0,0&-2, fX0,0&-2, gX0,0&-2 and hX0,0&-2, respectively.

The positions labelled with lower-case letters within un-shaded blocks represent chroma samples at eighth-pel sample fractional locations. The chroma location offset in fractional-sample units ( xFracC, yFracC ) specifies which of the generated chroma samples at full-sample and fractional-sample locations is assigned to the predicted chroma sample value predSampleLXC[ xC, yC ]. This assignment is done according to Table 8-3. The value of predSampleLXC[ xC, yC ] shall be the output.

Table 8-3 – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 1, b ), ( 2, c ), ( 3, d ), ( 4, e ), ( 5, f ), ( 6, g ), and ( 7, h ), respectively

| xFracC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXC[ xC, yC ] | B << shift3 | ba | ca | da | ea | fa | ga | ha | aY | bY | cY | dY | eY | fY | gY | hY |