| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F735 |
| --- | --- |

| Title: | Cross-check of Intel’s proposal (JCTVC-F500) on self derivation of motion estimation at decoder side |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports performance cross-check result of Intel's proposal on self derivation of motion estimation (SDME) at decoder side[1]. The verification tests have been conducted using RA settings of common test conditions[2]. It is confirmed that the results perfectly match with the ones reported by the proponent.

# Verified scheme

There are two proposals in JCTVC-F500 as follows:

- Proposal 1: SDME for Skip-mode without MV refinement

  - 9 candidate MVs are obtained from the zero MVs, spatial neighbor MVs and temporal neighbor MVs.

  - The candidate MVs are rounded to integer pixel positions.

  - The candidate MVs which achieves minimum SAD is selected as the final MV predictor.

- Proposal 2: SDME for Skip-mode with MV refinement

  - The MV predictor obtained by the Proposal 1 is further refined using two step 8-point MV refinement process within 3/4 pixel search range.

# Simulation Results

This cross-checking was conducted using modified HM3.0 software provided by the proponent. It is confirmed that the results presented in this section perfectly match the results provided by the proponent.

Table 1. BD-rate performance of proposal 1 against the anchor

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.6 | -0.6 | -0.1 | -0.2 | 0.1 |
| Class B | -0.5 | -0.6 | -0.5 | 0.1 | 0.2 | 0.1 |
| Class C | -0.5 | -0.4 | -0.5 | -0.2 | -0.2 | -0.3 |
| Class D | -0.7 | -0.8 | -0.8 | -0.4 | -0.3 | -0.4 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.6 | -0.6 | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 101% |  |  | 103% |  |  |
| Dec Time[%] | 109% |  |  | 111% |  |  |

Table 2. BD-rate performance of proposal 2 against the anchor

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -1.2 | -1.2 | -0.6 | -0.6 | -0.5 |
| Class B | -1.3 | -1.1 | -1.1 | -0.8 | -0.5 | -0.6 |
| Class C | -1.1 | -1.1 | -1.1 | -0.8 | -0.8 | -0.9 |
| Class D | -1.4 | -1.5 | -1.6 | -1.2 | -1.0 | -1.1 |
| Class E |  |  |  |  |  |  |
| All | -1.2 | -1.2 | -1.2 | -0.8 | -0.7 | -0.7 |
| Enc Time[%] | 104% |  |  | 106% |  |  |
| Dec Time[%] | 122% |  |  | 123% |  |  |

# Conclusion

We confirmed that the implementation of Intel’s proposal on top of HM-3.0 is consistent with the descriptions, and all test results perfectly matched the results provided by the proponent.

References:

- Yi-Jen Chiu, Wenhao Zhang, Lidong Xu and Hong Jiang, “Report of self derivation of motion estimation techniques at video decoder side on HM3.0,” JCTVC-F500, Jul. 2011.

- Frank Bossen, “Common test conditions and software reference configurations,” JCTVC- E700, Mar. 2011.