| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F736 |
| --- | --- |

| Title: | On Performance Drawbacks of Intra Luma Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shevach Riabtsev Advanced Technology Center Building # 8/2 P.O. Box 15015 Haifa 31905, Israel | Tel: Email: | Shevach.Riabtsev@zoran.com |
| Source: | Zoran Microelectronics Ltd |  |  |

_____________________________

# Abstract

The intra luma prediction process of small blocks is challenging for parallelization, especially for ASIC solutions, due to strong dependency among blocks in intra prediction and the filtering process of top/left reconstructed samples.  We propose the JCT-VC re-consider the dependency issues to improve performance of future HEVC ASIC solutions.

# Introduction

The 4x4 luma intra prediction in HEVC is a completely serial method. Let’s consider the following 8x8 intra CU that is coded with 4x4 prediction:

L0

L1

L2

L3

Cb

Cr

Where L0 is the first 4x4 luma block in decoding order, L1 is the second one and so forth.

Intra predictors for L0 are computed from top and left neighboring CUs.

Then in order to determine predictors for L1 we should take in general case the reconstructed samples of L0 as well as the reconstructed samples of top neighboring CUs. In order to determine predictors for L2 we should take the reconstructed samples of L0 and/or L1 and/or left neighboring CU. In order to determine predictors for L3 we should take the reconstructed samples of L0, L1 and L2.

As one can see there is a strong dependency (serialization) in computation of intra predictors for general 4x4 intra mode. Indeed, the block L1 depends on the reconstruction samples of L0. The block L2 in turn depends on the reconstructed samples L0 and L1. The block L3 depends on the reconstructed samples of L0, L1 and L2. This dependency makes a parallelization of intra prediction to be challenging especially for ASIC implementations. Therefore we propose to re-consider 4x4 intra prediction in HEVC to minimize dependencies and improve performance respectively.

To elaborate our points let’s specify the following basic routines:

- CompIntraPred(Lx, ModeLx) – compute intra predictors of 4x4 block Lx for a given intra mode ModeLx. Herein ‘x’ stands for 0,1,2 or 3.

- Recons(Lx) – compute Lx reconstructed block.

Let’s denote T0 – maximal processing time for CompIntraPred(Lx, ModeLx) and T1 is processing time for Recons(Lx).

Generally the whole intra prediction process for CU luma blocks is outlined below:

CompIntraPred(L0,ModeL0)

Recons(L0)

CompIntraPred(L1,ModeL1)

Recons(L1)

CompIntraPred(L2,ModeL2)

Recons(L2)

CompIntraPred(L3,ModeL3)

Recons(L3)

Total processing time is 4*T0 + 4*T1.

In some specific cases we can parallelize the intra prediction process:

- Case 1 -  Mode L2 is horizontal.

In such case intra prediction of the block L2 does not depend on reconstructed samples of L0 and L1 and can be processed in parallel.

CompIntraPred(L0,ModeL0)  ||  CompIntraPred(L2, HorizontalMode)

Recons(L0) || Recons(L2)

CompIntraPred(L1,ModeL1)

Recons(L1)

CompIntraPred(L3,ModeL3)

Recons(L3)

Herein || denotes the parallel processing. Total processing time is 3*T0 + 3*T1.

- Case 2: ModeL1 is vertical or Diagonal_Down_left.

In such case intra prediction of the block L1 does not depend on reconstructed samples of L0. Therefore L1 can be processed in parallel to L0.

CompIntraPred(L0,ModeL0)  ||  CompIntraPred(L1,ModeL1)

Recons(L0) || Recons(L1)

CompIntraPred(L2,ModeL2)

Recons(L2)

CompIntraPred(L3,ModeL3)

Recons(L3)

Total processing time is 3*T0 + 3*T1.

# Comments on CE6.d

We have studied the proposal on parallel intra prediction in CE6.d (JCTVC-E315).  Consider a 16x16 LCU with four 8x8 CUs. Let’s denote the worst-case performance of intra prediction for a single 8x8 block as T. Then we need a single SIMD processor and 4xT time to process of all 8x8 luma blocks. In a parallel intra prediction schema (e.g. in those presented in E315) we need two SIMDs and 2xT time to process the luma blocks.  To keep real-time decoding of 3D HD streams or 60 fps HD streams improvement of performance by twice is a crucial issue, otherwise the clock rate of SIMD processors gets so high that this is barely feasible develop such processors.

# Patent rights declaration(s)

Zoran Microelectronics Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Acknowledgements

The author would like to thank Andrew Segall for his discussion related to this paper.

# Conclusion

We propose the JCTVC to reduce the serial dependencies and increase parallelism in intra prediction to improve performance of future decoders.