| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F745 |
| --- | --- |

| Title: | BoG report on efficient binary representation of cu_qp_delta syntax for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono (NEC) Hirofumi Aoki (NEC) Yuzo Senda (NEC) 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN Kenji Kondo(Sony) Kazushi Sato(Sony) W12F 1-11-1 GCO Osaki, Shinagawa, 141-0032 Tokyo, Japan Jun Xu(Sony Electronics Inc.) 1730 N 1st Street San Jose, CA 95112 | Email: | chono@ct.jp.nec.com hao@bu.jp.nec.com y-senda@bc.jp.nec.com Kenji.Kondo@jp.sony.com Kazushi.Sato@jp.sony.com Jun.Xu@am.sony.com |
| Source: | [NEC Corp., Sony Corp., and Sony Electronics Inc.] |  |  |

_____________________________

# Summary and recommendation

JCTVC-F046 and JCTVC-F422 propose efficient binary representations of cu_qp_delta syntax for CABAC. Both proposals are based on the same idea that converts the absolute value to bin string and encodes sign bins by using bypass mode. Through the BoG discussion on efficient binary representation of cu_qp_delta syntax, the following are agreed:

- Both JCTVC-F046 and JCTVC-F422 halve both maximum and average of bin string length and reduce CABAC encoding/decoding complexity significantly;

- In addition, a truncated unary binarization in JCTVC-F046 assures that encoders do not send prohibited values;

- Coding efficiency and complexity of JCTVC-F046 and JCTVC-F422 are very comparable:

Based on the BoG discussion, it is recommended that JCTVC-F046 is adopted into the HM4.0.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- K. Chono, H. Aoki, and Y. Senda, "Efficient binary representation of cu_qp_delta syntax for CABAC", JCTVC-F046, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011.

- K. Kondo, K. Sato, and J. Xu, “Improvement of delta-QP Coding,” JCT-VC document JCTVC-F422, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.