| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F747r3 |
| --- | --- |

| Title: | Adaptation Parameter Set (APS) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger and Jill Boyce 433 Hackensack Ave., Hackensack, N.J. 07601, USA Yu-Wen Huang and Chia-Yang Tsai No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Ping Wu and Ming Li No. 50 Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 | Tel: Email: | Stephan Wenger +1-415-713-5473 {stephan,jill}@vidyo.com Yu-Wen Huang +886-3-5670766 ext. 26297 {chiayang.tsai, yuwen.huang}@mediatek.com Ping Wu +86-25-88014641 {ping.wu, li.ming42}@zte.com.cn |
| Source: | Vidyo, MediaTek, and ZTE |  |  |

_____________________________

# Abstract

This document proposes an Adaptation Parameter Set (APS), a picture layer activated parameter set intended for use with picture-adaptive data.  Provided are syntax and semantics, as well as pointers to documents providing more information on the utility of slice parameter sets—the predecessor in spirit of most aspects of the APS.  Syntax, semantics and decoding process are based on JCTVC-E603-d8.  (The authors also took the liberty to remove leftover text from H.264/AVC that doesn’t seem to apply to HEVC, nor is likely in our subjective opinion to apply in the future.

# Introduction

Proposed are syntax and semantics for an Adaptation Parameter Set (APS) that can be activated at the picture level, but is independent of the picture parameter set (PPS).  The purpose of the new parameter set is for coding of parameters associated with pictures that are likely to change from picture to picture, including the Adaptive Loop Filter (ALF) and Sample Adaptive Offset (SAO) parameters.  In contrast, a picture parameter set (PPS) collects data that can potentially change from picture to picture, but are unlikely to do so.  In separating the PPS data from the APS data, the transmission, and decoding of the likely-to-be-constant PPS data is with high probability avoided, saving bits and, in some architectures, processing cycles.

Parts of the concepts behind APS have first been proposed in JVT-C078 (wftp3.itu.int/av-arch/jvt-site/2002_05_Fairfax/JVT-C078.doc), close to a decade ago, and have been revisited in JCTVC-E281.

The mechanisms proposed in this document are similar to what was proposed in the above documents, with two noticeable exceptions.  First, the slice header contains always a reference to the PPS, but optionally also contains a second parameter set reference to the APS.  Parameter set references in the slice header are coded as ue(v).  Second, the possible presence of one or more APS in the bitstream, as well as the presence of the reference to an activated APS in the slice header is signaled by a combination of flags in the sequence parameter set (SPS).  Specifically, APS is present only when at least one of SAO or ALF parameters can be present in the bitstream, and at least one of the associated sequence parameter set flags (sample_adaptive_offset_enabled_flag and adaptive_loop_filter_enabled_flag) are on.

It is noted that there have been proposals at this meeting where, in the authors understanding, the related syntax elements would also best be coded in the APS.  Weighted prediction comes to mind.  See section 5 for recommendations where to put these new elements.

# Review of H.264/AVC High Level Syntax data structures

Please see JCT-VC E281 for an overview.

# Problem Statement

Please see JCT-VC E281, JCTVC-D128, JCTVC-E045, and JCTVC-F187 for an overview.

# Proposal

## Syntax

### Function byte_align()

In contrast to rbsp_trailing_bits(), which always inserts at least 1 bit, byte_align() inserts 0 to 7 bits until alignment is reached.  Editors, insert this somewhere in the functions.

| byte_align( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_one | f(1) |
| } |  |

### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

### Adaptation parameter set RBSP syntax

| Slice header syntax aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |
| if( aps_sample_adaptive_offset_flag ¦¦ aps_adaptive_loop_filter_flag ) { |  |
| aps_cabac_use_flag | u(1) |
| if( aps_cabac_use_flag ) { |  |
| aps_cabac_init_idc | ue(v) |
| aps_cabac_init_qp_minus26 | se(v) |
| } |  |
| } |  |
| /* Insert non-CABAC stuff above this line */ |  |
| if( aps_adaptive_loop_filter_flag ) { |  |
| alf_data_byte_count /* to enable skipping past data without parsing it */ | u(8) |
| /* byte_align() this byte align to happen between the non-CABAC and CABAC parts of the alf_param() Once there is an all CABAC alf_param(), enable this byte_align() */ |  |
| alf_param( ) |  |
| byte_align() |  |
| } |  |
| /* insert CABAC stuff below this line; make sure its byte-aligned */ |  |
| if( aps_sample_adaptive_offset_flag ) { |  |
| sao_data_byte_count /* to enable skipping past data without parsing it */ | u(8) |
| byte_align () |  |
| sao_param( ) |  |
| /* byte_align() this final byte align unnecessary as being taken care of by rbsp_trailing_bits() */ |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id |  |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag ) { |  |
| byte_align() |  |
| alf_cu_control_param( ) |  |
| byte_align() |  |
| } |  |
| } |  |

### Sample adaptive offset syntax

| sao_param( ) { | Descriptor |
| --- | --- |
| sample_adaptive_offset_flag | u(1)¦ae(v) |
| … |  |
| } |  |

### Adaptive loop filter syntax

| alf_non_entropy_coded_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_region_adaptation_flag | 2 | u(1) |
| alf_length_luma_minus_5_div2 | 2 | ue(v) |
| alf_no_filters_minus1 | 2 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |  |
| alf_start_second_filter | 2 | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |  |
| for (i=1; i< 16; i++) |  |  |
| alf_filter_pattern[i] | 2 | u(1) |
| } |  |  |
| if (AlfNumFilters > 1) |  |  |
| alf_pred_method | 2 | u(1) |
| alf_min_kstart_minus1 | 2 | ue(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |  |
| alf_golomb_index_bit[i] | 2 | u(1) |
| byte_align(); |  |  |
| for (i=0; i< AlfNumFilters; i++) |  |  |
| for (j=0; j< AlfCodedLengthLuma; j++) |  |  |
| alf_coeff_luma[i][j] |  | ge(v) |
| alf_chroma_idc | 2 | ue(v) |
| if ( alf_chroma_idc ) { |  |  |
| alf_length_chroma_minus_5_div2 | 2 | ue(v) |
| for( i = 0; i< AlfCodedLengthChroma; i++ ) |  |  |
| alf_coeff_chroma[i] |  | se(v) |
| } |  |  |
| } |  |  |

## Semantics

### Adaptive slice parameter set RBSP semantics

aps_id identifies the adaptation parameter set that is referred to in the slice header. The value of aps_id shall be in the range of 0 to TBD, perhaps level/profile dependent, inclusive.

aps_sample_adaptive_offset_flag equal to 1 specifies that the SAO is on for slices referred to the current APS; equal to 0 specifies that the SAO is off for slices referred to the current APS.  If there is no active APS, the aps_sample_adaptive_offset_flag value is inferred to be 0.

aps_adaptive_loop_filter_flag equal to 1 specifies that the ALF is on for slices referred to the current APS; equal to 0 specifies that the ALF is off for slices referred to the current APS.  If there is no active APS, the aps_adaptive_loop_filter_flag value is inferred to be 0.

aps_cabac_use_flag equal to 1 specifies that the CABAC decoding process shall be used for sao_param( ) when present, and alf_param( ) when present; equal to 0 specifies that the CAVLC decoding process shall be used for sao_param( ) when present, and alf_param( ) when present.

aps_cabac_init_idc specifies the index for determining the initialisation table used in the initialisation process for context variables of SAO and ALF. The value of cabac_init_idc shall be in the range of 0 to 2, inclusive.

aps_cabac_init_qp_minus26 specifies a quantization parameter minus 26 wherein the quantization parameter is used in the initialization process for context variables of SAO and ALF.

alf_data_byte_count specifies the number of bytes

sao_data_byte_point specifies the number of bytes

### Function byte_align()

bit_equal_to_one specifies a bit equal to 1.

## Decoding Process

### Order of sequence, picture, and adaptation parameter set RBSPs and their activation

This subclause specifies the activation process of adaptation, picture and sequence parameter sets for coded video sequences that conform to one or more of the profiles specified in Annex XXXX that are decoded using the decoding process specified in clauses XXXX and XXXX.

- NOTE 1 – The adaptation, sequence and picture parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded macroblock data. Adaptation, sSequence and picture parameter sets may, in some applications, be conveyed "out-of-band" using a reliable transport mechanism.

- An adaptation parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more coded pictures when at least one of sample_adaptive_offset_enabled_flag or adaptive_loop_filter_enabled_flag are equal to 1. Each adaptation parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one adaptation parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular adaptation parameter set RBSP results in the deactivation of the previously-active adaptation parameter set RBSP (if any).

- When an adaptation parameter set RBSP (with a particular value of aps_id) is not active and it is referred to by a coded slice NAL unit (using that value of aps_id), it is activated. This adaptation parameter set RBSP is called the active adaptation parameter set RBSP until it is deactivated by the activation of another adaptation parameter set RBSP. An adaptation parameter set RBSP, with that particular value of aps_id, shall be available to the decoding process prior to its activation.

- A picture parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more coded pictures. Each picture parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one picture parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular picture parameter set RBSP results in the deactivation of the previously-active picture parameter set RBSP (if any).

- When a picture parameter set RBSP (with a particular value of pic_parameter_set_id) is not active and it is referred to by a coded slice NAL unit (using that value of pic_parameter_set_id), it is activated. This picture parameter set RBSP is called the active picture parameter set RBSP until it is deactivated by the activation of another picture parameter set RBSP. A picture parameter set RBSP, with that particular value of pic_parameter_set_id, shall be available to the decoding process prior to its activation.

- Any picture parameter set NAL unit containing the value of pic_parameter_set_id for the active picture parameter set RBSP for a coded picture shall have the same content as that of the active picture parameter set RBSP for the coded picture unless it follows the last VCL NAL unit of the coded picture and precedes the first VCL NAL unit of another coded picture.

- A sequence parameter set RBSP includes parameters that can be referred to by one or more picture parameter set RBSPs or one or more SEI NAL units containing a buffering period SEI message. Each sequence parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one sequence parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular sequence parameter set RBSP results in the deactivation of the previously-active sequence parameter set RBSP (if any).

- When a sequence parameter set RBSP (with a particular value of seq_parameter_set_id) is not already active and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) or is referred to by an SEI NAL unit containing a buffering period SEI message (using that value of seq_parameter_set_id), it is activated. This sequence parameter set RBSP is called the active sequence parameter set RBSP until it is deactivated by the activation of another sequence parameter set RBSP. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id, shall be available to the decoding process prior to its activation. An activated sequence parameter set RBSP shall remain active for the entire coded video sequence.

- NOTE 2 – Because an IDR access unit begins a new coded video sequence and an activated sequence parameter set RBSP must remain active for the entire coded video sequence, a sequence parameter set RBSP can only be activated by a buffering period SEI message when the buffering period SEI message is part of an IDR access unit.

- Any sequence parameter set NAL unit containing the value of seq_parameter_set_id for the active sequence parameter set RBSP for a coded video sequence shall have the same content as that of the active sequence parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit and the first SEI NAL unit containing a buffering period SEI message (when present) of another coded video sequence.

- NOTE 3 – If adaptation parameter set RBSP, picture parameter set RBSP or sequence parameter set RBSP are conveyed within the bitstream, these constraints impose an order constraint on the NAL units that contain the picture parameter set RBSP or sequence parameter set RBSP, respectively. Otherwise (adaptation parameter set RBSP, picture parameter set RBSP or sequence parameter set RBSP are conveyed by other means not specified in this Recommendation | International Standard), they must be available to the decoding process in a timely fashion such that these constraints are obeyed.

When present, a sequence parameter set extension RBSP includes parameters having a similar function to those of a sequence parameter set RBSP. For purposes of establishing constraints on the syntax elements of the sequence parameter set extension RBSP and for purposes of determining activation of a sequence parameter set extension RBSP, the sequence parameter set extension RBSP shall be considered part of the preceding sequence parameter set RBSP with the same value of seq_parameter_set_id. When a sequence parameter set RBSP is present that is not followed by a sequence parameter set extension RBSP with the same value of seq_parameter_set_id prior to the activation of the sequence parameter set RBSP, the sequence parameter set extension RBSP and its syntax elements shall be considered not present for the active sequence parameter set RBSP.

All constraints that are expressed on the relationship between the values of the syntax elements (and the values of variables derived from those syntax elements) in sequence parameter sets, and picture parameter sets, or adaptation parameter sets, and other syntax elements are expressions of constraints that apply only to the active sequence parameter set, and the active picture parameter set and the active adaptation parameter set. If any sequence parameter set RBSP is present that is not activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream. If any picture parameter set RBSP is present that is not ever activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream.  If any adaptation parameter set RBSP is present that is not ever activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream.

- During operation of the decoding process (see clause XXX), the values of parameters of the active adaptation parameter set, the active picture parameter set, and the active sequence parameter set shall be considered in effect. For interpretation of SEI messages, the values of the parameters of the picture parameter set and sequence parameter set that are active for the operation of the decoding process for the VCL NAL units of the primary coded picture in the same access unit shall be considered in effect unless otherwise specified in the SEI message semantics.

# Open issues

- During our presentation, one open issues were discussed that need to be addressed by the editors in the context of other decisions made at the meeting.

## CABAC byte alignment, and alf_params ()

- Entropy coded data (in particular CABAC data) should be placed into a NAL unit at byte aligned positions.  Therefore, in order to avoid excessive waste of bits through byte alignment, the non-entropy coded data should be placed into the APS before the (byte aligned) potentially CABAC coded data.

- WD8, upon which this document is mostly based, includes both non-CABAC and CABAC data in alf_param().  Sao_param() contains only entropy coded data.  No other non-administrative data structures are currently placed in the APS.  As a result, the separation between non byte-aligned non-CABAC data and CABAC data, and the byte alignment, can be achieved by placing byte_align() functions just before the CABAC part of alf_params(), and placing alf_params() before sao_params().  This is what can be found in the document above.

- It was observed that the current software does use CABAC for all alf_params().  If, after this meeting, all alf_params() are indeed CABAC coded (as it is already in the current software), then the syntax can remain unchanged or, preferably, the sequence of alf_params() and sao_params() should be reversed so to reflect that SAO data, during slice decoding, is required before ALF data.

- If, however, the alf_param() syntax continues to include non-CABAC data, the sequencing of alf_params() and sao_params() as shown in this document should be maintained.  Further, into alf_params() a byte-align() should be inserted in the position as shown above.

- Worst case would be if there would be more than one data block (such as alf_param(), sao_param(), control info for weighted prediction, and similar) that includes both non-entropy coded and entropy coded data.  In this case, at least one of these blocks, and preferably all, should be split into two sub-blocks, one including the non-entropy coded data, the other including the entropy coded data.  The non-entropy coded data subblocks should be accumulated before the entropy coded subblocks, both ideally aligned with the use of the data in the decoding process.

## CABAC flush

- As per JCTVC-F-399, 131, the cabac state needs to be flushed at the end of the slice header.  Per meeting agreement (see report under document 377), no syntax (terminating_bit) is required.  However, the cabac flush needs to be inserted at an appropriate place in the slice (header?) decoding process.

- It is our understanding that no such flush is required for APS or other parameter sets.

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).