| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F749 |
| --- | --- |

| Title: | Cross-check of new results for parallel decoding for tiles (JCTVC-F594), |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Michael Horowitz Shilin Xu 1450-1055 W. Hastings Street Vancouver, BC V6E 2E9 Canada | Tel: Email: | +1 778.331.3432 x323 michael@ebriskvideo.com |
| Source: | eBrisk Video |  |  |

_____________________________

# Abstract

This contribution presents results of a cross-check of "New results for parallel decoding for Tiles," proposed by Sharp in document JCTVC-F594. The experiment associated with the contribution was successfully replicated under the common conditions for BD rate, PSNR, bit rate and decoding time for the partial results generated at the time this revision was written. The source code was found to be well written and compiled without problem. An issue was identified where an unnecessary byte was being added to bits embedded in the bitstream to implicitly identify the start of a tile. The extra byte resulted in a significant BD-rate penalty in some of the test cases.

# Introduction

The technique analyzed in "New results for parallel decoding for Tiles" was presented to the JCTVC first in “Tiles for Parallel Decoding”, JCTVC-E412. It is designed to facilitate decoder parallelization by signaling tile entry points in the bitstream. The approach supports sending tile entry points explicitly in the slice header (explicit mode) or by using markers in the bitstream (implicit mode).

# Experiment

The source code provided by the proponents was successfully compiled using a Visual C++ 2008 Expression Edition complier. The software was used for encoding and decoding according to common conditions [1] on a Windows cluster to get compression efficiency numbers.

## BD rate performance

We found a perfect match for all bitstreams between data provided by the proponents and the results we have generated at the time of writing of this revision. It can be noted that most losses occur for the implicit mode in Class E LC because the Class E sequences are coded at the lowest bit rate per tile. In part, the losses are overestimated as a result of an error described in the General Comments section below. Full results are available in the accompanying spreadsheets.

# Comments on source code

In general, we found the source code to be clearly written. For completeness, we note a few minor issues: lack of proper parameter value range checks in a couple of places, use of a variable that is always zero, and a distracting number of parameter values are written to the log files.

# General comments

A couple of issues were discovered during the review. One issue had to do with the fact that one bit was used to signal the mode of operation (i.e., explicit, implicit, and off) in the slice header rather than 2 bits.  A second issue had to do with the fact that in the implicit mode of operation, the software used a 4-byte start code where a 3-byte start code should have been used. The extra byte resulted in a significant BD-rate penalty in some of the test cases.

# References

- F. Bossen, “Common test conditions and software reference configurations” JCTVC-E600, Switzerland, March 2011