| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F752 |
| --- | --- |

| Title: | BoG report on de-quantization offset, adaptive reconstruction level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gaëlle Martin-Cocher (RIM) 1875 Buckhorn Gate, Mississauga, ON, L4W 5P1, Canada | Email: | gmartincocher@rim.com |
| Source: | [Research In Motion Ltd] |  |  |

_____________________________

# Summary and recommendation

In the CE4 subset 3 [1] JCTVC-F119 [2] and JCTVC-F276 [3] as well as a new proposal JCTVC-F610 [4] propose to define at the slice level, one or more offsets to be applied to the quantizer reconstruction level. The following concerns were raised by the committee and discussed in the BoG:

- Does the gain from those techniques come from bit allocation, what are their impacts on PSNR and RVM [5] values?

- Could “encoder only” techniques provide similar gain/complexity?

- How could these techniques be evaluated against the above?

The BoG came to the conclusion that a CE is needed to evaluate these proposals as well as combinations of them.

The following tests would be performed:

1)	Bit allocation/intra-boosting/reverse intra-boosting. (HM4 common condition [6])

•	Adjust proposed encoders so that the average bit rate for each class of frames (temporal level) is as close as possible to that of the anchor for each sequence. Intra picture remains the same. For each temporal level the lambda will be adjusted. A tolerance of 2% per temporal level is to be observed.

•	Compare the PSNR and the average bitrate for the sequence

2)	Encoder only technique (HM4 common condition [6])

This test aims at verifying if encoder only techniques could achieve similar functions/gains or if the gain could be cumulative.

The propose tests consist of using QP scaling. The current proposal is to use delta QP/Fractional offset (as illustrated in F610) and round the offset to the nearest QP value.

3)	Comparison to Anchor. (HM4 common condition [6])

•	Proposals with no lambda changes

•	One or multiple offsets.

4)	RDOQ Off + EEM [7]

This test aims at verifying if encoder only techniques could achieve similar functions/gains or if the gain could be cumulative.

•	Adaptive rounding/EEM would need to be integrated in HM.

•	A volunteer is needed

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- K. Sato et al, "Description of Core Experiment 4: Quantization", JCTVC-E704, Geneva, CH, March 2011.

- Xiang Li, Xun Guo, Shawmin Lei,"CE4 Subtest3: Adaptive De-Quantization Offset ", JCTVC-F119, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011.

- Xiang Yu, Jing Wang, Da-ke He, En-hui Yang, “CE4-subtest3.3.2: Quantization with Adaptive Reconstruction Levels” JCTVC-F276, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011.

- Xianglin Wang, Rajan Joshi, Geert Van Der Auwera, Marta Karczewicz, “Fine granularity QP change at slice level”  JCTVC-F610, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011.

- M. Horowitz, G. Bjontegaard, T.K. Tan, “Rate Variation Measure”, VCEG-AM10, Kyoto, Japan, Jan. 2010.

- F. Bossen Common HM test conditions and software reference configurations JCTVC-E700, 5th. JCTVC, Meeting, Geneva, Mar. 2011.

- Gary Sullivan, “Adaptive quantization encoding technique using an equal expected-value rule”, JVT-N011, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG, Hong Kong, China 18-21 Jan., 2005