| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F753 |
| --- | --- |

| Title: | BoG report on unified scans for the significance map and coefficient level coding in high efficiency (JCTVC-F288) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | BoG Report |  |  |
| Author(s) or Contact(s): | Rajan Joshi 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com |
| Source: |  |  |  |

_____________________________

# Abstract

This is a report of the activities of the break out group related to Unified scans for the significance map and coefficient level coding in high efficiency (JCTVC-F288).

# Attendance List

H. Sasai (Panasonic), M. Mrak (BBC), T. Nguyen (HHI), J. Sole (Qualcomm), R. Joshi (Qualcomm), C. Auyeung (Sony), H. Yu (Huawei), C. Lai (Huawei), C. Rosewarne (Canon), V. Sze (TI) D. He (RIM) T. Davies (Cisco), G. Martin- Cocher (RIM) X. Guo (Mediatek)

# Report

The objective of the break out group was to discuss adoption of JCTVC-F288. During the presentation of JCTVC-F288, some concerns were expressed. The break out group addressed these issues and tried to form a consensus.

The context initialization issue was discussed. Proponents shared some results when the context models for HM 3.0 are trained using class C and D and used to encode the same test sequences. It was shown that the performance improvement for trained HM.30 contexts over HM3.0 was very small (on average less than 0.05%).

The advantages of the proposal for software were discussed. It was agreed that for optimized software implementations, having unified scans would be beneficial over having two separate scans.

It was commented that the working draft text will be simpler if scans are unified.

The issue of hardware complexity was discussed. The proponents provided additional information to show that same reordering logic could be used for significance and level coding scans.

Sony still has some concerns about memory read/write for large scans. But they agreed that it could be implemented. Participants agreed that there could be a difference in the architectures of different companies.

It was also commented that 4x4 scan may be better to interface with other hardware blocks. Proponents clarified that for large block sizes, you have to wait for all the TU data is decoded and dequantized for the transform to be performed. This is very architecture dependent and it was agreed hat there could be a difference in the architectures of different companies.

The existing level coding has a parsing issue as it needs to know the intra mode. It was commented that the proposed solution makes the parsing of coefficient level independent of the intra mode. There was no consensus on the significance of this issue.

The proposal also reduced the number of contexts for the significance map coding. Context removal was not in the original proposal (E335). It was agreed that this simplification was desirable and the additional contexts were not needed in the current design.

# Recommendation

There was support for adoption of part 2 (unified scans) but no consensus.

One participant suggested that the adoption of reverse significance scan could be discussed in the breakout group on context reduction.