| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F759 |
| --- | --- |

| Title: | Report of the BoG on clean random access (CRA) picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang (BoG chair) Huawei Technologies 400 Crossing Blvd, 2nd Floor Bridgewater, NJ 08807, USA | Tel: Email: | +1 908 541 3518 yekui.wang@huawei.com |
| Source: | BoG on clean random access (CRA) picture |  |  |

_____________________________

# Abstract

This document provides the report of the BoG meeting on clean random access (CRA) picture at the 6-th JCTVC meeting. Changed definitions of clean decoding refresh (CDR) picture and inter prediction, provided in the BoG report, were agreed at the BoG meeting.

# Introduction

The related input documents are JCTVC-F381 and JCTVC-F464.

The BoG meeting was held at 13:30-14:00 on July 19, 2011, with the following participants:

- Peter Amon

- Thomas Davies

- Dzung Hoang

- Chanyal Kim

- Ajay Luthra

- Younqo Park

- Arturo Rodriguez

- Karsten Sühring

- TK Tan

- Wade Wan

- Ye-Kui Wang

The BoG agreed and recommends the following changed definitions of clean decoding refresh (CDR) picture and inter prediction.

# Text changes

The changes, in relative to HEVC WD3d8, provided below are proposed. For convenience, some related definitions that are not changed are also provided below.

The definitions of “inter prediction” and “intra prediction” are changed to cover also prediction of data elements, in addition to sample values (see the definition of prediction process and predictor). Actually, these changes also improves the definitions of “I slice”, “P slice”, and “B slice” to be more generic. If we keep the definitions of “inter prediction” and “intra prediction” unchanged but define the term “refer to” to cover sample values and data elements, then we may also additionally need to change the definitions of “I slice”, “P slice”, and “B slice”. If using the current definition of “inter prediction” and “intra prediction” is acceptable for the definitions of “I slice”, “P slice”, and “B slice”, then using “inter prediction” (i.e., not “refer to”) obviously also acceptable for the definition of “CRA picture”.

3.xx		B slice: A slice that may be decoded using intra prediction or inter prediction using at most two motion vectors and reference indices to predict the sample values of each block.

3.xx		clean random access (CRA) access unit: An access unit in which the primary coded picture is a CRA picture.

3.xx		clean random access (CRA) picture: A coded picture containing only I slices and for which each slice of the CRA picture has nal_unit_type equal to 4, all coded pictures that follow the CRA picture both in decoding order and output order shall not use inter prediction from any picture that precedes the CRA picture either in decoding order or output order; and any picture that precedes the CRA picture in decoding order also precedes the CRA picture in output order.

3.xx		I slice: A slice that is decoded using intra prediction only.

3.xx 	inter prediction: A prediction derived from decoded samples or data elements of reference pictures other than the current decoded picture.

3.xx		intra prediction: A prediction derived from the decoded samples or data elements of the same decoded slice.

3.xx		P slice: A slice that may be decoded using intra prediction or inter prediction using at most one motion vector and reference index to predict the sample values of each block.

3.xx		prediction: An embodiment of the prediction process.

3.xx		prediction process: The use of a predictor to provide an estimate of the sample value or data element currently being decoded.

3.xx		predictor: A combination of specified values or previously decoded sample values or data elements used in the decoding process of subsequent sample values or data elements.

3.xx		random access: The act of starting the decoding process for a bitstream at a point other than the beginning of the stream.

# 4  Abbreviations

For the purposes of this Recommendation | International Standard, the following abbreviations apply:

…

CRA		Clean Random Access

…

Table 7-1 – NAL unit type codes, syntax element categories, and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-IDR and non-CRA picture slice_layer_rbsp( ) | VCL |
| 2-3 | Reserved | n/a |
| 4 | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 5 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 10-11 | Reserved | n/a |
| 12 | Filler data filler_data_rbsp( ) | non-VCL |
| 13-23 | Reserved | n/a |
| 24..31 | Unspecified | non-VCL |