WD Text of JCTVC-F760, change is highlighted

8.3.3.1.6 Specification of Intra_FromLuma prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	neighbouring chroma samples pC[ x, y ], with x, y = -1..nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 35.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + log2( nS ) – 14 )		(8-1)

pY’[ x, y ] = (recSamplesL[ 2x-1, 2y+1 ] + 2*recSamplesL[ 2x, 2y+1 ] + recSamplesL[ 2x+1, 2y+1 ] + 2) >> 2, with x = 0… nS-1, y = -1		(8-2)

pY’[ x, y ] = (recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ]) >> 1, with x = -1… nS-1, y = 0… nS-1	(8-3)

- Variables L, C, LL, LC and k2 are derived as follows:

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k3		(8-4)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k3		(8-5)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3	(8-6)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k3	(8-7)

k2 = log2( (2*nS) >> k3 )		(8-8)

- Variables a and b are derived as:

a1 	= ( LC << k2 ) – L*C		(8-9)
a2 	= ( LL << k2 ) – L*L		(8-10)
k1 	= Max( 0, log2( abs( a2 ) ) – 5 ) – Max( 0, log2( abs( a1 ) ) – 14 ) + 2	(8-11)
a1s	= a1 >> Max(0, log2( abs( a1 ) ) – 14 )		(8-12)
a2s	= abs( a2 >> Max(0, log2( abs( a2 ) ) – 5 ) ) 		(8-13)

a	= a2s < 1 ? 0 : Clip3( -215, 215-1, a1s*lmDiv + ( 1 << ( k1 – 1 ) ) >> k1 )	(8-14)

a	= a >> Max(0, log2( abs( a ) ) – 6 )		 (8-15)

k  = 13 - Max(0, log2( abs( a ) ) – 6 )

b	= ( L – ( ( a*C ) >> k1 ) + ( 1 << ( k2 – 1 ) ) ) >> k2	(8-16)

where lmDiv is specified in Error: Reference source not found with the input a2s.

- The values of the prediction samples predSamples[ x, y ] are derived as:

predSamples[ x, y ] = Clip1C ( ( ( pY’[ x, y ] * a ) >> k ) + b ), with x, y = 0..nS-1	(8-17)