| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F760 |
| --- | --- |

| Title: | BoG report on simplification of intra_chromaFromLuma mode prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen | Email: | jianle.chen@samsung.com |
| Source: | [BoG report on simplification of intra_chromaFromLuma mode prediction] |  |  |

_____________________________

This contribution summarizes the activities of the Break out Group (BoG) on simplification of intra_chromaFromLuma mode prediction.

# Attendees

| Name | Email | Company |
| --- | --- | --- |
| Jianle Chen | jianle.chen@samsung.com | Samsung |
| Shawmin Lei Xun Guo | shawmin.lei@mediatek.com xun.guo@mediatek.com | MediaTek |
| Sze, Vivienne | sze@ti.com | TI |
| Kazushi SATO | Kazushi.Sato@jp.sony.com | Sony |
| Yi-jen Chiu@intel.com | yi-jen.chiu@intel.com | Intel |
| Kazuo Sugimoto | sugimoto.Kazuo@ak.mitsubishielectric.co.jp | Mitsubishi |

# Summary and Recommendation

In this JCT-VC meeting, JCTVC-F121 (MediaTek), JCTVC-F233 (TI) and JCTVC-F431 (Sony) and JCTVC-F494 (Samsung) propose simplification solutions for intra_chromaFromLuma mode prediction. The purpose of this BoG meeting is to find the optimal combination of them.

Based on BoG discussion, the following combination is recommended to be adopted in HM4.0.

- BugFix

- Using one line reference pixel of PU top boundary

  - Left boundary pixel use [1, 1] vertical filter for dowsampling

  - Top boundary pixels use [1, 2 ,1] horizontal filter for downsampling

  - Both filtering process can be performed simultaneously )

- Alpha 7 bits

The experiment results of above combination is shown in following table.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.3 | 0.1 | 0.0 | 0.2 | 0.0 |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class C | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

The BoG also recommends the methods of following proposals to be tested in context of CE for next meeting for further complexity reduction or performance enhancement.

- JCTVC-F173 - an improvement to chroma intra prediction from luma

- JCTVC-F431 - complexity reduction of chroma intra prediction by reconstructed luma samples

- JCTVC-F502 - cross-channel techniques to improve intra chroma prediction

# Reference

JCTVC-F121, “Intra Chroma LM Mode with Reduced Line Buffer”, M. Guo, X. Guo, Y.-W. Huang, S. Lei (MediaTek)

JCTVC-F173, “An improvement to chroma intra prediction from luma”, A. Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi)

JCTVC-F233, “Luma-based chroma intra prediction simplification”, M. Budagavi, A. Osamoto (TI)

JCTVC-F431, “Complexity Reduction of Chroma Intra Prediction by Reconstructed Luma Samples”, K. Sato (Sony)

JCTVC-F494, “Complexity reduction of chroma intra LM prediction mode”, J. Chen, V. Seregin, T. Lee, E. Alshina (Samsung)

JCTVC-F502, “Cross-channel techniques to improve intra chroma prediction”, Y.-J. Chiu, Y. Han, L. Xu, W. Zhang, H. Jiang (Intel)