| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F763 M21418 |
| --- | --- |

| Title: | BoG report on review of deblocking filter related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Minhua Zhou Andrey Norkin | Email: Email: | zhou@ti.com andrey.norkin@ericsson.com |
| Source: | BoG on review of deblocking filter related contributions |  |  |

_____________________________

This contribution summarizes the activities of the Break out Group (BoG) on review of de-blocking filter related contributions.

# Attendees

Geert Van Der Auwera (geertv@qualcomm.com)

Jinho Lee (jinosoul@etri.re.kr)

Thomas Wedi (Thomas.Wedi@eu.panasonic.com)

Matthias Narroschke (Matthias.Narroschke@eu.panasonic.com)

Arturo Rodriguez (rodrigaa@cisco.com)

Andrey Norkin (andrey.norkin@ericsson.com)

Kenneth Andersson (kenneth.r.andersson@ericsson.com)

Byeungwoo Jeon  (bjeon@skku.edu)
Jungyeop Yang (binbak1@ece.skku.ac.kr)

Shawmin Lei (shawmin.lei@mediatek.com)

Hui Yong Kim (hykim5@etri.re.kr)

Sung-Chang Lim (sclim@etri.re.kr)

Masaru Ikeda (Masaru.Ikeda@jp.sony.com)

Teruhiko Suzuki (teruhikos@jp.sony.com)

Sekiguchi Shunichi (Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp)

Tomoo Yamakage (tomoo.yamakage@toshiba.co.jp)

Mangesh Sadafale (mangesh@ti.com)

Vivienne Sze (sze@ti.com)

Minhua Zhou (zhou@ti.com)

# Review of non-CE related contributions

## Proposals

JCTVC-F044 Improving Deblock-filter Performance with SKIP-CU [M. Sadafale (TI)]

This contribution advocates to skip deblocking around edges if CUs of both sides are skipped to lower average case deblocking complexity. The suggested changed does not lead to coding loss.

Decision: Encourage further study.

JCTVC-F053 Deblocking Filter with Reduced Pixel Line Buffers for LCU-based Processing [C.-W. Hsu, J. An, X. Guo, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

Deblocking filter (DF) contains horizontal filtering on vertical edges and vertical filtering on horizontal edges. When the largest coding units (LCUs) are processed by DF in a raster scan order, each LCU needs pixels from its upper LCU. Therefore, pixel line buffers of sizes proportional to the picture width are required. In HM-3.0, for luma vertical filtering, four intermediate pixels and four reconstructed pixels on each side of an edge are used, which requires eight luma line buffers; for chroma vertical filtering, two intermediate pixels on each side of an edge are used, which requires two chroma line buffers. In this contribution, two vertical filtering methods were proposed only for the processing of horizontal LCU edges in order to reduce line buffers while the DF in HM-3.0 was applied for rest edges. The first method could reduce four luma line buffers by using intermediate pixels to replace reconstructed pixels for filtering decisions. Almost no change of BD-rate, run time, and visual quality was reported for the first method. The second method used only reconstructed pixels below horizontal LCU edges for filtering decisions and avoids changing pixels above horizontal LCU edges. The second method could remove all the luma and chroma line buffers dedicated for DF and reportedly caused 0.3% BD-rate increase with roughly unchanged run time and subjective quality.

Method 1: on/off decision on intermediate pixels to reduce line buffer for on/off decision.

It was remarked that storing on/off decision could be an alternative to avoid line buffer.

It was commented that the on/off decision for line buffer is small.

Decision: Further study is encouraged.

Method 2: reducing the line buffer from 4 lines to 0 by not using upper three lines and reusing intra-prediction buffer on LCU horizontal edges. Filter is redesigned for filters around the horizontal LCU boundary.

It was remarked that artifacts may occur because of different filters.

Proponents claimed that non-expert viewers did not see subjective quality difference but experts did.

Decision: further study in CE

JCTVC-F120 Simplification of Chroma Deblocking Filter in F118 [Q. Huang, J. An, X. Guo, S. Lei (MediaTek)]

A modified chroma deblocking filter is proposed in JCTVC-F118, where the filtering unit size for chroma is determined according to the sampling format. Moreover, filtering decision is also added for chroma boundary. This contribution is to simplify the chroma filtering decision in JCTVC-F118 by reusing the filtering decision results of luma component. It is reported that on average BD-Rate reductions of 1.7%, 2.9%, 2.9%, 3.2%, 2.9% and 3.5% are achieved for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD and LC-LD, respectively. The results are reported to be similar to that of JCTVC-F118. It is also reported that the decoding time of proposed method is slightly reduced compared with that of JCTVC-F118 and HM 3.0, and the subjective quality is almost the same to that of JCTVC-F118.

This contribution proposes a simplification of chroma de-blocking filter on/off line decision based on luma on/off decision. Chroma filter uses 4x4 block size.

Decision: further study in CE

JCTVC-F175 Signaling of boundary filtering strength of deblocking filter for intra [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

This contribution proposes signaling of boundary filtering strength of deblocking filter for intra at the slice header. The proposed method signals the strength for luma and chroma separately as the difference from the default strength value. The proposed method is implemented on the HM-3.0 software, and simulations using the value of -2 for the boundary filtering strength for intra luma are conducted using common test configurations[1]. It is reportedly confirmed that the proposed method achieves 0.7, 0.4 and 0.1% BD-rate reduction on average for All intra, Random access and Low delay high efficiency configurations, and 0.5, 0.4 and 0.1% BD-rate reduction for each low complexity configurations respectively.

This method provides flexibility of controlling filter strength (Bs) for deblocking around intra edges by adding two slice level offsets (bSL, bSC), use case is intra refreshment in which boundary strength different from 3 and 4 may be needed.

It was remarked that similar tools (such as Toshiba JCTVC-D192 and ETRI JCTVC-F258) have already been studied in CE12.

Decision: further study in CE

JCTVC-F214 Parallel deblocking improvement [M. Ikeda, J. Tanaka, T. Suzuki (Sony)]
This contribution proposes the following two tools to improve deblocking parallel processing:

Method 1: Both horizontal (H) and vertical (V) filtering process applied to reconstructed samples. The average BD-rate is reportedly -0.3% for AI_HE, -0.1% for AI_LC, -0.2% for RA_HE, -0.1% for RA_LC, -0.2% for LD_HE and -0.1% for LD_LC.

It was remarked that this proposal can potentially resolve the misalignment between parallel on/off decision and sequential filtering.

### Method 2: On/off  filter on/off decision using the central non-deblocked samples (moving from line d2&d5 to d3&d4). The average BD-Rate for luma in the second method is reportedly -0.1% for AI_HE, -0.1% for AI_LC, -0.1% for RA_HE, -0.1% for RA_LC, 0.0% for LD_HE and 0.0% for LD_LC

Decision: further study in CE, separate study should be conducted for both methods

JCTVC-F215 Vertical tap length reduction to reduce line memory in deblocking filter [M. Ikeda, T. Suzuki (Sony)]

This contribution proposes to reduce strong intra filter tap length by one to reduce de-blocking line buffer size from four lines to three lines. The proposed change does not cause coding loss.

It was remarked that filter symmetry is changed that may impact visual quality.

It was remarked that modified filter of shorter tap only needs to apply at horizontal LCU boundaries to achieve line buffer reduction.   The current design has changed filter at all CU boundaries.

Decision: further study in CE

JCTVC-F256 Improving Deblock filter efficiency [M. Sadafale (TI)]

The contributions propose the following changes:

### Part 1  modify Chroma filter to share luma filter logic and apply Chroma filter to edges even if both sides belong to inter CU. The average BD-Rate for chroma U in the second method is reportedly -0.2% for AI_HE, -0.5% for AI_LC, -0.6% for RA_HE, -0.6% for RA_LC, -0.6% for LD_HE and -0.9% for LD_LC. No BD-rate change has been observed for luma.

Decision: further study in CE

Part 2: removal of parallel on/off decision (same as JCTVC-F465)

There is a concern that two sets of data are needed to make the decisions/filtering whereas HM2.0 required only one set of data.

Decision: further study in CE

JCTVC-F359 Deblocking filtering modification for constrained intra prediction [J. Lee, S.-C. Lim, H. Y. Kim, J. S. Choi (ETRI)]

This contribution presents deblocking filter modification for constrained intra prediction. Constrained intra control that intra blocks in one frame are not affected by errors of another frame. To this end, constrained intra only allows neighboring samples from intra blocks for intra prediction. However, if an inter block is not correctly reconstructed, the errors of the inter block would be propagated to neighboring intra blocks by HM3.0 deblocking process. The proposal prevents the problem by disabling deblocking filtering on pixels inside intra block when its neighboring block is inter block. It is reported that the BD-rate increase due to the proposed method is negligible: 0.1% and 0.1% in RA-HE and RA-LC, respectively, and 0.0% and 0.0% in LD-HE and LD-LC, respectively.

### The proposal is to disable deblocking filter for intra block edge if neighboring block is inter when constrained_intra_pred_flag = 1.

It was remarked that in AVC works deblocking is performed for constrained intra prediction blocks.

It was remarked that ALF may also need to be disabled for constrained intra prediction blocks to avoid error propagation from inter block to intra block.

No consensus on whether this restriction is truly needed.

Decision: encourage further study

JCTVC-F405 Deblocking filter using adaptive weighting factors [M. Narroschke, A.-K. Seifert (Panasonic)]

In this contribution, adaptive weighting factors are introduced into the deblocking filter. Each weighting factor weights the value, by which the HM3.0 deblocking filter would modify a sample value. The weighting factors are estimated at the encoder side by minimization of the mean squared error between the deblocked signal and the original input signal. The encoded weighting factors are coded and transmitted to the receiver side. An average BD-bit rate reduction of 1.1% (I-HE: 1.7%, I-LC: 1.1%, RA-HE: 1.2%, RA-LC: 0.8%, LD-HE(B): 1.0%, LD-LC(B): 0.6%, LD-HE(P): 0.9%, LD-LC(P): 0.4%) is achieved compared to HM3.0 with approximately no encoder/decoder run time increases. Combined with the individual decisions and the evaluation of the second derivation at the block boundary in the decision process as proposed in JCTVC-F191, the average bit rate reduction is increased to 1.2% (I-HE: 1.7%, I-LC: 1.1%, RA-HE: 1.3%, RA-LC: 0.8%, LD-HE(B): 1.4%, LD-LC(B): 0.8%, LD-HE(P): 1.2%, LD-LC(P): 0.4%). It is proposed to evaluate this technique within the next CE on deblocking filters.

This proposal advocates to add weighting factors to derived delta (S‘ = S + delta (HM3.0) => S’ = S +w*delta ). A total of 28 weighting factors are quantized by 64 and transmitted in slice header. High-level disabling flag is defined.

It was remarked that the proposed algorithm introduces a slice of delay due to weights estimation based on minimization of the mean square error.

It was remarked that subjective quality impact should be evaluated.

Experts from Toshiba pointed out that fixing weights to 0.5 led to an average gain of 0.4% RA-HE and 1.0%  LD-HE in HM2.0.

Decision: further study in CE

JCTVC-F484 Improving Worst-Case performance of Deblock filter [Mangesh Sadafale (TI)]

This contribution proposes to remove motion vectors for boundary strength (Bs = 1) derivation process. Average BD-rate loss is reportedly 0.1% in RA-HE, 0.1% in RA-LC, 0.2% in LD-HE and 0.1% in LD-LC.

Decision: Recommend to study in the CE.

JCTVC-F556 CE6.b: SDIP harmonization with deblocking, MDIS and HE residual coding [G. Van der Auwera, J. Sole, Y. Zheng, X. Wang, I. S. Chong, R. Joshi, M. Karczewicz (Qualcomm)]

### This contribution proposes the harmonization SDIP + deblocking by the following changes by Modifying weak filters and adding two more sets of weak filters to accommodate small block sizes  of 4x16, 16x4, 8x2, 2x8. 16x1 and 1x16. The average BD-rate gain is reportedly 0.1% in both AI-HE and AI-LC.

Concerns were expressed regarding the complexity increase due to small block size filtering.

It was remarked that subjective quality improvement needs to be evaluated to justify the complexity increase.

Decision: further study in CE

## Cross-check documents

JCTVC-F219 Cross-verification of TI's proposal JCTVC-F044 [M. Ikeda, T. Suzuki (Sony)]

JCTVC-F531 Cross-verification of MediaTek's line buffer reduction proposal (F053) by Ericsson [Andrey Norkin (Ericsson)] [late upload 07-12]

JCTVC-F210 Cross-check of simplified filtering decision for chroma deblokcing filter (JCTVC-F120) [J. Xu (Microsoft)] [late upload 07-12]

JCTVC-F267 Cross-verification Results of Sony's Parallel Deblocking Improvement (JCTVC-F214) by SKT/SKKU [J. Yang, K. Won, B. Jeon (SKKU), J. Lim (SK Telecom)]

JCTVC-F445 Cross-check of Parallel deblocking improvement (JCTVC-F214) by Sony [M. Sadafale (TI)]

JCTVC-F444 Cross-check results for Sony’s proposal JCTVC-F215 [S. Esenlik, M. Narroschke (Panasonic)]

JCTVC-F220 Cross-verification of TI's proposal JCTVC-F256 [M. Ikeda, T. Suzuki (Sony)]

JCTVC-F651 Cross-check report on JCTVC-F359 by ETRI [V. Wahadaniah, C. S. Lim (Panasonic)] [late reg. 07-05, upload 07-07]

JCTVC-F722 Cross-verification of TI's JCTVC-F484 on improving worst case performance of deblock-filter V. Seregin, J. Chen (Samsung) [late reg. 07-12, upload 07-12]

JCTVC-F437 CE6.b Crosscheck Report of Qualcomm’s Proposal JCTVC-F556 [M. Guo, X. Guo (Mediatek)]

JCTVC-F774, Cross Check of Panasonic’s Proposal JCTVC-F405 [G. Van der Auwera]

# Summary of review report for non-CE contributions

| Contribution | Tools | BD-rate (%) | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| JCTVC-F044 (TI) | Skip de-blocking around a edge if CUs of both sides are skipped | 0.0 | No subjective viewing results available | Encourage further study. |
| JCTVC-F053 (MTK) | Tool 1 on/off decision on intermediate pixels to reduce line buffer for on/off decision | 0.0 | store on/off decision is alternative? On/off decision line buffer is small | Encourage further study. |
| JCTVC-F053 (MTK) | Tool 2 Reduce the line buffer from 4 lines to 0 by not using upper three lines and reusing intra-prediction buffer on LCU horizontal edges. Filter is redesigned for filters around the horizontal LCU boundary | 0.2 | Any visual artifacts because of different filters? Claimed that non-expert viewers did not see subjective quality difference but experts did. | Further study in CE |
| JCTVC-F120 (MTK) | Chroma on/off decision based on luma. Using same 4x4 chroma filtering same as F118 | -2.9 (U) | Simplification of chroma filter on/off decision | Further study in CE |
| JCTVC-F175 (Mitsubishi) | Adding two slice level offsets (bSL, bSC) to control filter strength (Bs) for intra edges | -0.4 (bSL=2 and bSC=0) | Currently intra edges Bs is fixed to 4 or 3, this allows to apply less strong filters on intra edges. Use case is intra refreshment. | Recommended for CE to study together with similar technology such as Toshiba D192 and ETRI F258 |
| JCTVC-F214 (SONY) | Tool 1 Both horizontal and vertical deblocking filtering performed on reconstructed samples in parallel, final output is obtained by weighting H/V outputs | -0.2 | This proposal could potentially resolve the misalignment between parallel on/off decision and sequential filtering | Further study in CE |
| JCTVC-F214 (SONY) | Tool 2 ### Filter on/off decision using the central non-deblocked samples in block boundary (move from line d2 d5 to d3&d4) | -0.1 |  | Further study in CE (Separate study should be done for tool 1 and tool 2 in CE) |
| JCTVC-F215 (SONY) | ### Reduce strong intra filter tap length by one to reduce de-blocking line buffer size by one line | 0.0 | Filter symmetry is impacted, the current design has changed filter at all CU boundaries. Recommended to apply new filter only at LCU boundary. | Further study in CE |
| JCTVC-F256 (TI) | Part1 Change chroma filter to share luma filter logic ### Apply chroma filter to edges even both sides belong to inter CU | -0.6 (U) |  | Further study in CE |
| JCTVC-F256 (TI) | ### Part 2 ### Remove parallel on/off decision | 0.0 | There is a concern that two sets of data are needed to make the decisions/filtering whereas HM2.0 required only one set of data. | Further study in CE |
| JCTVC-F359 (ETRI) | ### Disable deblocking filter for intra block if neighboring block is inter when constrained_intra_pred_flag = 1 | 0.1 | Avoid error propagation from inter to intra blocks for intra refreshment. AVC works without this constraint. How about ALF? | Encourage further study. |
| JCTVC-F405(Panasonic) | ### Adding weighting factors to derived delta. 28 weighting factor quantized by 64 and transmitted in slice header. S‘ = S + delta (HM3.0) => S’ = S +w*delta (proposed) Weights estimated with MSE optimization High-level disabling flag is defined. | -1.1 | Latency of one slice. Subjective quality impact should be evaluated. Information from Toshiba: fixing weights to 0.5 leads to a gain of 0.4% RA-HE, 1.0% LD-HE in HM2.0 | Further study in CE |
| JCTVC-F484 (TI) | ### Simplification of Bs derivation process by removing MVs for Bs = 1 derivation process | 0.1 |  | Further study in CE |
| JCTVC-F556 (QCom) | ### SDIP + deblocking harmonization Modified weak filters, added two more sets of weak filters to accommodate small block sizes of 16x4, 4x16, 8x2, 2x8. 16x1 and 1x16 | -0.1 | Concerns expressed regarding the complexity increase due to small block size filtering, subjective quality improvement needs to be evaluated to justify the complexity increase | Further study in CE |

# Summary of review report for CE12 related contributions

|  | tools | Comments | BD-rate | Subjective gain |
| --- | --- | --- | --- | --- |
| Ericsson/MediaTek (F118) | Separate on/off decision for two sides of a luma edge. 4x4 block-based chroma, Apply chroma Inter-edges, Chroma format dependent deblocking block size | How chroma delta is scaled? How scales are derived? Scales are fixed. No description on how chroma delta scaling is done. Filter response advantage in luma filter design (-1.2 %) Parallel decision for chroma? Yes. line buffer still the same, no reduction comment: gain in luma part may partially come from chroma gain. Intra TC offset = 2, inter offset = 0 is used Separate luma and chroma tests were done in previous April’11 Geneva meeting. | -1.2%/-2.6%/-2.7% | No loss |
| SKT/SKKU (F258) | Unified luma/chroma 8x8 based filter, Reduced number of filters Unified Bs derivation for intra/inter Modified filter type decision for luma Modified weak filter | . Filter design may have problem which could cause coding artifacts? Noise amplifying? SAO cleans up noise? Adding dependency, intra Bs calculation depends on intra prediction mode. Can changing Tc value achieve the same? Filtering two pixels each side of a chroma edge Intra TC offset = 1 or 3 based on intra prediction direction | -1.1%/-0.6%/-0.6% | Kimono QP = 32 case worse than anchor |
| SKT/SKKU (F262) | Luma F258, Chroma F118 |  | -1.2/-2.6%/-2.7% | Kimono QP = 32 case worse than anchor |
| Microsoft (F198) |  |  | 0.0% | Kimono QP = 32 case worse than anchor |
| Panasonic (F191) v1 | On/off decision done every other line (-0.2%). Decision is for each line. | Soft complexity slightly increased in on/off decision | -0.2% | Kimono QP = 32 case worse than anchor |
| Panasonic (F191) v2 | Evaluation of gradient at boundary |  | -0.5% | Further study in CE |
| Toshiba/Qualcomm (F143) (c) | Intra TC offset = 2, inter TC offset = -1 Adding slice level Tc offset | Intra TC offset = 2 is used in F118 as well | -0.3 | No loss |

Discussions:

Tc offset setting:

HM3.0 intra Tc offset = 4, inter Tc offset = 0, agreed on setting Intra TC offset to 2, inter TC offset = 0

Recommend to adopt setting intra TC offset to 2 of F143 into HM4.0. (-0.2% gain).

It was remarked that luma and chroma improvement should be tested separately.

Luma part of F118 which includes the luma filter design and the separate on/off decision for P/Q edge side: better filter response, no subjective quality loss, no significant complexity increase, -1.2% gain in luma.  More computational dependency? Being able to switch on/off filtering of either side may achieve the same goal?

Strong support from non-proponents to adopt the luma part of F118 which includes the luma filter design and the separate on/off decision for P/Q edge side. Some concerns expressed by some of CE12 proponents.

# Reference

[1]  V. Baroncini, A. Norkin, M. Narroschke, B.  Jeon, “Report on subjective viewing test for deblocking filter proposals”, JCTVC –F767, 6th Meeting: Torino, IT, 14-22 July, 2011.