| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F764 |
| --- | --- |

| Title: | Report of BoG meeting on CE3 MC interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032 Japan | Tel: Email: | +81-3-5435-3308 teruhikos@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Participants

Elena Alshina (Elena_a.alshina@samsung.com / Elena.alshina@gmail.com)

Yukihiro Bandoh (jctvc-te@lab.ntt.co.jp)

Jinale Chen, jianle.chen@samsung.com

Takeshi Chujoh (takeshi.chujoh@toshiba.co.jp)

Hsan Guermanzi (hsan@ebriskvideo.com)

Michael Horowitz (michael@ebriskvideo.com)

Jae Hoon Kim ()

Kenji Kondo (Kenji.Kondo@jp.sony.com)

Ajay Luthra (aluthra@motorola.com)

Shohei Matsuo (jctvc-te@lab.ntt.co.jp)

Koohyar Minoo (kminoo@motorola.com)

Teruhiko Suzuki (teruhikos@jp.sony.com)

Akiyuki Tanizawa (akiyuki.tanizawa@toshiba.co.jp)

Kemal Ugur (Kemal.Ugur@nokia.com)

# Related contributions

## Contributions

CE3 contributions:

JCTVC-F048 CE3: Modifications of region-based adaptive interpolation filter [S. Matsuo, Y. Bandoh, T. Ito, S. Takamura, H. Jozawa (NTT)]

JCTVC-F216 An Adaptive Interpolation Filtering Technique [F. Kossentini, N. Mahdi, H. Guermazi, M. A. Ben Ayed, M. Horowitz (eBrisk)]

JCTVC-F468 An Adaptive Interpolation Filtering Technique [F. Kossentini, N. Mahdi, H. Guermazi, M. A. Ben Ayed, M. Horowitz (eBrisk)]

JCTVC-F100 CE3: Results on MC boundary filter (tool 4) [K. Kondo, T. Suzuki (Sony)]

JCTVC-F101 CE3: Results on Bi/Uni MC filter (tool 5) [K. Kondo, T. Suzuki (Sony)]

JCTVC-F248 CE3: Interpolation filter with shorter tap-length for small PU's [K. Ugur, J. Lainema (Nokia)]

JCTVC-F315 CE3: Non-uniform tap length filtering for bidirectional prediction [T. Chujoh, T. Yamakage (Toshiba)]

JCTVC-F247 CE3: DCT derived interpolation filter test by Samsung. [E. Alshina, A.Alshin (Samsung)]

JCTVC-F576 CE3: Slice-type based adaptive interpolation filter tests by Motorola Mobility [J. Lou, K. Minoo, D. Baylon, K. Panusopone, L. Wang, A. Luthra (Motorola Mobility)]

JCTVC-F574 CE3: Fixed interpolation filter tests by Motorola Mobility [J. Lou, K. Minoo, D. Baylon, K. Panusopone, L. Wang, A. Luthra (Motorola Mobility)]

New contributions:

JCTVC-F242 Bilinear chroma interpolation for small block sizes [K. Ugur, J. Lainema (Nokia), K. Kondo, T. Suzuki (Sony)]

JCTVC-F599 On Chroma interpolation filters [Koohyar Minoo, Jian Lou, Ajay Luthra]

## Cross-check documents

JCTVC-F222 CE3: Cross-check report for Sony's test JCTVC-F# by eBrisk Video [F. Kossentini, N. Mahdi (eBrisk)]

JCTVC-F246 CE3: Cross-check report by Nokia for 2.2, 4.3, 4.6 [K. Ugur (Nokia)] [late upload 07-06]

JCTVC-F259 CE3: Cross-check report for test1 JCTVC-F315 [K. Kondo, T. Suzuki (Sony)]

JCTVC-F260 CE3: Cross-check report for test2.1 JCTVC-F247 [K. Kondo, T. Suzuki (Sony)]

JCTVC-F261 CE3: Cross-check report for test3 JCTVC-F248 [K. Kondo, T. Suzuki (Sony)]

JCTVC-F314 CE3 Subset 2.4: Cross-Verification of Samsung’s interpolation filter (JCTVC-F247) by Qualcomm [I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-F316 CE3: Cross-check report for Samsung's tool 2.3 (JCTVC-F247) [T. Chujoh, T. Yamakage (Toshiba)]

JCTVC-F317 CE3: Cross-check report for Sony's tool 5 (JCTVC-F101) [T. Chujoh, T. Yamakage (Toshiba)]

JCTVC-F322 CE3 : Cross-check for NTT's proposal on Region-Based Adaptive Interpolation Filter (JCTVC-F048) [T. Yoshino, K. Kawamura, S. Naito (KDDI)]

JCTVC-F344 Cross-check report on CE3 Subtest 4.1 and 4.4 (JCTVC-F100) [K. Chono, H. Aoki (NEC)]

JCTVC-F443 CE3: Cross-check by Samsung for Motorola Mobility tests on interpolation filters [E. Alshina (Samsung)] [late upload 07-07]

JCTVC-F568 CE3: Cross-check report for Samsung's test JCTVC-F247 by Motorola Mobility [J. Lou, K. Minoo, L. Wang (Motorola Mobility)]

JCTVC-F570 CE3: Cross-check report for eBrisk's test JCTVC-F216 by Motorola Mobility [J. Lou, L. Wang (Motorola Mobility)]

JCTVC-F685 Cross-check of JCTVC-F599: Chroma interpolation filters [Jinwen Zan, Dake He] [late reg. 07-07, missing prior]

# Complexity measurement

The complexity measurement to evaluate CE3 MC Interpolation filter was discussed. In CE3 at this meeting used Excel sheet to measure the average number of add/multiplication and the worst case memory bandwidth. However, the average was calculated with equal probability of all interpolation filters. It was pointed out that such average should be measured by using the bitstreams coded by common test conditions. BoG concluded to measure the following complexity measurements for the evaluation.

- Average number of add/multi in all bitstreams

- Average memory bandwidth

- Worst case memory bandwidth

It is mandatory to provide all the above information. The software and new Excel sheet to measure the complexity was provided by JCTVC-F247_r3 and attached to this BoG report too.

# Discussions

All proponents provided all complexity measurements by Tuesday. The proposals were evaluated with the complexity measurements. The followings are the summary of the review. The results are summarized in the attached Excel sheet.

## Fixed interpolation filter with modified compare to HM3.0 coefficients targeting for better tradeoff between performance and complexity

JCTVC-F247 CE3: DCT derived interpolation filter test by Samsung. [E. Alshina, A.Alshin (Samsung)]

Use shorter tap filter at all cases to reduce complexity. The average computation is reduced 16 %. The memory bandwidth is reduced 5% with 0.07% performance drop compared to HM 3. Combination with JCTVC-F248 can reduce memory bandwidth worst case to 41%.

Should be reviewed

JCTVC-F574 CE3: Fixed interpolation filter tests by Motorola Mobility [J. Lou, K. Minoo, D. Baylon, K. Panusopone, L. Wang, A. Luthra (Motorola Mobility)]

JCTVC-F599 On Chroma interpolation filters [Koohyar Minoo, Jian Lou, Ajay Luthra (Motorola Mobility)]

Proponent prefers JCTVC-F599 configuration. Coding gain is 0.48 % in luma but 0.12 % loss in chroma. No complexity increase compared with HM 3.0. Interest in coding gain from the floor. Combination with JCTVC-F248 proposal can reduce the worst case memory bandwidth to 50%. Questions for different from 4:2:0 color format on the asymmetry filter. It increases the complexity.

Should be reviewed

## Extended interpolation filters set for memory bandwidth (including peak case) reduction

JCTVC-F248 CE3: Interpolation filter with shorter tap-length for small PU's [K. Ugur, J. Lainema (Nokia)]

The worst case memory bandwidth is reduced 50 % with 0.05 % coding loss. Average complexity (both computation and memory bandwidth) is not improved. This should be compared with simple restriction of small block sizes (PU < 8x8). This proposal can be combined with any kind of interpolation filter. Question was raised that ME with two filters increases complexity.

Should be reviewed

JCTVC-F242 Bilinear chroma interpolation for small block sizes [K. Ugur, J. Lainema (Nokia), K. Kondo, T. Suzuki (Sony)]

This proposal extend JCTVC-F248 to chroma interpolation filter.

JCTVC-F100 CE3: Results on MC boundary filter (tool 4) [K. Kondo, T. Suzuki (Sony)]

Proponent`s preferred configuration.  Symmetric filter at SAO bound to reduce the worst case memory band width to 40% also reduce average memory band width 13%.  Coding loss is 0.17%. Nokia`s proposal reduces the worst case memory bandwidth 50%. Question was raised about different way of processing samples depending on sample position. Question was raised that ME with several filters increases complexity

Should be reviewed

## Variable filter coefficients are used (AIF) in order to improve coding efficiency

JCTVC-F048 CE3: Modifications of region-based adaptive interpolation filter [S. Matsuo, Y. Bandoh, T. Ito, S. Takamura, H. Jozawa (NTT)]

Complexity of filter design is the same as HM3. Coding gain is -0.67 % with no complexity increase. Encoding time is increased 8 %.

JCTVC-F216 An Adaptive Interpolation Filtering Technique [F. Kossentini, N. Mahdi, H. Guermazi, M. A. Ben Ayed, M. Horowitz (eBrisk)]

Use current HM 8 tap filter as default and use separable 8 tap AIF with same structure. 2 bit flag to switch default and AIF at Q pel and H pel. Send the 2 bit flag per slice. AIF filter coefficients are calculated per slice. Chroma: 16 options of default and AIF. AIF restores most loss sustained by simplified ALF.  -0.69 % gain with 4 % encoding time increase. Average and worst case complexity (computation and memory bandwidth) is the same as HM3.

JCTVC-F468 An Adaptive Interpolation Filtering Technique [F. Kossentini, N. Mahdi, H. Guermazi, M. A. Ben Ayed, M. Horowitz (eBrisk)]

Use Samsung's new filter design for default filter. Other is the same as F216. Proponent prefers this configuration.

Should be reviewed

## Extended interpolation filters set for overall complexity reduction

JCTVC-F315 CE3: Non-uniform tap length filtering for bidirectional prediction [T. Chujoh, T. Yamakage (Toshiba)]

The worst case number of operation is reduced. Use 6 tap asymmetry filter only for bi-pred, 2D case. Other filter, including chroma, is the same as HM3. 6 % reduction of average computation and 2% average memory bandwidth. The worst case memory bandwidth is the same as HM3. Coding loss is 0.13 %. If remove intermediate rounding, complexity is more reduced.

# Summary and recommendation

The complexity measurement for the evaluation of MC interpolation filter was agreed. All proponents need to provide the following information (mandatory)

- Average number of add/multi in all bitstreams

- Average memory bandwidth

- Worst case memory bandwidth

CE3 and related new contributions were reviewed. BoG identified 5 contributions, JCTVC-F247, F599, F248, F100, F468 valuable to review by the JCTVC planary. Those should be reviewed by JCTVC plenary and make a decision. Other contributions are continued to study by CE.