| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F768 |
| --- | --- |

| Title: | An observation of the subjective viewing test results for deblocking filter proposals |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | *Xun Guo, #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | xun.guo@mediatek.com shawmin.lei@mediatek.com |  |
| Source: | MediaTek Inc. |  |  |  |

_____________________________

# Abstract

This contribution provided one way to extract information out of the complicated subjective viewing test results for deblocking filter proposals. It reported that there were five proposals which had subjective quality at least the same as or better than the H3.0 anchor in all six test sequences.

# Introduction

An informal subjective viewing for the deblocking filtering was held during the Torino meeting on July 18, 2011. There were thirteen proposals tested and the anchor, in total 14 different proposals. The report of the subjective viewing report can be found in the document JCTVC-F767.

# An observation of the subjective viewing results

For each test sequence, we can mark as green color for those proposals that we can confidently say that they are better than the anchor. We mark as yellow color for those proposals which confident intervals overlap with the confident interval of the anchor. We then mark as red color for those proposals that we can confidently say that they are worse than the anchor. The result of these markings is shown in the following table, where BQM-32, BQM-37, Kimono32, Kimono37, Vidyo32, and Vidyo37 are six test sequences, MEAN is mean score, and CI is confident interval.

| BQM-32 | MEAN | CI |  | BQM-37 | MEAN | CI |  | Kimono 32 | MEAN | CI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 198 | 6,00 | 0,32 |  | 120 | 4,50 | 0,23 |  | 262 | 5,33 | 0,35 |
| 405 | 6,00 | 0,36 |  | 214 | 4,58 | 0,25 |  | 214 | 5,33 | 0,39 |
| 262 | 6,08 | 0,31 |  | 177 | 4,67 | 0,25 |  | 120 | 5,33 | 0,37 |
| 118 | 6,17 | 0,20 |  | 215 | 4,67 | 0,25 |  | 191 | 5,50 | 0,35 |
| HVC | 6,25 | 0,21 |  | HVC | 4,75 | 0,30 |  | 198 | 5,50 | 0,28 |
| 143 | 6,25 | 0,24 |  | 118 | 4,75 | 0,30 |  | 177 | 5,58 | 0,35 |
| 120 | 6,25 | 0,21 |  | 484 | 4,75 | 0,18 |  | 258 | 5,67 | 0,37 |
| 191 | 6,33 | 0,30 |  | 143 | 4,83 | 0,20 |  | 405 | 5,67 | 0,35 |
| 256 | 6,33 | 0,25 |  | 258 | 4,83 | 0,27 |  | 215 | 5,67 | 0,42 |
| 215 | 6,42 | 0,22 |  | 262 | 4,83 | 0,29 |  | 118 | 5,75 | 0,38 |
| 177 | 6,50 | 0,33 |  | 198 | 4,83 | 0,27 |  | 484 | 5,92 | 0,28 |
| 258 | 6,58 | 0,22 |  | 256 | 4,92 | 0,19 |  | 256 | 6,17 | 0,38 |
| 214 | 6,67 | 0,18 |  | 191 | 5,00 | 0,27 |  | 143 | 6,25 | 0,36 |
| 484 | 6,67 | 0,22 |  | 405 | 5,08 | 0,25 |  | HVC | 6,33 | 0,28 |
| Kimono 37 | MEAN | CI |  | Vidyo 32 | MEAN | CI |  | Vidyo 37 | MEAN | CI |
| 405 | 3,75 | 0,27 |  | 405 | 4,08 | 0,35 |  | 198 | 3,42 | 0,28 |
| 484 | 3,83 | 0,34 |  | HVC | 5,50 | 0,28 |  | 262 | 3,67 | 0,25 |
| 143 | 3,92 | 0,31 |  | 143 | 5,50 | 0,26 |  | 484 | 3,67 | 0,25 |
| 262 | 3,92 | 0,33 |  | 262 | 5,58 | 0,25 |  | 256 | 3,67 | 0,37 |
| 177 | 4,08 | 0,37 |  | 118 | 5,67 | 0,22 |  | HVC | 3,75 | 0,42 |
| 256 | 4,08 | 0,33 |  | 198 | 5,75 | 0,34 |  | 215 | 3,75 | 0,36 |
| 118 | 4,17 | 0,27 |  | 484 | 5,75 | 0,24 |  | 258 | 3,83 | 0,27 |
| 198 | 4,17 | 0,36 |  | 191 | 5,83 | 0,24 |  | 143 | 3,92 | 0,33 |
| 214 | 4,17 | 0,45 |  | 258 | 5,83 | 0,24 |  | 214 | 3,92 | 0,35 |
| 215 | 4,17 | 0,36 |  | 214 | 5,83 | 0,29 |  | 118 | 4,00 | 0,40 |
| HVC | 4,25 | 0,42 |  | 256 | 5,83 | 0,24 |  | 177 | 4,00 | 0,32 |
| 191 | 4,25 | 0,42 |  | 177 | 5,92 | 0,25 |  | 120 | 4,00 | 0,27 |
| 120 | 4,25 | 0,34 |  | 120 | 6,00 | 0,24 |  | 191 | 4,08 | 0,35 |
| 258 | 4,33 | 0,25 |  | 215 | 6,08 | 0,28 |  | 405 | 5,33 | 0,30 |

With the help of this table, one can easily see that most of proposals have similar subject quality as the anchor. However, if we want to identify those proposals that have subjective quality at least the same (marked as yellow) as or better (marked as green) than the H3.0 anchor in all six test sequences, there are only five proposals satisfy this criterion, namely, JCTVC-F215, JCTVC-F484, JCTVC-F118, JCTVC-256, and JCTVC-F143. This criterion can show the robustness aspect of the proposals in terms of subject quality.

# Conclusion

A deblocking proposal that has subjective quality at least the same as or better than the anchor in all six test sequences shows its robustness in terms of subjective quality. Five proposals that pass this criterion are JCTVC-F215, JCTVC-F484, JCTVC-F118, JCTVC-256, and JCTVC-F143. Note that JCTVC-F118 and JCTVC-F143 are included in CE12, and the other three are non-CE proposals.