| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22July, 2011 | Document: JCTVC-F771 |
| --- | --- |

| Title: | BoG report: Screen Content Coding (SCC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Oscar C. Au Dept. of ECE, Hong Kong University of Science and Technology Clearwater Bay, Hong Kong | Tel: +852-2358-7053 Email:eeau@ust.hk |  |
|  | Jizheng Xu Microsoft No. 5, Danling Street, Haidian District, Beijing 100080, China Haoping Yu Huawei Technologies (USA) 2330 Central Expressway Santa Clara, CA 95095 | Tel: +86-10-5917-3019 Email: jzxu@microsoft.com Tel:+1-408-330-4813 Email: haopingyu@huawei.com |  |
| Source: | HKUST, Microsoft, Huawei |  |  |

_____________________________

# Abstract

This contribution provides a summary of the SCC test material subjective viewing held at 1pm on 19 July, 2011, and the BoG meeting on Screen Content Coding (SCC) held at 6pm on 19 July, 2011.

# General Background

- An AHG on SCC was established between the 5th JCTVC meeting in Geneva and the 6th meeting in Torino to coordinate SCC related matters.

- A total of 18 SCC test sequences were submitted to the designated ftp site: 12 sequences from Huawei, 3 from BJUT and 3 from HKUST. As a result, subjective view of the test material by JCTVC experts was to be carried out.

- A BoG meeting was to be held to examine the copyright and terms of usage of the sequences, and to study and recommend test conditions for future SCC coding experiments.

# Summary of subjective viewing

- A subjective viewing of the 18 SCC test sequences and coded SCC results was held on Tuesday, 19 July, from 1pm to 2:30pm.

- About 20 JCTVC experts (from NEC, Apple, FastVDO, ZTE, Mitsubishi, Microsoft, Huawei, Sejong U, Leibniz U, SungKyunKwan U, Zhejiang U, Peking U, BJUT, HKUST) attended the session. Each sequence was played repeatedly to the satisfaction of the experts. The design ideas/principles were highlighted. Due to limited harddisk space, only a few coded sequences were shown: Huawei - ChinaSpeed (a video game sequence), BJUT – slideshow (a slide show presentation), HKUST - HKUST1 (a document editing sequence).

- Out of the 12 sequences from Huawei, 7 sequences are existing Class A/B/C/D/E sequences with a running banner added at the bottom. The banner size and the spacing are no longer multiples of 8. Two sequences are texting editing shots with video overlay. Another two sequences are purely text editing shots. One sequence is a game sequence. It was agreed that all the 12 sequences do not have copyright problems.

Comments/observations: In the coded version of the video game sequence (ChinaSpeed), some small important details (e.g. user ID, or scores) are badly distorted while the rest of the video seems reasonable. Some severe distortion of HM on screen content can be very localized lead to significant degradation on intelligibility, while the PSNR/BR may suggest good performance.

- Out of the 3 sequences from BJUT, one is document editing. One is web browsing. Both are good and have no copyright problem. The third one is a slide show presentation. An audience pointed out a minor problem, which BJUT promised to fix by next Monday. Eventually, BJUT fixed the problem by 21 July during the Torino meeting. Otherwise, there is no other copyright problem.

Comments/observations: The coded version of the SlideShow sequence shows some significant distortion – some low-contrast background texture and edges can be totally removed. Some numbers are smeared rather significantly.

- Out of the 3 sequences from HKUST, one is a circuit design shot. One is a web browsing shot. Both are fine in terms of copyright. The third on is on document editing. One audience points out a potential minor problem - a software icon may not be from Microsoft. Eventually, HKUST confirmed that the icon is from Microsoft. Otherwise, there is no other copyright problem.

Comments/observations: The audience comment that the circuit design shot is interesting but not challenging enough. More complicated motion should be added. The coded version of the document editing sequence contains badly smeared fonts in low contrast regions. Dotted line becomes solid line. The low-contrast buttons can be badly distorted after coding.

# Summary of BoG meeting

- A first BoG meeting on SCC was held on Tuesday, 19 July, from 6pm to 8pm. About 10 experts from Huawei, Microsoft, Beijing University of Technology, Leibniz University, Zhejiang University, and Hong Kong University of Science and Technology. (BBC and Peking University also gave input before the BoG.)

- There was a discussion on the choice of the sequences and test conditions. There was a consensus that, while the JCTVC chairs comment that some SCC sequences are not challenging enough, it was felt that some relatively challenging and representative sequences should be identified for SCC tools experiments between the Torino meeting and the next meeting. JCTVC members are encouraged to submit more challenging SCC test sequences in the next meeting.

- There was a short discussion on the color problem when 4:4:4 contents are converted to 4:2:0. It is suggested that, while 4:2:0 contents are selected for common test conditions, the 4:4:4 content should also be made available on the ftp site for 4:4:4/4:2:0 experiments.

- A second BoG meeting was held from 6:50pm to 7:15pm on 21 July to reduce the number of selected SCC sequences from 7 to 4. It was attended by Huawei (Haoping Yu, Wen Gao), Microsoft (Jizheng Xu), FastVDO (Pankaj Topiwala), BJUT (Wenpeng Ding) and HKUST (Oscar Au).

- The following 4 sequences are identified:

(1) SlideShow_1280x720_20.yuv (1280x720, 20 fps, 500 frames, 420, 8 bit) – slide presentation playback

(2) BasketballDrillText_832x480_50.yuv (832x480, 50 fps, 500 frames, 420, 8bit) - camera view with text overlay

(3) ChinaSpeed_1024x768_30.yuv  (1024x768, 30 fps, 500 frames, 420, 8 bit) – video game

(4) SlideEditing_1280x720_30.yuv  (1280x720, 30fps, 300 frames, 420, 8 bit) – slide editing

- The proposed SCC Test conditions are

  - the prevailing HM4.x common test conditions (Class A/B/C/D/E sequences)

  - the HM4.x common test conditions extended to the 4 selected SCC sequences

  - for any lossless coding proposals, the anchor should be created by running the HM4.x with the PCM mode on,

  - in addition to reporting BD-BR and BD-PSNR, an addition objective measure for localized distortion (related to worst-case SNR) is to be reported. This measure is added because often localized distortion (e.g. on buttons or fonts with low contrast) can have adverse effect on the intelligibility of the screen content. Huawei has submitted an input document (JCTVC-F773) on one possible measure. The measure is to be confirmed within one month after the Torino meeting.

- Our recommendation is to

  - Adopt the above 4 selected sequences for SCC tool experiments, and establish a new class of content, Class F, for such SCC sequences

  - Adopt the proposed test conditions for SCC tool experiments

  - Set up an AHG to coordinate the investigation the SCC tools, and to study and finalize the additional quality measure for localized distortion.

  - Make the 4 selected SCC test sequences available on the ftp site within 1 week after this JCTVC meeting

  - Encourage submission of more challenging SCC content