| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F772 |
| --- | --- |

| Title: | Report on subjective viewing test for configurations in JCTVC-F320 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vivienne Sze Vittorio Baroncini | Email | sze@ti.com |
| Source: | Texas Instruments, FUB |  |  |

_____________________________

# Abstract

This contribution is a report on informal subjective viewing for the configuration in JCTVC-F320 that was held during the Torino meeting on July 20, 2011. The goal of informal subjective viewing test was to gather information on ALF & SAO. In total, four configurations were evaluated in the subjective test.

# Test procedure

Test subjects were asked to selected one of two sequences for eight tests (Table 1). Each test compared the anchor (ALF on/ SAO on) with a different configuration. A total of 16 sequences were shown one at a time (note: anchor was always shown first) and the test subjects.  The test subject did not know the order that the sequences were shown (i.e. blind test).

The test subjects were given score sheets and asked to were asked to select the sequence that they felt had the best visual quality (1 or 2); they were not allowed to choose no difference.   The test procedure was as follows. The test subjects were given instructions first. The anchor and modified configuration were shown one after another with a five-second break for voting after each test sequence.

Note: due to resource shortage at this meeting the number of test sequences were reduced from six (in F320) to two. The following two sequences have been used in the test.

Sequence 1: BasketBallDrill, low-delay high-efficiency, QP 37
Sequence 2: Vidyo3, low-delay high-efficiency, QP 37

The sequences were encoded according to common test conditions E700, except ALF and/or SAO were disabled.  In one case only the luma filter in ALF was disabled.

The display used for test was LG 42LH5000.

Table 1. Subjective Viewing Test

| Test # | Sequence | ALF | SAO | Y-PSNR | U-PSNR | V-PSNR | bit-rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | BasketBallDrill | OFF | OFF | 31.1564 | 35.1887 | 34.6072 | 424.6616 |
| 2 | BasketBallDrill | ON | OFF | 31.4605 | 35.1196 | 34.716 | 430.8112 |
| 3 | BasketBallDrill | LUMA OFF ONLY | ON | 31.3222 | 35.103 | 34.7225 | 430.924 |
| 4 | BasketBallDrill | OFF | ON | 31.328 | 35.1757 | 34.5753 | 426.9992 |
| 5 | Vidyo3 | OFF | OFF | 34.8913 | 43.1526 | 41.295 | 222.6152 |
| 6 | Vidyo3 | ON | OFF | 35.2861 | 43.1383 | 41.1841 | 224.3336 |
| 7 | Vidyo3 | LUMA OFF ONLY | ON | 35.1231 | 43.1595 | 41.2209 | 226.9184 |
| 8 | Vidyo3 | OFF | ON | 35.1253 | 43.2002 | 41.2898 | 222.8648 |

Anchors

| Sequences | Y-PSNR | U-PSNR | V-PSNR | bit-rate |
| --- | --- | --- | --- | --- |
| BasketballDrill | 31.6128 | 35.091 | 34.7424 | 433.0296 |
| vidyo3 | 35.5108 | 43.1237 | 41.1811 | 225.2288 |

# Summary of Results

| Sequence |  | Votes |  | Votes |
| --- | --- | --- | --- | --- |
| BasketballDrill | anchor | 17 | sao off/ alf off | 8 |
| BasketballDrill | anchor | 17 | sao off/ alf on | 8 |
| BasketballDrill | anchor | 11 | sao on/ alf luma off | 14 |
| BasketballDrill | anchor | 10 | sao on/ alf off | 15 |
| vidyo3 | anchor | 16 | sao off/ alf off | 9 |
| vidyo3* | anchor | 18 | sao off/ alf on | 6 |
| vidyo3 | anchor | 7 | sao on/ alf luma off | 18 |
| vidyo3 | anchor | 16 | sao on/ alf off | 9 |

* for this sequence one vote was discarded