| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F900 WG11 Number: m21452 |
| --- | --- |

| Title: | Common test conditions and software reference configurations |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 650 496 4742 bossen@docomoinnovations.com |
| Source: | JCT-VC |  |  |

_____________________________

# Abstract

This document defines common test conditions and software reference configurations to be used in the context of core experiments (CE) conducted between the 6th and the 7th JCT-VC meetings. These common test conditions are also recommended for use in technical contributions to the 7th JCT-VC meeting, if applicable.

# Introduction

Common test conditions are desirable to conduct experiments in a well-defined environment and ease the comparison of the outcome of experiments.

This document defines 8 test conditions, reflecting a combination of high efficiency and low complexity, and of intra-only, random-access, and low-delay settings:

- Intra, high efficiency

- Intra, low complexity

- Random access, high efficiency

- Random access, low complexity

- Low delay, high efficiency

- Low delay, low complexity

- Low delay, high efficiency, P slices only (optional)

- Low delay, low complexity, P slices only (optional)

A subset of these test conditions might be used for a particular experiment. For example, when testing an intra coding tool, only intra configurations might be used. Also, when testing variations of a tool such as adaptive loop filtering (ALF), low-complexity configurations might be skipped as this tool is not enabled therein.

Version 4.0 of the common software is expected to be used for most experiments. More recent versions are encouraged where applicable. Availability of the software will be announced on the JCT-VC email reflector by August 19, 2011.

A version 4.1 of the common software will be provided roughly 3 weeks after version 4.0. It will additionally include tools that do not affect common conditions.

The following sections define test sequences, quantization parameter values, encoder configuration files, and compile-time options to be used.

People bringing input contributions should provide a set of results as complete as possible that apply to the proposal. Results should be reported using the attached Excel sheets.

# Test sequences

Table 1 defines the set of test sequences to be used for intra, random-access, and low-delay conditions. All frames (as defined by frame count in the table) shall be encoded for all sequences and test cases (class F sequences being optional).

Test sequences are available on ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/ (please contact the JCT-VC chairs for login information).

Table 1 Test sequences

| Class | Sequence name | Frame count | Frame rate | Bit depth | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | Traffic | 150 | 30fps | 8 | X | X |  |
| A | PeopleOnStreet | 150 | 30fps | 8 | X | X |  |
| A | Nebuta | 300 | 60fps | 10 | X | X |  |
| A | SteamLocomotive | 300 | 60fps | 10 | X | X |  |
| B | Kimono | 240 | 24fps | 8 | X | X | X |
| B | ParkScene | 240 | 24fps | 8 | X | X | X |
| B | Cactus | 500 | 50fps | 8 | X | X | X |
| B | BQTerrace | 600 | 60fps | 8 | X | X | X |
| B | BasketballDrive | 500 | 50fps | 8 | X | X | X |
| C | RaceHorses | 300 | 30fps | 8 | X | X | X |
| C | BQMall | 600 | 60fps | 8 | X | X | X |
| C | PartyScene | 500 | 50fps | 8 | X | X | X |
| C | BasketballDrill | 500 | 50fps | 8 | X | X | X |
| D | RaceHorses | 300 | 30fps | 8 | X | X | X |
| D | BQSquare | 600 | 60fps | 8 | X | X | X |
| D | BlowingBubbles | 500 | 50fps | 8 | X | X | X |
| D | BasketballPass | 500 | 50fps | 8 | X | X | X |
| E | Vidyo1 | 600 | 60fps | 8 | X |  | X |
| E | Vidyo3 | 600 | 60fps | 8 | X |  | X |
| E | Vidyo4 | 600 | 60fps | 8 | X |  | X |
| F | BaskeballDrillText | 500 | 50fps | 8 | X | X | X |
| F | ChinaSpeed | 500 | 30fps | 8 | X | X | X |
| F | SlideEditing | 300 | 30fps | 8 | X | X | X |
| F | SlideShow | 500 | 20fps | 8 | X | X | X |

Note: When the encoder operates in 8-bit mode (InternalBitDepth=8) for a 10-bit source, each 10-bit source sample x is converted prior to encoding to an 8-bit value (x+2) / 4 clipped to the [0,255] range. Similarly when the encoder operates in 10-bit mode (InternalBitDepth=10) for an 8-bit source, each 8-bit source sample x is converted prior to encoding to a 10-bit value 4*x. This behavior is built into the reference encoder and no external conversion program is required.

# Quantization parameter values

For each video sequence four quantization parameter values are to be used: 22, 27, 32 and 37. These values define the QP values used for the I-frames in a sequence (configuration files further define QP values used for other frames).

# Configuration files

The following sections define encoder configuration files to be used for each test case. Parameters to be changed for each test point are:

- InputFile to reflect the location of the source video sequence on the test system

- FrameRate to reflect the frame rate of a given sequence as per Table 1

- SourceWidth to reflect the width of the source video sequence

- SourceHeight to reflect the height of the source video sequence

- FramesToBeEncoded to reflect the frame count of a given sequence as per Table 1

- IntraPeriod to reflect the intra refresh period in the random access test cases. The intra refresh period is dependent on the frame rate of the source: a value 16 shall be used for sequences with a frame rate equal to 20fps, 24 for 24fps, 32 for 30fps, 48 for 50fps, and 64 for 60fps.

- QP to reflect the quantization parameter values defined in section 3.

- InputBitDepth to reflect the bit depth of a given sequence as per Table 1

These configuration files are provided in the cfg/ folder of version 4.0 of the common software package. There are 8 configurations provided as follows:

- “All Intra – High efficiency” (AI-HE): encoder_intra.cfg

- “Random access – High efficiency” (RA-HE): encoder_randomaccess.cfg

- “Low-delay B – High efficiency” (LB-HE): encoder_lowdelay.cfg

- “Low-delay P – High efficiency” (LP-HE, optional): encoder_lowdelay_P.cfg

- “All Intra – Low complexity” (AI-LC): encoder_intra_loco.cfg

- “Random access – Low complexity” (RA-LC): encoder_randomaccess_loco.cfg

- “Low-delay B – Low complexity” (LB-LC): encoder_lowdelay_loco.cfg

- “Low-delay P – Low complexity” (LP-LC, optional): encoder_lowdelay_P_loco.cfg

Sequence-specific parameters are to be found in the cfg/per-sequence/ folder.

# Compile-time settings

Compile-time settings are defined mostly in the TypeDef.h file located in the source/Lib/TLibCommon folder of the common software. The default settings provided in the source code should be used.