| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22July, 2011 | Document: JCTVC-F902 |
| --- | --- |

| Title: | Core Experiment 2: Motion Partitioning and OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng1 (Primary Coordinator), I. S. Chong2 (Coordinator) Il-Koo Kim3 (Coordinator) Wen-Hsiao Peng4 Krit Panusopone5 | Tel: Email: | +86-755-28422009 zhengxiaozhen@huawei.com ichong@qualcomm.com ilkoo.kim@samsung.com pawn@mail.si2lab.org krit@motorola.com |
| Source: | Huawei & HiSilicon Technologies1, Qualcomm2, Samsung3 , NCTU / ITRI4, Motorola Mobility5 |  |  |

_____________________________

- Introduction

This Core Experiment (CE) is an ongoing work of previous CE2: Motion Partitioning and OBMC in Error: Reference source not found. Its goal is to further investigate block partitioning, overlapped block motion compensation and non-square TU issues.

Several related techniques have been considered in the 6th JCTVC meeting, with promising BD-rate performance, but with some remaining concerns regarding encoder and decoder complexity.

The techniques considered in this CE are:

- Block partitioning, proposed in [2];

- Overlap Block Motion Compensation (OBMC), proposed in [3], [4]

- Non-Square TU, proposed in [5], [6], [7], [8]

Further improvements are required to find the best solution of those three techniques with the optimal trade-off coding efficiency/encoding complexity.

# Participants List

| Participant | Person, Email address |
| --- | --- |
| Qualcomm | Liwei Guo, liweig@qualcomm.com Xianglin Wang, xianglin@qualcomm.com I. S. Chong, ichong@qualcomm.com |
| Huawei & HiSilicon | Xiaozhen Zheng, zhengxiaozhen@huawei.com Haoping Yu, haopingyu@huawei.com |
| Samsung | Il-Koo Kim, ilkoo.kim@samsung.com |
| Tsinghua | Yuan Yuan, y-yuan08@mails.tsinghua.edu.cn |
| Intel | Y.-J Chiu, Yi-jen.chiu@intel.com W. Zhang, Wenhao.zhang@intel.com |
| NCTU/ITRI | Wen-Hsiao Peng, pawn@mail.si2lab.org Chun-Chi Chen, cheerchen.cs98g@g2.nctu.edu.tw Chung-Lin Lee, yoyoli.cs97g@nctu.edu.tw |
| Motorola Mobility | Krit Panusopone, krit@motorola.com Limin Wang, limin.wang@motorola.com Xue Fang, X.Fang@motorola.com Jae Hoon Kim, jaekim@motorola.com Wei-Ying Kung, Vivian.Kung@motorola.com |
| MediaTek | Yu-Wen Huang, yuwen.huang@mediatek.com Chih-Wei Hsu, cw.hsu@mediatek.com Chih-Ming Fu, chihming.fu@mediatek.com Yi-Wen Chen, yiwen.chen@mediatek.com Zhi Zhou, zhi.zhou@mediatek.com |
| SONY | Kazushi SATO, Kazushi.Sato@jp.sony.com |
| BBC | Marta Mrak, Marta.Mrak@bbc.co.uk |
| Mitsubishi | Kazuo Sugimoto, Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp Akira Minezawa, Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Nokia | Oguz Bici, ext-oguz.bici@nokia.com |
| Broadcom | Peisong Chen, peisongc@broadcom.com Wade Wan, wwan@broadcom.com |
| Fraunhofer HHI | Benjamin Bross, benjamin.bross@hhi.fraunhofer.de Philipp Helle, philipp.helle@hhi.fraunhofer.de Simon Oudin, simon.oudin@hhi.fraunhofer.de |
| Panasonic | Toshiyasu Sugio, sugio.toshiyasu@jp.panasonic.com Viktor Wahadaniah, viktor.wahadaniah@sg.panasonic.com |
| LG | Seungwook Park, seungwook.park@lge.com Yongjoon Jeon, yongjoon.jeon@lge.com Joonyoung Park, jy.park@lge.com |

# Tools under Test

At the next meeting cycle, the following three categories will be evaluated at CE2:

- Black Partitioning;

- Overlapped Block Motion Compensation (OBMC)

- Non-Square TU

## Subtest A: Block Partitioning

3.1.1 Non-Rectangular Motion Partitioning (NRMP)

The related JCTVC contribution is:

- JCTVC-F415 [2], by Huawei & HiSilicon, Tsinghua, Technicolor

Non-rectangular motion partitioning (NRMP) was proposed in JCTVC-F415. NRMP enables to partition a square block into 2 regions with non-rectangular shape. Limited sets of diagonal-only partitions (45 degree or 135 degree partitioning) and non-diagonal partitions will be tested. Figure 1 and figure 2 illustrate two NRMP approaches.

Figure 1: Example of 6 diagonal partitioning modes

Figure 2: Example of set of 6 not-only diagonal partitioning modes

## Subtest B: Overlapped Block Motion Compensation (OBMC)

3.2.1 OBMC for 2NxN and Nx2N Motion Partitions

The related JCTVC contribution is:

- JCTVC-F299, by Qualcomm

In OBMC, predictions in the overlapped regions are derived by weighted averaging prediction pixels from two different motion vectors. The overlapped region in the luma component is defined as 2 rows (columns) of pixels on both sides of the horizontal (vertical) partition boundary. For chroma components, the overlapped region is reduced to 1 row (column) of pixels. The weighting factors are (3/4, 1/4) for the pixels which are 1 row (column) apart from the boundary and (7/8, 1/8) for pixels which are 2 row (column) apart from the boundary. In this core experiments, OBMC will be tested with AMP and/or with non-rectangular motion partition and/or with regular motion partitions (e.g., 2NxN and Nx2N). Additional test with 1-row OBMC region (to reduce memory bandwidth) will also be performed.

3.2.2 OBMC with Motion Merging

The related JCTVC contribution is:

- JCTVC-F049, by NCTU / ITRI

JCTVC-F049 introduced an OBMC with Motion Merging technique. Conceptually, it performs OBMC for a 2Nx2N prediction unit (PU) based on two sets of motion parameters, one (Set #1) is derived from motion merging with a reduced candidate set containing only Left, Above and Col merging candidates and the other (Set #2) can be signaled either by merging or by explicit coding. Relative to HM3.0 anchor, this technique achieved a Y-BD-rate saving of 1.5%, 1.3%, 1.8%, and 2.2% for RA-HE, RA-LC, LB-HE, and LB-LC, respectively. When combined with Non-Square Quadtree Transform (NSQT, JCTVC-F410), it provided a much higher rate saving ranging from 0.9% to 7.6%. The improvements came with a 20-30% increase in encoding time and a 3-4% increase in decoding time. Discussion in this meeting showed some concerns about the increased memory bandwidth on the decoder side and its encoding complexity. This subtest aims to further study its performance-complexity trade-offs, including performance versus ME search criterion, performance versus the number of hypotheses used, and etc., and to seek a way to harmonize it with the other techniques studied in this CE.

## Subtest C: Non-Square TU

3.3.1 Non-Square Quadtree Transform (NSQT)

The related JCTVC contribution is:

- JCTVC-F410 & F412, by Tsinghua, Huawei & HiSilicon, Microsoft, USTC, Microsoft

At RQT's second and third level, non-square TU is used when the PU partition type is 2NxN, Nx2N or asymmetric partition. Let's define 2M = 1 << log2TrafoSize. Then, for the case of 2NxN, 2NxnU and 2NxnD PU type, TU size at RQT's second and third level will be 2Mx0.5M; for the case of Nx2N, nLx2N and nRx2N PU type, TU size at RQT's second and third level will be 0.5Mx2M. For 2Nx2N and NxN PU types, square TU with the size of 2Mx2M or MxM is used. Meanwhile, 2x8 and 8x2 transform are excluded at NSQT design due to hardware complexity.

According to NSQT design, a block is divided into four non-square TU along with vertical direction if PU partition type is 2NxN, or four non-square TU along with horizontal direction if PU partition type is Nx2N. At RQT’s third level, a block is divided into four non-square TU along with horizontal and vertical direction. This method preserves the residual quadtree structure, and coding performance can be improved. Figure 3 illustrate this implementation scheme.

At the next meeting cycle, the efficiency and complexity of 2x8 and 8x2 transform, and improvement of  entropy coding related to non-square TU will be evaluated.

Figure 3: Illustration of TU split at RQT's second and third level (2NxN, 2NxnU and 2NxnD PU type)

3.3.2 Non-Square Transform for 2NxN and Nx2N Motion Partitions

The related JCTVC contribution is:

- JCTVC-F563 Non-Square Transform for 2NxN and Nx2N Motion Partitions, by Qualcomm

2NxN and Nx2N transforms are introduced such that transform size can match non-square PU size 2NxN and Nx2N. The non-square transform 2NxN (Nx2N) is integrated in the current RQT structure as additional transform  choices for 2NxN (Nx2N) PU. The introduced transforms reuse existing transform cores in HM software and does not add new contexts/tables for coefficients coding. One example of the integration of RQT with 2NxN/Nx2N transform is as follows: For a CU with Nx2N partition, a. level 0 2Nx2N transform; b. level 0 Nx2N transform; c. level 1 NxN transform; level 2 0.5Nx0.5N transform for right top subblock.

Figure 4: RQT for Nx2N PU (a) 2Nx2N transform (level 0), (b) Nx2N transform (level 0), (c) NxN transform (level 1); (d) N/2xN/2 transform (level 2) for the right-top subblock

3.3.3 RQT with rectangular transform unit

The related JCTVC contribution is:

- JCTVC-F578, by Motorola Mobility

Non-square TU was proposed in JCTVC-F578 [8]. The tool consists of forming a non-square 2NxnU, 2NxnD, nLx2N, nRx2N TU to match the shape of non-square PU. Simplified RQT structure is used to represent the new non-square TU. The new simplified RQT allows non-square TU at the root level and performs a joint split decision for all non-square TUs at the root level. Figures 5-6 show examples of the TU structures based on F578 and NSQT for Nx2N PU. In this experiment, the proposed simplified structure will be modified to support NSQT and AMP. The new structure will also preserve the flexibility of NSQT and F578 structures.

Figure 5. An example of tree partitioning based on F578 for Nx2N PU

Figure 6. An example of tree partitioning based on NSQT for Nx2N PU

# Experiments

## Tests description

The goal of the tests of CE2 is to identify the best combination and settings.

The reference anchors will be generated using the HM4.0 with the tests conditions as specified in JCTVC-F900 [9].

## The test will contain Software

The CE tools are going to be implemented into the HM4.0 branch software.

## Test Sequences, Bit Rates and Coding Conditions

The CE tools will be tested and evaluated under the common official configurations recommended by JCT-VC at the conclusion of the 6th JCT-VC meeting in document JCTVC-F900. In particular the reference anchors will be generated using the recommended configuration files provided by the JCT-VC software group.

Restrictions / modifications for this CE:

- No test with intra-only configuration

- High efficiency configuration for Random Access and LowDelay_B

- Low complexity configuration for Random Access and LowDelay B

- High efficiency configuration for LowDelay P (optional)

- Low complexity configuration for LowDelay P (optional)

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and encoding/decoding run time comparisons as defined in document JCTVC-F900.

Following comparisons will be made:

- Proposal against HM4.0

## Tools to be evaluated

| Subtest | Description | Related document(s) | Cross-checker(s) |
| --- | --- | --- | --- |
| A.1 | Non-Rectangular Motion Partitioning | JCTVC-F415 |  |
| B.1 | OBMC for 2NxN and Nx2N Motion Partitions | JCTVC-F299 |  |
| B.2 | OBMC with Motion Merging | JCTVC-F049 | Samsung |
| C.1 | Non-Square Quadtree Transform | JCTVC-F410 & F412 | Fraunhofer HHI |
| C.2 | Non-Square Transform for 2NxN and Nx2N Motion Partitions | JCTVC-F563 | Motorola Mobility |
| C.3 | RQT with rectangular transform unit | JCTVC-F578 | Broadcom, Samsung |

# Time-line

- 19 August 2011		Final CE-description

- 20 August 2011		Start of tool integration

- 9 September 2011	Completion of tool integration and optimization, start of testing

- 4 November 2011 	Report results to coordinators

- 8 November 2011   	Input Documents upload

# References

- X. Zheng, P. Bordes, P. Chen, I-K Kim, “Core Experiment 2: Motion Partitioning and OBMC,” Doc. JCTVC-E702, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March 2011

- X. Zheng (HiSilicon), H. Yu (Huawei), S. Li, Y. He (Tsinghua Univ.), P. Bordes (Technicolor), “CE2: Non-rectangular motion partitioning”, Doc. JCTVC-F415, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

- L. Guo, P. Chen, I. S. Chong, R. Joshi, X. Wang, M. Karczewicz (Qualcomm), “CE2:Overlapped Block Motion Compensation for 2NxN and Nx2N Motion Partitions”, Doc. JCTVC-F299, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

- C.-C. Chen, Y.-Y. Chen, C.-L. Lee, W.-H. Peng, H.-M. Hang (NCTU/ITRI) “CE2: Report of OBMC with Motion Merging”, Doc. JCTVC-F049, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

- Y. Yuan (Tsinghua), X. Zheng (HiSilicon), X. Peng (USTC), J. Xu (Microsoft), L. Liu (HiSilicon), Y. Wang, X. Cao (Tsinghua Univ.), C. Lai, J. Zheng (HiSilicon), Y. He (Tsinghua Univ.), H. Yu (Huawei), “CE2: Non-Square Quadtree Transform for symmetric motion partitions”, Doc. JCTVC-F410, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

- Y. Yuan (Tsinghua), X. Zheng (HiSilicon), X. Peng (USTC), J. Xu (Microsoft), I.-K Kim (Samsung), L. Liu (HiSilicon), Y. Wang, X. Cao (Tsinghua Univ.), C. Lai, J. Zheng (HiSilicon), Y. He (Tsinghua Univ.), H. Yu (Huawei), “CE2: Non-Square Quadtree Transform for symmetric and asymmetric motion partitions”, Doc. JCTVC-F412, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

- L. Guo, J. Sole, R. Joshi, P. Chen, X. Wang, M. Karczewicz (Qualcomm) “Non-Square Transform for 2NxN and Nx2N Motion Partitions”, Doc. JCTVC-F563, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

- K. Panusopone, X. Fang, V. Kung, L. Wang(Motorola Mobility), “RQT with rectangular transform unit support”, Doc. JCTVC-F578, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.