| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F904r3 |
| --- | --- |

| Title: | Description of Core Experiment 4: Quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | K.Sato (Sony) M. Budagavi (TI) M. Coban (Qualcomm) H.Aoki (NEC) X.Li (MediaTek) | Email: | Kazushi.Sato@jp.sony.com madhukar@ti.com mcoban@qualcomm.com hao@bu.jp.nec.com x.li@mediatek.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

It is agreed that CE4: Quantization activities be continued. Topics of this CE are

- QP coding

- De-quantization offset

- Quantization Matrices

Mandatory of this CE is to evaluate the rate-distortion performance and complexity of each of the related proposals as well as seeking for possibility of improvement by harmonization of technologies, etc.

This document provides detailed description on each of the experiments.

# Participants

| Participant | Person, Email address | P | C |
| --- | --- | --- | --- |
| Canon | M. Shima (shima.masato@canon.co.jp) | X | X |
| Ericsson | R. Sjöberg (rickard.sjoberg@ericsson.com) | X | X |
| HiSilicon | J.Zheng (zhengjianhua@huawei.com) X.Zheng (zhengxiaozhen@huawei.com) |  | X |
| HKUST | F. Zou (fengzou@ust.hk) |  | X |
| Huawei | M. Yang (yangmingyuan@huawei.com) |  | X |
| I2R | C. Yeo (chyeo@i2r.a-star.edu.sg) |  | X |
| MediaTek | X. Li (x.li@mediatek.com) S.Liu (Shan.Liu@mediatek.com) X.Zhang (ximin.zhang@ mediatek.com) | X | X |
| Microsoft | J. Xun (jzxu@microsoft.com) |  | X |
| Mitsubishi | K.Sugumoto (Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp) | X | X |
| Motorola Mobility | K. Panusopone (krit@motorola.com) A. Luthra (aluthra@motorola.com) | X | X |
| NEC | H. Aoki (hao@bu.jp.nec.com) K.Chono (chono@ct.jp.nec.com) | X | X |
| Nokia | J.Kang (ext-jewon.kang@nokia.com) |  | X |
| Qualcomm | M. Coban (mcoban@qualcomm.com) R. Joshi (rajanj@qualcomm.com) G. Van der Auwera(geertv@qualcomm.com) X.Wang (xianglin@qualcomm.com) | X | X |
| RIM | X. Yu (xiayu@rim.com) | X | X |
| Samsung | J. Chen (jianle.chen@samsung.com) T. Lee (tammy.lee@samsung.com) E. Alshina (elena_a.alshina@samsung.com) | X | X |
| Sharp | Y.Yasugi (yasugi.yukinobu@sharp.co.jp) |  | X |
| Sony | K.Sato (Kazushi.Sato@jp.sony.com) T.Suzuki (teruhikos@jp.sony.com) J.Xu (Jun.Xu@am.sony.com) | X | X |
| TI | M.Zhou (zhou@ti.com) | X |  |

# Tools under Test

## Subtest1: QP Coding

### Signaling Unit Granularity

Subtest 1.1: JCTVC-F577: QP adaptation at sub_CU level (Motorola)

This proposal studies trade-off between level of granularity in QP adaptation and overhead bits. The proposal also investigates QP coding schemes when QP adaptation at sub_CU level is allowed. In this experiment, 2 coding methods will be tested.

1.1.a) The first method follows the current method in HM4.0, except for signaling bits to allow QP adaptation at sub_CU level, to serve as anchor.

1.1.b) The second method utilizes differential coding with QP computed at CU level served as a predictor as described in JCTVC-F577. QP derivation at CU level follows the method used in HM4.0.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 1.1.a | Motorola Mobility: K. Panusopone (krit@motorola.com) | Ericsson: R.Sjöberg (rickard.sjoberg@ericsson.com) |
| 1.1.b | Motorola Mobility: K. Panusopone (krit@motorola.com) | NEC: K.Chono (chono@ct.jp.nec.com) |

### Entropy Coding and Related

Subtest 1.2.a: JCTVC-F174: Signaling of Max and Min QP in slice (Mitsubishi)

This scheme signals maximum and minimum QP values (MaxQP, MinQP) within a slice at the slice header. A modified indexing of delta QP according to MaxQP and MinQP is applied so that delta_cu_qp which has smaller absolute value is assigned to smaller index value. The index value is then entropy coded with a modified binarization table for CABAC or a modified VLC table for CAVLC. Truncated Unary binarization with maximum index equal to MaxQP-MinQP is used for CABAC. For CAVLC, Concatenated Truncated Golomb code is used.

Subtest 1.2.b: JCTVC-F492: A table-based delta QP coding method (Ericsson)

In this subtest, the compression efficiency when using a sparse QP distribution shall be tested. A modified version of the HM4 software with adaptive QP scaling shall be used following the common conditions. The modification shall consist of a categorization of CUs into activity categories for which a constant QP is assigned, replacing the current CE method that produces a continuous QP distribution. The algorithm for categorization and category QP assignment shall result in similar QP metrics compared to captured broadcast AVC bitstreams that uses sparse QP distributions. The QP statistics shall include the following metrics:

- A = Average QP span per slice. The QP span of a slice is the value of the maximum QP minus the minimum QP used in that particular slice. The average QP span per slice is the average of all QP spans in the bitstream.

- B = Average number of different QP values used per slice. The number of different QP values for each slice is counted. Then the mean of these values is calculated for the bitstream.

The QP distribution can be indicated by the average QP span per slice divided by the average number of different QP values used per slice (A/B). It shows how sparse/condense the QPs are distributed in the bitstream. An AVC bitstream with A/B above 1.5 is considered having a sparse QP distribution.

The experiment to conduct (1.2.b) is to compare the QP coding method described in JCTVC-F492 (test) against the agreed CE4 QP coding method (anchor). The anchor shall use the unmodified CE4 QP scaling method that results in continuous QP distributions. The test shall use the categorization and category QP assignment modification that results in sparse QP distributions.

Subtest 1.2.c: JCTVC-F495: Higher granularity of quantization parameter scaling and adaptive delta QP signaling (Samsung)

Following methods proposed in JCTVC-F495 will be tested in HM4.0 and CE4 anchor software.

- “Fine granularity of quantization step size” will be tested in HM4.0 with common test configurations. Various QP assignment of each level picture in hierarchical GOP structure will be tested to achieve better coding efficiency with the fine quantization step size.

- The high-level syntax element dQP_scale in Slice head syntax. Various values of dQP_scale will be tested on top of “fine granularity of quantization step size in CE4 anchor software” to show the tradeoff between better rate/quality allocation and dQP bits overhead.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 1.2.a | Mitsubishi: K.Sugimoto (Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp) | NEC: K.Chono (chono@ct.jp.nec.com) |
| 1.2.b | Ericsson: R. Sjöberg (rickard.sjoberg@ericsson.com) | NEC: K.Chono (chono@ct.jp.nec.com) |
| 1.2.c | Samsung: J. Chen (jianle.chen@samsung.com) T. Lee (tammy.lee@samsung.com) | Sony: K.Sato (Kazushi.Sato@jp.sony.com) |

### Spatial QP Prediction

Subtest 1.3.a: JCTVC-F422: Improvement of delta-QP Coding (Sony)

QP of the CUs with CBP=0 is not reliable for QP prediction. Therefore it is proposed to regard QP of such CU as “unavailable”. Combination with spatial and/or temporal QP prediction algorithms that performs best with coding efficiency in this CE is also tested.

Subtest 1.3.b: JCTVC-F159: CE4 Subtest 2: QP prediction based on intra prediction (test 2.3.g) (NEC, Canon)

The proposed method uses intra prediction direction for deriving predicted QP. Intra prediction directions are divided into two groups as shown in Figure 2. If the approximate intra prediction direction of the current PU is upward, the QP value of the above neighbouring QU is assigned to the predicted QP. If the approximate intra prediction direction of the current PU is leftward, the QP value of left neighbouring QU is assigned to the predicted QP.

Figure 3.1.4.1: QP prediction based on intra prediction.

Subtest 1.3.c: JCTVC-F661: CE4 Subtest 2: Spatial QP prediction: combination of test 2.3.g, 2.3.f and 2.3.e (NEC, Canon, Sony)

Brief algorithm of the proposed method is described as follows.

- If the top-left-most CU is intra-coded and its intra prediction mode is directional, the subtest 1.4.b method based on intra prediction is employed.

- Otherwise, if at least either of the above CU or the left CU have the same prediction mode as that of the current CU, predicted QP is derived from those CUs. (See 2.3.f of JCTVC-F300)

- Otherwise, if at least either of the above CU or the left CU is available, predicted QP is derived from those CUs. (See 2.3.e of JCTVC-F420)

Subtest 1.3.d: JCTVC-F332: CE4 Subtest 2: QP prediction from spatially neighboring CUs (test 2.3.b) (Qualcomm)

We propose to use left top most neighbor (L) CU’s QP as the predictor for coding of the current CU’s QP. If the left neighbor is not available then the top left most (T) neighboring CU’s QP is used as the predictor, as show in 3.1.4.2.

Figure 3.1.4.2: CU level QP assignment

Hence delta QP calculations becomes

dQPC0 = QPC0 – QPA5

dQPC1 = QPC1 – QPC0

dQPC5 = QPC5 – QPA12

dQPA0 = QPA0 – QPB8

…………

If left CU is not available then the top CU’s QP is used as a predictor. If the neighboring predictor CU is a skipped CU, then the QP value of the predictor is the QP value of its predictor QP. Basically skip CU’s QP is assumed to of its predictor QP. Top QP predictor gets used at the beginning of each LCU row.

Subtest 1.3.e: JCTVC-F705: CE4 Subtest 2: Spatial QP prediction: combination of test 2.3.g with 2.3.b/2.3.c (Qualcomm)

Intra coded blocks use JCTVC-F159 method for QP prediction. Inter coded CUs use the left CU’s QP as the predictor if left CU is inter coded or both left and above CUs are intra coded, otherwise, above CU’s QP is used as the predictor. If left CU QP is not available, above CU’s QP is used.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 1.3.a | Sony: J.Xu (Jun.Xu@am.sony.com) K.Sato (Kazushi.Sato@jp.sony.com) | HiSilicon: X. Zheng (zhengxiaozhen@huawei.com) |
| 1.3.b | NEC: H. Aoki (hao@bu.jp.nec.com) Canon: M. Shima (shima.masato@canon.co.jp) | Mitsubishi: K.Sugimoto (Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp) |
| 1.3.c | NEC: H. Aoki (hao@bu.jp.nec.com) Canon: M. Shima (shima.masato@canon.co.jp) Sony: K. Sato (Kazushi.Sato@jp.sony.com) | Sharp: Y.Yasugi (yasugi.yukinobu@sharp.co.jp) Qualcomm: M. Coban (mcoban@qualcomm.com) |
| 1.3.d | Qualcomm: M. Coban (mcoban@qualcomm.com) | Motorola Mobility: K. Panusopone (krit@motorola.com) |
| 1.3.e | Qualcomm: M. Coban (mcoban@qualcomm.com) | I2R: C. Yeo (chyeo@i2r.a-star.edu.sg) |

### Temporal QP Prediction

Subtest 1.4: JCTVC-F103 + JCTVC-F499: QP prediction based on intra/inter prediction with QP buffer compression  (NEC, Qualcomm)

For inter CU, predicted QP is derived from QPs of a temporally neighboring CU directed by the motion vector as shown in Figure 3.1.5.1. Let Q(F, x) be the luma quantization parameter of the QU (quantization group of coding units) covering a luma location x of the frame F, QSlice(F, x) be the value of SliceQPY of the slice covering a luma location x of the frame F, FCurr be the current frame, FRef, be the reference frame, xTL be the location of the top-left luma sample of the current QU, and xRef be the location of the center luma sample of the reference block employed for motion compensation of the top-left PU of the current QU. The predicted QP value QPred of the current QU is derived as follows.

QPred = Q’(FRef, xRef) + QSlice(FCurr, xTL) – QSlice(FRef, xRef)

Q’(FRef, xRef) = average of Q(FRef, x) within the QP storage unit covering the luma location xRef

The offset term by slice QPs accounts for the inter-frame QP adaptation for rate control, quality control, etc. Temporal QP storage unit size can be signaled in the PPS header or set to a prefixed value.

Figure 3.1.5.1: QP prediction based on inter prediction.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 1.4 | NEC: H. Aoki (hao@bu.jp.nec.com) Qualcomm: M. Coban (mcoban@qualcomm.com) | Mitsubishi: K.Sugimoto (Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp) |

## Subtest 2: De-quantization Offset

Subtest 2.1: JCTVC-F119: Adaptive de-quantization offset (MediaTek)

In JCTVC-F119, an adaptive de-quantization offset was proposed. The basic idea is to adjust de-quantization offset for Y, U and V components at slice level based on the statistics of previously coded pictures. The range of normalized de-quantization offset for inter pictures is between 0 and 100% while the range for intra pictures is between -50% and 50%.

Subtest 2.2: JCTVC-F276: Adaptive reconstruction level (RIM)

ARL design shows three major features. The first is to compute the empirical centroid for reconstruction levels.  For a coded frame, the optimal reconstruction levels to minimize the quantization distortion can be computed as follows:

δi=1Nin=1Nic(i)n−i⋅q.   --- (1)

where c(i)ndenotes the n-th coefficient that is quantized to i, and Ni is the total number of coefficients that are quantized to i.

The second feature is to adaptively determine the number of offsets for representing the reconstruction set. In this contribution, a threshold Ti is set to test Ni. Only offsets corresponding to Ni >Ti  will be transmitted, while for all partitions corresponding to Ni <=Ti , an average is calculated and used for reconstruction, as

δ=Ni≤Tiδi⋅NiNi≤TiNi.    --- (2)

The third feature is to utilize the cross-frame correlation. The empirical centroids for one frame minimize the distortion for the current frame, yet it involves in an iterative/multi-pass process. This issue is addressed in ARL by utilizing the cross-frame correlation. Specifically, empirical centroids collected from one frame are transmitted and used for another frame.

Subtest 2.3: JCTVC-F610: Fine granularity QP offset (Qualcomm)

In this proposal, a method for sending offset values to modify the quantizer step size at the slice level is proposed. The step-size change is signaled as a percentage of the original quantizer step-size, resulting in much higher granularity compared to what could be achieved through a QP change. It is reported that the BD-rate for RA-HE and LB-HE configurations is -0.5% and -0.9%, respectively, and the BD-rate for RA-LC and LB-LC configurations is -1.1% and -1.8%, respectively.

Subtest 2.4: Combination of  JCTVC-F119, JCTVC-F276 and JCTVC-F610 (MediaTek,RIM,Qualcomm)

Combinations of JCTVC-F119, JCTVC-F276 and JCTVC-F610 such as different offset numbers versus different offset types are to be tested.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 2.1.a | MediaTek: X. Li (x.li@mediatek.com) | I2R: C. Yeo (chyeo@i2r.a-star.edu.sg) |
| 2.1.b |  | Huawei: M. Yang (yangmingyuan@huawei.com) |
| 2.1.c |  | HKUST: F. Zou (fengzou@ust.hk) |
| 2.1.d |  | RIM: X. Yu (xiayu@rim.com) |
| 2.2.a | RIM: X. Yu (xiayu@rim.com) | MediaTek: X. Li (x.li@mediatek.com) Microsoft: J. Xu (jzxu@microsoft.com) |
| 2.2.b |  | Huawei: M. Yang (yangmingyuan@huawei.com) Microsoft: J. Xu (jzxu@microsoft.com) |
| 2.2.c |  | Samsung: E. Alshina (elena_a.alshina@samsung.com) Microsoft: J. Xu (jzxu@microsoft.com) |
| 2.2.d |  | HKUST: F. Zou (fengzou@ust.hk) Microsoft: J. Xu (jzxu@microsoft.com) |
| 2.3.a | Qualcomm: R. Joshi (rajanj@qualcomm.com) G. Van der Auwera(geertv@qualcomm.com) X.Wang (xianglin@qualcomm.com) | MediaTek: X. Li (x.li@mediatek.com) |
| 2.3.b |  | Nokia: J.Kang (ext-jewon.kang@nokia.com) |
| 2.3.c |  | RIM: X. Yu (xiayu@rim.com) |
| 2.3.d |  | I2R: C. Yeo (chyeo@i2r.a-star.edu.sg) |

## Subtest 3: Quantization Matrices

Subtest 3.1: JCTVC-F362: Proposal to support quantization matrix in HEVC (Sony &TI)

An extended AVC quantization matrix method is proposed. It consists of the followings.

- Encoding of matrices in SPS and PPS

- Support 4x4, 8x8, 16x16 and 32x32 matrices

- DPCM + signed exp-Golomb code

- AVC inverse quantization process

The designing of the default matrix, especially for 16x16 and 32x32, are also evaluated. The performance of the default matrix defined in the attached Excel sheet is compared with flat matrix. Both subjective quality and BD-rate performance will be evaluated.

Subtest 3.2: JCTVC-F085: Further study on compact representation of quantization matrices (TI)

In JCTVC-F085, a compact representation of quantization matrices was proposed. The proposed algorithm contains four tools, 1) matrix down-sampling, 2) 135 degree symmetry processing, 3) 45 degree symmetry processing and 4) unsigned exp-golomb coding. Those four tools can be turned on/off with high-level control parameters and combined arbitrarily. Test results revealed that the algorithm provided up to 13-15x more compression when compared to the AVC quantization matrix compression method if all the compression tools are enabled. Compression methods to improve coding efficiency of quantization matrices proposed in F085 are tested.

Subtest 3.3: JCTVC-F475: Enhancement of quantization matrix coding for HEVC (Sony)

Since data size of quantization matrix, especially for large CU, is large and efficient encoding method to support quantization matrix is desired. Compression methods to improve coding efficiency of quantization matrices proposed in F475 are tested.

Assignment

| Test | Proponent (Contact Person) | Cross-Checker (Contact Person) |
| --- | --- | --- |
| 3.1 | TI: M.Zhou (zhou@ti.com) Sony: T.Suzuki (teruhikos@jp.sony.com) | Canon: M. Shima (shima.masato@canon.co.jp) HiSilicon: J. Zheng (zhengjianhua@huawei.com) |
| 3.2 | TI: M.Zhou (zhou@ti.com) | Canon: M. Shima (shima.masato@canon.co.jp) MediaTek: S.Liu (Shan.Liu@mediatek.com) HiSilicon: J. Zheng (zhengjianhua@huawei.com) |
| 3.3 | Sony: T.Suzuki (teruhikos@jp.sony.com) | Canon: M. Shima (shima.masato@canon.co.jp) MediaTek: X.Zhang (ximin.zhang@ mediatek.com) HiSilicon: J. Zheng (zhengjianhua@huawei.com) |

# Core Experiment Tasks

The following tasks shall be performed in this CE.

- The proposed methods are implemented on top of HM-4.0 and distributed to CE participants via e-mail exchange

- Improve the tools if possible by harmonizing the proposed methods, etc

- Conduct Test & Cross-Checking

- Report the results

# Time-line and Responsibilities

[Subtest 1]

T1:2011-Aug-19:	Final CE description and test condition definition

T2:2011-Aug-26:	HM-4.0 available

T3a:2011-Sep-02:	Release anchor result and patch for counting DQP code

T3b: 2011-Sep-23: 	Completion of tool integration, test configuration announcement

Start of cross-check

T4: 2011-Nov-01: 	Report cross-checkers’ results to coordinators

T5: 2011-Nov-08: 	Input document upload

T6: 2011-Nov-17: 	Summary report uploaded

[Subtest 2]

T1: 2011-Aug-19:	Final CE description and test condition definition

T2:	2011-Aug-26:	HM-4.0 available

T3a: 2011-Sep-14: 	Completion of EEM integration.

T3b: 2011-Oct-09: 	Completion of tool integration, test configuration announcement.

Start of cross-check

T4: 2011-Nov-01: 	Report cross-checkers’ results to coordinators

T5: 2011-Nov-08: 	Input document upload

T6: 2011-Nov-17: 	Summary report uploaded

[Subtest 3]

T1:2011-Aug-19:	Final CE description and test condition definition

T2:2011-Aug-26:	HM-4.0 available

T3: 2011-Sep-16: 	Completion of tool integration, test configuration announcement

Start of cross-check

T4: 2011-Nov-01: 	Report cross-checkers’ results to coordinators

T5: 2011-Nov-08: 	Input document upload

T6: 2011-Nov-17: 	Summary report uploaded

# Software Configuration and Evaluation Criteria

## Subtest 1: QP coding

QP modulation algorithm is needed for evaluation of each subtest.

Participants agreed that TM-5 Step3, which has already been agreed to be implemented on HM-4.0, is realistic enough as evaluation criteria. Current software applies modulation of QP by +-6 but by +-12 will be better to see the differences of coding efficiency with each of the proposals.

Note that this is not to enhance BD-performance but to control subjective quality. TM-5 Step3 may not be the best technique but most adaptive quantization algorithms used in commercial products apply more or less similar techniques, combining with other advanced methods like facial recognition, etc.

Different configuration from HEVC common test condition for HM-4.0 will be needed for CE4 subtest 1 x-checking experiment of the anchor will be done by the tester1 [NEC: H. Aoki (hao@bu.jp.nec.com)] and the tester 2 [Sony: K.Sato (Kazushi.Sato@jp.sony.com)] with the timeline described in section 5 of this document.

In addition test with QP map taken from a AVC encoder in the real world as seen in JCTVC-F492 is encouraged.

New CE on Quantization inherits the following evaluation criteria from the previous CE:

- BD performance including dQP bit increase/saving

- Enc/Dec time as complexity measure

- MaxCUDepthDQP=3 (8x8) as mandatory and 2 (16x16), 1(32x32), and 0(64x64) as option.

Additionally the proponent of Subtest 1.5 will provide data on amount of storage as measure of implementation cost.

## Subtest 2: De-quantization Offset

The proposals and combinations shall be tested in the following four cases.

- Performance compared to HM4

- HM4 shall be used as the anchor

- Experiments shall be conducted according to the common test conditions defined in JCTVC-E900

- Lambda refinement is not allowed.

- Performance compared to HM4 when bitrate is constrained at temporal level

- HM4 shall be used as the anchor

- Experiments shall be conducted according to the common test conditions defined in JCTVC-E900

- Proposed encoders shall be adjusted so that the average bitrate of each temporal level is as close as possible to that of the anchor for each sequence. The maximal bitrate mismatch of each temporal level shall be within 2%. Quantization of intra picture shall not be changed. Lambda tuning is allowed for the purpose of obtaining the target bitrate of each temporal level.

- Performance compared to QP scaling

- HM4 with adaptive QP scaling shall be used as the anchor. The adaptive QP scaling shall be obtained by rounding the proposed QP fractional offset (or reconstruction level) to the nearest integer QP value.

- Experiments shall be conducted according to the common test conditions defined in JCTVC-E900

- Performance compared to adaptive rounding/EEM (JVT-N011) when RDOQ off

- HM4 with RDOQ off and adaptive rounding/EEM (JVT-N011) on shall be used as the anchor.

- Experiments shall be conducted according to the common test conditions defined in JCTVC-E900 except that RDOQ shall be disabled and adaptive rounding/EEM shall be enabled.

## Subtest 3: Quantization Matrices

### Matrices to be tested

The matrices to test are included in the attached Excel sheet “JCTVC-F904_subtest3_Qmatrix.xlsx”. “Default” is the matrix coefficients for the default quantization matrix. The matrices “Symmetry1” and “Asymmetry1” are used to evaluate the coding performance of the proposed methods (3.1, 3.2 and 3.3) in the subtest 3.

### Evaluation of CE Results

Both coding performance and complexity are tested. The following tests will be conducted to evaluate coding performance.

Test 1: Results of compression performance of matrix itself (matrix only)

Test 2: Its impact on overall HM performance

In case of the test 1, the coding efficiency of matrix only is investigated. The proposed method is compared with AVC method. In case of lossy coding, coding error of matrix coefficients is also evaluated.

In the test 2, QMs are changed frame by frame, and are encoded at each frame. The matrices at each frame are generated by the software provided by the proponent. Then the impact on the overall performance can be studied. In this subtest, the ratio of the generated bits for matrix coefficients and the other remaining bits are measured. BD rate/PSNR are measured compared with AVC quantization method (Subtest 1: F362). In case of subtest 1, BD rate/PSNR are measured compared with flat matrix.

Complexity assessment is performed on the base of encoding / decoding complexity measurement.

For complexity assessment, complexity of each proposals will be compared. In addition, the impact on HM 4.0 will be evaluated. HM 4.0 software with the anchor configuration and the proposal implemented on the HM 4.0 software shall be used. For decoder complexity measurements, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

### Evaluation Criteria

- Compression ratio and coding error of quantization matrix coding

- Complexity (encoding / decoding time measure, memory storage, etc.)

Note 1: HM 4.0 shall be used as the anchor with AVC quantization matrix method.

# References

[1] “Description of Core Experiment 4 (CE4): Quantization”, JCTVC-E704, Mar 2011

[2] “CE4: Summary report of Core Experiment on quantization”, JCTVC-F024, Jul 2011