| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F905 |
| --- | --- |

| Title: | CAVLC entropy coding improvements |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Xianglin Wang Ping Wu Chanyul Kim | Tel: Email: | 1-858-651-5135 xianglin@qualcomm.com ping.wu@zte.com.cn dionism@samsung.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Participants

| Proponent/Partner | Contact | Email |
| --- | --- | --- |
| Sharp | Yukinobu Yasugi Tomoyuki Yamamoto | yasugi.yukinobu@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp |
| ZTE | Ping Wu Ming Li Wen Zhang | pwuzte@gmail.com li.ming42@zte.com.cn zhang.wen4@zte.com.cn |
| Panasonic | Virginie Drugeon | Virginie.Drugeon@eu.panasonic.com |
| Yonsei Univ. | Seongwan Kim Jaeho Lee Sangyoun Lee | knauer@yonsei.ac.kr jhlee82@yonsei.ac.kr syleee@yonsei.ac.kr |
| LG Electronics | Jaehyun Lim Jie Jia | jaehyun.lim@lge.com jie.jia@lge.com |
| Qualcomm | Liwei Guo Xianglin Wang Marta Karczewicz | liweig@qualcomm.com xianglin@qualcomm.com martak@qualcomm.com |
| Cisco | Arild Fuldseth | arilfuld@cisco.com |
| Nokia | Kemal Ugur Jani Lainema Antti Hallapuro | kemal.ugur@nokia.com jani.lainema@nokia.com antti.hallapuro@nokia.com |
| Microsoft | Jizheng Xu Bin Li | jzxu@microsoft.com v-lbi@microsoft.com |
| MediaTek | Yu-Wen Huang Shan Liu Ximin Zhang Zhi Zhou | yuwen.huang@mediatek.com Shan.Liu@mediatek.com ximin.zhang@mediatek.com zhi.zhou@mediatek.com |
| HHI | Tung Nguyen | tung.nguyen@hhi.fraunhofer.de |
| TI | Vivienne Sze | sze@ti.com |
| Zhejiang Univ | Xingguo Zhu | 05xxgcsgx@zju.edu.cn |
| NTT Docomo | TK Tan | ttk@pacific.net.sg |
| Sony | Munsi Haque Jun Xu | munsi.haque@am.sony.com Jun.Xu@am.sony.com |
| Samsung | Jianle Chen Ankur Saxena | jianle.chen@samsung.com asaxena@sta.samsung.com |
| NEC | Keiichi Chono | chono@ct.jp.nec.com |

# Related proposals

# Coefficient coding

In JCTVC-F458, it is proposed that intra chroma coefficients are coded using intra-block run-level index mapping instead of inter-block run-level index mapping. Currently in HM3.0 all chroma coefficients are coded using inter-block run-level index mapping. It is reported by doing so coding efficiency can be improved.

In JCTVC-F612, to improve coding efficiency of large transform block, it is proposed to send flags indicating if coefficients in a 4x4 sub-block are all zeros. Coding efficiency improvement is reported with the proposed scheme.

In JCTVC-F408, threshold for table switch in level mode coding is made dependent on factors such as block type (intra luma, P luma, B luma, or chroma) as well as RQT depth.

# VLC table modification

In JCTVC-F408, VLC tables are re-organized and accessed through certain parameters. It is reported that such a scheme can get rid of some tables saving VLC table numbers.

In JCTVC-F286, several truncated unary code tables (with different sizes) are introduced to remove the redundancy in run-mode coding.

In JCTVC-F466, a scheme is proposed to handle some exception cases with code-word larger than 32 bit in CAVLC which leads to an error, by appending fixed-length codeword above certain code number for a VLC table. It is asserted that it does not affect coding performance, but can cover the maximum level of quantized coefficients in CAVLC.

In JCTVC-F608, VLC table change is also proposed to address a case with codeword longer than 32bits by concatenating Golomb codes with two different parameters k.

# Codeword adaptation

In JCTVC-F395, difference counter was proposed to be used for CAVLC codeword mapping adaptation. In the proposed method, the difference of the occurrences of two successive code numbers is stored in a difference counter. Decoding table is swapped when the value of the associated counter equals to zero. It is asserted that the proposed scheme can simplify CAVLC adaptation by removing normalization process and sum counters.

# Core experiment test conditions

- Software

Use HM 4.0, which will be available approximately four weeks after Torino meeting, as base software.

- Coding conditions

The same as that specified in JCTVC-F900 using low complexity settings.

- Test set

- The same as that specified in JCTVC-F900.

- Evaluation criteria

Report coding efficiency improvement relative to reference software, evaluate coding complexity.

- Cross check

| Proposals | Cross-checkers |
| --- | --- |
| JCTVC-F408(Yonsei, Samsung) | Qualcomm |
| JCTVC-F395(Sharp) | Microsoft |
| JCTVC-F612(Qualcomm) | Sharp |
| JCTVC-F608(Qualcomm) | Yonsei, Samsung |
| JCTVC-F458(Samsung) | Sony |
| JCTVC-F286(Sony) | Samsung |
| JCTVC-F466(Samsung) | Qualcomm |

# Timelines

Aug. 20th, 2011: Deadline for sending email to coordinator expressing interest in participating in core experiment.

Aug. 20th, 2011: Final CE description.

Sep. 10th, 2011: Software available for cross-verification.

Nov. 8th, 2011: Report the verification results.

# References

- S. Kim, J. Lee, S. Lee (Yonsei Univ.), J. Chen, J. Park (Samsung) “CE5: Run and level mode coding improvement in CAVLC”, JCTVC-F408, Torino, Italy, Jul. 2011.

- T. Yamamoto (Sharp), “CAVLC Adaptation using difference counter”, JCTVC-F395, Torino, Italy, Jul. 2011.

- M. Karczewicz, Y. Zheng, L. Guo, X. Wang (Qualcomm) “Modifications to intra blocks coefficient coding with VLC”, JCTVC-F612, Torino, Italy, Jul. 2011.

- C. Kim, Y. Park (Samsung), “Improvement of CAVLC run- coding by prediction mode”, JCTVC-F458, Torino, Italy, Jul. 2011.

- J. Xu, A. Tabatabai (Sony), “Redundancy removal for Run-mode in CAVLC”, JCTVC-F286,  Torino, Italy, Jul. 2011.

- C. Kim, Y. Park(Samsung), “Handling for exception cases regarding to code-word larger than 32bit in CAVLC”, JCTVC-F466,  Torino, Italy, Jul. 2011.

- W. Zhang(ZTE), “Joint coding of CU splitting flag and inter modes based on HM 3.1”, JCTVC-F418,  Torino, Italy, Jul. 2011.