| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F906_r2 |
| --- | --- |

| Title: | CE6: Intra Coding Improvements |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Ali Tabatabai (Primary) Edouard Francois Keiichi Chono Haoping Yu Joshi, Rajan Jani Lainema | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | ali.tabatabai@am.sony.com edouard.francois@canon.crf.fr chono@ct.jp.nec.com haoping.yu@huawei.com rajanj@qualcomm.com jani.lainema@nokia.com |

_____________________________

# Introduction

This output document provides the description of Core Experiments 6 on “Intra prediction and improvements”. Proposals on intra coding and intra prediction are divided in several subsets of this CE:

- Subset CE6a:	Intra Chroma Prediction

- Subset CE6b:	Intra Mode Coding

- Subset CE6c:	Short Distance Intra Prediction (SDIP)

- Subset CE6ed:	Intra Prediction with secondary boundary

# Experimental Conditions

## Software

Tools under test in this CE will be implemented on top of HM 4.0 software that will be released by the software Ad-Hoc group after integrating changes adopted at the 6th JCT-VC meeting in Torino.

## Common Test Conditions

The common test conditions for all-Intra configuration as defined in the document JCTVC-F900 released by the software Ad-Hoc group will be used as anchor configuration for tests in all subsets. The participants have to consider either Intra High Efficiency or Intra Low Complexity configuration or both.

## Evaluation of CE Results

Results of this CE will be evaluated on the basis of BDR metric. In addition, subjective evaluation to support the advantage of the proposed tools is highly desirable.

## Evaluation of Complexity

For the complexity measurement, the anchor and the reference software with the tools implemented will be executed on the same machine and the computational time will be measured for each configuration. Then, a time factor is calculated which compares the reference software including the sub-CE test tool and the reference software without the tool.

# Timelines

| T1 | August 19, 2011 | HM 4.0 software available |
| --- | --- | --- |
| T2 | Torino + 4Wks | Deadline for sending email to coordinator expressing interest to participate in core experiment. |
| T3 | September 09, 2011 | Codes to be released by proponents |
| T4 | November 07, 2011 | Cross-verification completed CE. |
| T5 | November 21, 2011 | Report the verification results uploading. |

# Description of Tool Experiment

## CE6a: Intra Chroma Prediction

### Modified down-sampling filters for LM mode of chroma intra prediction (JCTVC-F173, JCTVC-F502)

JCTVC-F173 and JCTVC-F502 proposed the modified luma down-sampling filters to improve LM mode of chroma intra prediction. In LM mode of HM 3.0, the correlation parameters α and β are derived based on the down-sampled reconstructed luma pixels and the reconstructed chroma pixels. The purpose of this CE subset is to test the benefit of modified luma down-sampling filters in term of coding performance and complexity.

There are three test points of this sub-test.

- Modified down-sampling horizontal filters for the top neighboring luma line

- Modified down-sampling vertical filters for the left neighboring luma line

- Modified down-sampling filters for the prediction value of the inner pixels of the luma block

#### An improvement to chroma intra prediction from luma (JCTVC-F173)

In this scheme, horizontal down-sampling filter for luma samples is applied for chroma intra prediction. For the luma samples to calculate α and β, horizontal 1:0:1 filter is applied. For the internal luma samples to generate chroma prediction, horizontal 1:2:1 filter is applied.

Figure 1.  The position of samples used for calculation of the parameters and 

#### Cross-channel techniques to improve intra chroma prediction (JCTVC-F502)

This contribution reports two techniques to improve intra chroma prediction: a 2-dimensional 3x2 filter for LM intra chroma prediction mode, and a new intra chroma prediction mode based on the technique of local adaptive prediction selection (LAPS) process to produce the intra chroma prediction sample from the comparison of the reconstructed luma sample of target location to the local reconstructed luma samples in the neighborhood.

In the sub-test of downsampling filter, the filter set of 1:2:1, 1:0:1, 0:1:0 will be tested on the horizontal filters for the top neighboring luma line, the vertical filters for the left neighboring luma line and the filter for the prediction value of the inner pixels of the luma block.

#### Test plan

The test plan is to compare the coding performance of the above two test points and the combination of the test points to the common anchor of HM 4.0.

| CE6b: Modified down-sampling filters for LM mode of chroma intra prediction |  |  |  |
| --- | --- | --- | --- |
| Contribution | Tools | Proponent | Cross-check |
| JCTVC-F173 | improvement to chroma intra prediction from luma | Mitsubishi minezawa.akira@ds.mitsubishielectric.co.jp sugimoto.kazuo@ak.mitsubishielectric.co.jp | Intel yi-jen.chiu@intel.com ITRI shcho@etri.re.kr |
| JCTVC-F502 | Cross-channel techniques to improve intra chroma prediction | Intel yi-jen.chiu@intel.com lidong.xu@intel.com yu.han@intel.com | Mitsubishi minezawa.akira@ds.mitsubishielectric.co.jp ITRI shcho@etri.re.kr |

#### Reference

- Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi), “An improvement to chroma intra prediction from luma”, JCTVC-F173, Torino, Italy, July 2011

- Y.-J. Chiu, Y. Han, L. Xu, W. Zhang, H. Jiang (Intel), “Cross-channel techniques to improve intra chroma prediction”, JCTVC-F502, Torino, Italy, July 2011

### Complexity Reduction of LM mode of chroma intra prediction (JCTVC-F233, JCTVC-F431)

#### Sub-sampling portion of neighboring pixels (JCTVC-F233&F431)

F233+F431 proposes that reference luma and chroma samples are subsampled before use in calculation of alpha and beta. It is reported that the number of multiplications is reduced.

#### An improvement to chroma intra prediction from luma (JCTVC-F431)

In the current HEVC specification, the order of transmission is coefY followed by coefU and coefV in the unit of CU. The max size of CU is 128x128 for luma and 64x64 for re-sampled luma. These re-sampled pixels need to be stored only for LM_CHROMA and cannot be used for other processes such as deblocking. This additional buffer is one of the concerns for implementation.

In this test, methods to alleviate this bottleneck such as storing 8 bit samples instead of 10 bit will be studied.

#### Test plan

The test plan is to compare the coding performance of the above two test points and the combination of the test points to the common anchor of HM 4.0.

| CE6b: Complexity Reduction of LM mode of chroma intra prediction |  |  |  |
| --- | --- | --- | --- |
| Contribution | Tools | Proponent | Cross-check |
| JCTVC-F233 JCTVC-F431 | Sub-sampling portion of neighboring pixels | TI & Sony sze@ti.com, kazushi.sato@jp.sony.com | Panasonic sasai.hisao@jp.panasonic.com |
| JCTVC-F431 | Improvement to chroma intra prediction from luma | Sony kazushi.sato@jp.sony.com | Qualcomm rajanj@qualcomm.com Panasonic sasai.hisao@jp.panasonic.com |

## CE6b: Intra Mode Coding

The goal of this experiment is to enhance Intra mode coding of the HEVC Test Model. This is not limited to improving coding efficiency, but also relates to harmonization and complexity reduction. Two main technical topics are addressed:

- Most Probable Modes (MPM) derivation

- Remaining mode coding

The following proposals are considered in this experiment.

### MPM derivation related proposals

In this first set of proposals, the derivation of MPMs is modified compared to the HM design.

#### Luma Intra Prediction Mode Coding (JCTVC-F062)

In this test, additional most probable modes (MPMs) are added to improve coding efficiency. The additional MPMs are generated according to neighboring modes. The redundant bit of remaining mode coding might also be removed for higher efficiency.

#### Parsing friendly intra mode coding (JCTVC-F459)

In JCTVC-F459, a total set of four MPMs is derived. First, two MPMs are derived as in HM4.0. The additional MPMs are derived depending on the value of the two first MPMs. If the above and the left neighboring blocks have the same intra prediction mode, two closest available intra modes of the left block intra mode are inserted to complete the MPMs set. Otherwise, if the above and left neighboring blocks have different intra prediction mode, Planar mode, plus DC mode or one closest available intra mode are added.

#### Differential Coding of Intra Modes (DCIM) (JCTVC-F566)

JCTVC-F566 proposes DCIM (Differential Coding of Intra Modes) which utilizes neighboring reconstructed pixels to derive dominant edge and differentially signal the direction used for prediction. The performance of DCIM on the latest reference software i.e., HM 4.0 will be evaluated. Taking account for the recent adoptions and discussions, harmonization with SDIP as well as harmonization with MPM improvement will be tested.

### Remaining mode coding related proposals

This second set of proposals relates to remaining mode coding. Three very similar solutions are proposed:

- JCTVC-F091 by Sony

- JCTVC-F106 by LG

- JCTVC-F269 by Canon.

The approach relates to 4x4 to 32x32 PUs. It consists in using ranking tables related to the modes and can apply to CABAC and CAVLC. Two ranking tables of size 18 (for 4x4 PUs) and 35 (for 8x8 to 32x32 PUs) are defined. When the mode is not one of the MPMs, the rank of the mode is first decremented if it is larger than the ranks of the MPMs. The decremented value is then coded. Then the ranking table is updated based on the mode value, in the same way as currently done in CAVLC design of current HM.

The difference between JCTVC-F091, JCTVC-F106 and JCTVC-F269 are the following:

- In JCTVC-F091, same binarization and VLC tables, based on exp-Golomb code, are used. The modes are initially ranked according to their values. The ranking tables are updated only when remaining mode is coded.

- In JCTVC-F106 and JCTVC-F269, binarization and VLC tables are pre-determined. The ranking tables are updated when MPM or remaining mode is coded.

- JCTVC-F106 only applies to CAVLC but uses different VLC tables or initial ranking tables than JCTVC-F269.

### List of configuration and combinations to be evaluated

The following combinations will be explored.

- MPMs derivation

  - Different number of MPMs

  - Different MPMs ordering and derivation rules

- Remaining mode coding
The solution combining proposals JCTVC-F106 and JCTVC-F269 and JCTVC-F091 will be evaluated.
The following experiments will be conducted:

  - Specific binarization and VLC tables

  - Unified Exp-Golomb-based binarization and VLC tables

- Combinations of 1) and 2)
Combinations of Solutions 1) and 2) will be tested in order to find the best combination.

| CE6c: Intra Mode Coding |  |  |  |
| --- | --- | --- | --- |
| Contribution | Tools | Proponent | Cross-check |
| JCTVC-F062 | MPMs derivation | MediaTek peter.chuang@mediatek.com yuwen.huang@mediatek.com | Qualcomm wchien@qualcomm.com Sony ehsan.maani@am.sony.com Santa Clara Univ. cli1@scu.edu |
| JCTVC-F459 | MPMs derivation | Qualcomm wchien@qualcomm.com | Canon edouard.francois@canon.cfr.fr MediaTek peter.chuang@mediatek.com, Santa Clara Univ. cli1@scu.edu JVC-Kenwood fukushima.shigeru@jvckenwood.com, kumakura.toru@jvckenwood.com |
| JCTVC-F091 | Remaining mode coding | Sony ehsan.maani@am.sony.com | Santa Clara Univ. cli1@scu.edu |
| JCTVC-F106 | Remaining mode coding | LG jy.park@lge.com | Santa Clara Univ. cli1@scu.edu |
| JCTVC-F269 | Remaining mode coding | Canon edouard.francois@canon.cfr.fr | Santa Clara Univ. cli1@scu.edu |
| JCTVC-F566 | DCIM MPM & SDIP Harmonization | Sony ehsan.maani@am.sony.com Sharp yamamoto.tomoyuki@sharp.co.jp | Huawei laichangcai@huawei.com |
| JCTVC-F062, JCTVC-F459, JCTVC-F091, JCTVC-F106, JCTVC-F269 | Combinations of MPMs derivation and Remaining mode coding | MediaTek, Qualcomm, Sony, LG, Canon | MediaTek peter.chuang@mediatek.com ETRI shcho@etri.re.kr I2R hltan@i2r.a-star.edu.sg Sharp yamamoto.tomoyuki@sharp.co.jp |

### Reference

- T.-D. Chuang, C.-Y. Chen, M. Guo, X. Guo, Y.-W. Huang, S. Lei, “Luma Intra Prediction Mode Coding”, JCTVC- F062, Torino, July 2011.

- E. Maani, A. Tabatabai, “Unifying binarizations of Intra modes in HE and LC”, JCTVC- F091, Torino, July 2011.

- J. Park, B. Jeon, “CAVLC coding for Intra Pred mode”, JCTVC- F106, Torino, July 2011.

- E. Francois, S. Pautet, C. Gisquet, “Modified Intra Mode Coding”, JCTVC- F269, Torino, July 2011.

- W.-J. Chien, X. Wang, M. Karczewicz, “Parsing friendly intra mode coding”, JCTVC- F459, Torino, July 2011.

- E. Maani, T. Yamamoto, A. Tanizawa, T. Shiodera, V. Drugeon, “CE6.c: Differential Coding of Intra Modes”, JCTVC-F566, Torino, Italy, July 2011

- E. Maani, W. Liu, “Differential Coding of Intra Modes (DCIM)”, JCTVC-B109, Geneva, Switzerland, July 2010

## CE6c: Short Distance Intra Prediction

The goal of this experiment is to test the coding efficiency and encoding/decoding running time of the harmonized SDIP based on the HM4.0 software. This is not limited to improving coding efficiency, it also relates to harmonization and complexity reduction. Five main subtests are addressed here:

- Tradeoff of coding efficiency and encoding/decoding running time of SDIP

- Chroma coding scheme for SDIP mode

- SDIP harmonization with HE residual coding

- LM mode harmonization on SDIP

- Intra prediction mode coding with CAVLC on SDIP

### CE6.b Test Summary and spec text of SDIP (JCTVC-F532)

This CE subset is to conduct further tests, based on the comments received in the discussions, on the tradeoff of coding efficiency and encoding/decoding running time of the harmonized SDIP integrated on the HM4.x software platform.

The additional gain reported in JCTVC-F505 (A chroma coding scheme for SDIP mode) as well as the harmonization in HE residual coding described in JCTVC-F556 will also be tested.

### A chroma coding scheme for SDIP mode (JCTVC-F505)

A chroma prediction method for SDIP mode proposed in JCTVC-F505 will be tested. The proposed method consists of  three components, using the shortest codeword for LM, a new prediction mode SDM (i.e. SDIP direct mode), and candidate mode pruning process. In this proposal, the LM mode always uses the shortest codeword in all possible candidate mode sets. With SDM, chroma component shares the same PU partition and prediction modes of luma component. Therefore no overhead is transmitted to signal the chroma PU mode and the prediction mode of each PU. With the candidate mode pruning process, luma prediction mode used by only one of the four SDIP partitions inside a CU is excluded from the candidate mode set of chroma component.

### CE6.b: Report on SDIP Harmonization with Deblocking, MDIS, MDCS, and HE Residual Coding (JCTVC-F556)

In the harmonization with HE residual coding part, the mapping between non-square blocks to square blocks is not used. Instead, several modifications in the current ‘square-based’ entropy coding are described in order to deal with the coding of non-square coefficient blocks. Modifications are in the following parts: scans, coding of the last significant coefficient, significance map coding and coefficient level coding.

### LM mode harmonization on SDIP (JCTVC-F110)

LM mode harmonization method and chroma mode coding for SDIP based on JCTVC-F110 will be tested. When SDIP is enabled, luma modes, LM mode, mode 0~2 and planar mode are arranged by turns in the chroma mode candidate list. The arrangement of chroma mode coding candidates including LM mode will be investigated in this sub-test.

### Intra prediction mode coding with CAVLC on SDIP (JCTVC-F111)

A CAVLC joint coding scheme for split mode and partitioning types on SDIP will be tested. When SDIP is enabled, split mode and four kinds of partitioning type exist for intra prediction. Similar CAVLC joint coding scheme and counters as used for inter mode coding is also applied to intra mode coding.

### Test plan

#### Test configurations for 4.3.1 (F532) and 4.3.2 (F505)

1) The test configurations in 4.3.1 will be set up based on the comments received from the discussions in the last meeting. In particular, the following three test cases will be conducted:

Case 1: SDIP (with all harmonization listed in JCTVC-532) without 1x16/16x1 TU;

Case 2: Case 1 with fast SDIP optimization algorithm;

Case 3: Case 1 simplification with non-square TU only (removing non-square PU).

2) The chroma coding scheme proposed in F505 (Subtest 4.3.2)  will be implemented on and compared to the software provided by Subtest 4.3.1 Case 1 and/or Case 2. In particular, the following two cases will be tested

Case 1: the chroma coding scheme proposed in JCTVC-F505

Case 2: disable the SDIP direction mode (SDM) in case 1 to show the performance of SDM only

#### Test configurations for 4.3.3 (F556)

### The SDIP integration in HM4.0 is necessary to test the method for HE residual coding for non-square blocks described in F556. The method will be implemented on and compared to the software provided by Subtest 4.3.1 (all cases).

#### Test configurations for 4.3.4 (F110) and 4.3.5 (F111)

- Chroma mode coding

  - Best chroma mode order reflecting the LM mode modification in the previous meeting.

  - Combination of a) and the new chroma tools from F505

- CAVLC coding for the split and partitioning type

  - Specific VLC tables and counters

- Combinations of 1) and 2)

| CE6d: Short Distance Intra Prediction |  |  |  |
| --- | --- | --- | --- |
| Contribution | Tools | Proponent | Cross-check |
| JCTVC-F532 | CE6.b Test Summary and Spec Text of SDIP | Huawei laichangcai@huawei.com, andy.songjin@huawei.com, haitao.yang@huawei.com, haoping.yu@huawei.com Microsoft jzxu@microsoft.com Tsinghua Univ. caoxr05@mails.tsinghua.edu.cn | Sony jun.xu@am.sony.com Orange Lab. joelb.jung@orange-ftgroup.com |
| JCTVC-F505 | Chroma coding scheme for SDIP mode | Huawei andy.songjin@huawei.com haitao.yang@huawei.com yangmingyuan@huawei.com haoping.yu@huawei.com laichangcai@huawei.com | Samsung jianle.chen@samsung.com |
| JCTVC-F556 | SDIP Harmonization with Deblocking, MDIS, MDCS, and HE Residual Coding | Qualcomm geertv@qualcomm.com, joels@qualcomm.com | Huawei/HiSilicon laichangcai@huawei.com |
| JCTVC-F110 | LM mode harmonization on SDIP | LG jaehyun.lim@lge.com, bm.jeon@lge.com | MediaTek peter.chuang@mediatek.com Huawei/HiSilicon laichangcai@huawei.com |
| JCTVC-F111 | Intra prediction mode coding with CAVLC on SDIP | LG | MediaTek peter.chuang@mediatek.com Huawei/HiSilicon laichangcai@huawei.com |

### Reference

- X. Cao, et. Al, “CE6.b Test Summary and Spec Text of SDIP”, JCTVC-F532, Torino, Italy, July 2011

- Jin Song, et. al., “A chroma coding scheme for SDIP mode”, JCTVC-F505, Torino, Italy, July 2011

- G. Van der Auwera, et. al., “CE6.b: Report on SDIP Harmonization with deblocking, MDIS, MDCS, and HE residual coding”, JCTVC-F556, Torino, Italy, July 2011

- J. Lim, B. Jeon, “CE6.b Test 4: LM mode harmonization on SDIP”, JCTVC-F110, Torino, Italy, July 2011

- J. Lim, B. Jeon, “Intra prediction mode coding with CAVLC on SDIP”, JCTVC-F111, Torino, Italy, July 2011

## CE6d: Intra prediction with secondary boundary

### Direction based Angular Intra Prediction (JCTVC-F122)

In this scheme, a direction based angular intra prediction is proposed, which includes two techniques: the gradient based prediction and bi-directional prediction. In the first one, the gradient information between neighboring reference samples along the intra prediction direction is used to improve the quality of a set of intra prediction modes, which totally include 8 modes (from Vertical to Vertical+3 and from Horizontal to Horizontal+3). In the second one, the reference samples at both ends of the direction is used to improve the quality of a set of intra prediction modes, which totally include 10 modes (from Vertical +4 to Vertical +8 and from Horizontal +4 to Horizontal +8). A simple interpolation is used to achieve the fractional pixels according to the angular directions.

### An improved intra vertical and horizontal prediction (JCTVC-F172)

In this scheme, difference between two reference samples in left or above prediction direction is scaled and added to corresponding prediction samples in intra vertical or horizontal prediction mode. For intra vertical prediction mode, prediction samples are derived by applying the following process:

S'(x,y)=S(x,−1)+(S(−1,y)−S(−1,−1))2 (x=0)S(x,−1) (x≥1)

where x, y represent horizontal and vertical pixel positions within a PU, S(x,y) represents reference sample value and S’(x,y) represents prediction sample value.

For intra horizontal prediction mode, prediction samples are derived by applying the following process:

S'(x,y)=S(−1,y)+(S(x,−1)−S(−1,−1))2 (y=0)S(−1,y) (y≥1)

### Intra planar and angular prediction smoothing (JCTVC-F358)

Planar prediction smoothing: In this scheme, a filter is applied to the boundary of planar prediction as described in JCTVC-F358. The process can be described as follows:

rpDst[0] = ( pSrc[-1] + 2*rpDst[0] + pSrc[-srcStride] + 2 ) >> 2;

for (k=0, l=1; l<blkSize; l++)
	  rpDst[k*dstStride+l] = ( pSrc[l-srcStride] + 3*rpDst[k*dstStride+l] + 2 ) >> 2;

for (k=1, l=0; k<blkSize; k++)
	  rpDst[k*dstStride+l] = ( pSrc[k*srcStride-1] + 3*rpDst[k*dstStride+l] + 2 ) >> 2;

Angular prediction smoothing: In this scheme, a diagonal 2-tap filter is applied to the secondary boundary depending on the intra prediction direction as described in JCTVC-F358. The process can be described as follows:

if ( intraPredAngle > 0 )
 {
    for (k=0;k<blkSize;k++)
      pDst[k*dstStride] = ( 3*pDst[k*dstStride] + refSide[k+2] + 2 ) >> 2;
 }

### Directional intra prediction smoothing (JCTVC-F456)

In this scheme, the secondary prediction boundary undergoes a diagonal 2-tap filtering depending on the intra prediction direction as described in JCTVC-F456. The process can be described as follows:

if ( abs ( intraPredAngle ) > 9 )

{

Int iFiltOff = intraPredAngle > 0 ? 2 : 0;

for (k=0;k<blkSize;k++)

pDst[k*dstStride] = ( 3*pDst[k*dstStride] + refSide[k+iFiltOff] + 2 ) >> 2;

}

### Category 1: Filtering for vertical and horizontal predictions

| CE6e: Intra prediction with secondary boundary |  |  |  |
| --- | --- | --- | --- |
| Contribution | Tools | Proponent | Cross-check |
| JCTVC-F172 | Intra horizontal and vertical prediction edge smoothing | Mitsubishi minezawa.akira@ds.mitsubishielectric.co.jp | Nokia jani.lainema@nokia.com |
| JCTVC-F122 | Gradient based intra prediction for vertical and horizontal set (totally 8 modes) | MediaTek xun.guo@mediatek.com, shawmin.lei@mediatek.com, mei.guo@mediatek.com | Nokia jani.lainema@nokia.com Sony cheung.auyeung@am.sony.com |

### Category 2: Filtering for other angular predictions

| CE6e: Intra prediction with secondary boundary |  |  |  |
| --- | --- | --- | --- |
| Contribution | Tools | Proponent | Cross-check |
| JCTVC-F122 | Bi-direction intra angular prediction | MediaTek xun.guo@mediatek.com, shawmin.lei@mediatek.com, mei.guo@mediatek.com | Nokia jani.lainema@nokia.com Toshiba akiyuki.tanizawa@toshiba.co.jptaichiro.shiodera@toshiba.co.jp |
| JCTVC-F358 | Intra angular prediction smoothing | ETRI jinosoul@etri.re.kr, hykim5@etri.re.kr, sclim@etri.re.kr | Nokia jani.lainema@nokia.com |
| JCTVC-F456 | Directional intra prediction smoothing | Nokia jani.lainema@nokia.com | Toshiba akiyuki.tanizawa@toshiba.co.jp Huawei laichangcai@huawei.com I2R hltan@i2r.a-star.edu.sg Inria laurent.guillo@irisa.fr, ronan.boitard@inria.fr |

In this category each technique is evaluated as proposed in 6th JCT-VC, Torino meeting contributions. In addition the following results are generated to identify the aspects providing best trade-offs between complexity and coding efficiency:

- Proposed technique with sub-pixel interpolation switched off for the secondary reference

- Proposed technique applied to a maximum of one row/column for each block

### Category 3: Filtering for planar prediction

| CE6e: Intra prediction with secondary boundary |  |  |  |
| --- | --- | --- | --- |
| Contribution | Tools | Proponent | Cross-check |
| JCTVC-F358 | Intra planar prediction smoothing | ETRI jinosoul@etri.re.kr, hykim5@etri.re.kr, sclim@etri.re.kr | Nokia jani.lainema@nokia.com |

### Category 4: Combination of Category 1, 2, and 3

Test the combination of each of the most promising results from Category 1, 2, and 3.

| CE6e: Intra prediction with secondary boundary |  |  |  |
| --- | --- | --- | --- |
| Contributions | Tools | Proponent | Cross-check |
| JCTVC-F172, JCTVC-F122, JCTVC-F358 , JCTVC-F456 | Combination of Category 1, 2 and 3 | Mitsubishi, MediaTek, ETRI, Nokia, | Huawei laichangcai@huawei.com Qualcomm geertv@qualcomm.com |

### Reference

- M. Guo, X. Zhao, X. Guo and S. Lei, “Direction based Angular Intra Prediction”, JCTVC-F122, Torino, Italy, July 2011

- Akira Minezawa, et.al., “An improved intra vertical and horizontal prediction”, JCTVC-F172, Torino, Italy, July 2011

- Jinho Lee, et.al., “Mode dependent filtering for intra predicted sample”, JCTVC-F358, Torino, Italy, July 2011

- Jani Lainema, et.al., “Directional intra prediction smoothing”, JCTVC-F456, Torino, Italy, July 2011