| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F906 |
| --- | --- |

| Title: | CE6: Intra Coding Improvements |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Ali Tabatabai (Primary) Edouard Francois Keiichi Chono Haoping Yu Tung Nguyen Jani Lainema | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | ali.tabatabai@am.sony.com edouard.francois@canon.crf.fr chono@ct.jp.nec.com haopingyu@huawei.com tung.nguyen@hhi.fraunhofer.de jani.lainema@nokia.com |

_____________________________

# Introduction

This output document provides the description of Core Experiments 6 on “Intra prediction and improvements”. Proposals on intra coding and intra prediction are divided in several subsets of this CE:

- Subset CE6a:	Intra Luma Prediction

- Subset CE6b:	Intra Chroma Prediction

- Subset CE6c:	Intra Mode Coding

- Subset CE6d:	Short Distance Intra Prediction

- Subset CE6e:	Intra Prediction with secondary boundary

Note that proposals related to SDIP are listed in the SDIP subset.

# Experimental Conditions

## Software

Tools under test of this CE will be implemented on the top of HM 4.0 software that is released by the software Ad-Hoc group after integrating adopted changes by the 6th JCT-VC meeting at Torino.

## Common Test Conditions

The common test condition for all-Intra configuration as defined in the document JCTVC-F800 released by the software Ad-Hoc group will be used as anchor configuration for tests in all subsets. The participants have to consider either Intra High Efficiency or Intra Low Complexity or both.

## Evaluation of CE Results

Results of this CE will be evaluated on the basis of BDBR metric. In addition, subjective evaluation to support the advantage of the proposed tools is highly desirable.

## Evaluation of Complexity

For the complexity measurement, the anchor, the reference software and the reference software with the tool implemented will be executed on the same machine and the computational time will be measured for each configuration. Then, a time factor is calculated which compares the reference software including the sub-CE test tool and the reference software without the tool.

# Timelines

| T1 | July xx, 2011 | HM 4.0 software available |
| --- | --- | --- |
| T2 | Torino + 2Wks | Deadline for sending email to coordinator expressing interest to participate in core experiment. |
| T3 | November xxx, 2011 | Cross-verification completed CE. |
| T4 | November yyy, 2011 | Report the verification results uploading. |

# Description of Tool Experiment

## CE6a: Intra Luma Prediction

There are no proposals in this subset.

## CE6b: Intra Chroma Prediction

### Modified down-sampling filters for LM mode of chroma intra prediction (JCTVC-F173, JCTVC-F502)

JCTVC-F173 and JCTVC-F502 proposed the modified luma down-sampling filters to improve LM mode of chroma intra prediction. In LM mode of HM 3.0, the correlation parameters α and β are derived based on the down-sampled reconstructed luma pixels and the reconstructed chroma pixels. The purpose of this CE subset is to test the benefit of modified luma down-sampling filters in term of coding performance and complexity.

There are three test points of this sub-test.

- Modified down-sampling horizontal filters for the top neighboring luma line

- Modified down-sampling vertical filters for the left neighboring luma line

- Modified down-sampling filters for the prediction value of the inner pixels of the luma block

#### An improvement to chroma intra prediction from luma (JCTVC-F173)

In this scheme, horizontal down-sampling filter for luma samples is applied for chroma intra prediction. For the luma samples to calculate α and β, horizontal 1:0:1 filter is applied. For the internal luma samples to generate chroma prediction, horizontal 1:2:1 filter is applied.

Figure 1.  The position of samples used for calculation of the parameters and 

#### Cross-channel techniques to improve intra chroma prediction (JCTVC-F502)

This contribution reports two techniques to improve intra chroma prediction: a 2-dimensional 3x2 filter for LM intra chroma prediction mode, and a new intra chroma prediction mode based on the technique of local adaptive prediction selection (LAPS) process to produce the intra chroma prediction sample from the comparison of the reconstructed luma sample of target location to the local reconstructed luma samples in the neighborhood.

In the sub-test of downsampling filter, the filter set of 1:2:1, 1:0:1, 0:1:0 will be tested on the horizontal filters for the top neighboring luma line, the vertical filters for the left neighboring luma line and the filter for the prediction value of the inner pixels of the luma block.

#### Test plan

The test plan is to compare the coding performance of the above three test points and of the combination of the test points to the common anchor on the HM 4.0.

#### Participants

| Participant | Contact | Role |
| --- | --- | --- |
| Mitsubishi Electronics | Minezawa.Akira@ds.MitsubishiElectric.co.jp Sugimoto.Kazuo@ak.mitsubishielectronic.co.jp | P |
| Intel | Yi-Jen Chiu, yi-jen.chiu@intel.com Lidong Xu, lidong.xu@intel.com Yu Han yu.han@intel.com | P |

#### Reference

- Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi), “An improvement to chroma intra prediction from luma”, JCTVC-F173, Torino, Italy, July 2011

- Y.-J. Chiu, Y. Han, L. Xu, W. Zhang, H. Jiang (Intel), “Cross-channel techniques to improve intra chroma prediction”, JCTVC-F502, Torino, Italy, July 2011

### Complexity Reduction of LM mode of chroma intra prediction (JCTVC-F233, JCTVC-F431)

#### Sub-sampling portion of neighboring pixels (JCTVC-F233&F431)

F233+F431 proposes that reference luma and chroma samples are subsampled before use in calculation of alpha and beta. It is reported that the number of multiplications is reduced.

#### An improvement to chroma intra prediction from luma (JCTVC-F431)

In the current HEVC specification order of transmission is coefY followed by coefU and coefV in the unit of CU. As max size of CU is 128x128 in luma resolution and 64x64 in re-sampled luma resoulution.. This re-sampled pixels should be stored only for LM_CHROMA and cannot be used for other processes like deblocking, etc so increase in this additional buffer is one of the concerns for implementation.

In this test methods to alleviate this bottleneck such as storing with not 10 bit but 8 bit will be studied.

#### Test plan

The test plan is to compare the coding performance of the above three test points and of the combination of the test points to the common anchor on the HM 4.0.

#### Participants

| Participant | Contact | Role |
| --- | --- | --- |
| TI | sze@ti.com | P |
| Sony | Kazushi.Sato@jp.sony.com | P |

## CE6c: Intra Mode Coding

The goal of this experiment is to enhance Intra mode coding of the HEVC Test Model. This is not limited to improving coding efficiency, but also relates to harmonization and complexity reduction. Two main technical topics are addressed:

- Most Probable Modes (MPM) derivation

- Remaining mode coding

The following proposals are considered in this experiment.

### MPM derivation related proposals

In this first set of proposals, the derivation of MPMs is modified compared to the HM design.

#### Luma Intra Prediction Mode Coding (JCTVC-F062)

In this test, additional most probable modes (MPMs) are added to improve coding efficiency. The additional MPMs are generated according to neighboring modes. The redundant bit of remaining mode coding might also be removed for higher efficiency.

#### Intra prediction mode coding based on direction difference (JCTVC-F339)

In JCTVC-F339, plural MPM sets are defined and one of the MPM sets is adaptively selected using direction difference between leftMode and aboveMode. This process is applied only for 4x4, 8x8, 16x16 and 32x32 prediction units. The process for 64x64 prediction units that have only 3 modes is not changed from HM3.0.

Brief decoding process for intra prediction mode consists of following ordered steps.

- Calculate direction difference between leftMode and aboveMode(DiffDir).

- Determine the MPM set by DiffDir.

- Decode prev_intra_pred_flag

- If prev_intra_pred_flag is equal to 1, decode mpm_idx using selected MPM set.

- Otherwise (prev_intra_pred_flag is equal to 0), decode rem_intra_pred_mode. The coding trees of the rem_intra_pred_mode are switched by the number of MPM candidates because the increase of the number of MPM candidates causes the decrease of the number of the modes needed to be decoded as remainder.

#### Parsing friendly intra mode coding (JCTVC-F459)

In JCTVC-F459, a total set of four MPMs is derived. First, one or two MPMs are first derived as in current HM design. The additional MPMs are derived depending on the value of the one or two first MPMs. If the above and the left neighboring blocks have the same intra prediction mode, Planar or DC mode plus two closest available intra modes of the left block intra mode are inserted to complete the MPMs set. Otherwise, if the above and left neighboring blocks have different intra prediction mode, Planar mode, plus one closest available intra mode are added.

#### Differential Coding of Intra Modes (DCIM) (JCTVC-F566)

JCTVC-F566 proposes DCIM (Differential Coding of Intra Modes) which utilizes neighboring reconstructed pixels to derive dominant edge and differentially signal the direction used for prediction. The performance of DCIM on the latest reference software i.e., HM 4.0 will be evaluated. Taking account for the recent adoptions and discussions, harmonization with SDIP as well as harmonization with MPM improvement will be tested.

### Remaining mode coding related proposals

This second set of proposals relates to remaining mode coding. Three very similar solutions are proposed:

- JCTVC-F091 by Sony

- JCTVC-F106 by LGE

- JCTVC-F269 by Canon.

The approach relates to 4x4 to 32x32 PUs. It consists in using ranking tables related to the modes. Two tables of size 18 (for 4x4 PUs) and 35 (for 8x8 to 32x32 PUs) are defined. When the mode is not one of the MPMs, the rank of the mode is first decremented if it is larger than the ranks of the MPMs. The decremented value is then coded. Then the ranking table is updated based on the mode value, in the same way as currently done in CAVLC design of current HM.

The difference between JCTVC-F091, JCTVC-F102 and JCTVC-F269 are the following:

- In JCTVC-F091, same binarization and VLC tables, based on exp-Golomb code, are used. The modes are initially ranked according to their values. The ranking tables are updated only when remaining mode is coded.

- In JCTVC-F106 and JCTVC-F269, binarization and VLC tables are pre-determined. The ranking tables are updated when MPM or remaining mode is coded.

- JCTVC-F106 only applies to CAVLC but uses different VLC tables or initial ranking tables than JCTVC-F269.

#### List of configuration and combinations to be evaluated

The following combinations will be explored.

- MPMs derivation

  - Different number of MPMs

  - Different MPMs ordering and derivation rules

- Remaining mode coding
The solution combining proposals JCTVC-F106 and JCTVC-F269 and JCTVC-F091 will be evaluated.
The following experiments will be conducted:

  - Specific binarization and VLC tables

  - Unified Exp-Golomb-based binarization and VLC tables

- Combinations of 1) and 2)
Combinations of Solutions 1) and 2) will be tested in order to find the best combination.

### Participants

| Participant | Contact | Role |
| --- | --- | --- |
| Canon | edouard.francois@canon.cfr.ft | P |
| LGE | Jy.park@lge.com | P |
| MediaTek | peter.chuang@mediatek.com yuwen.huang@mediatek.com | P |
| Qualcomm | wchien@qualcomm.com | P |
| Sony | ehsan.maani@am.sony.com | P |
| Sharp | yamamoto.tomoyuki@sharp.co.jp | P |
| Huawei | laichangcai@huawei.com | C |

### Reference

- T.-D. Chuang, C.-Y. Chen, M. Guo, X. Guo, Y.-W. Huang, S. Lei, “Luma Intra Prediction Mode Coding”, JCTVC- F062, Torino, July 2011.

- E. Maani, A. Tabatabai, “Unifying binarizations of Intra modes in HE and LC”, JCTVC- F091, Torino, July 2011.

- J. Park, B. Jeon, “CAVLC coding for Intra Pred mode”, JCTVC- F106, Torino, July 2011.

- E. Francois, S. Pautet, C. Gisquet, “Modified Intra Mode Coding”, JCTVC- F269, Torino, July 2011.

- T. Kumakura, S. Fukushima, “Intra prediction mode coding based on direction difference”, JCTVC- F339, Torino, July 2011.

- W.-J. Chien, X. Wang, M. Karczewicz, “Parsing friendly intra mode coding”, JCTVC- F459, Torino, July 2011.

- E. Maani, T. Yamamoto, A. Tanizawa, T. Shiodera, V. Drugeon, “CE6.c: Differential Coding of Intra Modes”, JCTVC-F566, Torino, Italy, July 2011

- E. Maani, W. Liu, “Differential Coding of Intra Modes (DCIM)”, JCTVC-B109, Geneva, Switzerland, July 2010

## CE6d: Short Distance Intra Prediction

This CE subset is to conduct further tests, based on the comments received in the discussions, on the coding efficiency and the encoding/decoding running time of the SDIP integrated on the HM4.x software platform.

### A chroma coding scheme for SDIP mode (JCTVC-F505)

A chroma prediction method for SDIP mode proposed in JCTVC-F505 will be tested. The proposed method is composed of two components, a new prediction mode i.e. SDM (SDIP direct mode), and candidate mode pruning process. With SDM, chroma component shares the same PU partition and prediction modes of luma component. Therefore no overhead is transmitted to signal the chroma PU mode and the prediction mode of each PU. With the candidate mode pruning process, luma prediction mode used by only one of the four SDIP partitions inside a CU is excluded from the candidate mode set of chroma component.

### CE6.b: Report on SDIP Harmonization with Deblocking, MDIS, MDCS, and HE Residual Coding (JCTVC-F556)

In the harmonization with HE residual coding part, the mapping between non-square blocks to square blocks is not used. Instead, several modifications in the current ‘square-based’ entropy coding are described in order to deal with the coding of non-square coefficient blocks. Modifications are in the following parts: scans, coding of the last significant coefficient, significance map coding and coefficient level coding.

### Participants

| Participant | Contact | Role |
| --- | --- | --- |
| Huawei | andy.songjin@huawei.com haitao.yang@huawei.com haoping.yu@huawei.com | P |
| Qualcomm | geertv@qualcomm.com joels@qualcomm.com | P |

### Reference

- Jin Song, et. al., “A chroma coding scheme for SDIP mode”, JCTVC-F505, Torino, Italy, July 2011

- G. Van der Auwera, et. al., “CE6.b: Report on SDIP Harmonization with deblocking, MDIS, MDCS, and HE residual coding”, JCTVC-F556, Torino, Italy, July 2011

## CE6e: Intra prediction with secondary boundary

### Direction based Angular Intra Prediction (JCTVC-F122)

In this scheme, a direction based angular intra prediction is proposed, which includes two techniques: the gradient based prediction and bi-directional prediction. In the first one, the gradient information between neighboring reference samples along the intra prediction direction is used to improve the quality of a set of intra prediction modes, which totally include 8 modes (from Vertical to Vertical+3 and from Horizontal to Horizontal+3). In the second one, the reference samples at both ends of the direction is used to improve the quality of a set of intra prediction modes, which totally include 10 modes (from Vertical +4 to Vertical +8 and from Horizontal +4 to Horizontal +8). A simple interpolation is used to achieve the fractional pixels according to the angular directions.

### An improved intra vertical and horizontal prediction (JCTVC-F172)

In this scheme, difference between two reference samples in left or above prediction direction is scaled and added to corresponding prediction samples in intra vertical or horizontal prediction mode. For intra vertical prediction mode, prediction samples are derived by applying the following process:

S'(x,y)=S(x,−1)+(S(−1,y)−S(−1,−1))2 (x=0)S(x,−1) (x≥1)

where x, y represent horizontal and vertical pixel positions within a PU, S(x,y) represents reference sample value and S’(x,y) represents prediction sample value.

For intra horizontal prediction mode, prediction samples are derived by applying the following process:

S'(x,y)=S(−1,y)+(S(x,−1)−S(−1,−1))2 (y=0)S(−1,y) (y≥1)

### Intra planar and angular prediction smoothing (JCTVC-F358)

Planar prediction smoothing: In this scheme, a filter is applied to the boundary of planar prediction as described in JCTVC-F358. The process can be described as follows:

rpDst[0] = ( leftColumn[0] + 2*rpDst[0] + topRow[0] + 2 ) >> 2;

for (k=0, l=1; l<blkSize; l++)
  rpDst[k*dstStride+l] = ( topRow[l] + 3*rpDst[k*dstStride+l] + 2 ) >> 2;

for (k=1, l=0; k<blkSize; k++)
  rpDst[k*dstStride+l] = ( leftColumn[k] + 3*rpDst[k*dstStride+l] + 2 ) >> 2;

Angular prediction smoothing: In this scheme, a diagonal 2-tap filter is applied to the secondary boundary depending on the intra prediction direction as described in JCTVC-F358. The process can be described as follows:

if ( intraPredAngle > 0 )
 {
    for (k=0;k<blkSize;k++)
      pDst[k*dstStride] = ( 3*pDst[k*dstStride] + refSide[k+2] + 2 ) >> 2;
 }

### Directional intra prediction smoothing (JCTVC-F456)

In this scheme, the secondary prediction boundary undergoes a diagonal 2-tap filtering depending on the intra prediction direction as described in JCTVC-F456. The process can be described as follows:

if ( abs ( intraPredAngle ) > 9 )

{

Int iFiltOff = intraPredAngle > 0 ? 2 : 0;

for (k=0;k<blkSize;k++)

pDst[k*dstStride] = ( 3*pDst[k*dstStride] + refSide[k+iFiltOff] + 2 ) >> 2;

}

### Category 1: Filtering for vertical and horizontal predictions

| Contribution | Tools | Tester | Cross-check |
| --- | --- | --- | --- |
| JCTVC-F172 (Mitsubishi) | Intra horizontal and vertical prediction edge smoothing | Mitsubishi |  |
| JCTVC-F122 (MediaTek) | Gradient based intra prediction for vertical and horizontal set (totally 8 modes) | MediaTek |  |

### Category 2: Filtering for other angular predictions

| Contribution | Tools | Tester | Cross-check |
| --- | --- | --- | --- |
| JCTVC-F122 (MediaTek) | Bi-direction intra angular prediction | MediaTek |  |
| JCTVC-F358 (ETRI) | Intra angular prediction smoothing | ETRI |  |
| JCTVC-F456 (Nokia) | Directional intra prediction smoothing | Nokia |  |

In this category each technique is evaluated as proposed in 6th JCT-VC, Torino meeting contributions. In addition the following results are generated to identify the aspects providing best trade-offs between complexity and coding efficiency:

- Proposed technique with sub-pixel interpolation switched off for the secondary reference

- Proposed technique applied to a maximum of one row/column for each block

### Category 3: Filtering for planar prediction

| Contribution | Tools | Tester | Cross-check |
| --- | --- | --- | --- |
| JCTVC-F358 (ETRI) | Intra planar prediction smoothing | ETRI |  |

### Category 4: Combination of Category 1, 2, and 3

Test the combination of each of the most promising results from Category 1, 2, and 3.

### Participants

| Participant | Contact | Role |
| --- | --- | --- |
| MediaTek | xun.guo@mediatek.com shawmin.lei@mediatek.com mei.guo@mediatek.com | P |
| Mitsubishi Electric Corporation | Minezawa.Akira@ds.MitsubishiElectric.co.jp | P |
| ETRI | jinosoul@etri.re.kr | P |
| Nokia | jani.lainema@nokia.com | P |

### Reference

- M. Guo, X. Zhao, X. Guo and S. Lei, “Direction based Angular Intra Prediction”, JCTVC-F122, Torino, Italy, July 2011

- Akira Minezawa, et.al., “An improved intra vertical and horizontal prediction”, JCTVC-F172, Torino, Italy, July 2011

- Jinho Lee, et.al., “Mode dependent filtering for intra predicted sample”, JCTVC-F358, Torino, Italy, July 2011

- Jani Lainema, et.al., “Directional intra prediction smoothing”, JCTVC-F456, Torino, Italy, July 2011