| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F907_r2 WG11 Number: m21439 |
| --- | --- |

| Title: | Description of Core Experiment 7: Additional Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories Chuohao Yeo Institute for Infocomm Research Rajan Joshi Qualcomm Felix Fernandes Samsung Electronics Co., Ltd. | Email: | cohen@merl.com chyeo@i2r.a-star.edu.sg rajanj@qualcomm.com felix.f@sta.samsung.com |
| Source: | CE7 Coordinators |  |  |

_____________________________

# Abstract

This document defines Core Experiment 7 (CE7) on Additional Transforms, to be performed for the upcoming 7th JCT-VC meeting November 2011. During the 6th JCT-VC meeting in July, 2011, three secondary transforms JCTVC-F224, JCTVC-F294, and JCTVC-F554, were recommended for a more thorough assessment. This assessment will be used to determine whether the improvement in compression efficiency is worthwhile given the associated increase in complexity. A mode-dependent DCT/DST for chroma, JCTVC-F553 was also recommended for a more thorough assessment including factors such as additional hardware complexity.

# Introduction

For the July 2011 JCT-VC meeting in Torino, Core Experiment 7 (CE7) [1] investigated the performance of several proposed non-core transforms. JCT-VC made the following recommendations:

- Three proposals, JCTVC-F224 [2], JCTVC-F294 [3], and JCTVC-F554 [4] apply secondary transforms after the core transform in the encoder. In the decoder, the inverse secondary transform is performed before the inverse core transform. A more thorough assessment is needed in order to determine whether the increase in compression efficiency is worthwhile given the associated increase in complexity.

- JCTVC-F553 [5] applies the mode-dependent 4x4 DCT/DST to chroma. Additional information related to hardware complexity was requested for this proposal.

These transforms operate on Intra Transform Units only.

# Participant list

| Name | Affiliation | Email |
| --- | --- | --- |
| Robert Cohen (coordinator) | Mitsubishi Electric Research Labs | cohen@merl.com |
| Chuohao Yeo (coordinator) | Institute for Infocomm Research | chyeo@i2r.a-star.edu.sg |
| Rajan Joshi (coordinator) | Qualcomm | rajanj@qualcomm.com |
| Felix Fernandes (coordinator) | Samsung | felix.f@sta.samsung.com |
| Elena Alshina | Samsung | elena_a.alshina@samsung.com |
| Ankur Saxena | Samsung | asaxena@sta.samsung.com |
| Il-Koo Kim | Samsung | ilkoo.kim@samsung.com |
| Youji Shibahara | Panasonic | shibahara.youji@jp.panasonic.com |
| Takahiro Nishi | Panasonic | nishi.takahiro@jp.panasonic.com |
| Marta Mrak | BBC | Marta.Mrak@bbc.co.uk |
| Pankaj Topiwala | FastVDO | pankaj@fastvdo.com |
| Akiyuki Tanizawa | Toshiba | akiyuki.tanizawa@toshiba.co.jp |
| Jun Yamaguchi | Toshiba | jun2.yamaguchi@toshiba.co.jp |
| Atsuro Ichigaya | NHK | ichigaya.a-go@nhk.or.jp |
| Shinichi Sakaida | NHK | sakaida.s-gq@nhk.or.jp |
| Haitao Yang | Huawei | haitao.yang@huawei.com |
| Tung Nguyen | HHI | tung.nguyen@hhi.fraunhofer.de |
| Madhukar Budagavi | TI | madhukar@ti.com |
| TK Tan | NTT DOCOMO | ttk@pacific.net.sg |
| Kiran Misra | Sharp Labs of America | misrak@sharplabs.com |
| Jewon Kang | Nokia | ext-jewon.kang@nokia.com |

# Description of tools

The following tools shall be evaluated for CE 7. Full details can be found in the related documents. Bold text indicates the primary contact for the corresponding entry.

| Tool # | Participants | Document | Tool Description and cross-checkers |
| --- | --- | --- | --- |
| 1 | Y. Shibihara, T. Nishi | JCTVC-F224 | A mode-dependent secondary transform operates on the low-frequency coefficients output by the HM core transform in the encoder. The secondary transforms is a 4x4 pseudo-DST and operates on 8x8 and larger blocks. |
|  | Cross-checkers: |  | 1. A. Saxena, F. Fernandes 2. R. Cohen 3. … |
| 2 | A. Saxena, F. Fernandes | JCTVC-F554 | Depending on the intra prediction mode, a secondary transform may operate on the low-frequency coefficients of the 8x8 and larger HM DCT. Two versions of this mode-dependent secondary transform will be evaluated: an 8-point version, and a 4-point version. The latter version is derived from the existing 4-point DCT and 4-point DST already in Working Draft 3. |
|  | Cross-checkers: |  | 1. Y. Shibihara, T. Nishi 2. M. Mrak 3. … |
| 3 | Z. Ma, F. Fernandes, E. Alshina, A. Alshin | JCTVC-F294 | After the 8x8, 16x16, or 32x32 core transform is performed in the encoder, one of four 8x8 secondary Rotational Transforms (ROT) may be applied to the low-frequency transform coefficients output by the core transform. The decision as to which ROT to use is signalled by the encoder. Simplifications for improving complexity/performance tradeoffs will be investigated. |
|  | Cross-checkers: |  | 1. Y. Shibihara, T. Nishi 2. P. Topiwala 3. R. Joshi 4. … |
| 4. | A. Saxena, F. Fernandes, E. Alshina, J. Chen | JCTVC-F553 | The 4x4 mode-dependent DCT/DST adopted in Working Draft 3 for luma blocks is extended to operate on chroma blocks. |
|  | Cross-checkers: |  | 1. Y. Shibihara, T. Nishi 2. C. Yeo … |

# Core Experiment tasks

The objectives for this Core Experiment are to:

- Specify the software platform, common software configurations and common test conditions for these experiments;

- Evaluate compression efficiency and complexity of the additional transforms studied in this CE;

- Provide informal comments regarding subjective differences, if any.

- Investigate any unique interactions among the additional transforms and coefficient scanning schemes;

- Report the results and conclusions of these experiments.

# Test conditions

The following test conditions shall be used for this CE:

## Software

HM version 4.0 shall be used for these experiments, as specified in JCTVC-F900 Error: Reference source not found.

## Test configurations

This CE shall use the same encoder and decoder configurations as described in JCTVC-F900. Experiments shall be run for the Intra High-Efficiency, Intra Low-Complexity, Random Access High-Efficiency, and Random Access Low-Complexity coding configurations.

## Test sequences

The test sequences as described in JCTVC-F900 shall be used for these tests.

## Training of transforms (if applicable)

The test sequences used from Section 5.3 shall not be used for training any proposed additional transforms. Proponents shall report which sequences they use for training of their transforms, if applicable.

# Evaluation Criteria

This section contains details about how the transforms shall be evaluated. If a proposed transform includes scan orders not already included in HM, simulation results at a minimum shall be reported for the additional transform without these additional scan orders. Results with the additional scan orders may be reported as well. This is done to help JCT-VC determine the benefits of these additional scans. Proponents are welcome to include additional results at their discretion.

## Compression efficiency

The spreadsheet associated with the common conditions JCTVC-F900 [6] shall be used to evaluate coding efficiency. This spreadsheet includes a summary of BD metrics and encoder/decoder run times, along with detailed rate and PSNR tables and plots, and possibly other data. Data associated with the CE7 configurations described above in Section 5.2 shall be reported. The references for these computations shall be generated from the unmodified HM 4.0 software. Participants are free to provide additional data at their discretion.

## Complexity evaluation

### Tool description

In the JCT-VC text document associated with each proposed tool, the abstract shall include a brief text description of the proposed additional transform, along with a brief summary of the compression performance and complexity evaluation.

### Additional transform description

The additional transforms for each transform size shall be described in such a way that makes it clear what operations are being performed. Pseudo-code or data-flow diagrams are preferred. The number of additional transform cores to be added shall also be reported.

To enable a clear understanding of these tools relative to the HM 4.0 reference, proponents shall provide block diagrams showing all ways in which their transforms are instantiated in the decoder. Each unique instantiation and the conditions (e.g. mode or signal) that trigger that instantiation shall be described. Example diagrams are shown in the figures below. Participants are free to copy and modify these figures. If the encoder instantiation is not a straightforward inverse of the decoder implementation, then a separate figure for the encoder shall be provided.

Conditions

Decoder Operations

HM 4.0 NxN Inverse DCT

HM 4.0 Inverse Quant

4-Point Column Transforms

4-Point Row Transforms

8x8 and larger TUs when IntraPredMode = 3,4,7,10,11,14,15,18,19,20,21,26,27,28,29,34

HM 4.0 NxN Inverse Transform

HM 4.0 Inverse Quant

4x4 TUs; 8x8 and larger TUs when IntraPredMode = 2

HM 4.0 NxN Inverse DCT

HM 4.0 Inverse Quant

4-Point Column Transforms

8x8 and larger TUs when IntraPredMode = 0,5,6,12,13,22,23,24,25

HM 4.0 NxN Inverse DCT

HM 4.0 Inverse Quant

4-Point Row Transforms

8x8 and larger TUs when IntraPredMode = 1,8,9,16,17,30,31,32,33

Figure 1. Example decoder operations for mode-dependent secondary transforms.

HM 4.0 NxN Inverse DCT

HM 4.0 Inverse Quant

Vert. Inverse 8x8 ROTi

Horz. Inverse 8x8 ROTi

ROT Index i

0

1,2,3,4

HM 4.0 NxN Inverse Transform

HM 4.0 Inverse Quant

Conditions

4x4 TUs; 8x8 and larger TUs for which RD search signals i = 0

8x8 and larger TUs for which RD search signals i = 1,2,3,4

Decoder Operations

Figure 2. Example decoder operations for ROT

### Arithmetic operations

For each size of each inverse transform, the worst-case number of arithmetic operations per square transform unit (multiplies, adds, and shifts) shall be reported. These counts shall be based upon the pseudo-code or data-flow diagrams. The operations of the final scaling performed during the inverse transform process shall also be included in the operations count. Data for the JCTVC-E243 partial butterfly implementation are provided in Table 1 for reference. Note that the number of additions reported includes the offset before the shift. The correct number of shifts is also shown.

The following table format shall be used. The contents of the shaded areas are fixed, and the non-shaded areas shall contain data for each of the proposed transforms. Additional columns may be added as needed. If the forward and inverse transforms do not have similar operation counts, forward transform data shall also be reported in a separate column. For secondary transforms applied in addition to the core HM transform, the operation counts of the secondary transform shall be preceded by a “+” sign. Example entries for additional transforms are shown in the last two columns of Table 1.

In addition to worst-case operation counts, averaged operation counts shall be provided to give an indication of the additional complexity incurred under typical operating conditions. For the mode-dependent tools shown in Figure 1, the average complexity shall be computed as a weighted average based upon the number of modes for which that configuration is used. For example, Figure 1 shows four different configurations that are used for a total of 35 Intra prediction modes. If the four configurations require, for example, +0, +32, +32, and +64 multiplies respectively, then the average number of multiplies for the tool would be (0 × 1/35) + (32 × 9/35) + (32 × 9/35) + (64 × 16/35) = 46 (rounded). So, the worst-case number of multiplies for this secondary transform would be reported as +64, and the average case would be +46. Note that HM 4.0 may have additional prediction modes than those shown in this example figure, so the final weightings may need to be based upon 1/36 or 1/37. For the rotational transform of Figure 2, an equiprobable distribution shall be assumed, so the weighting shall be 1/5 for each of the five cases.

Table 1. Operation count examples

| Transform size |  | JCTVC-E243 partial butterfly implementation of 2-D inverse DCT | Example transform that replaces the core HM 2-D transform | Example secondary transform applied after 8x8 and larger HM 2-D transforms. For 4x4, a replacement transform is used |
| --- | --- | --- | --- | --- |
| 4x4 | Mults | 48 | 72 | 128 |
|  | Adds | 96 | 160 | 144 |
|  | Shifts | 32 | 80 | 32 |
| 8x8 | Mults | 352 | 1024 | +128 |
|  | Adds | 576 | 1024 | +128 |
|  | Shifts | 128 | 352 | +16 |
| 16x16 | Mults | 2752 |  | +128 |
|  | Adds | 3712 |  | +128 |
|  | Shifts | 512 |  | +16 |
| 32x32 | Mults | 21888 |  | +128 |
|  | Adds | 25856 |  | +128 |
|  | Shifts | 2048 |  | +16 |

### Bit precision

Report the maximum bit precision needed for each transform size.

In the decoder, because the output of the secondary inverse transform is input to the core inverse transform, proponents shall provide information that confirms that the data output from the secondary inverse transform does not exceed 16 bits.

### Memory requirements

For each transform size, report any additional memory requirements in bytes.

### Quantization

Report the presence and number of additional quantization matrices, for each transform size.

Any additional bit-depth required by use of these additional quantization matrices shall also be reported.

### R-D optimization

If any additional rate-distortion loops or iterations are being performed in the encoder or decoder, they shall be described.

### Encoder/Decoder run times

Encoder and decoder run times shall be reported using the common spreadsheet as described in Section 6.1.

### Other statistics

If present, memory transfers, reordering, and other data operations not included above shall be reported.

## Subjective effects

If sequences coded using a proposed additional transform exhibit subjective improvements or degradations, proponents and/or cross-verifiers shall describe these subjective differences. These descriptions are to be considered as informal comments, as formal subjective testing is not required by CE7 at this time.

# Timeline

Aug 19, 2011: Final CE7 description uploaded

August 23, 2011: HM 4.0 available

September 13, 2011: Software distributed to all CE7 participants so cross-checking can begin

November 4, 2011 or earlier: Proponents report results to CE7 coordinators

November 8, 2011: Input documents uploaded

# References

- R. Cohen, C. Yeo, R. Joshi and F. Fernandes, “CE7: Summary Report of Core Experiment on Additional Transforms,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F027, 6th JCT-VC Meeting, Torino, IT, Jul. 2011.

- T. Shibihara and T. Nishi, “CE7: Mode Dependent 2-step Transform for Intra Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F224, 6th JCT-VC Meeting, Torino, IT, Jul. 2011.

- Z. Ma, F. Fernandes, E. Alshina and A. Alshin, “CE 7: Experimental Results for the Rotational Transform,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F294, 6th JCT-VC Meeting, Torino, IT, Jul. 2011.

- A. Saxena and F. Fernandes, “On secondary transforms for intra prediction residual,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F554, 6th JCT-VC Meeting, Torino, IT, Jul. 2011.

- A. Saxena, F. Fernandes, E. Alshina and Jianle Chen, “Mode-Dependent DCT/DST for 4x4 Chroma Blocks,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F553, 6th JCT-VC Meeting, Torino, IT, Jul. 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th JCT-VC Meeting, Torino, IT, July 2011.