| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F908_r2 WG11 Number: m21446 |
| --- | --- |

| Title: | Description of Core Experiment 8 (CE8): Non-deblocking loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage (Primary) In Suk Chong Matthias Narroschke Yu-Wen Huang |  | tomoo.yamakage@toshiba.co.jp ichong@qualcomm.com Matthias.Narroschke@eu.panasonic.com yuwen.huang@mediatek.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

The goal of this Core Experiment (CE) is to enhance non-deblocking loop filtering techniques in the HEVC Test Model.  This is not limited to improving coding efficiency, but also reducing complexity and harmonization of the various schemes that are technically feasible based on the understanding that the current situation of JCT-VC activity is in “collaborative phase”.

Techniques are evaluated according to BD-rate gain, complexity and/or subjective picture quality. The following items are categorized as subtests:

- Block level filter adaptation with directional feature (BADIR).  4 proposals.

Sharp/Qualcomm/MediaTek/Toshiba (JCTVC-F384, JCTVC-F522)

Sharp/Qualcomm/MediaTek/Toshiba (JCTVC-F384, JCTVC-F522)

Samsung (JCTVC-F301)

Mitsubishi (JCTVC-F179)

- Sub-frame delay adaptive loop filter: Online vs. Fixed (parametric).  2 proposals.

Cisco (JCTVC-F542)

Sony (JCTVC-F285)

- Line Memory reduction in ALF and SAO decoding.  6 proposals.

Panasonic (JCTVC-F272)

MediaTek (JCTVC-F054, JCTVC-F055)

Panasonic/MediaTek (JCTVC-F272, JCTVC-F054, JCTVC-F055)

MediaTek (JCTVC-F054)

TI (JCTVC-F232)

MediaTek (JCTVC-F093)

- Filter shapes and coefficient constraints, for both luma and chroma.  3 proposals.

MediaTek/Qualcomm/Toshiba (JCTVC-F157)

Samsung/eBrisk (JCTVC-F303)

TI (JCTVC-F235)

- Prediction of filter coefficients.  3 proposals.

MediaTek/Qualcomm/Toshiba (JCTVC-F321)

Peking Univ. (JCTVC-F498)

Ericsson (JCTVC-F076)

- Filter switching reduction at LCU boundary for LCU friendly decoding.  1 proposal.

Panasonic (JCTVC-F271)

- Chroma filter control.  1 proposal.

eBrisk (JCTVC-F042)

- Other in-loop filters.  1 proposal.

NTT (JCTVC-F047)

The software basis for this CE is HM 4.0, and the experiments are conducted based on the test conditions in JCTVC-F900.  Details are described in section 3.

Variants of the number of design passes for ALF coefficients in HM 4.0 will be also tested as an additional activity of CE8.

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 | Chia-Yang Tsai Chih-Ming Fu Ching-Yeh Chen Shawmin Lei Xun Guo Yu-Wen Huang (coordinator) Matthias Narroschke (coordinator) Thomas Wedi Semih Esenlik Marta Karczewicz In Suk Chong (coordinator) Wei-Jung Chien Wang (PoLin) Lai Felix C. Fernandes Elena Alshina Il-Koo Kim Tomohiro Ikai Tomoyuki Yamamoto Yukinobu Yasugi Andrew Segall Takeshi Chujoh Tomoo Yamakage (Primary coordinator) Andrey Norkin Kenneth Andersson Faouzi Kossentini Michael Horowitz Hsan Guermazi Madhukar Budagavi Minhua Zhou Shun-ichi Sekiguchi Kazuo Sugimoto Ali Tabatabai Ehsan Maani Masaru Ikeda Y.-J. Chiu L. Xu W. Zhang Han Yu B. Bross T. K. Tan Chuohao Yeo Yih Han Tan Kemal Ugur Oguz Bici Arild Fuldseth Siwei Ma Masaaki Matsumura Jinho Lee Hui Yong Kim Sung-Chang Lim Ju Ock Lee Joo-Hee Moon Jeongyoen Lim Edouard Francois Seungwook Park Yong-Joon Jeon Joonyoung Park | MediaTek MediaTek MediaTek MediaTek MediaTek MediaTek Panasonic Panasonic Panasonic Qualcomm Qualcomm Qualcomm Samsung Samsung Samsung Samsung Sharp Sharp Sharp Sharp Toshiba Toshiba Ericsson Ericsson eBrisk Video eBrisk Video eBrisk Video TI TI Mitsubishi Mitsubishi Sony Sony Sony Intel Intel Intel Intel HHI DOCOMO I2R I2R Nokia Nokia Cisco Peking Univ. NTT ETRI ETRI ETRI Sejong Univ. SK Telecom SK Telecom Canon LGE LGE LGE | chiayang.tsai@mediatek.com chihming.fu@mediatek.com chingyeh.chen@mediatek.com shawmin.lei@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com Matthias.Narroschke@panasonic.de Thomas.Wedi@eu.panasonic.com Semih.Esenlik@eu.panasonic.com martak@qualcomm.com ichong@qualcomm.com wchien@qualcomm.com wlai@sta.samsung.com ffernandes@sta.samsung.com elena_a.alshina@samsung.com ilkoo.kim@samsung.com ikai.tomohiro@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp yasugi.yukinobu@sharp.co.jp asegall@sharplabs.com takeshi.chujoh@toshiba.co.jp tomoo.yamakage@toshiba.co.jp andrey.norkin@ericsson.com kenneth.r.andersson@ericsson.com faouzi@ebriskvideo.com michael@ebriskvideo.com hsan@ebriskvideo.com madhukar@ti.com zhou@ti.com Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp Ali.Tabatabai@am.sony.com Ehsan.Maani@am.sony.com Masaru.Ikeda@jp.sony.com Yi-jen.chiu@intel.com Lidong.xu@intel.com Wenhao.zhang@intel.com yu.han@intel.com benjamin.bross@hhi.fraunhofer.de ttk@pacific.net.sg chyeo@i2r.a-star.edu.sg yhtan@i2r.a-star.edu.sg Kemal.Ugur@nokia.com ext-oguz.bici@nokia.com arilfuld@cisco.com swma@jdl.ac.cn matsumura.masaaki@lab.ntt.co.jp jinosoul@etri.re.kr hykim5@etri.re.kr sclim@etri.re.kr jolee@sju.ac.kr jhmoon@sejong.ac.kr jeongyeon@sk.com edouard.francois@crf.canon.fr Seungwook.park@lge.com yongjoon.jeon@lge.com jy.park@lge.com | Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent |

# Experimental Conditions

## Software

HM 4.0 shall be used, which will be released on Aug. 19, 2011, except subtet e (prediction of filter coefficients).  Since APS, which provides the capability of temporal direct prediction of ALF coefficients, will be integrated in HM 4.1, subtest e will start three weeks after the release of HM4.1.

Regarding encoding algorithm, the number of design passes for ALF coefficients in all subtests shall follow those described in JCTVC-F900 except subtest b.  In addition, it is encouraged proponents to provide additional results such as harmonization of proposals.

In the event that HM 4.0 has serious defects affecting CE8 experimental results, an alternative version will be used.  The selection of an appropriate version will be consulted with the HM software AHG chair.

## Test Sequences, Bit Rates and Coding Conditions

Unless it is described as optional in JCTVC-F900, test all test sequences under the configurations described in JCTVC-F900.  All High Efficiency cases (AI, RA, LDB and LDP) shall be tested.

If a proposal intend to adopt ALF in Low Complexity cases, it is mandatory for the proposal to provide experimental results in Low Complexity cases.

Since some proposals in subtest c (JCTVC-F232 and JCTVC-F093) test the improvement of the SAO LCU boundary pixel processing, coding conditions of multiple slices per picture are requested.  Details are described in section 6.3.

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures, complexity and/or functionality.

BD-rate gain/loss and runtime numbers shall be provided according to JCTVC-F900.

For the complexity measurement, it is highly recommended that the HM 4.0 software with the anchor configuration and the proposal implemented on the HM 4.0 software will be executed on the same (or identically configured) machine(s), and the computational time will be measured for each test sequence and test case. Relative computational time is calculated against the anchor. Maximum number of vertical taps of filtering and the number of coefficients shall be also reported.

# Subtest a: Block level filter adaptation with directional feature (BADIR)

## Functionality Addressed

- Complexity reduction and/or coding efficiency enhancement for filter adaptation

## Expected Gains

- Enhance current block based adaptive loop filter

- Complexity reduction

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Number of coefficients

- Maximum number of vertical taps

Note: In subtest a, it is only allowed to modify block-based pixel classification methods from the anchor.  No change of chroma processing is allowed.

## Tool Description

### (1) 2-D mergeable syntax (JCTVC-F384, Sharp and Qualcomm; JCTVC-F522, Qualcomm, Toshiba, MediaTek, and Sharp)

This evaluates 2-D mergeable syntax for filter adaptation to improve coding efficiency and encoder design. This technique enables 2-D merge for BA mode and RA mode in place of current 1-D merge. Filter coefficients prediction may use 2-D dimensionality.

#### Bitstream organization

The classification syntax is changed to enable 2-D merge.  Filter coefficients prediction syntax may be changed.

### (2) Directional feature calculation on subset of pixels (JCTVC-F384 Sharp and Qualcomm; JCTVC-F522, Qualcomm, Toshiba, MediaTek, and Sharp)

#### This evaluates a directional feature calculation on subset of pixels to improve coding efficiency. This technique uses the internal and the surrounding pixels (window pixels) of the 4x4 block unit and may change the computational direction to diagonal directions from horizontal/vertical directions. In addition, it may use additional block adaptation mode.  This also evaluates several ways of feature calculation (i.e., using different subset of pixels) which have different complexity and performance trade off.

#### Bitstream organization

The range of adaptation mode flag may be changed.

### (3) Filter adaptation with directional feature (JCTVC-F301, Samsung)

In the 6th JCTVC meeting, a proposal (JCTVC-F301) on simplification of block-based filter adaptation (BA) of ALF was adopted.  It was recommended by the committee to further improve efficiency-complexity trade-off.  This evaluates modifications of filter adaptation (e.g. BA features on varies pixel positions, improving classification methods, etc.)

#### Bitstream organization

Pixel group information signaling may be changed.

### (4) Pixel classification for ALF based on edge direction (JCTVC-F179, Mitsubishi)

The Laplacian Activity is mapped to index value using one of three tables shown as Table 2 to Table 4 in JCTVC-F179 instead of Table 1 in JCTVC-F179 depending on the Direction so that more classification levels are assigned to “the others” category.  4 levels are assigned to “horizontal edge” and “vertical edge”, and 8 levels are assigned to “the others” catetory respectively, instead of assigning 5 levels for each category in HM-4.

#### Bitstream organization

Pixel group information signalling may be changed.

## Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| SHARP | Tomohiro Ikai ikai.tomohiro@sharp.co.jp |
| Samsung | Wang (PoLin) Lai wlai@sta.samsung.com |
| Mitsubishi | Kazuo Sugimoto Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |

Proponent(s)						Cross-checker(s)

- Sharp/Qualcomm					Samsung

- Sharp/Qualcomm					Mitsubishi, Samsung

- Samsung						Qualcomm

- Mitsubishi						Sharp

# Subtest b: Sub-frame delay adaptive loop filter: Online vs. Fixed (parametric)

## Functionality Addressed

- Low latency capability of adaptive loop filtering in encoder and/or decoder

## Expected Gains

- Low latency ALF

- Complexity reduction

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Number of coefficients

- Maximum number of vertical taps

Note: In subtest b, encoding techniques will be different from those in HM 4.0.  Therefore, it is mandatory to describe the encoding algorithm in detail in the contribution text.

## Tool Description

### (1) Low complexity adaptive loop filter with sub-frame delay (JCTVC-F542, Cisco)

This experiment evaluates ALF where estimation and transmission of the ALF coefficients is performed on a block-by-block basis, typically 64x64 or 128x128 (or rectangular size).  The ALF process is single-pass within the picture and without any dependency between the current block and future blocks in processing order.  The ALF targets a complexity operation point that is significantly lower than that of HM 4.0-ALF.

#### Bitstream organization

Coefficient coding and other signalling syntaxes may be changed.

### (2) Parametric Adaptive Loop Filter (JCTVC-F285, SONY)

A set of fixed filters is used instead of the traditional online-trained Weiner filters.  The encoder chooses the best (in RD sense) filter for a Coding Unit (CU) and transmits the index of the filter in the bit stream to the decoder.  The selection of the best filter at the encoder requires only a single-pass processing for each Large CU (LCU).  The classification of the filters is 2D and it is based on direction and bandwidth parameters.

In this CE it is further investigated about the potential of these kinds of filters as a set of Default Fixed ALF filters with lower memory access and delay.

CE will include:

- Symmetry and shape alignment between default fixed filter set and adopted ALF shape in HM4.0

- Compression efficiency tradeoffs between filter shapes, number of filters, computational bit depth

- SPS/APS based harmonization of  ALF and default fixed filter set

- RDO based fixed vs. temporally predicted ALF filters

#### Bitstream organization

Coefficient coding and other signalling syntaxes may be changed.

## Participants

| Participant | Contact |
| --- | --- |
| Sony | Ali Tabatabai Ali.Tabatabai@am.sony.com Ehsan Maani Ehsan.Maani@am.sony.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| Mitsubishi | Shun-ichi Sekiguchi Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp Kazuo Sugimoto Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |
| Cisco | Alird Fuldseth arild.fuldseth@cisco.com |
| TI | Minhua Zhou zhou@ti.com |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |

Proponent(s)						Cross-checker(s)

(1) Cisco (F542)					Sony, Qualcomm

(2) Sony (F285)						TI, Cisco, Mitsubishi, Toshiba

# Subtest c: Line Memory reduction in ALF and SAO decoding

## Functionality Addressed

- Line buffer/external memory access in ALF and/or SAO decoding

- Processing of LCU and slice boundary pixels in SAO

## Expected Gains

- Complexity reduction, in particular line buffer/external memory access reduction

- LCU and slice boundary visual artifact removal

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Degree of memory reduction

- Number of coefficients

- Maximum number of vertical taps

- Required line memory

- Subjective picture quality

Note: If the filter shape is changed (e.g. filter shape/symmetory/size, pixels to be used for filtering, etc.), individual results need to be provided for this change.  This would be required for comparison with those of subtest d.

Note: For subtests 6.c.5 (JCTVC-F232) and 6.c.6 (JCTVC-F093), the multiple slice coding conditions (SliceMode=1) is used.  For these tests, the timeline is delayed two weeks from the regular timeline.

Note: An informal subjective viewing may be conducted in this subtest in order to check the visual artifact caused by the data discontinuity (e.g. a special treatment of filtering at vertical boundaries).  Proponents and cross-checkers are encouraged to check the subjective picture quality.

## Tool Description

### (1) Line Memory reduction in ALF and SAO decoding (JCTVC-F272, Panasonic)

The proposal reduces line memory requirements by using pre-deblocked (non-deblocked or only horizontally deblocked) samples at the LCU boundaries. In other words, at the bottom LCU boundary, the pre-deblocked samples that are already available in the memory are used as the input to SAO and ALF instead of the fully-deblocked samples (horizontally and vertically deblocked), for which waiting and thus line memory would be necessary.

In this core experiment, reduction in line memory will be investigated.

#### Bitstream organization

No change in the bitstream.

### (2) Line Memory reduction in ALF and SAO decoding (JCTVC-F054, JCTVC-F055, MediaTek)

In JCTVC-F054, ALF with padding at virtual boundaries was proposed to remove all the line buffers. The general concept is to process a to-be-filtered pixel on one side of a virtual boundary without using any pixel from the other side.  Repetitive padding is used to replace pixels from the other side.  Besides, a boundary handling technique is applied to try to avoid visible boundaries.

In F055, the similar concept in F054 is applied to SAO.  Repetitive padding is used to replace pixels from the other side of virtual boundary.

In this experiment, reduction in line memory and boundary handling are investigated.

#### Bitstream organization

No change in bitstream.

### (3)Line Memory reduction in ALF and SAO decoding (JCTVC-F272 by Panasonic, JCTVC-F054 and JCTVC-F055 by MediaTek)

At the bottom LCU boundary, the available pre-deblocked (non- or partially deblocked) pixels are used as the input of SAO and ALF in JCTVC-F272, while the repetitively padded pixels are used as the input of ALF and SAO in JCTVC-F054 and JCTVC-F055. In this test, a harmonized scheme is investigated. For example, for those samples under the virtual LCU boundary inside LCU, the partially deblocked pixels can be used; for those samples above the virtual LCU boundary inside LCU, the repetitively padded pixels can be used. The boundary handling technique can be applied to avoid visual artifacts.

In this experiment, the harmonization between JCTVC-F272 and JCTVC-F054/JCTVC-F055 for ALF/SAO is investigated.

#### Bitstream organization

No change in the bitstream.

### (4) Harmonization between Line Memory Reduction and Filter Shape design (JCTVC-F054, MediaTek)

In F054, a to-be-filtered pixel on one side of a virtual boundary can be processed without using any pixel from the other side by using repetitive padding. In other words, the constraint on the number of vertical filter taps does not exist.  New filter shape will be tested along with the line memory reduction scheme.

In this experiment, the harmonization between line memory reduction and filter shape design is investigated.

#### Bitstream organization

No change in the bitstream.

### (5) SAO LCU boundary pixel processing (JCTVC-F232, TI)

Techniques that do not use line buffer and/or pixels across LCU/slice will be investigated. Combination of following filter kernels will be tested.  Pixel categorization will appropriately modified to match the support of the filter kernel.

#### Bitstream organization

No change in bitstream.

### (6) SAO LCU boundary pixel processing (JCTVC-F093, MediaTek)

This contribution proposed to replace the skipping technique used in SAO by the padding technique. At image boundaries, if a to-be-filtered pixel requires a pixel out of image boundaries, the pixel out of image boundaries will be replaced by the to-be-filtered pixel. At slice boundaries when the slice-independent in-loop filtering flag is enabled, if a to-be-filtered pixel requires a pixel from any other slice, the pixel from any other slice will be replaced by the to-be-filtered pixel.

In this experiment, padding technique is investigated to replace skipping technique for slice and image boundary pixels in SAO.

#### Bitstream organization

No change in bitstream.

## Participants

| Participant | Contact |
| --- | --- |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| Panasonic | Semih Esenlik Semih.Esenlik@eu.panasonic.com |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| TI | Madhukar Budagavi madhukar@ti.com |

Proponent(s)						Cross-checker(s)

(1) Panasonic (F272)					MediaTek

(2) MediaTek (F054, F055)				Qualcomm, Panasonic

(3) Panasonic/MediaTek (F272, F054, F055)		NTT

(4) MediaTek (F054)					Toshiba

(5) TI (F232)						MediaTek

(6) MediaTek (F093)					TI

# Subtest d: Filter shapes and coefficient constraints, for both luma and chroma

## Functionality Addressed

- Complexity reduction and/or improved coding efficiency

- Maximum number of vertical taps for both luma and/or chroma

- Maximum number of coefficients for both luma and/or chroma

## Expected Gains

- Reduction in worst case computational complexity and/or memory requirements

- Evaluate the implication of using the same or different chroma filters compared to luma filters

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Maximum number of vertical taps for both luma and chroma

- Maximum number of coefficients for both luma and chroma

- Required line memory

## Tool Description

### (1) Luma and chroma filter shapes for ALF (JCTVC-F157, Toshiba/MediaTek/Qualcomm)

Luma and chroma filter shapes including the modification of the number of taps are studied that may consider the line memory reduction proposed in JCTVC-F054 and JCTVC-F055.

#### Bitstream organization

Signalling the number of filter coefficients and shapes may be modified.

### (2) Filter shapes for luma and chroma (JCTVC-F303, Samsung/eBrisk Video)

#### In the 6th JCTVC meeting, filter shapes with vertical-size 5 and up to 9 coefficients were adopted for luma and chroma (JCTVC-F303).  This evaluates trade-off between filter size/coefficients and coding efficiency (e.g. further study for shrinking the chroma filter size).  Possible harmonization with JCTVC-F272, F054 is also considered to resolve line-buffer issue.

#### Bitstream organization

### (3) Chroma ALF with reduced vertical filter size (JCTVC-F235, TI)

#### It is reported that vertical size of ALF filter determines the memory bandwidth/line buffer requirements in ALF decode and that reducing the vertical size of the filter leads to reduction in memory bandwidth/line buffer requirements.  In this experiment, filters with reduced vertical size (e.g. Nx3) will be investigated to reduce memory bandwidth/line buffer requirements.

#### Bitstream organization

Signaling filter coefficients may be modified to support shapes of chroma filter.

## Participants

| Participant | Contact |
| --- | --- |
| eBriskVideo | Faouzi Kossentini faouzi@ebriskvideo.com |
| TI | Madhukar Budagavi madhukar@ti.com |
| Samsung | Wang (PoLin) Lai wlai@sta.samsung.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |

Proponent(s)						Cross-checker(s)

(1) Toshiba/MediaTek/Qualcomm (F157)		TI

(2) Samsung (F303)					Qualcomm

(3) TI (F235)						Samsung

# Subtest e: Prediction of filter coefficients

## Functionality Addressed

- Improved coding efficiency

## Expected Gains

- Coding efficiency improvement, in particular low resolution sequences and/or low bitrate cases

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Level of error resilience/propagation

## Tool Description

### (1) Reusing fixed set of coefficients in APS and APS control(JCTVC-F321, Toshiba/Qualcomm/MediaTek)

In HM 4, APS is adopted to signal ALF coefficients.  In this experiment, three schemes are tested; i) APS management scheme (e.g. maximum number of APSs to be stored, how to share APS with multiple slices/pictures) and ii) difference coding of ALF coefficients.

#### Bitstream organization

i) can be done by adding syntax to remove unused APS from the memory or by implicitely according to decoding process.  ii) can be done by changing APS syntax or slice header syntax to whether coefficients are directly signalled or differences from its reference are signalled.

### (2) Adaptive Loop Filter Merge in Temporal Domain (JCTVC-F498, PKU)

In this experiment we will explore the coding mechanism with APS for ALF parameters.

#### Bitstream organization

Add syntax to control the coding of ALF parameters in APS.

### (3) Spatial prediction of filter coefficients (JCTVC-F076, Ericsson)

### In this experiment it is tested what can be gained by spatial prediction of some luma filter/difference coefficients from sum of other filter/difference coefficients of the same filter.

### Bitstream organization

Can be done implicitely by the decoding process and explicitly to indicate if non-centre filter coefficients are spatially predicted.

## Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |
| PKU | Xinfeng Zhang xfzhang@jdl.ac.cn Siwei Ma swma@pku.edu.cn |
| Ericsson | Kenneth Andersson kenneth.r.andersson@ericsson.com |

Proponent(s)						Cross-checker(s)

(1) Toshiba/MediaTek/Qualcomm (F321)		PKU

(2) PKU (F498)						Ericsson

(3) Ericsson (F076)					Toshiba

# Subtest f: Filter switching reduction at LCU boundary for LCU friendly decoding

## Functionality Addressed

- Complexity reduction without losing improved coding efficiency

## Expected Gains

- Reduction in computational complexity

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

- Number of line buffers

- Number of switching compared to the anchor when LCU-based and frame-based decoding

## Tool Description

### (1) Grid displacements for in-loop filtering (JCTVC-F271, Panasonic)

The contribution proposes a method to reduce the complexity of LCU- (Largest Coding Unit) based filtering operations via displacements in the filtering grids. The proposed method applies to Sample Adaptive Offset (SAO) and Adaptive Loop Filter (ALF).

Fig. 1: Filter application window gets shifted due to the latency in deblocking filter.

In HM3.0 the filtering grids of SAO and ALF are aligned with the LCU boundaries. However during the LCU-based decoding, application windows of the filters get shifted due to the latency in deblocking filter. The contribution proposes to align the filtering grids of SAO and ALF with their respective application windows. As a result filter switching required in LCU-based decoding is minimized, thus simplifying hardware implementations.

#### Bitstream organization

No change in the bitstream.

## Participants

| Participant | Contact |
| --- | --- |
| Panasonic | Semih Esenlik Semih.Esenlik@eu.panasonic.com |

Proponent(s)						Cross-checker(s)

(1) Panasonic (F271)					eBrisk

# Subtest g: Chroma filter control

## Functionality Addressed

- Complexity reduction and improved chroma coding efficiency

- Use of luma-ALF CU adaptivity map to guide chroma-ALF filter design and application

## Expected Gains

- Reduction in average computational complexity

- Evaluate the implication of using the same or different chroma filters compared to luma filters

## Evaluation Criteria

- Coding efficiency: BD-rate

- Encoding and decoding time

Note: In subtest g, it is only allowed to modify chroma filter control from the anchor.

## Tool Description

### (1) ALF chroma filtering using same ALF luma filtering structure (JCTVC-F042, eBrisk)

Following the ALF chroma filtering method described in JCTVC-F042, an investigation will be conducted into the use of the Luma ALF shape and/or map for chroma ALF filtering in the context of HM 4.0.  Simulation results will be presented that will illustrate the complexity-performance tradeoffs of the proposed method relative to that of HM 4.0.

#### Bitstream organization

No change is needed.

## Participants

| Participant | Contact |
| --- | --- |
| eBrisk Video | Faouzi Kossentini faouzi@ebriskvideo.com |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |

Proponent(s)						Cross-checker(s)

(1) eBrisk (F042)					MediaTek

# Subtest h: Other in-loop filters

## Functionality Addressed

- Improving coding efficiency and subjective picture quality

## Expected Gains

- Improving coding efficiency and subjective picture quality

## Evaluation Criteria

- Coding efficiency: BD-rate

- Subjective picture quality

- Encoding and decoding time

- Required line memory

Note: In subtest h, if there is any modification in the existing in-loop filtering in the current design, it is requested to report individual results.

## Tool Description

### (1) In-loop filter based on non-local means filter (JCTVC-F047, NTT)

This evaluates an adaptive loop filtering for enhancing existing loop filter tools with NLM. In order to reduce the memory bandwidth and the complexity, this provides a method for getting rid of line buffer and an adaptive control algorithm for filtering parameters such as template size and filter tap.

#### Bitstream organization

Coefficient coding and other signaling syntax may be changed.

## Participants

| Participant | Contact |
| --- | --- |
| NTT | Masaaki Matsumura matsumura.masaaki@lab.ntt.co.jp |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com |
| Intel | Lidong Xu lidong.xu@intel.com |

Proponent(s)						Cross-checker(s)

(1) NTT (F047)						MediaTek, Intel

# Time-line and Responsibilities

T1:		2011-Aug-19:	Finalize CE description and upload

T2:		2011-Aug-19:	HM 4.0 available

T3.a:	2011-Sept–13:	Software and draft contribution text available to all CE participants and start cross-checking for regular tests

T4.a:	2011-Sept-20:		Report proponents’ results to Coordinators

T5.a:	2011-Sept-27:		Report cross-checkers’ coding (and code review) results and comments to Coordinators

T3.b:	2011-Sept–27:	Software and draft contribution text available to all CE participants and start cross-checking only for subtest 8.c.5 and 8.c.6

T4.b:	2011-Oct-4:		Report proponents’ results to Coordinators

T5.b:	2011-Oct-11:		Report cross-checkers’ coding (and code review) results and comments to Coordinators

T3.c:	HM4.1 + 3 weeks:	Software and draft contribution text available to all CE participants and start cross-checking only for subtest e

T4.c:	HM4.1 + 4 weeks:	Report proponents’ results to Coordinators

T5.c:	HM4.1 + 5 weeks:	Report cross-checkers’ coding (and code review) results and comments to Coordinators

T6:		2011-Nov-8:		Contribution documents upload, and report the document number and title to coordinators

T7: 	2011-Nov-17		Summary report upload

Note: Time-line changed to reflect the release date of HM 4.0.

# References

[1]	“Common HM test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.

[2]	“High Efficiency Video Coding (HEVC) Test Model 4 (HM 4) Encoder Description”, JCTVC-F802, Torino, July 2011.

[3]	“High Efficiency Video Coding (HEVC) text specification Working Draft 4”, JCTVC-F803, Torino, July 2011.