| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-E910 WG11 Document: m21433 |
| --- | --- |

| Title: | CE10: Core transform design |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala FastVDO LLC Madhukar Budagavi Texas Instruments Arild Fuldseth Cisco Rajan Joshi Qualcomm Elena Alshina Samsung Electronics | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-443-455-0394 pankajtva@yahoo.com madhukar@ti.com arilfuld@cisco.com rajanj@qualcomm.com elena.alshina@samsung.com |
| Source: | FastVDO LLC, Texas Instruments, Cisco, Qualcomm, Samsung Electronics |  |  |

_____________________________

# 1  Objective

This CE shall investigate the design of core spatial transforms, and study the tradeoff between coding efficiency and complexity, return evidence and a recommendation for design selection. The anchor for comparison shall be the transforms in the HM. Proposals that use dynamic range control methods (such as clipping) should incorporate them into the complexity analyses below.

# Participant list

| Name |  | Affiliation |  | Email |
| --- | --- | --- | --- | --- |
| Pankaj Topiwala |  | FastVDO LLC |  | pankajtva@yahoo.com |
| Madhukar Budagavi Mangesh Sadafale |  | Texas Instruments Texas Instruments |  | madhukar@ti.com mangesh@ti.com |
| Arild Fuldseth |  | Cisco |  | arilfuld@cisco.com |
| Rajan Joshi |  | Qualcomm |  | rajanj@qualcomm.com |
| Il-Koo Kim Elena Alshina |  | Samsung Electronics |  | ilkoo.kim@samsung.com elena.alshina@samsung.com |
| Dzung Hoang |  | Zenverge |  | Dzung.Hoang@Zenverge.com |
| David Flynn |  | BBC |  | davidf@rd.bbc.co.uk |
| Louie Kerofsky |  | Sharp |  | lkerofsky@sharplabs.com |
| Jian Lou |  | Motorola |  | lou@motorola.com |
| Robert Cohen |  | Mitsubishi Electric Research Laboratories |  | cohen@merl.com |
| Tung Nguyen |  | HHI |  | tung.nguyen@hhi.fraunhofer.de |
| Ankur Saxena |  | Samsung Telecommunications America |  | asaxena@sta.samsung.com |
|  | TK Tan |  | NTT DOCOMO, Inc. | tktan.p@rd.nttdocomo.co.jp |

# 2  Description of tools

JCTVC-F446, A. Fuldseth, G. Bjøntegaard (Cisco), M. Sadafale, M. Budagavi (TI), Transform design for HEVC with 16 bit intermediate data representation.

This contribution proposes a set of transform matrices for HEVC, covering all transform sizes from 4x4 to 32x32. The proposed transform design is reported to have the following properties: 16 bit data representation before and after each transform stage (applies to both IBDI off and IBDI on), 16 bit multipliers for all internal multiplications, no need for correction of different norms of basis vectors during quantization/dequantization, all transform sizes above 4x4 can reuse arithmetic operations for smaller transform sizes, and implementations using either pure matrix multiplication or a combination of matrix multiplication and butterfly structures are possible. This experiment will evaluate the coding performance and complexity of the transform.

JCTVC-F251, E. Alshina, A. Alshin, I.-K. Kim (Samsung), P. Topiwala (FastVDO), Full factorized core transform proposal by Samsung/FastVDO.

This proposal develops a set of transforms that, it is claimed, support all three forms of implementation: matrix multiplication, partial butterfly and full-factorization, and identical in terms of performance.  This reported equivalence allows choosing the implementation according to platform: i.e. matrix multiplication form may be preferred for SIMD implementations while full-factorization may be preferred for platforms with strict limitations for number of multiplications. By utilizing Loeffler’s factorization in the 32pt transform, it is reported that a 4X reduction in the number of multiplications compared to partial butterfly, anda 12X reduction compared to matrix multiplication,  is achieved.

JCTVC-F363, Wei Dai, Madhu Krishnan, Pankaj Topiwala (FastVDO), (FastVDO), Ilkoo Kim (Samsung), FastVDO-Samsung Core Transform Proposal

This submission is an update of E253, a joint proposal from FastVDO and Samsung. This version employs the 4-pt and 8-pt transforms from the Cisco/TI design, currently incorporated in the HM 3.0 software, and 16-pt and 32-pt transforms of FastVDO and Samsung design, which are variants of the designs from E253. It is reported that the compression efficiency of this design is comparable to that of the HM 3.0 transforms, while evidence is provided to indicate that the computational complexity of the larger transforms of size 16 and 32, which dominate the complexity burden, is significantly lower than those of the HM 3.0 design. Thus, it is claimed that this transform architecture provides a favorable tradeoff between complexity and performance.

JCTVC-F352, R. Joshi, Y. Reznik, J. Sole, M. Karczewicz (Qualcomm), Scaled orthogonal integer transforms supporting recursive factorization structure

In this contribution, 8, 16, and 32 point transforms are proposed. These transforms are scaled orthogonal integer transforms supporting a recursive factorization structure. The 4-point transform is unchanged from HM 3.0. The 8, 16 and 32-point transforms use 192, 1152, and 5888 multiplications, respectively, compared to 352, 2752, and 21888 multiplications for the corresponding transforms in HM 3.0. A multiplication-free implementation for the proposed transforms is also provided.

# 3  Core Experiment Tasks

3.1. Coding efficiency

3.1.1. For proposals that are updated (tentative date: 8/5/11, unnecessary otherwise), measure the RD performance of candidate transform designs using common conditions E700 and HM 3.0 software. If a transform design uses clipping etc. on intermediate variables, this clipping should be included in coding efficiency tests. (For convenience, we are planning to keep the HM3.0 for RD testing purposes.)

3.1.2. Also test at low and high QP values.   Provide BD-Rate results for the following QP sets: 1, 5, 9, 13 and 36, 42, 47, 51 to see if there is unusual behavior at these uncommon test points.  Run this test on 2 sec of video to speed up this test. Number of frames to encode is provided in  table 1 below.

B.1.3 (Optional). Also, report BD-rate performance using “12 bit input” (InternalBitDepth=12) with high efficiency settings. Provide BD-rate results for the low-, normal-, and high QP ranges described in B1.1 and B1.2.

Table 1. Number of test frames for 3.1.2.

| Sequence name | No. frames to encode |
| --- | --- |
| Traffic | 65 |
| PeopleOnStreet | 65 |
| Kimono | 49 |
| Nebuta | 129 |
| SteamLocomotive | 129 |
| ParkScene | 49 |
| Cactus | 97 |
| BasketballDrive | 97 |
| BQTerrace | 129 |
| BasketballDrill | 97 |
| BQMall | 129 |
| PartyScene | 97 |
| RaceHorses (C) | 65 |
| BasketballPass | 97 |
| BQSquare | 129 |
| BlowingBubbles | 97 |
| RaceHorses (D) | 65 |
| Vidyo1 | 129 |
| Vidyo3 | 129 |
| Vidyo4 | 129 |

Suggested Cross-check Schedule (as necessary)

| Proponent | Cross-Check Normal Qp | Cross-Check Low/High Qp |
| --- | --- | --- |
| Cisco/TI (F446) | Qualcomm | Samsung/FastVDO |
| Samsung/FastVDO butterfly (F251) | Motorola | Zenverge ¦ Sharp |
| FastVDO/Samsung Lifting (F363) | Cisco/TI | Qualcomm |
| Qualcomm (F352) | Samsung/FastVDO | Cisco/TI |

## 3.2. Complexity Preliminaries

Analyze the complexity of candidate transform designs using the following parameters:

- Arithmetic operations

- Software execution speed

- Dynamic range

- Parallelizability on SIMD

- Number of sequential operations

- Quantization/dequantization matrices (optional)

- HW

Proponents are encouraged to provide additional complexity analyses voluntarily.

3.2.1. Arithmetic Operations

Report the number of arithmetic operations needed for implementing the 2D inverse transform, in terms of multiplications, additions, shifts, compares. Optionally, multiplications using 16-bit and 32-bit multipliers can be counted separately. If the design allows flexibility in the way it is implemented, report the range of arithmetic operations available (for example, methods with and without multiplications).  Include in the count TPE and final rounding if used.

## 3.3. Hardware and Software Complexity Tests

## 3.3.1. Preliminaries.

1. Any updates to proposals - by 8/5/11.

2. Test all proposals simultaneously.

3. Quantization to be included in test (decoder side).  Details to be finalized.

4. Test forward and inverse 32-pt.

## 3.3.2. HW Test.

a. Test using a Software Tool (Catapult C).

b. fixed set of test frequencies. 150, 250, 500 MHz.  45 nm. (Details tbd).

c. measure gate counts, latencies, throughputs.

For test purposes, we will need cooperation in test configurations and code.

Volunteer. Samsung. (Elena Alshina and team).  Will prepare Samsung code, accept other codes. Run all tests. Will create and distribute cleaned up C code for all proposals.       (9/2/11)

Volunteer:  STMicro (Daniele Alfonso).  Assist in preparing codes, run tests as necessary. Will use Synopsis Symphony (instead of Catapult C).

Volunteer:  TI. (Mangesh Sadafale) Cross-check settings, configurations, input code (review RTL).

---------------------------------------------------------

## 3.3.3. SW (SIMD) - Intel Nehalem Platform, Linux OS.

Test

1.  Common Case.  Starting Assumption: complexity in SIMD is a matter of the architecture of the transform - whether it is Matrix Multiply, Partial Butterfly, or Full Factorization.

2.  Test. One of each architecture type: Matrix Mult, Partial Butterfly, and Full Factorization.

3.  Justify the Assumption with evidence.

Example: Test all full factorization 8-pt transforms, show equivalence.

4.  (Optional) Test fast matrix multiplication algorithms (e.g., Strassen) for SIMD implementation.

Volunteer: David Flynn (BBC).

9/2/11.  Test Results:

a. Clock cycles to do each size transform (2D).

b. Code size (bytes) for each transform.

# 4  Timeline  Teleconferences / Milestones

8/5/11.    Discuss code drop, issues. Confirm test plans.

8/19/11.  Discuss partial progress, share intermediate results, and feedback from proponents.

8/26/11.  Any issues in progressing SIMD test.

9/2/11.   Test conclusion and distribution of results. Confirm cleaned up C code dist. for HW test.

9/20/11.  Any feedback from proponents on SIMD.  Completion of HW test and feedback from cross-checkers.

9/20/11 – 11/5/11.  Any discussion.

11/5/11.  Upload of CE Report and Test Results.