| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F911 |
| --- | --- |

| Title: | Description of Core Experiment (CE11): Coefficient scanning and coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Vivienne Sze (primary) Tung Nguyen Jianle Chen Joel Sole Krit Panusopone | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-214-567-4005 sze@ti.com +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de +82-31-279-8826 jianle.chen@samsung.com +1-858-845-2429 joels@qualcomm.com +1-858-404-3618 krit@motorola.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document defines Core Experiment 11 (CE11) on coefficient scanning and coding, to be performed for the upcoming November 2011 JCT-VC meeting. The CE is divided into 2 subtests. The 1st subtest consists of experiments on coefficient coding; specifically context modeling/selection for syntax elements related to transform coefficients. The 2nd subtest consists of experiments on various coefficient scanning methods for CAVLC only, CABAC only and for both CAVLC and CABAC.

# Introduction

Various methods have been proposed for coding the transform coefficients of the residual signal to reduce complexity and/or improve the coding efficiency. This core experiment evaluates the coding efficiency and complexity impact of:

- Context modeling/selection for syntax elements related to transform coefficients in HE configuration

- B.	Transform coefficient scanning order methods for CAVLC and/or CABAC.

The objectives of this CE include:

- Investigation of the impact of context selection schemes for transform coefficient coding on the coding efficiency and complexity

- Coding efficiency and complexity measurement of transform coefficient scanning order methods

Complexity measures are described in Section 6.2.

# Participants List

| Company | Participants | Proponent | Cross-Checker |
| --- | --- | --- | --- |
| TI | Vivienne Sze, sze@ti.com | X | X |
| Qualcomm | Marta Karczewicz, martak@qualcomm.com Liwei Guo, liweig@qualcomm.com Yunfei Zheng, zhengy@qualcomm.com Xianglin (Shawn) Wang, xianglin@qualcomm.com Joel Sole, joels@qualcomm.com | X | X |
| Bejing Univ. Tech | Wenpeng Ding, wpding@gmail.com | X |  |
| Motorola Mobility | Yue Yu, yyu@motorola.com | X |  |
| Huawei | Jin Song, andy.songjin@huawei.com Haitao Yang, haitao.yang@huawei.com Haoping Yu, haoping.yu@huawei.com | X |  |
| Canon | Chris Rosewarne, Chris.Rosewarne@cisra.canon.com.au | X | X |
| MediaTek | Xun Guo, xun.guo@mediatek.com Shih-Ta Hsiang, shih-ta.hsiang@mediatek.com | X |  |
| Samsung | Ankur Saxena, asaxena@sta.samsung.com Jianle Chen, jianle.chen@samsung.com v.seregin@samsung.com, tammy.lee@samsung.com |  | X |
| Mitsubishi | Kazuo Sugimoto, Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp Akira Minezawa, Minezawa.Akira@ds.MitsubishiElectric.co.jp |  |  |
| I2R | Yeo Chuo Hao, chyeo@i2r.a-star.edu.sg |  | X |
| Nokia | Jewon Kang, ext-jewon.kang@nokia.com |  |  |
| Panasonic | Hisao Sasai, sasai.hisao@jp.panasonic.com |  | X |
| Sony | Cheung Ayeung, Cheung.Auyeung@am.sony.com |  | X |
| HHI | Tung Nguyen, tung.nguyen@hhi.fraunhofer.de |  | X |
| LGE | Seungwook.park@lge.com yongjoon.jeon@lge.com jy.park@lge.com |  |  |

# Tools under Test

The following tools will be considered:

| Topic | Proponent | Document | Tool description | Cross-checker |
| --- | --- | --- | --- | --- |
| Context Reduction | TI | JCTVC-F132 | Reduction in context used for significant_coeff_flag and coefficient level | Qualcomm |
| Context Reduction | Bejing Univ. Tech. | JCTVC-F148 | Context simplification for coefficients entropy coding | TI |
| CAVLC Scan | Qualcomm | JCTVC-F612 | Modifications to intra blocks coefficient coding with VLC | I2R |
| CAVLC Scan | Huawei | JCTVC-F501 | Mode dependent coefficient scan for inter blocks | Samsung, Motorola Mobility |
| CABAC Scan | Qualcomm | JCTVC-F550 | Removal of the parsing dependency of residual coding on intra mode | TI, Panasonic |
| CABAC Scan | Qualcomm | JCTVC-F552 | Parallel Processing of Residual Data in HE | Canon |
| CABAC Scan | Canon | JCTVC-F186 | Predicted neighbour for context selection of significant_coeff_flag for parallel processing for 16x16 and 32x32 TUs | Qualcomm |
| CAVLC & CABAC Scan | Motorola Mobility | JCTVC-F569 | Adaptive Scan for Large Blocks for HEVC | HHI |
| CAVLC & CABAC Scan | MediaTek | JCTVC-F124 | Extended Mode-Dependent Coefficient Scanning | Sony |

## Sub Experiment CE.A: Context modeling/selection for transform coefficient related syntax elements

### JCTVC-F132: “Reduction in contexts used for significant_coeff_flag and coefficient level”, V. Sze (Texas Instruments)

JCTVC-F133 proposes removal of several contexts used for CABAC entropy coding of syntax elements significant_coeff_flag coding and coefficient level.  In this core experiment, only the context reduction of coefficient level is tested. The contexts for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag are each reduced by changing the threshold of c1 and c2.

### JCTVC-F148: “Context simplification for coefficients entropy coding”, X. Che, W. Ding, Y. Shi (Beijing Univ. Tech.)

JCTVC-F148 proposes to reduce the contexts used for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag. The contexts with similar probability will be grouped together to reduce the number of contexts needed.

## Sub Experiment CE.B: Coefficient Scanning Methods

### CE.B1: CAVLC only

#### JCTVC-F612: “Modifications to intra blocks coefficient coding with VLC”, M. Karczewicz, Y. Zheng, L. Guo, X. Wang (Qualcomm)

In HM3.0, for block 16x16 and larger, only zigzag scan is used. In F612, it’s proposed to extend the vertical and horizontal scan to 16x16 and 32x32 block for CAVLC.

#### JCTVC-F501: “Mode dependent coefficient scan for inter blocks”, J. Song, M. Yang, H. Yang, H. Yu, X. Zheng (Huawei)

A mode dependent coefficient scan method is proposed for inter-coded blocks in JCTVC-F501. For all TUs inside a CU, one scan pattern out of the three scan patterns, i.e. zigzag, horizontal, and vertical scans, is selected based on the PU partition mode of the current CU. Additional test will be performed using new horizontal and new vertical scans as described in JCTVC-F569.

### CE.B2: CABAC only

#### JCTVC-F186: “Predicted neighbour for context selection of significant_coeff_flag for parallel processing for 16x16 and 32x32 TUs”, C. Rosewarne, M. Maeda (Canon)

This contribution presents a method for context selection, modified from the approach integrated in HM-3.0. The context selection is modified when horizontal or vertical scan patterns are used in the 16x16 or 32x32 Transform Unit. In these locations, a value for a neighbouring significant coefficient flag, immediately preceding a current significant coefficient flag along the scan pattern, is predicted. Use of the predicted value instead of the neighbouring value enables context index determination to occur earlier in the bitstream parsing process, facilitating parallel processing of the significance map.

JCTVC-F550: "Removal of the parsing dependency of residual coding on intra mode", J. Sole, Y. Zheng, W.-J. Chien, X. Wang, R. Joshi, M. Karczewicz (Qualcomm)

In HEVC, parsing the residual data in CABAC depends on the 2-D coefficient location in a block (X and Y coordinates) rather than the 1-D coefficient index relative to the scanning order. The scan pattern to apply depends on the intra prediction mode, so the parsing process cannot be performed without deriving the intra mode. This contribution studies the removal of this dependency by making the significance map coding dependant on the scan position instead of block-based position and the last coefficient coding independent of the scan.

#### JCTVC-F552: “Parallel Processing of Residual Data in HE”, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)

Coding of the coefficient level in HEVC is performed in 3 passes on subsets of 16 coefficients in a reverse scan order. JCTVC-F552 extends these passes to include the significance map coding by interleaving significance map and coefficient level coding for each subset. The subset size can be varied from 1 to the TU size with nearly identical performance. Several values for the subset size are analyzed in the CE.

### CE.B3: CAVLC & CABAC

#### JCTVC-F569: “Adaptive Scan for Large Blocks for HEVC”, Y. Yu, K. Panusopone, J. Lou, L. Wang (Motorola Mobility)

The contribution JCTVC-F569 by Motorola Mobility proposed an adaptive scan for big block size for intra coding. Specially, reverse zigzag, new horizontal or new vertical scan is used for big block size for low complexity configuration. For high efficiency configuration, forward zigzag, new horizontal or new vertical scan is used for significant map. For level coding, a big block is partitioned into multiple 4x4 blocks. Within each 4x4 block, reverse zigzag, horizontal or vertical scan is used. Moreover, forward zigzag, horizontal or vertical scan is also used to scan multiple 4x4 blocks within a big block. In this CE, more tests will be conducted to investigate coefficient scan for intra TU using either zigzag scan or wavefront scan in combination with new horizontal and new vertical scans with implicit signaling through its prediction direction.

#### JCTVC-F124: “Extended Mode-Dependent Coefficient Scanning”, X. Zhao, X. Guo, M. Guo, S. Lei (MediaTek), S. Ma, W. Gao (PKU)

In this contribution, an extended mode-dependent coefficient scanning (MDCS) method is proposed. Based on the MDCS in HM 3.0, this proposal makes two extensions. First, a modified MDCS is applied to larger block sizes including 16×16 and 32×32, in which the horizontal scanning order and vertical scanning order are different from the current ones. The new horizontal and vertical scanning orders are offline derived using an image correlation model. And the horizontal order can be achieved by transposing the vertical order. A new mode dependent mapping table is also used. Second, MDCS is also applied for scanning of the absolute values in CABAC, which only uses reverse zigzag scanning now.

# Software and Configuration

## Software

The CE will use the HM-4.0 software that is recommended by the Software Ad Hoc group (JCTVC-F900). Proponents are requested to provide software that is can be compiled under Windows and Linux platforms. Also, the proponents are requested to provide software that fulfils the HEVC software guidelines (JCTVC-F688).

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended by the Software Ad Hoc group.

# Timeline and Responsibilities

HM 4.0 software available within four weeks after the meeting (Aug. 23 , 2011) – Note: software release delayed by 4 days.

=

| T1 | Aug, 19, 2011 | Final CE-description |
| --- | --- | --- |
| T2 | Three weeks after HM-4.0 software release (Sept. 13 , 2011) | Completion of tool integration; release software & results for cross-check |
| T3 | Two weeks before start of meeting (Nov. 8 , 2011) | Complete cross-check; input document upload |

# Definition of Performance Measurement Criteria

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment.

## Coding Performance Measurements

The objective rate-distortion measurements using 4-point BD-PSNR and BD-Rate for all tools in section 3 are used under common conditions.

Proposals in sub experiment CE.A, as per the request of the BoG on context reduction (JCTVC-F746), should also be tested with low QP and with RDOQ off. Context reduction methods should be test with high efficiency configurations only.

Proposals in sub experiment CE.B1 relating to coefficient scanning methods should be tested with low complexity configurations only.

Proposals in sub experiment CE.B2 relating to coefficient scanning methods should be tested with high efficiency configurations only.

Proposals in sub experiment CE.B3 relating to coefficient scanning methods should be tested with both high efficiency and low complexity configurations.

## Complexity Measurements

For the complexity measurement, the anchor, the reference software and the reference software with the proposed method implemented will be executed on the same machine and the computational time will be measured. A time factor will then be calculated between the reference software and the reference software with the proposed method implemented.

However, the complexity analysis does not solely rely on the encoding and decoding time measurements of a particular implementation (reference software) of the working draft (e.g. should describe need for LUT, additional logic, memory access, memory storage, etc.)

Core experiment participants who investigate parallelization aspects are requested to provide a description showing how the parallelization is improved and which architecture(s) would see the benefit of the parallelization. Average and worst case throughput gain from parallelized methods should be considered.

In addition, any tools proposing a different method of scanning are requested to report

- Scheme of selecting the scan pattern during encoding needs to be described.

- Scheme of updating the scan pattern during parsing needs to be described.