| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F912_r3 WG11 Number: m20237 |
| --- | --- |

| Title: | Description of Core Experiment 12: Deblocking Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Andrey Norkin (primary) Xun Guo Byeungwoo Jeon Matthias Narroschke | Email: | andrey.norkin@ericsson.com xun.guo@mediatek.com bjeon@skku.edu matthias.narroschke@eu.panasonic.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

The goal of this Core Experiment (CE) is to enhance deblocking filtering techniques in the HEVC Test Model. This is not limited to improving coding efficiency and subjective quality, but also reducing complexity, parallelization and harmonization of various schemes that are technically feasible. Techniques covered by this CE are deblocking filter improvements, deblocking filter parallelization, line memory reduction, complexity reduction and high-level syntax for signaling the deblocking filter parameters.

The contributions in this CE are organized into the following subtests:

Subtest 1: Subjective and objective quality improvements.

- Deblocking for large size blocks (JCTVC-F198, Microsoft)

- Chroma deblocking filter (JCTVC-F118, MediaTek, Ericsson)

- Simplified chroma deblocking (JCTVC-F120, MediaTek)

- Deblocking filter adaptive to intra-prediction directions (JCTVC-F258, JCTVC-E417, SKT/SKKU)

- Deblocking of new non-square blocks (JCTVC-F556, Qualcomm)

- Modification of decisions for deblocking (JCTVC-F191, Panasonic)

- Unification of luma and chroma filtering (JCTVC-F256, part 1, TI)

Subtest 2: Parallel deblocking decision solutions.

- Both horizontal and vertical filtering applied to reconstructed samples (JCTVC-F214 tool 1, Sony)

- Filter decision based on central non-deblocked samples in block boundary (JCTVC-F214 tool 2, Sony)

- Investigating parallel decisions (JCTVC-F256, part 2, TI)

Subtest 3: Line memory reduction.

- Vertical tap length reduction to reduce line memory (JCTVC-F215, Sony)

- Line memory reduction by reducing pixels above horizontal LCU edges (JCTVC-F053, MediaTek)

- Motion data line memory reduction by 2:1 motion data compression (JCTVC-F060, MediaTek)

Subtest 4: Reduction of computational complexity of deblocking filter.

- Decreasing worst-case complexity of Bs decisions (JCTVC-F484, TI)

Subtest 5: Controlling strength of deblocking filter at slice level.

- Deblocking filter parameter adjustment at slice level (JCTVC-F143, Toshiba, Qualcomm)

- Signaling of boundary filtering strength of deblocking filter for intra blocks (JCTVC-F175, Mitsubishi Electric)

- Deblocking filter using adaptive weighting factors (JCTVC-F405, Panasonic)

For the proposals in this CE, the impact of modifications on subjective and objective quality is evaluated. The complexity of the proposed schemes relative to the current HEVC deblocking is also evaluated. In addition, subjective viewing should be conducted according to the suggestions of viewing experts. 

The proponents are encouraged to test harmonization of various proposals in this CE in order to find the best solution and provide a basis for convergence of the proposals towards a recommendation.

The software basis for this CE is HM 4.0, and the experiments are conducted based on the test conditions in JCTVC-F900 [1].

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 | Andrey Norkin (primary coordinator) Shawmin Lei Xun Guo (coordinator) Yu-Wen Huang Jicheng An Qian Huang Jizheng Xu Xiaoyan Sun Matthias Narroschke (coordinator) Jeongyeon Lim Jungyoup Yang Byeungwoo Jeon (coordinator) Kenneth Andersson Teruhiko Suzuki Masaru Ikeda Tomoo Yamakage Vadim Seregin In Suk Chong Geert Van Der Auwera Minhua Zhou TK Tan Jani Lainema Oguz Bici Jianle Chen Vittorio Baroncini Simon Oudin Han Yu Chih-Wei Hsu Mangesh Sadafale Sugimoto.Kazuo Shun-ichi Sekiguchi Chih-Wei Hsu Chia-Yang Tsai Zhongbo Shi Kwanghyun Won Jinho Lee Hui Yong Kim Sung-Chang Lim Tzu-Der Chuang Ching-Yeh Chen Seungwook Park Yongjoon Jeon Joonyoung Park Sukho Lee | Ericsson MediaTek MediaTek MediaTek MediaTek MediaTek Microsoft Microsoft Panasonic SK Telecom SKKU SKKU Ericsson Sony Sony Toshiba Samsung Qualcomm Qualcomm Texas Instruments NTT Docomo Nokia Nokia Samsung FUB HHI Intel MediaTek Texas Instruments Mitsubishi Electric Mitsubishi Electric MediaTek MediaTek USTC SKKU ETRI ETRI ETRI MediaTek MediaTek LG LG LG ETRI | andrey.norkin@ericsson.com shawmin.lei@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com jicheng.an@mediatek.com qian.huang@mediatek.com jzxu@microsoft.com xysun@microsoft.com Matthias.Narroschke@panasonic.de jeongyeon@sktelecom.com binbak1@ece.skku.ac.kr bjeon@skku.edu kenneth.r.andersson@ericsson.com teruhikos@jp.sony.com Masaru.Ikeda@jp.sony.com tomoo.yamakage@toshiba.co.jp v.seregin@samsung.com ichong@qualcomm.com geertv@qualcomm.com zhou@ti.com ttk@pacific.net.sg jani.lainema@nokia.com ext-oguz.bici@nokia.com jianle.chen@samsung.com vittorio@fub.it simon.oudin@hhi.fraunhofer.de yu.han@intel.com cw.hsu@mediatek.com mangesh@ti.com Sugimoto.Kazuo@ ak.MitsubishiElectric.co.jp Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp cw.hsu@mediatek.com chiayang.tsai@mediatek.com v-zhshi@microsoft.com wkh12345@gmail.com jinosoul@etri.re.kr hykim5@etri.re.kr sclim@etri.re.kr peter.chuang@mediatek.com chingyeh.chen@mediatek.com seungwook.park@lge.com yongjoon.jeon@lge.com jy.park@lge.com shlee99@etri.re.kr | Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent |

# Experimental Conditions

  - Software

HM 4.0 shall be used (will be provided on August 19, 2011).

  - Test Sequences, Bit Rates and Coding Conditions

The recommended sequences, configurations and test cases described in JCTVC-F900 will be used. Providing results for additional sequences is encouraged but not mandated. Results for all six mandatory test cases shall be provided.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and complexity as well as subjective quality. Complexity assessment is performed on the base of decoding complexity measurement. For complexity assessment, HM 4.0 software with the anchor configuration and the proposal implemented on the HM 4.0 software shall be used. For decoder complexity measurements, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

Subjective viewing during the Geneva meeting should be performed and results provided during the meeting in order to ensure that the proposals do not cause degradation of subjective quality. The test sequences and test conditions should be determined before the beginning of cross-check.

# Subtest 1. Subjective and objective quality improvements

## Functionality addressed

- Reduction of blocking artifacts

- Improvement of coding efficiency

- Improvements in subjective quality

## Expected gains

- Subjective quality improvement

- BD-rate reduction

## Evaluation criteria

- Subjective video quality

- BD-rate relative to the anchor(s)

- Complexity (decoding time measurement, memory storage, etc.)

Note 1: HM 4.0 shall be used as the anchor. 

Note 2: Cross-checking includes informal viewing of proposals. All cross-checkers shall report the informal viewing results (without scoring) and their equipments (e.g., monitor, etc). If a room, equipment, and time-slot for informal viewing are available at the Geneva meeting, informal viewing will be conducted. The test sequences, QP values, and coding conditions for the potential informal viewing at the Geneva meeting are chosen from the sequences, configurations and test cases described in JCTVC-F900. The proponents should be ready to provide the requested sequences. The selection should be conducted by CE12 participants including non-proponents.

Note 3: The proponents are encouraged to provide additional results for separate tools (groups of tools) in their proposals if the proposal consists of several independent modifications of the HM software. The possibility to turn on and off these groups of tools should be provided in the software. This specifically applies to adjustments of the thresholds and tables as well as to modifications to the encoder control and implementations, which could also be applied to improve the anchor. The cross-checkers should have a possibility to assess, which part of the coding gain comes from the thresholds/tables adjustment or from modifications to the encoder control or implementations. The cross-checkers are encouraged to perform the subjective viewing of the separate tools/combinations of tools in order to ensure that removing or adding certain tools from the proposal does not degrade the subjective quality.

## Deblocking for large size blocks (JCTVC-F198, Microsoft)

There are two deblocking mode in the proposed scheme, the HEVC deblocking mode (Mode A), which performs exactly the same filtering as in the current HEVC deblocking, and the extra smoothing deblocking mode (Mode B), which performs a strong and long distance filtering across certain transform unit (TU) boundaries. The details of Mode B are introduced as follows.

The variances of 16x16 sub-blocks in a LCU (64x64) will be calculated as following way and used for deblocking mode decision. An extra smoothing deblocking mode (Mode B) is supposed to be applied in large smooth regions with small variation.

For deblocking Mode B, a strongly filtering is performed along lines perpendicular to the TU boundary.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

  - Chroma deblocking filter (JCTVC-F118, MediaTek/Ericsson)

### General Concept

In this contribution, a modified deblocking filter for chroma is proposed, which was originally proposed in JCTVC-E079.  There are three major modifications. First, instead of only filtering on a 8x8 chroma grid as in HEVC, this contribution proposes to filter on a chroma grid that adapts to the chroma sampling format. For the 4:2:0 sampling format, chroma filtering is performed on a 4x4 grid. Second, a filtering decision is added for chroma boundaries to make sure the accuracy of the filter, which is performed independently for U and V components. For 4:2:0 sampling format, the filtering decision is done on the 4-pixel block boundary. And the 2nd and 3rd lines are used in the filtering decision. Third, the filtering coefficients are also revised to further improve the performance.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

  - Simplified chroma deblocking (JCTVC-F120, MediaTek)

### General Concept

A modified chroma deblocking filter is proposed in JCTVC-F118, where the filtering unit size for chroma is determined according to the sampling format. Moreover, filtering decision is also added for chroma boundaries. This contribution is to unify the chroma filtering decision in JCTVC-F118 with luma component. For inter frames, such as P and B frames, the filtering decision of chroma is exactly the same as that of luma. For intra frames, chroma first reuses the gradient computed for luma to compare with a new threshold. Then the filtering decision is made based on the comparison.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

## Deblocking filter adaptive to intra-prediction directions (JCTVC- F258, JCTVC-E417, SKT/SKKU)

### General concept

For intra coded blocks, bS has a value of 3 or 4, the bS value is utilized to control the strength of deblocking filter. It is proposed to derive bS value for intra blocks based on relative directions of intra prediction and/or deblocking block boundary. Also, for intra blocks, different tc values are proposed to be defined for different bS values. Also, the weak filter in HM is proposed to be modified as in JCTVC-F258.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

  - Deblocking of new non-square blocks (JCTVC-F556, Qualcomm)

### General concept

Similar to SDIP (short-distance intra-prediction), NSQT (non-square quadtree transform ) and AMP (asymmetric motion partitioning) can split CUs  into non-square PUs or TUs such as 16x4, 4x16. The present deblocking filter in the HM addresses blocking artifacts on the 8x8 minimum block basis and does not deblock artifact edges located at positions that are not multiple of 8 pixel locations. The HM deblocking filter also supports parallel implementations of deblocking decisions and independent filtering for vertical or horizontal edges. The present HM deblocking decisions and filters have supports (number of pixels used for computations or to be modified) that maximally employ 4 pixels on each side of the edge. If a non-square block has a dimension that is smaller than 8, the deblocking decisions and filters potentially overlap between nearby edges and filtering dependencies may be introduced. Therefore, the goal of this experiment is to develop a solution for deblocking of, for example, 16x4 or 4x16 blocks, while improving subjective/objective quality and maintaining the properties of the HM deblocking filter such as parallelism.

### Bitstream organization

No need to modify the bitstream syntax (i.e. only requires the modification of decoding process).

  - Modification of decisions for deblocking (JCTVC-F191, Panasonic)

### General concept

A modification of the decisions of the deblocking filter is evaluated. For the decision to enable or disable the deblocking filter, this includes the evaluation of the gradient change at the block boundary and performing individual decisions for each line.

### Bitstream organization

No modification of the bitstream syntax, only the decoding process is modified.

  - Unification of luma and chroma filtering (JCTVC-F256, part 1, TI)

### General concept

This contribution proposes a unification of luma and chroma filter. This includes having same filter coefficient precision, similar filter length and apply filter to Inter chroma edge with similar filter decision.

### Bitstream organization

No modification of the bitstream syntax, only the decoding process is modified.

### Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Xun Guo xun.guo@mediatek.com |
| Microsoft | Jizheng Xu jzxu@microsoft.com Xiaoyan Sun xysun@microsoft.com |
| SKT/SKKU | Byeungwoo Jeon bjeon@skku.edu Jungyoup Yang binbak1@ece.skku.ac.kr |
| Ericsson | Andrey Norkin andrey.norkin@ericsson.com |
| Panasonic | Matthias Narroschke Matthias.Narroschke@eu.panasonic.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com Geert Van Der Auwera geertv@qualcomm.com |
| TI | Mangesh Sadafale mangesh@ti.com |
| ETRI | Jinho Lee jinosoul@etri.re.kr Hui Yong Kim hykim5@etri.re.kr Sung-Chang Lim sclim@etri.re.kr Sukho Lee shlee99@etri.re.kr |
| Sony | Teruhiko Suzuki teruhikos@jp.sony.com Masaru Ikeda Masaru.Ikeda@jp.sony.com |

Cross-check assignment

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| MediaTek/Ericsson (F118) | Microsoft |
| MediaTek (F120) | Microsoft |
| SKT/SKKU (F258) | ETRI, Ericsson |
| Microsoft (F198) | MediaTek |
| Panasonic (F191) | Sony, Ericsson |
| Qualcomm (F556) | MediaTek |
| TI (F256 part 1) | Sony |

# Subtest 2: Parallel deblocking decisions

  - Functionality addressed

- Filter decisions friendly to parallel processing and/or complexity

  - Expected gains

- Balance of both flexibility of parallel processing and complexity without subjective and objective quality degradation

  - Evaluation criteria

- BD-rate relative to the anchor

- Complexity (decoding time measure, memory storage, etc.)

- Impact on subjective quality

- Degree of achieved parallelization

- Number of operations in the critical path of the deblocking filter

- Comparison with decisions on both reconstructed and unreconstructed samples (HM-2.0 and JCTVC-E224)

Note 1: HM 4.0 shall be used as the anchor.

Note 2: Cross-checking includes informal subjective viewing of proposals. All cross-checkers shall report the informal viewing results (without scoring) and their equipments (e.g., monitor, etc). If a room, equipment, and a time-slot for informal viewing are available in the Geneva meeting, informal viewing will be conducted. The selection of test sequences, QP values, and coding conditions for the potential informal viewing in the Geneva meeting are decided before the cross-check start date. The selection should be conducted by CE12 participants including non-proponents.

  - Both horizontal and vertical filtering applied to reconstructed samples (JCTVC-F214 tool 1, Sony)

This proposal applies both H and V filtering processes to reconstructed samples as shown in the figure below. The output samples for luma are obtained by the weighted sum of the output of both H and V filter. The weights are fixed values that depend on the position from each block boundary. Both H and V filtering processes use the same filter and decisions as HM-3.0 respectively, and none of the parameters β, tc etc. are changed. Therefore it is possible to apply this proposal to a deblocking filter that is modified as suggested in the other contribution. The output samples for chroma are also obtained by sum of both H and V filter. The coefficient for the addition is also a fixed value depending on the position from each block boundary.

Figure 1: Both H and V filtering process

  - Filter decision based on central non-deblocked samples in block boundary (JCTVC-F214 tool2, Sony)

This proposal modifies the decision for reconstructed samples by changing the samples used for filtering decisions to the central samples only (Line#3 and Line#4) as shown in the Figure below. The decisions include both “deblocking on/off decision” and “strong/weak filter selection”.

Deblocking on/off decision is performed by the central samples only. Strong/weak filter selection for 8 lines is also decided by the central samples only. All parameters (β, tc and so on) are not changed.

Figure 2: parallel decision with central lines

  - Investigating parallel decisions (JCTVC-F256, Part-2, TI)

This contribution proposes the filter decisions to be done in the same way as in HM-2.0, as it removes the sequential dependency on horizontal decision (ON/OFF and strong/weak) and horizontal filtering.

This proposal also requests to check the other options to avoid the effect of “parallel decision” in the CE.

## Participants

| Participant | Contact |
| --- | --- |
| Sony | Teruhiko Suzuki teruhikos@jp.sony.com Masaru Ikeda Masaru.Ikeda@jp.sony.com |
| TI | Mangesh Sadafale mangesh@ti.com |
| Panasonic | Matthias Narroschke matthias.narroschke@eu.panasonic.com |
| Ericsson | Andrey Norkin andrey.norkin@ericsson.com |
| Samsung | Vadim Seregin v.seregin@samsung.com Jianle Chen jianle.chen@samsung.com |

Cross-check assignment

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| Sony tool 1 (F214) | Panasonic, TI |
| Sony tool 2 (F214) | Panasonic, TI, Ericsson |
| TI (F256 part 2) | Panasonic, Samsung |

# Subtest 3: Line memory reduction

## Functionality Addressed

- Reduction of required line memory and complexity

## Expected Gains

- Balance of both flexibility of parallel processing and complexity without subjective and objective quality degradation

## Evaluation Criteria

- BD-rate relative to the anchor(s)

- Complexity (decoding time measure, control, etc.)

- Impact on subjective quality

- Degree of achieved line memory reduction

Note 1: HM 4.0 shall be used as the anchor.

Note 2: Cross-checking includes informal viewing of proposals. All cross-checkers shall report the informal viewing results (without scoring) and their equipments (e.g., monitor, etc). If a room, equipment, and a time-slot for informal viewing are available at the Geneva meeting, informal viewing will be conducted. The selection of test sequences, QP values, and coding conditions for the potential informal viewing in the Geneva meeting are decided before the cross-check start date. The selection should be conducted by CE12 participants including non-proponents.

  - Vertical tap length reduction to reduce line memory (JCTVC-F215, Sony)

This proposal suggests reduction of a vertical filter tap length to reduce the size of deblocking line buffer. Vertical filtering for a horizontal edge is asymmetrical with less filter taps used for the upper block.

  - Line memory reduction by reducing pixels above horizontal LCU edges (JCTVC-F053, method 2,  MediaTek)

In the method 2 of JCTVC-F053 proposal, pixels from the upper side of the edge (p’ pixels) are not used in the filtering decisions for horizontal LCU edges. Therefore, the proposed filtering on/off decision is done according to:

d=q'22−2⋅q'12+q'02+q'25−2⋅q'15+q'05<β2

The proposed strong/weak filter selection is done according to:

d<β>>3andq'3i−q'0i<β>>4andp'0i−q'0i<5⋅tc+1>>1

where p’ and q’ pixels are reconstructed pixels. Moreover, the filtering operations for both luma and chroma are also modified for horizontal LCU edges.

## Motion data line memory reduction by 2:1 motion data compression (JCTVC-F060, MediaTek)

In the JCTVC-F060 proposal, a 2:1 motion data compression method to reduce the motion data line memory is proposed. The motion data includes the inter prediction direction, reference picture index, and motion vector (MV). In JCTVC-F060, for every two 4x4 blocks, one piece of the motion data of these two 4x4 blocks can be stored in the line buffer. The stored motion data represents the motion data of these two blocks. Since the motion data are compressed, it will affect the boundary strength decision of deblocking filtering. The effects of the proposal on subjective quality are studied in this CE. The 2:1 motion data compression will be tested with three different selection methods: always taking left, always taking right, and taking left and right in interleaving. The boundary strength decision will also be modified accordingly to enhance the subjective quality. The boundary strength decision can only take the decompressed motion data.

## Participants

| Participant | Contact |
| --- | --- |
| Sony | Teruhiko Suzuki teruhikos@jp.sony.com Masaru Ikeda Masaru.Ikeda@jp.sony.com |
| MediaTek | Yu-Wen Huang yuwen.huang@mediatek.com Chih-Wei Hsu cw.hsu@mediatek.com Chia-Yang Tsai chiayang.tsai@mediatek.com Tzu-Der Chuang peter.chuang@mediatek.com Ching-Yeh Chen chingyeh.chen@mediatek.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com Geert Van Der Auwera geertv@qualcomm.com |

Cross-check assignment

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| Sony (F215) | MediaTek |
| MediaTek (F053) | Sony |
| MediaTek (F060) | Qualcomm |

# Subtest 4. Reduction of computational complexity of deblocking filter

## Functionality addressed

- Reducing computational complexity of deblocking filtering

## Expected gains

- Lower computational complexity (e.g. reducing worst case complexity)

## Evaluation criteria

- BD-rate relative to the anchor(s)

- Complexity (decoding time measurement)

- Check that no degradation of subjective video quality occurs

  - Decreasing worst-case complexity of deblocking filter (JCTVC-F484, TI)

This contribution proposes to remove motion vectors dependency in boundary strength (Bs = 1) derivation process. The proposal decreases the worst case complexity by decreasing the number of operations and memory required to calculate Bs based on motion vectors. The proposal therefore suggests removing the deblocking filtering at block boundaries when the adjacent blocks have different motion vectors pointing at the same reference frame.

### Participants

| Participant | Contact |
| --- | --- |
| TI | Mangesh Sadafale mangesh@ti.com |
| Sony | Teruhiko Suzuki teruhikos@jp.sony.com Masaru Ikeda Masaru.Ikeda@jp.sony.com |
| Ericsson | Andrey Norkin andrey.norkin@ericsson.com |

Cross-check assignment

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| TI (F484) | Sony, Ericsson |

# Subtest 5. Controlling deblocking filter strength at slice level

## Functionality addressed

- Encoder optimization capability for subjective picture quality

- High-level syntax for sending deblocking filter parameters at slice level

- Improvements in subjective quality

- Improvements in coding efficiency

## Expected gains

- Give options to encoder to control deblocking filter strength in order to improve subjective picture quality

- Improve coding efficiency

## Evaluation criteria

- Flexibility and clarity of parameter signaling

- Subjective video quality improvements

- Encoding and decoding time

- BD-rate relative to the anchor(s)

Note 1: HM 4.0 shall be used as the anchor. 

Note 2: The proponents are encouraged to test harmonization of the proposals in this subtest.

Note 3: Cross-checking includes informal viewing of proposals. All cross-checkers shall report the informal viewing results (without scoring) and their equipments (e.g., monitor, etc). If a room, equipment, and time-slot for informal viewing are available at the Geneva meeting, informal viewing will be conducted. The test sequences, QP values, and coding conditions for the potential informal viewing at the Geneva meeting are chosen from the sequences, configurations and test cases described in JCTVC-F900. The proponents should be ready to provide the requested sequences. The selection should be conducted by CE12 participants including non-proponents.

An additional test is added for evaluation of subjective effects of controlling deblocking filtering strength. In this test, higher QP values (e.g. 42 and 47) are tested to study the effectiveness of deblocking parameters adjustment. When higher QP values are tested, the parameters may be adjusted for each encoding.

  - Deblocking filter parameter adjustment in slice level (JCTVC-F143, Toshiba, Qualcomm)

### General concept

In the current HM deblocking filter, there are two tables,  and tc, that affect application and strength of deblocking filter. This experiment studies the necessity and effectiveness of introduction of deblocking filter parameter adjustment in slice level, which is a similar concept to AVC.

### Adjustment method

### In the proposed deblocking filter parameter adjustment, indices (QP) for tc tables and/or  tables are offset by variables signaled in slice header.

### Bitstream organization

 and tc offsets shall be signaled in slice header.

  - Signaling of boundary filtering strength of deblocking filter for intra blocks (JCTVC-F175, Mitsubishi Electric)

### General concept

In this scheme, a subtract offset for the boundary strength for intra luma, bSL, and a subtract offset for the boundary strength for intra chroma, bSC, are signaled at the slice header, thus the boundary strength of the deblocking filter is set max(bS-bSL, 0) for luma and max(bS-bSC, 0) for chroma.

### Bitstream organization

Parameters are coded within the slice header

  - Deblocking filter using adaptive weighting factors (JCTVC-F405, Panasonic)

### General concept

Adaptive weighting factors are introduced into the deblocking filter. Each weighting factor weights the value (delta), by which the HM deblocking filter would modify a sample value at the corresponding filtering position. The weighting factors are estimated at the encoder side. The estimated weighting factors are coded and transmitted to the receiver side.

### Bitstream organization

Weighting factors are coded within the slice header.

### Participants

| Participant | Contact |
| --- | --- |
| Mitsubishi | Sugimoto.Kazuo Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp Shun-ichi Sekiguchi Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |
| Panasonic | Matthias Narroschke matthias.narroschke@eu.panasonic.com |
| Sony | Teruhiko Suzuki teruhikos@jp.sony.com Masaru Ikeda Masaru.Ikeda@jp.sony.com |

Cross-check assignment

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| Mitsubishi (F175) | Toshiba |
| Toshiba (F143) | Mitsubishi |
| Panasonic (F405) | Sony |

# Time-line and Responsibilities

2011-Aug-19:	HM4.0 software expected to be available

2011-Aug-19:	CE12 description finalized and uploaded

2011-Sep–13:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-checker(s)

2011-Sep-17:	Proponents report results to coordinators

2011-Sep-27:	Cross-checkers report results (including code review) to coordinators

2011-Nov-8:	Contribution documents uploaded, the document number and title are reported to coordinators

2011-Nov-17:	Summary report uploaded

Changes in time-line reflect the change in HM-4.0 release date.

# Previous Contributions

JCTVC-F198	[Z. Shi, X. Sun, J. Xu (USTC, Microsoft)] CE12, Subset 1: Report of Deblocking for Large Size Blocks.

JCTVC-F118 	[A. Norkin, K. Andersson, R. Sjöberg (Ericsson), J. An, Q. Huang, X. Guo, Y.-W. Huang, S. Lei (MediaTek)] CE12: Ericsson's and MediaTek's deblocking filter.

JCTVC-F120 	[Q. Huang, J. An, X. Guo, S. Lei (MediaTek)] Simplification of Chroma Deblocking Filter in JCTVC-F118.

JCTVC-F258	[J. Yang, K. Won, B. Jeon, J. Lim (SK Telecom and Sungkyunkwan University)] CE12: SK Telecom/SKKU Deblocking Filter.

JCTVC-E417	[J. Yang, K. Won, B. Jeon, J. Lim (SK Telecom and Sungkyunkwan University)] CE12 Subset1: SKT/SKKU Deblocking Filter.

JCTVC-F191 	[M. Narroschke, T. Wedi (Panasonic)] CE12: Results for decisions for deblocking.

JCTVC-F143 	[T. Yamakage, S. Asaka, T. Chujoh (Toshiba), M. Karczewicz, I. S. Chong (Qualcomm)] ] CE12: Deblocking filter parameter adjustment in slice level.

JCTVC-F405	[M. Narroschke, A.-K. Seifert (Panasonic)] Deblocking filter using adaptive weighting factors.

JCTVC-F175	[K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] Signaling of boundary filtering strength of deblocking filter for intra.

JCTVC-F484	[M. Sadafale (TI)] Improving worst-case performance of deblocking filter

JCTVC-F556	[G. Van der Auwera, J. Sole, Y. Zheng, X. Wang, I. S. Chong, R. Joshi, M. Karczewicz (Qualcomm)] CE6.b: SDIP Harmonization with Deblocking, MDIS and HE Residual Coding.

JCTVC-F256	[M. Sadafale (TI)] Improving deblocking filter efficiency.

JCTVC-F214	[M. Ikeda, J. Tanaka, T. Suzuki (Sony)] Parallel deblocking improvement.

JCTVC-F215	[M. Ikeda, T. Suzuki (Sony)] Vertical tap length reduction to reduce line memory in deblocking filter.

JCTVC-F053	[C.-W. Hsu, J. An, X. Guo, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)] Deblocking Filter with Reduced Pixel Line Buffers for LCU-based Processing.

JCTVC-F060	[T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, and S. Lei (MediaTek)] Reducing Line Buffers for Motion Data and CABAC.

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-E900, Torino, July 2011.

- “High Efficiency Video Coding (HEVC) Test Model 4 (HM 4) Encoder Description”, JCTVC-F802, Torino, July 2011.

- “High Efficiency Video Coding (HEVC) text specification Working Draft 4”, JCTVC-F803, Torino, July 2011.