| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F913 WG11 Number: m21437 |
| --- | --- |

| Title: | Description of Core Experiment CE13: Motion data parsing robustness and throughput |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 92130 ISSY LES MOULINEAUX France | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com |
|  | B. Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin |  | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

# Introduction

This Core Experiment relates to motion data parsing robustness and throughput. During the 6th JCT-VC meeting, a simplified version of JCTVC-F470 has been adopted in HM4.0. The goal of this CE is to evaluate various configurations or simplifications of different methods proposed for HM3.0, and evaluate them in the context of HM4.0.

# Participants List

| Participant | Person, Email address | P | C |
| --- | --- | --- | --- |
| Broadcom | Y. Yu (yongyu@broadcom.com) W. Wan (wan@broadcom.com) |  |  |
| Canon | P. Onno (patrice.onno@crf.canon.fr) G. Laroche (guillaume.laroche@crf.canon.fr) |  |  |
| HHI | B. Bross (benjamin.bross@hhi.fraunhofer.de) |  |  |
| JVC Kenwood | S. Fukushima (fukushima.shigeru@jk-holdings.com) H. Nakamura (nakamura.hiroya@jk-holdings.com) H. Takehara (takehara.hideki@jk-holdings.com) M. Ueda (ueda.motoharu@jk-holdings.com) |  |  |
| MediaTek | Jian-Liang Lin (jl.lin@mediatek.com) Yi-Wen Chen (yiwen.chen@mediatek.com) Yu-Wen Huang (yuwen.huang@mediatek.com) Shawmin Lei (shawmin.lei@mediatek.com) | X |  |
| Orange Labs | J. Jung (joelb.jung@orange-ftgroup.com) |  | X |
| Samsung | J. Chen (chanelwind@gmail.com, jianle.chen@samsung.com) T. Lee (tammy.lee@samsung.com) | X |  |

Table 1: participant list.

# Parsing robustness and throughput: related contributions

JCTVC-F052 - [MediaTek] - CE9: Results of Experiment ROB04

#### This experiment evaluates the parsing robustness solution based on redundancy removing and replacement. This technique includes two tools.

#### The first tool is replacing the redundant MVPs by other substitutes, and the second tool is reducing redundant MVPs without comparing motion vector values.

#### This experiment will test each tool separately and the combination of both tools.

|  | Tool1 (Redundancy Replacement) | Tool2 (Redundancy Removing w/o comparing MV) | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- |
| Test 1 | O | X | MediaTek |  |
| Test 2 | X | O | MediaTek |  |
| Test 3 | O | O | MediaTek |  |

O: tested, X: not tested

Provide a more detailed definition of Tool2, and describe how it interacts with the robustness scheme of HM4.

JCTVC-F068 – [TI] - A study on HM3.0 parsing throughput issue

This contribution advocates adding high-level parameters in slice header to signal the maximum truncated unary table size for merge_idx and mvp_idx coding, and constraining the complexity of merge/skip list pruning process. The following tests will be carried out:

- coding efficiency study with different maximum truncated unary table sizes signaled in slice header

- reducing the number of MVP candidates for the second pruning process in the merge/skip list derivation process

- test coding efficiency of merge/skip list derivation process with single step of MVP pruning

JCTVC-F083 – [TI] - CE9:  Evaluation results on disabling TMVP from AMVP list construction (ROB05)

May be can be shifted to CE9.

In this test, the coding efficiency of TMVP in the AMVP list will be quantified. The following the following three tests will be carried out, 1) disabling TMVP from the AMVP but retaining it in ME as one of initial motion vector, 2) disabling TMVP from both the AMVP and ME, 3) disabling TMVP from AMVP, ME and merge/skip mode. Removing TMVP from the AMVP would reduce the AMVP implementation cost and improve parsing robustness.

JCTVC-F341 – [Broadcom] - Decoder Performance Restrictions due to Merge/MVP Index Parsing

Previous contributions have asserted that the present method of decoding Merge Index and MVP Index have significant negative impacts on CABAC and CAVLC performance and implementation in high-performance decoder applications.  This contribution quantifies some of this impact. It suggests that the present method of Merge/MVP Index encoding will severely limit achievable CABAC decoder bit rates and CAVLC decoder performance.

Experiments description: two tools are included in this experiment:

- Replace duplicated MVP with other simple values to reduce complexity after pruning based on information that can be derived directly from syntax values.

- Post process (for example re ordering) the MVP list based on available information by the time of MV construction.

The kind of post-processing applied needs to be clearly described.

JCTVC-F402 – [Samsung] - MVP index parsing with fixed number of candidates

This contribution targeted on improving motion information parsing robustness and throughput. In the proposed method, the candidates number of AMVP and Merge list are fixed and signaled in slice or picture head syntax. Additional motion vectors are adjunct to the current merge and AMVP list if MV candidates’ number in the list is less than the specified number. The additional MV is generated by modification based on the existing MV in the candidate list or using Zero MV. It is proposed to test the proposed method based on HM4.0 in this CE to further improve the coding efficiency.

Initial description. A much more detailed description has been requested. How does this interact with the solution adopted in HM4.

JCTVC-F474 – [Canon] CE9: Description of experiments ROB01 and ROB02

This contribution proposes two technologies to deal with the parsing robustness issue and parsing throughput issue.

The first tool is related to the determination of the number of candidates in the set. The number of candidates in the set depends only on the data which comes from the current slices and not from the data of another slice. Consequently the number of candidates is decoupled from the motion vector value.

The second tool improves the coding efficiency of the first one by replacing the duplicate candidates by new candidates with different values (non-redundant). This offers the possibility to efficiently use the signaling bits which have to be transmitted to solve the parsing issue and which may cause coding efficiency loss.

Experiment proposed:

The goal of the experiment is to find new candidates to both improve the coding efficiency of HM4.0 and limit the complexity of new derived non-duplicate candidates.

JCTVC-F713 – [Canon, MediaTek] - CE9: Result about the combination of experiments ROB02 and ROB04

#### This experiment evaluates the combination of ROB02 and ROB04 to achieve the parsing robustness. In JCTVC-F474 (ROB02), Canon presented two tools to solve the parsing issue related to the temporal motion prediction for both Inter and Skip/Merge modes. The first tool consists in forcing the availability of the temporal candidate and the second tool consists in replacing the duplicate candidates by candidates with different (non-redundant) values. In JCTVC-F052 (ROB04), a basic scheme with parsing robustness by skipping the process of removing redundant motion vector predictors (MVPs) and forcing the availability of the temporal MVP was firstly provided by MediaTek. To reduce the bit rate increases, the redundancy removing and replacement are proposed in ROB04.

This experiment will test the combination of ROB02 and ROB04 and the simplification of the combination. The combination of both experiments will be done as follows. For the Merge mode, the concept of Motion ID is used to determine the list size of motion vector predictors. If some candidates in the list are duplicates according to the Motion ID value, they are replaced by non-duplicate candidates with the same process as ROB02. Regarding the AVMP mode exactly the same process as described for ROB02 is used.

# Experiment Descriptions

To be done before CE description upload (Timeline T1)

| Experiment |  |  | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- |
| Anchor |  |  | - | - |
| A.1 |  |  |  |  |
| A.2 |  |  |  |  |
| A.3 |  |  |  |  |

Table 2: Summary of the experiments.

# Test Sequences / Test Points / SW Configuration

Simulation results will be generated for sequences and coding conditions specified in the document JCTVC-E700. Latest version of the HM (HM 4.0), available on April 15th will be used for evaluation.

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-Rate).

Report the compression factor of all the MV data (MV fields, Indexes and mode).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration)

- Decoding time (compared to the default configuration)

# Time-line

| T1 | 2011-August 5 | Final CE description and description of proposals |
| --- | --- | --- |
| T2 | HM4.0 is available | Start of tool integration |
| T3 | T2 + 2 weeks | Completion of tool integration, test configuration announcement Start of testing |
| T4 | 2011-June 22 | Testing complete Reporting of all results to CE13 participants complete |
| T5 | 2011-July 1 | Input documents uploaded |
| T6 | T5 + 3 days | CE13 summary report uploaded |

Table 3: time-line