| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F913 WG11 Number: m21437 |
| --- | --- |

| Title: | Description of Core Experiment CE13: Motion data parsing robustness and throughput |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 92130 ISSY LES MOULINEAUX France | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange.com |
|  | B. Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
|  | J. Chen SAMSUNG ELECTRONICS CO., LTD. | Tel: Email: | +82 10 279 8826 jianle.chen@samsung.com |
|  | P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 Cesson-Sevigne, FRANCE | Tel: Email: | +33 2 99 87 68 92 patrice.onno@crf.canon.fr |
|  | M. Zhou 12500 TI Blvd., MS8649 Dallas, TX-75093 | Tel: Email: | +1 214 480 3816 zhou@ti.com |
| Source: | CE coordinators |  |  |

# Introduction

This Core Experiment relates to motion data parsing robustness and throughput. During the 6th JCT-VC meeting, a simplified version of JCTVC-F470 has been adopted in HM4.0. The goal of this CE is to evaluate various configurations or simplifications of different methods proposed for HM3.0, and evaluate them in the context of HM4.0.

# Participants List

| Participant | Person, Email address | Tester | Observer |
| --- | --- | --- | --- |
| BBC | M. Mrak (Marta.Mrak@bbc.co.uk) | X |  |
| Broadcom | Y. Yu (yongyu@broadcom.com) W. Wan (wwan@broadcom.com) | X |  |
| Canon | P. Onno (patrice.onno@crf.canon.fr) G. Laroche (guillaume.laroche@crf.canon.fr) | X |  |
| ETRI | S.-C. Lim sclim@etri.re.kr J. Lee jinosoul@etri.re.kr H. Y. Kim hykim5@etri.re.kr |  |  |
| Fujitsu | K. Kazui (kazui.kimihiko@jp.fujitsu.com) | X |  |
| HHI | B. Bross (benjamin.bross@hhi.fraunhofer.de) S. Oudin (simon.oudin@hhi.fraunhofer.de) |  | X |
| I2R | T. Y. Han (yhtan@i2r.a-star.edu.sg) | X |  |
| JVC Kenwood | S. Fukushima (fukushima.shigeru@jk-holdings.com) H. Nakamura (nakamura.hiroya@jk-holdings.com) H. Takehara (takehara.hideki@jk-holdings.com) M. Ueda (ueda.motoharu@jk-holdings.com) | X |  |
| LGE | Y. Jeon (yongjoon.jeon@lge.com) S. Park (seungwook.park@lge.com) J. park (jy.park@lge.com) | X |  |
| MediaTek | J-L. Lin (jl.lin@mediatek.com) Yi-Wen Chen (yiwen.chen@mediatek.com) Yu-Wen Huang (yuwen.huang@mediatek.com) S. Lei (shawmin.lei@mediatek.com) | X |  |
| Mitsubishi | K. Sugimoto (Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp) Y. Itani (Itani.Yusuke@dx.MitsubishiElectric.co.jp) |  |  |
| Nokia | O. Bici (ext-oguz.bici@nokia.com) |  | X |
| NTT DOCOMO | Y. Suzuki (suzukiyos@rd.nttdocomo.co.jp) | X |  |
| Orange Labs | J. Jung (joelb.jung@orange-ftgroup.com) | X |  |
| Qualcomm | Y. Zheng (zhengy@qualcomm.com) X. Wang (Xianglin@qualcomm.com) | X |  |
| Panasonic | T. Sugio (sugio.toshiyasu@jp.panasonci.com) | X |  |
| Samsung | J. Chen (chanelwind@gmail.com, jianle.chen@samsung.com) T. Lee (tammy.lee@samsung.com) | X |  |
| Sony | K. Sato (Kazushi.Sato@jp.sony.com) |  | X |

Table 1: participant list.

# Parsing robustness and throughput: related contributions

## JCTVC-F052 - [MediaTek] - CE9: Results of Experiment ROB04

This experiment evaluates the parsing robustness solution based on redundancy removing and replacement. This technique includes two tools:

- The first tool is replacing the redundant MVPs by other substitutes.

- The second tool is adaptive determination of the list size.

In Tool 1, for Inter mode and Merge/Merge-Skip mode, the redundant MVP is replaced by the MVP plus a constant values or rounding the MVP to half or integer precision. Moreover, for inter mode, the MVP derived from A0, A1, B0, B1, B2 can be used as the substitutes to reach the maximum size of candidate list.

|  | Tool 1 | Tool 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Redundancy Replacement | Fixed List Size | Adaptive List Size by Motion ID | Adaptive List Size by Prediction Modes | Adaptive List Size by Merge Index | Adaptive List Size by PU Size | Tester 1 | Tester 2 |
| HM4.0 Reference | Off | (5,2) | Off | Off | Off | Off | - | - |
| Test 1 | On | (5,2) | Off | Off | Off | Off | MediaTek | Canon |

In JCTVC-F052, motion identification (ID) is first introduced to describe the inheritance process of motion information passed from a coded PU to the current PU to follow the trail of the MV. A unique motion ID is assigned for each new MV created in the Inter mode or in the Merge and Merge-Skip modes merged with a new temporal MVP or merged with the combined bi-predictive candidate, scaled bi-predictive candidate and zero vector candidate. The motion ID can be inherited through the Merge and Merge-Skip modes merged with a spatial candidate. By comparing the motion ID, the size of the MVP candidate list can then be adaptively determined.

During the inheritance process of motion ID, some coded information of neighboring PUs is utilized to decide the size of candidate list for current PU. The utilized coded information includes prediction modes (Inter or Intra), merge index, PU size and so on. To simplify the method of motion ID which adaptively adjusts the size of the candidate list, three simplified schemes of motion ID are also tested in this experiment:

- For each PU, the list size is determined by the number of neighboring PUs which are coded in Inter mode.

- For each PU, the list size is determined by the largest merge index of neighboring PUs.

- For each PU, the list size is determined according to the size of current PU.

The performance of fixed list size will also be tested to compare the coding efficiency with the adaptive list size. But the specific number of the fixed list size is not determined yet at this moment. The corresponding tests are defined as follow:

|  | Tool 1 | Tool 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Redundancy Replacement | Fixed List Size | Adaptive List Size by Motion ID | Adaptive List Size by Prediction Modes | Adaptive List Size by Merge Index | Adaptive List Size by PU Size | Tester 1 | Tester 2 |
| HM4.0 Reference | Off | (5,2) | Off | Off | Off | Off | - |  |
| Test 2a | Off | (M,N) | Off | Off | Off | Off | MediaTek |  |
| Test 2b | Off | Off | On | Off | Off | Off | MediaTek | Samsung |
| Test 2c | Off | Off | Off | On | Off | Off | MediaTek | Canon |
| Test 2d | Off | Off | Off | Off | On | Off | MediaTek |  |
| Test 2e | Off | Off | Off | Off | Off | On | MediaTek |  |

To evaluate the combination of tool 1 and tool 2, some tests are also conducted to study the combining effect:

|  | Tool 1 | Tool 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Redundancy Replacement | Fixed List Size | Adaptive List Size by Motion ID | Adaptive List Size by Prediction Modes | Adaptive List Size by Merge Index | Adaptive List Size by PU Size | Tester 1 | Tester 2 |
| Test 3a | On | (M,N) | Off | Off | Off | Off | MediaTek |  |
| Test 3b | On | Off | On | Off | Off | Off | MediaTek |  |
| Test 3c | On | Off | Off | On | Off | Off | MediaTek | Canon |
| Test 3d | On | Off | Off | Off | On | Off | MediaTek |  |
| Test 3e | On | Off | Off | Off | Off | On | MediaTek |  |

## JCTVC-F068 – [TI] - A study on HM3.0 parsing throughput issue

This contribution advocates adding high-level parameters in slice header to signal the maximum truncated unary table size for merge_idx and mvp_idx coding, and constraining the complexity of merge/skip list pruning process. The following tests will be carried out:

### Coding efficiency study with different maximum truncated unary table sizes signaled in slice header.

  - Proposed method: the maximum table size for AMVP is fixed to 2 as specified in HM4.0, the maximum code size for merge/skip MVP list, maxNumMergeCand, can vary from 1 to 5 and is signaled in slice header. The truncated unary tables used for merge_idx coding depend on the value of maxNumMergeCand. If maxNumMergeCand =1, merge_idx is not present in bitstream. Merge/skip MVP list construction remains unchanged but maxNumMergeCand varies.

  - HM4.0 method: the encoder motion estimation is modified so that the encoder selects only those merge MVP candidates with merge_idx in the range of [0: maxNumMergeCand-1]. Merge/skip MVP list construction remains unchanged.

The following tests are defined to quantify the coding efficiency difference between HM4.0 method and the proposed method when encoder can only support a reduced set of sub-pel search for merge estimation (namely 1, 2, 3, 4)

|  | maxNumMerge Cand | Truncated unary table for merge_idx coding | Encoder restriction | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- | --- |
| HM4.0 Reference | 5 | 0 / 10 / 110 / 1110 / 1111 | No | - | - |
| Test 1a | 4 | 0 / 10 / 110 /111 | No | TI | Orange Labs |
| Test 2a | 3 | 0 / 10 / 11 | No | TI | Panasonic |
| Test 3a | 2 | 0 / 1 | No | TI | Panasonic |
| Test 4a | 1 | N/A | No | TI | Panasonic |
| Test 1b | 5 | 0 / 10 / 110 / 1110 / 1111 | Yes, merge_idx restricted to [0:3] in ME | TI | Panasonic |
| Test 2b | 5 | 0 / 10 / 110 / 1110 / 1111 | Yes, merge_idx restricted to [0:2] in ME | TI | Panasonic |
| Test 3b | 5 | 0 / 10 / 110 / 1110 / 1111 | Yes, merge_idx restricted to [0:1] in ME | TI | Panasonic |
| Test 4b | 5 | 0 / 10 / 110 / 1110 / 1111 | Yes, merge_idx restricted to 0 in ME | TI | Panasonic |

### Reduction of the number of candidates for the 2nd pruning process

The 2nd pruning process removes the redundant MVP candidates from the resulting MVP candidates after the first pruning process, the combined bi-directional, non-scaled bi-directional, and ZERO MVP candidates. In the current HM4.0, the second pruning process has a total of 12 MVP candidates in the worst case. In the test, a smaller worst case number of MVPs will be investigated to lower the implementation cost of the second pruning process without harming coding efficiency. Compared to the HM4.0 design, the only change here is that the maximum number of combined bi-predictive MVP candidates is reduced. In test1 and test2 of table below the actual number of non-scaled MVP candidate, i.e. n (n<=1), and the actual number of zero MVP candidates, i.e. z (z<=2), are first determined, then the maximum number of combined bi-predictive MVP candidates is reduced from 5 to 4–n–z and 5-n-z, respectively (maxNumMergeCand fixed to 5 in the test).

|  | Maximum number of MVP candidates after the 1st pruning process | Maximum number of combined bi-predictive MVP candidates | Maximum number of non-scaled bi-predictive MVP candidates | Maximum number of zero MVP candidates | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- | --- | --- |
| HM4.0 reference | 4 | 5 | 1 | 2 | - | - |
| Test 1 | 4 | 4 – n - z | 1 (n) | 2 (z) | TI | Panasonic |
| Test 2 | 4 | 5 – n - z | 1 (n) | 2 (z) | TI | Panasonic |

### Combined tests for 1) and 2)

Will be carried out if results of 1) and 2) are promising.

|  | maxNumMergeCand | Truncated unary table for merge_idx coding | Maximum number of combined bi-predictive MVP candidates | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- | --- |
| HM4.0 Reference | 5 | 0 / 10 / 110 / 1110 / 1111 | 5 | - | - |
| Test 1b | 4 | 0 / 10 / 110 /111 | 5 – n – z or 4 – n - z | TI | Panasonic |
| Test 2b | 3 | 0 / 10 / 11 |  | TI | Panasonic |
| Test 3b | 2 | 0 / 1 |  | TI | Panasonic |
| Test 4b | 1 | N/A |  | TI | Panasonic |

## JCTVC-F341 – [Broadcom] - Decoder Performance Restrictions due to Merge/MVP Index Parsing

A larger number of additional bi-predictive MVP candidates can cause significant amount of cost in hardware decoder design. This test attempts to find the best trade-off between the number of combined and non-scaled additional MVP candidates. This test will use reduced values of maximum number of combined bi-predictive MVP candidates and maximum number of non-scaled bi-predictive MVP candidates to excise the coding efficiency and complexity trade-offs. These values are defined in HM4.0 software. In HM4.0 the combined MVP candidates are added first followed by adding the non-scaled MVP candidate. These tests will also exam the possible difference if the priority order of combined and non-scaled MVP candidates is swapped in the merge/skip MVP list.

Experiment is summarized in the following table.

|  | Maximum number of combined bi-predictive MVP candidates | Maximum number of non-scaled bi-predictive MVP candidates | Combined bi-predictive MVP candidates first | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- | --- |
| HM4.0 Reference | 5 | 1 | yes | - | - |
| Test 1a | 1 | 1 | yes | Broadcom | Panasonic? |
| Test 2a | 1 | 2 | yes | Broadcom | Panasonic? |
| Test 3a | 2 | 1 | yes | Broadcom | Panasonic? |
| Test 4a | 2 | 2 | yes | Broadcom | Panasonic? |
| Test 1b | 1 | 1 | no | Broadcom | Panasonic? |
| Test 2b | 1 | 2 | no | Broadcom | Panasonic? |
| Test 3b | 2 | 1 | no | Broadcom | Panasonic? |
| Test 4b | 2 | 2 | no | Broadcom | Panasonic? |

JCTVC-F402 – [Samsung] - MVP index parsing with fixed number of candidates

This contribution targeted on improving motion information parsing robustness and throughput. In the proposed method, the candidates number of AMVP and Merge list are fixed and signaled in slice or picture head syntax. Additional motion vectors are inserted to the current merge and AMVP list if MV candidates’ number in the list is less than the specified number. The additional MV is generated by modification based on the existing MV in the candidate list or using Zero MV. It is proposed to test the proposed method based on HM4.0 in this CE to further improve the coding efficiency. The following tests will be carried out:

- Slice-based adaptive maximum truncated unary table sizes. The table size of each slice can be different in a sequence and is signaled in slice header. It is planned to derive merge list size of current slice according to merge list candiates statistics of previous coded slices at encoder side. The specific threshold is not determined yet at this moment.

- Inserting additional bi-directional merge candidates by combination of available uni-directional motion with zero motion.

- Inserting additional merge candidates by adding ±1 offset to each component of existing MV candidate.

- Inserting additional AMVP candidates by using motion information of pre-defined position but not scanned in the first stage. As an example, the AMVP scanning order of above neighboring block is B0, B1, B2. In current design,  if B0 is used as the candidates, the information of B1 and B2 is skipped. In our design, the motion information of B1 and B2 will be utilized as additional candidates if the candidates number from the first AMVP deriving stage is less than the fixed value.

|  | Tool1 (Adaptive table size) | Tool2 (Additional bi-directional merge candidates) | Tool3 (Additional merge candidates with offset) | Tool4 (Addtinal AMVP candidates) | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- | --- | --- |
| HM4.0 Reference | Off | Off | Off | Off | - | - |
| Test 1 | On | Off | Off | Off | Samsung | Orange Labs |
| Test 2 | Off | On | Off | Off | Samsung | Canon |
| Test 3 | Off | Off | On | Off | Samsung | Fujitsu |
| Test 4 | Off | Off | Off | On | Samsung | MediaTek |
| Test 5 | On | On | On | On | Samsung | MediaTek |

## JCTVC-F474 – [Canon] CE9: Description of experiments ROB01 and ROB02

This contribution proposes two technologies to deal with the parsing robustness issue and parsing throughput issue.

- The first tool is related to the determination of the number of candidates in the set. The number of candidates in the set depends only on the data that comes from the current slices and not from the data of another slice. Consequently the number of candidates is decoupled from the motion vector value. Documents F274 and E219 described in detail the implementation of this tools.

- The second tool improves the coding efficiency of the first one by replacing the duplicate candidates by new candidates with different values (non-redundant). This offers the possibility to efficiently use the signaling bits which have to be transmitted to solve the parsing issue and which may cause coding efficiency loss. In particular, the non-redundant predictors are generated by adding offsets to the first predictor.

HM4.0 uses 3 types of new predictors for the Merge modes as described in F470: the combined predictors, the scaled predictors and the Zero predictors. All these new predictors need to be generated and compared to one another. This process significantly increases the complexity of the Merge derivation process compared to HM3.0. Especially, both combined and scaled predictors need to be generated after a first pruning process.

Experiments proposed:

2 kinds of experiments are proposed. Their goals are to improve the coding efficiency of HM4.0 and limit the complexity of new derived non-redundant candidates.

The aim of the first one is to evaluate the efficiency of Tool 2 and to limit the complexity of the motion vector derivation of the Merge modes. The main difference with F474 is that the non-redundant predictors (applied on Merge AND Inter modes) are limited to the “offset predictors” as defined in F474 and F402. These non-redundant predictors are used to replace the existing one, in order to reduce the complexity and to improve the parallelization of the Merge mode derivation process. The following table summarizes these experiments.

|  | Merge mode |  |  | AMVP/Merge |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Maximun number of combined predictors | Scaled predictors | Zero predictors | Non-redundant predictors | Tester 1 | Tester 2 |
| HM4.0 | 12 | On | On | Off | - | - |
| Test 1 | 12 | On | Off | On | Canon | ETRI |
| Test 2 | 12 | Off | Off | On | Canon | Orange Labs |
| Test 3 | 3 | Off | Off | On | Canon | Samsung |
| Test 4 | 2 | Off | Off | On | Canon | ETRI |

The aim of the second kind of experiments is to evaluate the coding efficiency of Tool 1 and Tool 2 for the Inter mode. For Tool 2, the new non-redundant predictors are generated by adding offsets to the first predictor. In first experiment, the Merge mode is not modified. In the second experiment, the best compromise between the coding efficiency and the complexity of the first kind of experiments is used for the Merge mode. In Test 3 and in Test 4, the maximum number of derived combined predictors is respectively limited to 3 and 2 instead of 12 possible combined predictors in the HM4.0 design.

|  | Merge mode | AMVP | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- |
| Test 5 | HM4.0 | Tool 1 and Tool 2 | Canon | MediaTek |
| Test 6 | 3.5 Test N (N = 1, 2, 3 or 4 ) | Tool 1 and Tool 2 | Canon | MediaTek |

# Test Sequences / Test Points / SW Configuration

Simulation results will be generated for sequences and coding conditions specified in the document JCTVC-F900. Latest version of the HM (HM 4.0) will be used for evaluation.

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-Rate).

Report the compression factor of all the MV data (MV fields, Indexes and mode).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration)

- Decoding time (compared to the default configuration)

# Time-line

| T1 | 4 weeks after the meeting (2011-August 19) | Final CE description and description of proposals |
| --- | --- | --- |
| T2 | HM4.0 is available (2011-August 26) | Start of tool integration |
| T3 | 3 weeks after HM4.0 is available (2011-September 16) | Completion of tool integration, test configuration announcement Start of testing |
| T4 | 2011-October 7 | Testing complete Reporting of all results to CE13 participants complete |
| T5 | 2011-October 21 | Input documents uploaded |
| T6 | 2011-October 28 | CE13 summary report uploaded |

Table 3: time-line