| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G002 WG11 Number: m22604 |
| --- | --- |

| Title: | JCT-VC AHG report: HEVC Draft and Test Model editing (AHG 2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Benjamin Bross (Fraunhofer HHI) Ken McCann (Samsung/ZetaCast) Woo-Jin Han (Kyungwon University) Jens-Rainer Ohm (RWTH Aachen) Shun-ichi Sekiguchi (Mitsubishi) Gary J. Sullivan (Microsoft) Thomas Wiegand (Fraunhofer HHI) | Email: | benjamin.bross@hhi.fraunhofer.de ken@zetacast.com hurumi@gmail.com ohm@ient.rwth-aachen.de Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp garysull@microsoft.com thomas.wiegand@hhi.fraunhofer.de |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC ad hoc group on HEVC Draft and Test Model editing (AHG2) between the 6th JCT-VC meeting in Torino (14-22 July, 2011) and the 7th Meeting in Geneva (21-30 November, 2011).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| HEVC Draft and Test Model editing (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-F802 HEVC Test Model 4 (HM 4) Encoder Description - Produce and finalize JCTVC-F803 HEVC text specification Working Draft 4 - Gather and address comments for refinement of these documents - Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to mismatches between software and text | B. Bross K. McCann (co-chairs) W.-J. Han J.-R. Ohm S. Sekiguchi G. J. Sullivan T. Wiegand (vice chairs) | No |

# Background

The fourth High Efficiency Video Coding (HEVC) test model (HM4) was developed from the third HEVC test model (HM3), following the decisions taken at the 6th JCT-VC meeting in Torino (14-22 July, 2011).

# Method of Working

Two editorial teams were formed to work on the two documents that were to be produced:

- JCTVC-F802 HM4: HEVC Test Model 4 Encoder Description [1]

  - Ken McCann

  - Shun-ichi Sekiguchi

  - Benjamin Bross

  - Woo-Jin Han

- JCTVC-F803 WD4: Working Draft 4 of High Efficiency Video Coding [2]

  - Benjamin Bross

  - Woo-Jin Han

  - Jens-Rainer Ohm

  - Gary J. Sullivan

  - Thomas Wiegand

Editing JCTVC-F803 was assigned a higher priority than editing JCTVC-F802.

# Status

One draft of JCTVC-F802 and six drafts of JCTVC-F803 were published by the Editing AHG between the 6th JCT-VC meeting in Torino (14-22 July, 2011) and the 7th Meeting in Geneva (21-30 November, 2011).  JCTVC-F802 still needs significant further improvement, whilst the final draft of JCTVC-F803 is reasonably complete.

The main changes in JCTVC-F803, relative to the previous JCTVC-E603, were:

- Removed inferred merge (JCTVC-F082)

- Incorporated slice header flag to disable 4x4 inter partitions (JCTVC-F744)

- Incorporated modified rounding in MV scaling (JCTVC-F142)

- Removed intermediate amvp spatial candidates redundancy check (JCTVC-F050)

- Incorporated reducing the number of spatial mv scalings to 1 (JCTVC-F088)

- Incorporated spatial merge candidate positions unification (JCTVC-F419)

- Incorporated one reference list check for temporal mvp (JCTVC-F587)

- Incorporated AMVP/merge parsing robustness with simplifications (JCTVC-F470)

- Incorporated unified availability check for intra (JCTVC-F477)

- Incorporated generic interpolation filter (JCTVC-F537)

- Incorporated non-square quadtree transform NSQT (JCTVC-F412)

- Incorporated asymmetric motion partitions AMP (JCTVC-F379)

- Incorporated CBF redundancy reduction (JCTVC-C277)

- Incorporated modified delta QP binarization (JCTVC-F745)

- Incorporated diagonal coefficient scanning in CABAC (JCTVC-F129)

- Incorporated parallel context processing for coefficient levels in CABAC (JCTVC-F130)

- Incorporated context sharing for significant_coeff_flag of 16x16 and 32x32 transforms (JCTVC-F132)

- Incorporated unified scans (JCTVC-F288)

- Incorporated sample adaptive offset (JCTVC-E049)

- Incorporated sample adaptive offset for chroma (JCTVC-F057)

- Incorporated sample adaptive offset offset accuracy (JCTVC-F396)

- Incorporated updated ALF slice padding (JCTVC-D128)

- Incorporated updated ALF slice padding due to ALF filter shape change (JCTVC-F303/F042)

- Incorporated updated ALF slice padding due to unified luma and chroma filter shapes (JCTVC-F157)

- Incorporated ALF filter using subset of pixels (JCTVC-F301)

- Incorporated modified deblocking process for luma (JCTVC-F118)

- Incorporated modified tc_offset in deblocking process (JCTVC-F143)

- Incorporated MDIS and pixel position change of planar (JCTVC-F483)

- Incorporated availability check removal for intra DC filtering (JCTVC-F178)

- Incorporated size-independent intra DC filtering (JCTVC-F252)

- Incorporated modified MDIS table (JCTVC-F126)

- Incorporated simplified intra_FromLuma prediction (JCTVC-F760)

- Incorporated fixed number of MPM (JCTVC-F765)

- Incorporated SAO boundary processing (JCTVC-F232)

- Minor bug in deriving sample positions in SAO process was fixed

- Bug in coding tree syntax table related to the initialization of variable IsCuQpDeltaCoded was fixed

- Incorporated modified last significant coefficient position coding in CABAC (JCTVC-F375)

- Incorporated modified mvd coding in CABAC (JCTVC-F455)

- Incorporated reduced number of contexts in CABAC (JCTVC-F746)

- Incorporated high-level syntax cleanup (JCTVC-F714)

- Incorporated NAL unit type and CDR (CRA) (JCTVC-F462/464)

- Incorporated adaptation parameter set (APS) (JCTVC-F747)

Some specific open issues remain for JCTVC-F803:

- Should support for monochrome, 4:2:2 and 4:4:4 (with and w/o separate color planes) be included from the start? Currently, it has been left in the text as it doesn't seem to affect much text.

- Handling of the term "frame". We are currently leaning towards changing all occurrences of "frame" to "picture" (removed or marked all occurrences of "field")

- Large size table (de-scaling matrices) is not inserted yet.

- Slice-header syntaxes and their semantics are not completed yet. Also possible modifications that maybe necessary because of larger treeblocks (64x64) compared to macroblocks (16x16) are not yet considered.

- Clipping is applied to temporally scaled motion vector in Error: Reference source not found. Do we need this? (c.f. AVC seems not)

- High-level syntax items are significantly different from software in terms of both meaning and position. Needs to be unified.

- Use of bin string and bit string should be consistent.

- MD5 hash (JCTVC-E490)

- ALF coefficient derivation from syntax elements

- Improve text quality by considering: strict use of terms “unit” and “block” – block = a rectangular 2D array (one component), unit = collective term for specifying information for both luma and chroma. Don’t use the term ‘unit’ by itself – always use the term ‘unit’ with prefix – coding unit, prediction unit or transform unit.

- Syntax functions cuAddress and granularity_block_boundary (fine granularity slices, JCTVC-E483) are missing

- Do we need SPS flag for disabling PCM mode? Currently PCM mode is turned on for all cases.

- Software seems to have SPS syntax to turn on/off combined reference list (JCTVC-D421), but currently text does not.

- Both variables IntraPredMode and IntraPredModeC are used in the syntax table but the actual derivations are specified in the decoding process. Maybe it’s better to move them to the semantics section.

- Binarization of intra_chroma_pred_mode reflecting codeword switching and luma-based chroma intra prediction (JCTVC-E266) is missing.

- Clarification on the use of intra_chroma_pred_mode and IntraPredModeC is needed. The former specifies the syntax item to indicate how to determine the chroma intra prediction mode (IntraPredModeC) as 0 (to Intra_FromLuma), 1 (to Intra_Vertical), 2 (to Intra_Horizontal), 3 (Intra_DC or Intra_Planar) or 4 (re-use luma mode). The latter specifies chroma intra prediction mode, which is actually mapped to the specific prediction process.

- The reference list construction and the decoded picture buffer management are not fully described. This is closely related to the work of the reference picture buffering and list construction AHG.

- NSQT text does not match the software w.r.t. non-square transforms. In HM, e.g.16x4 and 4x16 blocks are both transformed as 16x4 block and in case of 4x16 block, the transformed 16x4 sample are reordered to 4x16 after transform.

- NSQT mapping of coefficient level after inverse scan from square to non-square always uses zigzag scan and may be harmonized with the different scans used for Intra (horizontal, vertical), CAVLC (zigzag) and CABAC (diagonal).

- Chroma QP derivation process is missing. Can be taken from H.264/AVC and modified accordingly.

- The variable qPL used in deblocking filter process has to be defined by, e.g. setting it to QPY.

- The Chroma QP should be considered in chroma edge filtering instead of luma QP.

- The tc and beta values of the deblocking filter should be explicitly scaled in the text since their values are provided in the table for the 8-bit case.

- There is no CABAC parsing described for the following syntax elements (ae(v)): sao_flag_cb, sao_flag_cr, sao_split_flag, sao_type_idx and sao_offset.

- Deblocking filter text seems incorrect. Some horizontal edges are not filtered mainly due to the parallel deblocking process. Usage of indices xDk and XPOS also has to be cross-verified with the software behaviour.

- Software-text mismatch of JCTVC-F252: SW applies DC filtering to 64x64 intra block but WD does not.

- There are input/output parameter inconsistencies between function call and actual functions of intra prediction. Function call uses sample position, block, size and chroma index while some functions do not use some parameters.

- PPS flag disabling the temporal motion vector predictor missing in both WD and HM (JCTVC-F744).

The general list of key HEVC issues that need to be addressed was identified to be:

- Entropy coding architecture (see AHG9)

- Transform and dynamic range (see AHG5 and AHG7)

- Picture buffering and high-level syntax (see AHG21)

- Picture resolution adaptation (see AHG18)

- Non-4:2:0 colour formats (see AHG20)

- 10 bit vs. 8 bit decoding capability

- 4x4 inter-picture prediction?

- Simplification of MV coding

- In-loop filtering clean-up

- Profiles and Levels

- Parallel processing clean-up

# Recommendations

The recommendations of the HEVC Draft and Test Model Editing AHG are to:

- Approve the edited JCTVC-F802 and JCTVC-F803 documents as JCT-VC outputs

- Continue to edit the HEVC WD and Test Model documents to ensure that all agreed elements of HEVC are fully described

- Compare the HEVC documents with the HEVC software and resolve any discrepancies that may exist, in collaboration with the Software AHG

- Continue to improve the editorial consistency of HEVC WD and Test Model documents

- Ensure that, when considering the addition of new tools to HEVC, properly drafted text for addition to both the HEVC Working Draft and the HM Test Model (if appropriate) is made available in a timely manner

# References

- JCT-VC, “HM4: High Efficiency Video Coding (HEVC) Test Model 4 Encoder Description”, JCTVC- F802, JCT-VC Meeting, Torino, July 2011

- JCT-VC, “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, JCT-VC Meeting, Torino, July 2011