| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G003 |
| --- | --- |

| Title: | AHG Report: Software development and HM software technical evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Frank Bossen David Flynn Karsten Sühring | Tel: Email: | +1 650 496 4742 bossen@docomoinnovations.com |
| Source: | AHG chairs |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AhG on Software development and HM software technical evaluation that have taken place between the 6th and 7th JCT-VC meeting. Activities focused on integration of tools adopted at the 6th meeting into a common code base.

# Introduction

Mandates given to the AhG are as follows:

- Coordinate development of the HM software and its distribution to JCT-VC members

- Produce documentation of software usage for distribution with the software

- Prepare and deliver HM 4.0 software version and the reference configuration encodings according to JCTVC-F900 based on common conditions suitable for use in most core experiments (expected within four weeks after the meeting).

- Prepare and deliver HM 4.1 software (and additional "dot" version software releases as appropriate) and appropriate software branches that include additional items not integrated into the 4.0 version (expected within three weeks after the 4.0 software release).

- Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis.

- Coordinate with HEVC Draft and Test Model editing AhG to identify any mismatches between software and text

A brief summary of activities related to each mandate is given below.

- Development of the software was coordinated with the parties needing to integrate changes. Several tracks were typically pursued in parallel to speed up development. The distribution of the software was made through the SVN servers set up at HHI and BBC, as announced on the jct-vc email reflector.

- Version 4.0 of the software was delivered according to schedule and reference configuration encodings were provided according to the common test conditions through an ftp site at the BBC (ftp://ftp.kw.bbc.co.uk/hevc/hm-4.0-anchors/).

- All tools integrated into version 4.0 were individually retested. No unexpected result was observed.

- Version 4.1 of the software was delivered ahead of the 7th JCT-VC meeting.

- A number of mismatches between text and software were identified.

# History of revisions

Multiple versions of the HM software were produced and announced on the jct-vc email reflector. The following sections give a brief summary of the changes made for each version. A detailed history of changes made to the software can be viewed at http://hevc.kw.bbc.co.uk/trac/timeline

Released versions of the software are available on the SVN server set up at the following URL: https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number where version_number corresponds to one of the versions described below (e.g., HM-2.0). Intermediate code submissions can be found on a variety of branches available at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name where branch_name corresponds to a branch (e.g., HM-2.0-dev).

## Version 4.0

Version 4.0 was released on August 23, 2011. It includes all the changes adopted at the 6th JCT-VC meeting that affect common test conditions. This release was announced on the email reflector, along with performance data w.r.t. version 3.0 as follows:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | -8.2% | -9.4% | -1.6% | -7.1% | -7.6% |
| Class B | -0.3% | -8.5% | -9.8% | -1.5% | -7.3% | -7.8% |
| Class C | -0.1% | -8.1% | -8.5% | -1.6% | -8.6% | -9.5% |
| Class D | -0.4% | -6.0% | -6.5% | -1.6% | -7.6% | -8.4% |
| Class E | -0.2% | -11.5% | -10.4% | -1.6% | -7.3% | -7.8% |
| Class F | 0.3% | -7.4% | -9.0% | -2.0% | -7.9% | -9.8% |
| Overall | -0.2% | -8.2% | -8.9% | -1.6% | -7.6% | -8.5% |
|  | -0.2% | -8.8% | -9.6% | -1.6% | -8.3% | -9.1% |
| Enc Time[%] | 64% |  |  | 93% |  |  |
| Dec Time[%] | 78% |  |  | 99% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -2.5% | -17.3% | -17.7% | -3.4% | -14.7% | -15.8% |
| Class B | -1.5% | -16.0% | -15.6% | -3.1% | -13.5% | -12.0% |
| Class C | -1.8% | -14.2% | -14.6% | -3.0% | -12.2% | -13.4% |
| Class D | -2.1% | -11.7% | -12.3% | -3.0% | -9.7% | -10.0% |
| Class E |  |  |  |  |  |  |
| Class F | -0.8% | -12.4% | -14.8% | -2.8% | -11.8% | -14.6% |
| Overall | -1.7% | -14.4% | -15.0% | -3.0% | -12.4% | -13.1% |
|  | -1.7% | -14.7% | -15.4% | -3.0% | -12.8% | -13.5% |
| Enc Time[%] | 93% |  |  | 109% |  |  |
| Dec Time[%] | 88% |  |  | 93% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -5.0% | -23.7% | -24.8% | -7.7% | -23.5% | -23.8% |
| Class C | -6.5% | -22.8% | -23.9% | -7.7% | -21.3% | -23.2% |
| Class D | -7.2% | -20.2% | -20.9% | -8.0% | -18.9% | -19.8% |
| Class E | -5.9% | -21.6% | -25.6% | -8.0% | -25.1% | -24.8% |
| Class F | -4.0% | -19.3% | -22.2% | -5.2% | -17.3% | -21.0% |
| Overall | -5.7% | -21.7% | -23.5% | -7.3% | -21.1% | -22.5% |
|  | -5.6% | -22.1% | -23.9% | -7.3% | -21.4% | -22.7% |
| Enc Time[%] | 98% |  |  | 113% |  |  |
| Dec Time[%] | 87% |  |  | 95% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3.3% | -23.5% | -24.4% | -4.6% | -23.8% | -23.1% |
| Class C | -4.9% | -22.0% | -23.1% | -5.6% | -20.3% | -22.4% |
| Class D | -5.8% | -18.9% | -19.8% | -6.3% | -18.3% | -19.3% |
| Class E | -4.8% | -21.0% | -23.4% | -4.4% | -25.6% | -24.2% |
| Class F | -2.6% | -18.7% | -21.7% | -3.7% | -16.8% | -20.2% |
| Overall | -4.2% | -20.9% | -22.5% | -4.9% | -20.9% | -21.8% |
|  | -4.2% | -21.3% | -22.9% | -4.9% | -21.1% | -22.1% |
| Enc Time[%] | 97% |  |  | 122% |  |  |
| Dec Time[%] | 84% |  |  | 91% |  |  |

It can be noted that substantial coding efficiency improvements are achieved for all configurations, and for low-delay configurations in particular.

## Version 4.1

Version 4.1 was released on November 10, 2011. It includes all tools adopted at the 6th JCT-VC. This release happened much later than planned, as the integration of some tools (tiles, wavefront) took much longer than anticipated. This release was announced on the email reflector, along with performance data w.r.t. version 4.0 as follows:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -2.0% | -2.7% | 0.0% | -1.3% | -1.9% |
| Class B | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class D | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | -0.3% | -0.1% | 0.0% | -0.4% | -0.1% |
| Class F | 0.0% | -0.2% | -0.1% | 0.0% | -0.4% | -0.2% |
| Overall | 0.0% | -0.4% | -0.5% | 0.0% | -0.4% | -0.4% |
|  | 0.0% | -0.4% | -0.5% | 0.0% | -0.4% | -0.4% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -2.6% | -3.2% | 0.0% | -1.6% | -2.0% |
| Class B | 0.2% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | 0.3% | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% |
| Class D | 0.7% | 0.6% | 0.5% | 0.3% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.3% | 0.1% | 0.1% | 0.1% | -0.2% | -0.1% |
| Overall | 0.3% | -0.4% | -0.5% | 0.1% | -0.4% | -0.4% |
|  | 0.3% | -0.4% | -0.4% | 0.1% | -0.3% | -0.4% |
| Enc Time[%] | 106% |  |  | 107% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.1% | -0.2% | 0.1% | 0.0% | -0.1% |
| Class C | 0.4% | 0.2% | 0.2% | 0.1% | 0.2% | -0.2% |
| Class D | 0.8% | 0.6% | 0.4% | 0.3% | 0.7% | 0.0% |
| Class E | 1.0% | -0.3% | 0.7% | 0.3% | -0.5% | 0.3% |
| Class F | 0.3% | 0.1% | 0.2% | 0.3% | -0.1% | -0.4% |
| Overall | 0.5% | 0.2% | 0.2% | 0.2% | 0.1% | -0.1% |
|  | 0.5% | 0.1% | 0.2% | 0.2% | 0.1% | -0.1% |
| Enc Time[%] | 107% |  |  | 108% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.1% | 0.0% | 0.0% | 0.1% | -0.3% |
| Class C | 0.4% | 0.3% | 0.1% | 0.1% | -0.1% | 0.1% |
| Class D | 0.7% | 0.1% | 0.4% | 0.3% | 0.5% | 0.1% |
| Class E | 0.9% | 0.0% | -0.1% | 0.3% | -0.4% | -0.3% |
| Class F | 0.4% | 0.3% | 0.2% | 0.2% | 0.0% | -0.2% |
| Overall | 0.5% | 0.2% | 0.1% | 0.2% | 0.0% | -0.1% |
|  | 0.5% | 0.2% | 0.1% | 0.2% | 0.1% | -0.1% |
| Enc Time[%] | 106% |  |  | 107% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

It can be noted that a slight coding efficiency degradation is observed. The reason of this degradation should be further identified.

## Other branches

In addition to the regular HM development process, one branch was created to expose tools to a wider audience:

- HM-4.0-dev-ahg21-picbuffer which contains an implementation of the picture buffer management technique studied in AHG 21

# Potential mismatches with text

It is likely that the draft text does not match the implementation of the NSQT tool. This topic should be further discussed during the 7th JCT-VC meeting.

# Recommendations

- Continue to develop reference software based on HM version 4.1 and improve its quality, in particular for the loop filters such as ALF

- Continue to identify bugs and discrepancies with text, and address them