| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 20-30 November, 2011 | Document: JCTVC-G007 WG11 Number: m22635 |
| --- | --- |

| Title: | JCT-VC AHG report: Transform dynamic range (AHG 7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | A. Segall (Sharp) E. Alshina (Samsung) | Email: | asegall@sharplabs.com elena_a.alshina@samsung.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports the progress of the AhG on Transform Dynamic Range (AHG7) between the 6th JCT-VC meeting in Torino (14-22 July, 2011) and the 7th JCT-VC meeting in Geneva (20-30 November, 2011)

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| Transform Dynamic Range (AHG7) (jct-vc@lists.rwth-aachen.de) - Study methods for managing transform dynamic range - Find solutions guaranteeing 16 bit buffers in inverse transform without overflow and with minimal clipping operations - Study hardware and software aspects of inverse transform implementations | A. Segall, E. Alshina | No |

# Email Activity

The JCTVC reflector was used by the AhG for discussion and e-mail were identified by including [AHG7] in the subject line.  A kick-off message was sent out on November 2, 2011.  A total of 34 emails relating to the work of AHG7 were exchanged on the reflector.

# Scope

The AhG was primarily devoted to discussing needs and solutions for managing the dynamic range within the inverse transform.  Three locations of dynamic range concern were identified.  The locations consisted of:

- Dynamic range of dequantized coefficient

- Dynamic range following the first (or column) inverse transform

- Dynamic range following the second inverse transform

Three methods for dynamic range management were identified.  The methods consisted of:

- Bit-stream limits

- Normative clipping

- Modified shifting

# Consensus

The following consensus was reached:

- Dynamic range restriction should be defined for the dequantized coefficients.

- Dynamic range restriction should be defined after first inverse transform

- Dynamic range restriction should not be defined after second inverse transform, if input to second inverse transform is restricted to 16-bits

- Dynamic range following the first transform can exceed 16-bit in the worst case

- Clipping is preferred to restrict dynamic range for the dequantized coefficients and after first inverse transform.

# Input Contributions

JCTVC-G719 Structured Level Limits 	L. Kerofsky, A. Segall, K. Misra (Sharp), E. Alshina, A. Alshin (Samsung)

JCTVC-G782 AhG7: Overflow Prevention in HEVC inverse transform 	E. Alshina, A. Alshin, J.H. Park, (Samsung)

JCTVC-G856 AHG7: IDCT output range after T+Q+IQ+IT with valid residual inputs M. Zhou (TI)

JCTVC-G865 AHG7: New Results for Guaranteeing 16-bit Transform Dynamic Range Kiran Misra, Louie Kerofsky, Andrew Segall (Sharp)

# Recommendations

The recommendations of the AHG are:

- Review input contributions at the 7th JCT-VC meeting

- Include dynamic range restrictions for dequantized coefficients and first inverse transform in HEVC design

- Do not include dynamic range restriction following second inverse transform in HEVC design