| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G008 |
| --- | --- |

| Title: | JCT-VC AHG report: Reference pictures memory compression (AHG 8) |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document] |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono (Chair) NEC Corporation Takeshi Chujoh Toshiba Corporation Dzung Hoang Zenverge Incorporated ChongSoon Lim Panasonic Corporation Ali Tabatabai Sony Corporation Minhua Zhou Texas Instruments Inc., USA | Email: | chono@ct.jp.nec.com takeshi.chujoh@toshiba.co.jp dzung.hoang@zenverge.com ChongSoon.Lim@sg.panasonic.com Ali.Tabatabai@am.sony.com zhou@ti.com |
| Source: | [Reference pictures memory compression AHG] |  |  |

_____________________________

# Abstract

This document summarizes the AHG activities between 6th Meeting: Torino, 14-22 July, 2011, and the current 7th Meeting: Geneva, 21-31 November, 2011.

# Mandate

| AHG Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Reference pictures memory compression (jct-vc@lists.rwth-aachen.de) - Study motion compensation memory access bandwidth of HM design and proposed reference picture memory compression schemes - Study reference picture memory compression schemes and other motion compensation memory access reduction schemes - Study data format alignment between reference picture memory compression and display processing - Study the visual quality impact of reference picture memory compression - Report on conclusions reached | K. Chono (chair), T. Chujoh, D. Hoang, C. S. Lim, A. Tabatabai, M. Zhou (vice chairs) | N |

# Related activities and input contributions

Related to Mandate 1:

Decoder-side motion compensation memory access bandwidth of HM4.0 Anchor streams was measured by using an HM-4.0 patch based on HM2.0 branch software:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/HM-2.0-ahg-memory

The HM-4.0 patch and results were distributed to JCTVC reflector on August 30th. The results are summarized as follow:

- In RA settings, decoder-side MC memory access bandwidths of HM3.0 and HM4.0 are comparable (0.17% increase in HE case and 0.52% decrease in LC case) because no new tools that drastically increase or decrease motion compensation memory access bandwidth are adopted into HM4.0;

- In LD settings, decoder-side MC memory access bandwidths of HM4.0 are increased by 5.58% and 2.32% for HE and LC cases compared to those of HM3.0; the increase seems to be from changes in the common test conditions about reference picture list.

The detailed results are found in an attached XLS sheet “JCTVC-G008-MCBandwidth.xls.”

There is one informative contribution on memory bandwidth between 8-bit and 10-bit coding from Toshiba and NEC (JCTVC-G430). The contribution reports memory bandwidth differences between HM-4.0 8-bit and 10-bit coding using two typical DDR2 memory configurations. The parameter for memory bandwidth measurements and its results are shown in Table 1 and Table 2, respectively. The results reveal that the memory bandwidth increase of 10-bit coding compared to 8-bit coding is an average of 37% and a maximum of 47%. The contribution also asserts that proposals on memory bandwidth reduction in CE3 are considered for HM adoption.

Table 1		Parameter set for memory bandwidth measurement

|  | Access block size for luma | Access block size for chroma | Alignment | Burst length |
| --- | --- | --- | --- | --- |
| Case 1 (2D) | 4x2 | 4x2 | 64-bit | 128-bit |
| Case 2 (1D) | 8x1 | 8x1 | 32-bit | 64-bit |

Table 2		Summary of results (10-bit compared to 8-bit)

|  | Random Access HE |  |  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3.7% | -19.6% | -20.8% |  |  |  |  |  |  |
| Class B | -2.9% | -9.2% | -8.8% | -3.0% | -10.1% | -11.1% | -2.8% | -9.8% | -11.3% |
| Class C | -1.6% | -3.6% | -4.3% | -1.7% | -4.6% | -5.0% | -1.5% | -4.1% | -4.3% |
| Class D | -1.1% | -3.2% | -3.5% | -1.1% | -5.9% | -5.7% | -0.9% | -4.9% | -4.9% |
| Class E |  |  |  | -6.6% | -17.2% | -13.7% | -5.9% | -17.7% | -14.1% |
| Overall | -2.4% | -8.9% | -9.3% | -2.9% | -9.0% | -8.7% | -2.6% | -8.7% | -8.5% |
|  | -2.4% | -8.9% | -9.3% | -2.8% | -9.0% | -8.7% | -2.6% | -8.7% | -8.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 111% |  |  | 111% |  |  | 113% |  |  |
| MemBand(2D) Ave.[%] | 34% |  |  | 34% |  |  | 36% |  |  |
| MemBand(2D) Max.[%] | 48% |  |  | 49% |  |  | 45% |  |  |
| MemBand(1D) Ave.[%] | 34% |  |  | 34% |  |  | 36% |  |  |
| MemBand(1D) Max.[%] | 48% |  |  | 48% |  |  | 44% |  |  |

Related to Mandate 2:

There is one technical proposal from Qualcomm (JCTVC-G328). The contribution proposes to signal the output bit depth of decoded pictures so that less required decoded buffer memory is needed for certain bitstreams for which the ideal output bit depth is less than the bit depth of the decoded pictures. The contribution describes new SPS parameter to signal output bit depth and a new DPB management.

Related to Mandate 3 and 4:

There was no discussion among reflector. There is no contribution document.

# Recommendations

The AHG recommends reviewing related input documents listed below.

# List of related contributions

JCTVC-G328 [Qualcomm]	AHG8: Bit depth of output pictures

JCTVC-G430[Toshiba and NEC]	An experimental comparison of memory bandwidth between 8-bit and 10-bit coding