| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 20-30 November, 2011 | Document: JCTVC-G009 WG11 Number: m 22605 |
| --- | --- |

| Title: | JCT-VC AHG report: Entropy Coding Architecture (AHG 9) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | K. McCann (Samsung/ZetaCast) A. Fuldseth (Cisco) J. Lainema (Nokia) D. Marpe (Fraunhofer HHI) A. Segall (Sharp) K. Sugimoto (Mitsubishi) V. Sze (Texas Instruments) W. Wan (Broadcom) X. Wang (Qualcomm) | Email: | ken@zetacast.com arild.fuldseth@cisco.com Jani.Lainema@nokia.com detlev.marpe@hhi.fraunhofer.de asegall@sharplabs.com Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp sze@ti.com wwan@broadcom.com xianglin@qualcomm.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC ad hoc group on Entropy Coding Architecture (AHG9) between the 6th JCT-VC meeting in Torino (14-22 July, 2011) and the 7th JCT-VC meeting in Geneva (20-30 November, 2011).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| Entropy coding architecture (AHG9) (jct-vc@lists.rwth-aachen.de) - Study the entropy coding complexity and compression characteristics of single entropy coder architectures, scalable entropy coder architectures and switchable entropy coder architectures - Consider the potential for introducing scalability in the entropy coding options in the current HM and other potential means of harmonization of HE and LC entropy coding designs - Characterize entropy coding throughput, memory, silicon area, power requirements, etc. - Study interdependencies between entropy coding and other processes and the consequences of these interdependencies - Define practical use cases and corresponding HM configurations to enable testing of different entropy coding architectures in relevant environments - Study and develop approaches for hardware and software evaluation of entropy coding methods - Identify and discuss additional issues on entropy coding | K. McCann (chair), A. Fuldseth, J. Lainema, D. Marpe, A. Segall, K. Sugimoto, V. Sze, W. Wan, X. Wang (vice chairs) | No |

# Email Activity

A kick-off message was sent out on 5 August 2011.  A total of 42 emails relating to the work of AHG9 were exchanged on the reflector.

# Scope

This AHG was set up to investigate architectural issues relating to entropy coding.  The goal is to gather information and perform preliminary analysis to help enable a decision on the HEVC entropy coding architecture to be taken at the 7th JCT-VC meeting.

Two basic approaches to the entropy coding architecture were considered:

- Single entropy coder technology with single operating point

- Encoder architecture with two (or more) technologies or operating points

Multiple operating points could be achieved through some form of switchable or scalable architecture. In principle, such an architecture could support a large number of operating points.  However, no interest was expressed in examining more than 2 operating points.

# Switchable and Scalable Architectures

A purely switchable architecture could be characterized as:

- Multiple operating points supported by replacing major elements of the entropy coding system

- Tools are used in a lower complexity operating point that are not used at a higher efficiency operating point

- When implemented in software, a significant number of lines of code that are active when operating  in a lower complexity mode are no longer active when operating in a higher efficiency mode

- When implemented in non-programmable hardware, significant areas of silicon that are active when operating in a lower complexity mode are no longer active when operating in a higher efficiency mode.

A purely scalable architecture could be characterized as:

- A single, logically coherent entropy system that is able to support multiple operating points, e.g. by varying the degree of adaptability that is used

- The tools used in a lower complexity operating point are largely a subset of the tools used at a higher efficiency operating point

- When implemented in software, the majority of the lines of code that are active when operating  in a lower complexity mode would remain active when operating in a higher efficiency mode

- When implemented in non-programmable hardware, the majority of the areas of silicon that are active when operating in a lower complexity mode would remain active when operating in a higher efficiency mode.

However, it was recognized that the distinction between scalability and switchability was somewhat blurred and it did not appear to be possible to devise a pass/fail test that could unambiguously determine if a particular system should be labelled as scalable or switchable.

# Compression Efficiency Measurements

Differences in compression efficiency would ideally be evaluated through a process of formal subjective testing and the generation of Mean Opinion Score (MOS) values.  However, formal subjective testing is a very expensive and time-consuming process, so throughout the development of the HEVC Test Model (HM), coding performance was evaluated by objective rate-distortion measurements using PSNR, BD-PSNR and BD-Rate [1][2].

The experience of the HEVC Call for Proposals (CfP) was that the results of objective testing correlated reasonably well with those of subjective testing.   Furthermore, no major discrepancies between the actual visual quality and that which would be expected by the objective measurements have been observed so far during the HM development process.  There was therefore perceived to be only a very low risk of drawing incorrect conclusions by basing compression efficiency assessment on objective measures.

# Indicators of Complexity

## Software Run-Time

Encoder and Decoder run-time measurements have been used to provide a quick indicator of complexity for the CfP and the subsequent HM development process.  However, it is recognized that a difference in run-time is only a very approximate indication of a difference in complexity and that care needs to be taken to ensure that the comparison is done at an equal level of software optimization.

Accurate complexity measurements would be obtained only by comparing run-times of fully optimized code, but even this would only relate to the complexity of one particular software implementation.  Furthermore, the complexity of a hardware implementation is likely to be quite different from that of a software implementation.

## Hardware Complexity

It does not appear to be possible to define a single parameter that provides an accurate generic measurement of complexity.  Instead, a number of issues need to be considered, the relative importance of which will depend on the chosen implementation: throughput, silicon area, memory requirements, power requirements, error handling and buffer management, etc.

# Recommendations

The recommendations of the AHG on Entropy Coding Architecture are:

- HEVC should include only a single entropy coding technology with a single operating point unless adding a second option provides a significantly different  performance/complexity trade-off which substantially facilitates the use of HEVC in a class of applications for which it would not otherwise provide an appropriate solution

- JCT-VC should analyse input contributions relating to entropy coding with the aim of making a decision on the HEVC entropy coding architecture during the 7th JCT-VC meeting

# References

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11