# Config file syntax for flexible cyclic coding structure

It is proposed to remove the following config file parameters:

RateGOPSize                   : 8           # GOP size used for QP assignment

NumOfReference                : 4           # Number of reference frames

NumOfReferenceB_L0            : 2           # Number of reference frames for L0 for B-slices

NumOfReferenceB_L1            : 2           # Number of reference frames for L1 for B-slices

HierarchicalCoding            : 1           # Hierarchical B coding ON/OFF

LowDelayCoding                : 0           # Low-delay coding structure

GPB                           : 1           # Replace P-slice by B-slice using two same directions

NRF                           : 1           # Mark non-reference for highest temporal layer

BQP                           : 0           # Use hier-B style QP assignment for hier-P structure

The following config file parameter is kept:

GOPSize                       : 8           # GOP Size (number of B slice = GOPSize-1)

Directly following the line containig GOPSize there shall be GOPSize lines, each containing Slice type, POC, QPoffset, lambda factor, temporal_id, reference buffer size, ref_pic, #ref_pics and a list of #ref_pics reference pictures. The reference picture list contains of delta poc numbers relative the current poc. For instance, if the reference picture list is (-1, -5, -9, -13) and the current POC is 16, the DPB shall contain reference pictures with POC 15, 11, 7 and 3.

Example of the HM-4.0 LD picture structure with one most recent reference picture and three low-QP reference pictures:

GOPSize                       : 4

#        Type POC QPoffset QPfactor temporal_id ref_buf_size ref_pic #ref_pics reference pictures

Frame1:  B    1   3        0.4624    0           4            1       4         -1 -5 -9 -13

Frame2:  B    2   2        0.4624    0           4            1       4         -1 -2 -6 -10

Frame3:  B    3   3        0.4624    0           4            1       4         -1 -3 -7 -11

Frame4:  B    4   1        0.578     0           4            1       4         -1 -4 -8 -12

All reference pictures that are included in the Buffer Description are listed. This then includes pictures that are not used for the current picture but are kept in the DPB to be used for decoding of future pictures. The encoder assumes that all reference pictures that are of the same temporal_id or lower than the current frame will be used for prediction. Here is an example of the HM-4.0 RA setting:

GOPSize                       : 8

#        Type POC QPoffset QPfactor temporal_id ref_buf_size ref_pic #ref_pics reference pictures

Frame1:  B    8   1        0.442    0           4            1       4         -8 -10 -12 -14

Frame2:  B    4   2        0.3536   0           2            1       3         -4 -6 4

Frame3:  B    2   3        0.3536   0           2            1       4         -2 -4 2 6

Frame4:  B    6   3        0.3536   0           2            1       5         -2 -4 -6 -8 2

Frame5:  B    1   4        0.68     0           2            0       6         -1 -3 1 3 5 7

Frame6:  B    3   4        0.68     0           2            0       5         -1 -3 1 3 5

Frame7:  B    5   4        0.68     0           2            0       4         -1 -3 1 3

Frame8:  B    7   4        0.68     0           2            0       4         -1 -3 -5 1

The two new syntax elements MaxNumberOfReorderPictures and MaxNumberOfReferencePictures introduced in F493 are calculated from the coding structure and sent to the decoder as the lowest possible number of each parameter.

Reference pictures before IDR and open-GOP I-pictures are automatically removed from the signalling by the encoder.