| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G031 WG11 Number: m 22139 |
| --- | --- |

| Title: | CE1: Entropy Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Rajan Joshi Qualcomm Elena Alshina Samsung Electronics Co., Ltd. Hisao Sasai Panasonic Corporation Heiner Kirchhoffer Fraunhofer HHI Jani Lainema Nokia | Email: | rajanj@qualcomm.com elena_a.alshina@samsung.com sasai.hisao@jp.panasonic.com heiner.kirchhoffer@hhi.fraunhofer.de jani.lainema@nokia.com |
| Source: | CE1 coordinators |  |  |

_____________________________

# Abstract

The purpose of Core Experiment 1 (CE1) is to evaluate different entropy coding techniques. The entropy coding techniques studied included binary arithmetic coding (BAC), variable-to-variable length codes (V2V) and variable-to-fixed length codes (V2F). Additionally, proposals to simplify and improve current binary arithmetic coding were also studied. This document, JCTVC-G031, provides a summary of activities and results for this core experiment.

# Introduction

The purpose of Core Experiment 1 (CE1) is to evaluate different entropy coding techniques. The experiment consisted of 3 subtests. In subtest A, simplifications to CABAC were explored. These included delaying state update and simplification of the contexts used for skip and split modes. In subtest B, improvements to the probability update mechanism for the binary arithmetic coder were explored. In subtest C, various entropy coding techniques were studied under different constraints.

# Participant list

| Name | Affiliation | Email |
| --- | --- | --- |
| Rajan Joshi (coordinator) | Qualcomm | rajanj@qualcomm.com |
| Elena Alshina (coordinator) | Samsung | elena_a.alshina@samsung.com |
| Hisao Sasai (coordinator) | Panasonic | sasai.hisao@jp.panasonic.com |
| Heiner Kirchhoffer (coordinator) | Fraunhofer HHI | Heiner.Kirchhoffer@hhi.fraunhofer.de |
| Jani Lainema (coordinator) | Nokia | Jani.Lainema@nokia.com |
| Gaelle Martin-Cocher | Research In Motion | gmartincocher@rim.com |
| Kazuo Sugimoto | Mitsubishi Electric Corporation | Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Thomas Davies | Cisco | thdavies@cisco.com |
| Arild Fuldseth | Cisco | arilfuld@cisco.com |
| Tzu-Der (Peter) Chuang | MediaTek | peter.chuang@mediatek.com |
| Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |
| Shih-Ta Hsiang | MediaTek | shih-ta.hsiang@mediatek.com |
| Alexander Alshin | Samsung | alexander_b.alshin@samsung.com |
| Jianle Chen | Samsung | jianle.chen@samsung.com |
| Joel Sole Rojals | Qualcomm | joels@qualcomm.com |
| Tomoo Yamakage | Toshiba | tomoo.yamakage@toshiba.co.jp |
| Yeo Chuo Hao | I2R | chyeo@i2r.a-star.edu.sg |
| Xiaozhen Zheng | Huawei | zhengxiaozhen@huawei.com |
| Frank Bossen | Docomo | bossen@docomoinnovations.com |

# Summary of experimental results

## Subtest A

### Proposals targeting CABAC throughput improvement by probability up-date delay

Test results summary (BD-rate difference is averaged across AI-HE., RA-HE, LD(B)-HE, class F is not included):

| Proponent | Description | BD-rate (Y) | BD-rate, (U) | BD-rate, (V) |
| --- | --- | --- | --- | --- |
| Qualcomm JCTVC-G576 | Delay 1 bin (all syntax elements) | 0,1% | 0,1% | 0,2% |
|  | Delay 2 bins (all syntax elements) | 0,6% | 0,4% | 0,5% |
|  | Delay 3 bins (all syntax elements) | 1,0% | 0,4% | 0,7% |
| Panasonic JCTVC-G349 | Delay all coefficient coding parameters until end of block | 0,2% | 0,1% | 0,1% |
|  | Delay all coefficient coding parameters except for significant_coeff_flag" parameters until end of block | 0,1% | -0,1% | -0,1% |
|  | Delay "last_significant_coeff_x" and "last_significant_coeff_y" parameters until end of block | 0,0% | -0,1% | -0,1% |
|  | Delay "coeff_abs_level_greater1_flag"and "coeff_abs_level_greater2_flag" parameters until end of block | 0,1% | 0,0% | 0,0% |
|  | Delay "significant_coeff_flag" parameters until end of block | 0,2% | 0,1% | 0,2% |

Related documents:

JCTVC-G576 “CE1: Delayed state update for CABAC”, J. Sole, R. Joshi, and M. Karczewicz (Qualcomm)

Based on: JCTVC-F552: “Parallel processing of residual data in HE”, J. Sole, R. Joshi, and M. Karczewicz (Qualcomm)

Test description: JCTVC-F552 proposes delaying the context state update by one (or more bins). In this experiment, the BD-rate performance for various delays was evaluated. The delayed context update is applied uniformly for all syntax elements (not only residual coding).

Summary of tests results:

| All Intra HE. Y; U; V. Class A; 0,2%; 0,3%; 0,4%. Class B; 0,1%; 0,2%; 0,3%. Class C; 0,1%; 0,2%; 0,2%. Class D; 0,1%; 0,2%; 0,1%. Class E; 0,1%; 0,3%; 0,2%. Overall; 0,1%; 0,2%; 0,2%. 0,1%; 0,2%; 0,2%. Enc T[%]; 104%. Dec T[%]; 106%. Random Access HE. Y; U; V. Class A; 0,2%; 0,3%; 0,1%. Class B; 0,1%; 0,3%; 0,1%. Class C; 0,1%; 0,1%; 0,1%. Class D; 0,1%; 0,1%; -0,2%. Class E. Overall; 0,1%; 0,2%; 0,0%. 0,1%; 0,1%; 0,0%. Enc T[%]; 102%. Dec T[%]; 103%. Low delay B HE. Y; U; V. Class A. Class B; 0,1%; -0,1%; -0,2%. Class C; 0,1%; 0,1%; 0,0%. Class D; 0,0%; 0,2%; 0,2%. Class E; 0,0%; 0,1%; 1,2%. Overall; 0,1%; 0,1%; 0,2%. 0,1%; 0,0%; 0,1%. Enc T[%]; 101%. Dec T[%]; 104%. | All Intra HE. Y; U; V. 0,3%; 0,2%; 0,2%. 0,2%; 0,2%; 0,3%. 0,1%; 0,2%; 0,2%. 0,1%; 0,2%; 0,2%. 0,3%; 0,3%; 0,3%. 0,2%; 0,2%; 0,2%. 0,2%; 0,2%; 0,2%. 106%. 106%. Random Access HE. Y; U; V. 1,0%; 0,7%; 0,0%. 0,9%; 0,5%; 0,5%. 0,7%; 0,5%; 0,5%. 0,5%; 0,6%; 0,2%. 0,8%; 0,6%; 0,3%. 0,8%; 0,6%; 0,3%. 104%. 100%. Low delay B HE. Y; U; V. 1,0%; 0,7%; 0,7%. 0,7%; 0,4%; 0,5%. 0,5%; 0,3%; 0,7%. 1,8%; 1,0%; 2,3%. 0,9%; 0,6%; 0,9%. 0,9%; 0,6%; 0,9%. 105%. 99%. | All Intra HE. Y; U; V. 0,3%; 0,3%; 0,6%. 0,2%; 0,2%; 0,6%. 0,2%; 0,2%; 0,3%. 0,2%; 0,2%; 0,3%. 0,3%; 0,4%; 0,9%. 0,2%; 0,3%; 0,5%. 0,2%; 0,3%; 0,6%. 105%. 105%. Random Access HE. Y; U; V. 1,4%; 0,5%; 0,4%. 1,3%; 0,4%; 0,5%. 0,9%; 0,3%; 0,7%. 0,8%; 0,4%; 0,4%. 1,1%; 0,4%; 0,5%. 1,1%; 0,4%; 0,5%. 104%. 100%. Low delay B HE. Y; U; V. 1,6%; 0,7%; 0,6%. 1,0%; 0,6%; 0,5%. 0,7%; 0,2%; 0,3%. 3,2%; 1,0%; 3,0%. 1,5%; 0,6%; 0,9%. 1,5%; 0,7%; 0,9%. 103%. 100%. |
| --- | --- | --- |

Cross-check document number: JCTVC-G822 (Samsung)

JCTVC-G349 “CE1: SubsetA: Parallel context processing for coefficient coding using block-based context updates H. Sasai, T. Nishi (Panasonic)”

Based on: JCTVC-E226: ''Parallel context processing for significance map using block-based context updates", H. Sasai, T. Nishi (Panasonic)

Test description: JCTVC-E226 proposes block based context updating for the significance map. Proposed technique is aimed to improve the throughput of the entropy coder for CABAC. The context updates make difficult to increase throughput due to their serial dependencies. The proposed modifications have been implemented in HMv4 and their coding efficiencies were evaluated for coefficient cording parameters "last_significant_coeff_x","last_significant_coeff_y","significant_coeff_flag","coeff_abs_level_greater1_flag"and"coeff_abs_level_greater2_flag" respectively. The parallelization capability by the proposal comes at a cost of less than 0.1% performance loss.

Summary of tests results:

| Test1; Test2; Test3. All Intra HE. Y; U; V. Class A; 0,3%; 0,4%; 0,3%. Class B; 0,2%; 0,4%; 0,4%. Class C; 0,2%; 0,3%; 0,2%. Class D; 0,2%; 0,2%; 0,2%. Class E; 0,2%; 0,6%; 0,4%. Overall; 0,2%; 0,4%; 0,3%. 0,2%; 0,4%; 0,3%. Enc T[%]; 101%. Dec T[%]; 102%. Random Access HE. Y; U; V. Class A; 0,2%; 0,1%; -0,2%. Class B; 0,1%; 0,2%; 0,3%. Class C; 0,2%; 0,0%; 0,1%. Class D; 0,1%; 0,2%; -0,2%. Class E. Overall; 0,2%; 0,1%; 0,0%. 0,2%; 0,1%; 0,0%. Enc T[%]; 100%. Dec T[%]; 101%. Low delay B HE. Y; U; V. Class A. Class B; 0,1%; -0,1%; -0,2%. Class C; 0,2%; 0,2%; -0,3%. Class D; 0,1%; -0,2%; -0,1%. Class E; 0,2%; -0,5%; 0,3%. Overall; 0,1%; -0,2%; -0,1%. 0,1%; -0,2%; -0,1%. Enc T[%]; 101%. Dec T[%]; 100%. Low delay P HE. Y; U; V. Class A. Class B; 0,1%; 0,2%; -0,3%. Class C; 0,2%; 0,0%; 0,0%. Class D; 0,2%; -0,6%; -0,1%. Class E; 0,2%; -0,7%; -0,6%. Overall; 0,2%; -0,2%; -0,3%. 0,2%; -0,2%; -0,3%. Enc T[%]; 101%. Dec T[%]; 101%.; All Intra HE. Y; U; V. 0,2%; 0,2%; 0,1%. 0,0%; 0,1%; 0,1%. 0,0%; 0,1%; 0,1%. 0,0%; 0,0%; 0,0%. 0,0%; 0,1%; 0,1%. 0,1%; 0,1%; 0,1%. 0,1%; 0,1%; 0,1%. 101%. 102%. Random Access HE. Y; U; V. 0,0%; -0,3%; -0,4%. 0,0%; 0,0%; 0,0%. 0,0%; 0,0%; 0,0%. 0,1%; 0,0%; -0,1%. 0,0%; -0,1%; -0,1%. 0,0%; -0,1%; -0,1%. 100%. 101%. Low delay B HE. Y; U; V. 0,0%; -0,2%; -0,3%. 0,0%; 0,0%; -0,2%. 0,0%; -0,1%; -0,3%. 0,1%; -0,4%; 0,2%. 0,0%; -0,2%; -0,2%. 0,0%; -0,1%; -0,2%. 100%. 99%. Low delay P HE. Y; U; V. 0,0%; 0,1%; -0,1%. 0,0%; 0,1%; -0,1%. 0,0%; -0,3%; -0,7%. 0,1%; -0,4%; -0,4%. 0,0%; -0,1%; -0,3%. 0,0%; -0,1%; -0,3%. 101%. 101%.; All Intra HE. Y; U; V. 0,0%; -0,1%; 0,0%. 0,0%; 0,0%; 0,0%. 0,0%; 0,0%; 0,0%. 0,0%; 0,0%; -0,1%. 0,0%; -0,1%; 0,1%. 0,0%; 0,0%; 0,0%. 0,0%; 0,0%; 0,0%. 100%. 101%. Random Access HE. Y; U; V. 0,0%; 0,1%; -0,3%. 0,0%; 0,0%; 0,2%. 0,0%; -0,1%; 0,1%. 0,0%; -0,2%; 0,0%. 0,0%; -0,1%; 0,0%. 0,0%; -0,1%; 0,0%. 100%. 100%. Low delay B HE. Y; U; V. 0,0%; 0,0%; -0,4%. 0,0%; -0,1%; -0,2%. 0,0%; 0,1%; -0,2%. 0,1%; -0,9%; 0,0%. 0,0%; -0,2%; -0,2%. 0,0%; -0,2%; -0,2%. 100%. 99%. Low delay P HE. Y; U; V. 0,0%; -0,1%; 0,0%. 0,0%; 0,1%; -0,2%. 0,0%; 0,0%; -0,1%. 0,1%; -0,9%; -1,7%. 0,0%; -0,2%; -0,4%. 0,0%; -0,1%; -0,4%. 101%. 100%.. |  |  |
| --- | --- | --- |

| Test 4 | Test 5 |
| --- | --- |
| All Intra HE. Y; U; V. Class A; 0,2%; 0,2%; 0,1%. Class B; 0,0%; 0,1%; 0,1%. Class C; 0,0%; 0,1%; 0,1%. Class D; 0,0%; 0,0%; 0,0%. Class E; 0,0%; 0,1%; 0,1%. Overall; 0,1%; 0,1%; 0,1%. 0,1%; 0,1%; 0,1%. Enc T[%]; 101%. Dec T[%]; 101%. Random Access HE. Y; U; V. Class A; 0,1%; 0,1%; -0,4%. Class B; 0,0%; 0,1%; 0,0%. Class C; 0,1%; 0,0%; 0,0%. Class D; 0,1%; 0,1%; -0,2%. Class E. Overall; 0,0%; 0,1%; -0,2%. 0,0%; 0,1%; -0,2%. Enc T[%]; 100%. Dec T[%]; 100%. Low delay B HE. Y; U; V. Class A. Class B; 0,0%; 0,0%; 0,0%. Class C; 0,0%; -0,1%; -0,2%. Class D; 0,1%; -0,2%; 0,0%. Class E; 0,1%; -0,2%; 0,1%. Overall; 0,1%; -0,1%; 0,0%. 0,1%; 0,0%; -0,1%. Enc T[%]; 101%. Dec T[%]; 100%. Low delay P HE. Y; U; V. Class A. Class B; 0,0%; 0,3%; 0,0%. Class C; 0,0%; 0,0%; 0,1%. Class D; 0,0%; 0,1%; 0,0%. Class E; 0,1%; -0,2%; -0,9%. Overall; 0,1%; 0,1%; -0,2%. 0,1%; 0,1%; -0,2%. Enc T[%]; 101%. Dec T[%]; 100%. | All Intra HE. Y; U; V. 0,2%; 0,2%; 0,1%. 0,2%; 0,3%; 0,3%. 0,2%; 0,2%; 0,2%. 0,1%; 0,2%; 0,1%. 0,2%; 0,4%; 0,3%. 0,2%; 0,3%; 0,2%. 0,2%; 0,3%; 0,2%. 100%. 101%. Random Access HE. Y; U; V. 0,1%; 0,0%; 0,1%. 0,1%; 0,3%; 0,3%. 0,2%; 0,1%; 0,2%. 0,2%; 0,1%; 0,2%. 0,1%; 0,1%; 0,2%. 0,2%; 0,1%; 0,2%. 100%. 100%. Low delay B HE. Y; U; V. 0,1%; 0,0%; -0,5%. 0,1%; -0,1%; -0,1%. 0,1%; 0,4%; 0,4%. 0,2%; -0,4%; 0,8%. 0,1%; 0,0%; 0,1%. 0,1%; 0,0%; 0,0%. 100%. 100%. Low delay P HE. Y; U; V. 0,1%; 0,2%; -0,3%. 0,1%; 0,1%; 0,1%. 0,2%; 0,3%; -0,4%. 0,2%; 0,3%; -0,1%. 0,2%; 0,2%; -0,2%. 0,2%; 0,2%; -0,2%. 101%. 101%. |

Cross-check document number: JCTVC-G471 (MediaTek).

### Proposals targeting to line buffer removal for CABAC

Test results summary (BD-rate difference is averaged across AI-HE., RA-HE, LD(B)-HE, class F is not included):

| Proponent | Description | BD-rate, Y | BD-rate, U | BD-rate, V |
| --- | --- | --- | --- | --- |
| MediaTek JCTVC-G200 | Split LCU | 0,0% | 0,0% | -0,1% |
|  | Skip LCU | 0,0% | 0,0% | 0,0% |
|  | Spli&Skip LCU | 0,0% | 0,0% | -0,1% |
| Samsung JCTVC-G769 | Split CU | 0,1% | 0,0% | 0,0% |

Related documents: G829

JCTVC-G200 “CE1.A.3: Reducing line buffers for CABAC” T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)

Based on: JCTVC-F060: "Reducing Line Buffers for CABAC", T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)

Test description: In HM4.0, only two syntax elements, split_coding_unit_flag and skip_flag, need a line buffer to store the data from the upper LCU. In this test, reducing split_coding_unit _flag and skip_flag line buffer is proposed. The context formation of skip_flag is modified to avoid using upper block data and to only use left block data when the upper block belongs to the upper LCU. It is proposed to test the removal of the line buffer for split_coding_unit _flag and skip_flag in CE1.

Summary of test results:

| Split | Skip | Split&Skip |
| --- | --- | --- |
| All Intra HE. Y; U; V. Class A; 0,0%; -0,1%; -0,1%. Class B; 0,0%; -0,1%; -0,1%. Class C; 0,0%; -0,1%; -0,1%. Class D; 0,0%; 0,0%; -0,1%. Class E; 0,0%; -0,1%; -0,1%. Overall; 0,0%; -0,1%; -0,1%. 0,0%; -0,1%; -0,1%. Enc T[%]; 101%. Dec T[%]; 100%. Random Access HE. Y; U; V. Class A; -0,1%; -0,2%; 0,1%. Class B; 0,0%; 0,0%; -0,2%. Class C; 0,0%; -0,1%; 0,0%. Class D; 0,1%; 0,1%; 0,0%. Class E. Overall; 0,0%; 0,0%; -0,1%. 0,0%; -0,1%; 0,0%. Enc T[%]; 100%. Dec T[%]; 99%. Low delay B HE. Y; U; V. Class A. Class B; 0,0%; -0,1%; -0,3%. Class C; 0,1%; -0,1%; 0,0%. Class D; 0,1%; 0,1%; -0,5%. Class E; 0,1%; 0,5%; -0,6%. Overall; 0,1%; 0,0%; -0,3%. 0,1%; 0,0%; -0,3%. Enc T[%]; 100%. Dec T[%]; 99%. | All Intra HE. Y; U; V. Random Access HE. Y; U; V. 0,0%; -0,1%; -0,1%. 0,1%; 0,1%; 0,1%. 0,0%; -0,1%; 0,1%. 0,0%; -0,1%; -0,1%. 0,0%; 0,0%; 0,0%. 0,0%; 0,0%; 0,0%. 100%. 99%. Low delay B HE. Y; U; V. 0,1%; -0,1%; 0,0%. 0,0%; -0,1%; -0,1%. 0,0%; 0,4%; 0,2%. 0,0%; 0,0%; 0,4%. 0,0%; 0,1%; 0,1%. 0,0%; 0,0%; 0,0%. 100%. 99%. | All Intra HE. Y; U; V. 0,0%; -0,1%; -0,1%. 0,0%; -0,1%; -0,1%. 0,0%; -0,1%; -0,1%. 0,0%; 0,0%; -0,1%. 0,0%; -0,1%; -0,1%. 0,0%; -0,1%; -0,1%. 0,0%; -0,1%; -0,1%. 100%. 100%. Random Access HE. Y; U; V. 0,0%; -0,2%; 0,2%. 0,0%; 0,1%; -0,2%. 0,1%; 0,0%; 0,1%. 0,1%; 0,2%; 0,1%. 0,0%; 0,0%; 0,0%. 0,1%; 0,0%; 0,0%. 100%. 100%. Low delay B HE. Y; U; V. 0,1%; 0,0%; -0,3%. 0,1%; 0,1%; 0,0%. 0,1%; 0,0%; -0,3%. 0,1%; -0,3%; -0,1%. 0,1%; 0,0%; -0,2%. 0,1%; 0,0%; -0,2%. 100%. 101%. |

Cross-check document number: JCTVC-G298 (Panasonic).

JCTVC-G769 “CE1 subtest A: Line buffer removal for CU split flag context model” T. Lee, J. Chen, J. H. Park (Samsung)

Based on: JCTVC-F497: "Simplified context model selection for block level syntax coding", J. Chen and T. Lee (Samsung)

Test description: This contribution targeted reducing line buffers of CABAC engine by removing interdependency of neighboring block in current context model selection method. Context of split_coding_unit_flag and skip_flag is selected based on value of left block and current CU_depth.

Summary of test results:

| Split (MTK) | Split SEC |
| --- | --- |
| All Intra HE. Y; U; V. Class A; 0,0%; -0,1%; -0,1%. Class B; 0,0%; -0,1%; -0,1%. Class C; 0,0%; -0,1%; -0,1%. Class D; 0,0%; 0,0%; -0,1%. Class E; 0,0%; -0,1%; -0,1%. Overall; 0,0%; -0,1%; -0,1%. 0,0%; -0,1%; -0,1%. Enc T[%]; 101%. Dec T[%]; 100%. Random Access HE. Y; U; V. Class A; -0,1%; -0,2%; 0,1%. Class B; 0,0%; 0,0%; -0,2%. Class C; 0,0%; -0,1%; 0,0%. Class D; 0,1%; 0,1%; 0,0%. Class E. Overall; 0,0%; 0,0%; -0,1%. 0,0%; -0,1%; 0,0%. Enc T[%]; 100%. Dec T[%]; 99%. Low delay B HE. Y; U; V. Class A. Class B; 0,0%; -0,1%; -0,3%. Class C; 0,1%; -0,1%; 0,0%. Class D; 0,1%; 0,1%; -0,5%. Class E; 0,1%; 0,5%; -0,6%. Overall; 0,1%; 0,0%; -0,3%. 0,1%; 0,0%; -0,3%. Enc T[%]; 100%. Dec T[%]; 99%. | All Intra HE. Y; U; V. 0,0%; -0,1%; 0,0%. 0,0%; 0,0%; 0,0%. 0,0%; 0,0%; -0,1%. 0,0%; 0,0%; 0,0%. 0,1%; 0,0%; 0,0%. 0,0%; 0,0%; 0,0%. 0,0%; 0,0%; -0,1%. 100%. 100%. Random Access HE. Y; U; V. 0,0%; -0,4%; -0,1%. 0,0%; 0,1%; 0,0%. 0,1%; 0,0%; 0,1%. 0,0%; 0,1%; 0,0%. 0,0%; 0,0%; 0,0%. 0,0%; -0,1%; 0,0%. 100%. 100%. Low delay B HE. Y; U; V. 0,0%; 0,0%; -0,1%. 0,1%; 0,0%; 0,1%. 0,0%; 0,0%; -0,1%. 0,4%; -0,3%; 0,7%. 0,1%; 0,0%; 0,1%. 0,1%; -0,1%; 0,1%. 100%. 99%. |

Cross-check document number: JCTVC-G472  (MediaTek).

## Subtest B

Test results summary (BD-rate difference is averaged across AI-HE., RA-HE, LD(B)-HE, class F is not included):

| Proponent | Description | BD-rate, (Y) | BD-rate, (U) | BD-rate, (V) |
| --- | --- | --- | --- | --- |
| Samsung JCTVC-G764 | Single parameter probability update model | -0,5% | -0,4% | -0,4% |
|  | Multi-parameter probability update model | -0,9% | -1,0% | -1,0% |

Related documents: G326, G413, G547

JCTVC-G764 “CE1 (subset B): Multi-parameter probability up-date for CABAC”. A. Alshin, E. Alshina, JH Park (Samsung)

Based on: JCTVC-F254: "Multi-parameter probability up-date for CABAC". E. Alshina and A. Alshin (Samsung)

Test description: JCTVC-F254 reported an improved method for probability update for CABAC. In this experiment, two subtests will be conducted. In the 1st subtest, single probability will be used with window size of 16, 32, 64, and 128. In the 2nd subtest, combination of two probabilities with different window size will be explored. Slight changes to the design such as reduced number of bits for probability (12 instead of 15) and reduced LUT size for interval sub-division will also be investigated

Summary of tests results (single parameter probability update model):

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.8% | 0.0% | 0.0% | -0.8% | 0.8% | 1.5% |  |  |  |
| Class B | -0.6% | -0.7% | -0.6% | -0.5% | -0.2% | 0.0% | -0.4% | 0.2% | 0.0% |
| Class C | -0.6% | -0.9% | -0.8% | -0.4% | -0.6% | -0.3% | -0.4% | -0.3% | -0.5% |
| Class D | -0.7% | -1.5% | -1.5% | -0.4% | -0.6% | -1.1% | -0.4% | -0.4% | -0.6% |
| Class E | -0.6% | -0.8% | -0.9% |  |  |  | -0.1% | -1.5% | -0.6% |
| Overall | -0.7% | -0.8% | -0.8% | -0.6% | -0.2% | 0.0% | -0.3% | -0.4% | -0.4% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |

Summary of tests results (multi-parameter probability update model):

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.1% | -0.7% | -0.7% | -1.2% | -0.2% | -0.3% |  |  |  |
| Class B | -0.9% | -1.1% | -1.2% | -1.0% | -0.7% | -0.7% | -0.8% | -0.8% | -1.1% |
| Class C | -0.9% | -1.2% | -1.2% | -0.9% | -0.9% | -0.9% | -0.8% | -0.7% | -1.2% |
| Class D | -0.9% | -1.7% | -1.7% | -0.9% | -1.1% | -1.3% | -0.8% | -1.0% | -1.0% |
| Class E | -0.9% | -1.8% | -1.8% |  |  |  | -0.9% | -2.0% | -0.4% |
| Overall | -0.9% | -1.3% | -1.3% | -1.0% | -0.7% | -0.8% | -0.8% | -1.1% | -0.9% |
| Enc Time[%] | 103% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 102% |  |  |

Cross-check document number: JCTVC-G553 (HHI)

## Subtest C

Test results summary (BD-rate difference is averaged across AI, RA, and LD, class F is not included):

|  |  | Description | HE |  | LC |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Proponent |  | BD-rate, (Y) | BD-rate, (U+V)/2 | BD-rate, (Y) | BD-rate, (U+V)/2 |
|  |  | BAC, LC configuration |  |  | -5.99% | +4.47% |
| 1 | HHI (JCTVC-G633) | BAC, LCmod, 8-bit init |  |  | -0.45% | +8.78% |
| 2 | HHI (JCTVC-G633) | V2V, LCmod, Multi-bin, 8-bit init, TBC |  |  | -0.45% | +7.80% |
| 3 | HHI (JCTVC-G633) | V2V, LCmod, Multi-bin, Low delay, 8-bit init, TBC |  |  | -0.14% | +8.13% |
| 4 | HHI (JCTVC-G633) | V2V, 8-bit init, TBC | +0.14% | -0.74% | -5.85% | +3.78% |
| 5 | HHI (JCTVC-G633) | V2V, LowDelay, 8-bit init, TBC | 0.32% | -0.58% |  |  |
| 6 | HHI (JCTVC-G633) | BAC, 8-bit init | -0.24% | -0.18% | -6.20% | 4.45% |
| 7 | HHI (JCTVC-G633) | BAC, 8-bit init, Alt. PMU, TBC | -0.79% | -0.58% |  |  |
| 8 | Mitsubishi (JCTVC-G458) | V2F, 8-bit init |  |  | -3.27% | 7.44% |
| 9 | Mitsubishi (JCTVC-G458) | V2F, LowDelay, 8-bit init |  |  | -3.27% | 7.44% |
| 10 | Mitsubishi (JCTVC-G458) | V2F, MC mod., LowDelay, 8-bit init |  |  | -3.04% | 9.48% |
| 11 | Samsung/HHI (JCTVC-G771) | V2V, 8-bit init, Alt. PMU, TBC |  |  |  |  |
| 12 | Cisco (JCTVC-G210) | BAC, RDOQ off, RDO PMU off |  |  | -5.20% | -0.16% |
| 13 | Cisco (JCTVC-G233) | BAC, RDOQ off PMU off |  |  | 5.99% | 4.63% |
| 14 | HHI (JCTVC-G633) | BAC, LCmod, 8-bit init, RDOQ off |  |  | -0.31% | +2.63 |
| 15 | HHI (JCTVC-G633) | V2V, LCmod, 8-bit init, Multi-bin, RDOQ off |  |  | -0.27% | +1.97 |
| 16 | withdrawn |  |  |  |  |  |
| 17 | Docomo (JCTVC-G763) | BAC, TBC | 0.0% | 0.0% | -5.96% | +4.52% |

Related documents: G155, G547, G569, G659, G833, G837

### List of tools

| Short name | Description |
| --- | --- |
| V2F | PIPE using V2F codes |
| V2V | PIPE using V2V codes |
| BAC | Binary arithmetic coding engine of CABAC in HM (anchor) |
| LC mod. | Low complexity configuration for context model derivation and modeling |
| MC mod. | Medium complexity configuration for context model derivation |
| PMU off | Disabling of probability model update |
| Multi bin | Joint coding of sequences of bins |
| Low delay | LowDelay buffer control for PIPE |
| 8-bit Init | Context model initialization using 8 bit initialization |
| Alt. PMU | Alternative method for probability model update |
| RDOQ off | RDO_Q is disabled |
| RDO PMU off | Sub-LCU probability model update off during RDO mode selection process |
| TBC | Table-based counting of bits during RDO |

### Combination of tools tested in each test.

| Test | V2F | V2V | BAC | LC mod. | MC mod. | PMU off | Multi bin | Low delay | 8-bit Init | Alt. PMU | RDOQ off | RDO PMU off | TBC | Config. tested |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | x | x |  |  | n/a |  | x |  |  |  |  | LC |
| 2 |  | x |  | x |  |  | x |  | x |  |  |  | x | LC |
| 3 |  | x |  | x |  |  | x | x | x |  |  |  | x | LC |
| 4 |  | x |  |  |  |  |  |  | x |  |  |  | x | HE, LC |
| 5 |  | x |  |  |  |  |  | x | x |  |  |  | x | HE |
| 6 |  |  | x |  |  |  | n/a |  | x |  |  |  |  | HE, LC |
| 7 |  |  | x |  |  |  | n/a |  | x | x |  |  | x | HE |
| 8 | x |  |  |  |  |  |  |  | x |  |  |  |  | LC |
| 9 | x |  |  |  |  |  |  | x | x |  |  |  |  | LC |
| 10 | x |  |  |  | x |  |  | x | x |  |  |  |  | LC |
| 11 |  | x |  |  |  |  |  |  | x | x |  |  | x | HE |
| 12 |  |  | x |  |  |  |  |  |  |  | x | x |  | LC |
| 13 |  |  | x |  |  | x |  |  |  |  | x |  |  | LC |
| 14 |  |  | x | x |  |  | n/a |  | x |  | x |  |  | LC |
| 15 |  | x |  | x |  |  | x |  | x |  | x |  | x | LC |
| 16 | x |  |  | x |  |  | x |  | x |  | x |  |  | LC |
| 17 |  |  | x |  |  |  |  |  |  |  |  |  | x | HE, LC |

### Detailed description of the tools

V2F

The PIPE coder and the set of V2F codes as defined in JCTVC-F176 are used.

V2V

The PIPE coder and the set of V2V codes as defined in JCTVC-F268 are used.

BAC

The Binary arithmetic coding engine of CABAC in HM is used (anchor configuration).

LC mod.

The following tools of the low complexity configuration of JCTVC-F268 are used:

- Disabling of probability model update for all context models

- Disabling of context model derivation for non-residual syntax elements (skip_flag, no_residual_data_flag, and split_coding_unit_flag) Note: Context model derivation for no_residual_data_flag should have already been disabled in HM 4.0, but was not due to a bug.

- Context modeling for syntax elements coeff_abs_level_greaterX_flag (with X=1,2)

MC mod.

The following tools of the low complexity configuration of JCTVC-F268 are used:

- Disabling of context model derivation for non-residual syntax elements

PMU off

Probability model update is disabled for all context models.

Multi bin

Sequences of bins are encoded or decoded jointly according to JCTVC-F268 if the following conditions are fulfilled:

- All bins use the same context model

- Probability model update is not used for this context model

Low delay

The low delay buffer control as described in JCTVC-A032 is used for PIPE-based coding.

8-bit Init

Context model initialization using the 8 bit initialization scheme as described in JCTVC-F268 is used.

Alt. PMU

Alternative method for probability model update according to JCTVC-F254 with optimized parameters is used.

RDOQ off

RDO_Q is disabled (non-normative change).

RDO PMU off

Sub-LCU probability model update is disabled during RDO mode selection process as described in JCTVC-F162 (non-normative change).

TBC

Table-based bit counting is used to estimate the bit count during RDO process without performing entropy coding. Only the state update is performed if necessary.

Please note that for LC mod. and MC mod. cases, contexts and scans as specified in HM 4.0 are used. The only changes are those specified under LC mod. and MC mod.

Test 1: BAC, LCmod, 8-bit init

Cross-check document number: JCTVC-G072 (I2R), JCTVC-G459 (Mitsubishi)

Test 2: V2V, LCmod, Multi-bin, 8-bit init, TBC

Cross-check document number: JCTVC-G072 (I2R), JCTVC-G459 (Mitsubishi)

Test 3: V2V, LCmod, Multi-bin, Low delay, 8-bit init, TBC

Cross-check document number: JCTVC-G459 (Mitsubishi)

Test 4: V2V, 8-bit init, TBC

Cross-check document number: JCTVC-G072 (I2R)

Test 5: V2V, LowDelay, 8-bit init, TBC

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.3% | -0.4% | -0.5% | 0.2% | -0.3% | -0.4% |  |  |  |
| Class B | 0.4% | -0.5% | -0.6% | 0.2% | -0.5% | -0.7% | 0.3% | -0.7% | -1.2% |
| Class C | 0.4% | -0.6% | -0.7% | 0.2% | -0.4% | -0.5% | 0.3% | -0.2% | -0.3% |
| Class D | 0.4% | -1.3% | -1.6% | 0.4% | -0.5% | -0.9% | 0.3% | 0.1% | -0.5% |
| Class E | 0.4% | -1.1% | -1.0% |  |  |  | 0.3% | -0.2% | 0.0% |
| Overall | 0.4% | -0.8% | -0.9% | 0.3% | -0.4% | -0.6% | 0.3% | -0.3% | -0.6% |
| Enc Time[%] | 97% |  |  | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Cross-check document number: JCTVC-G766 (Qualcomm)

Test 6: BAC, 8-bit init

|  | All Intra HE |  |  |  |  | Random Access HE |  |  |  |  | Low delay B HE |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  | Y | U |  | V |  | Y | U |  |  | V |
| Class A | 0.0% | 0.0% |  | 0.0% |  | -0.1% | 0.1% |  | -0.2% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% |  | -0.1% |  | -0.2% | 0.2% |  | 0.0% |  | -0.2% | 0.1% |  |  | -0.2% |
| Class C | 0.0% | -0.3% |  | -0.5% |  | -0.3% | -0.2% |  | -0.2% |  | -0.3% | 0.2% |  |  | -0.2% |
| Class D | 0.0% | -1.2% |  | -1.4% |  | -0.4% | -0.6% |  | -1.0% |  | -0.6% | 0.3% |  |  | 0.3% |
| Class E | -0.1% | -0.2% |  | 0.0% |  |  |  |  |  |  | -0.9% | -0.3% |  |  | -0.1% |
| Overall | 0.0% | -0.3% |  | -0.4% |  | -0.3% | -0.1% |  | -0.3% |  | -0.5% | 0.1% |  |  | 0.0% |
| Enc Time[%] | 100% |  |  |  |  | 100% |  |  |  |  | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  | 101% |  |  |  |  | 100% |  |  |  |  |
|  | All Intra LC |  |  |  |  | Random Access LC |  |  |  |  | Low delay B LC |  |  |  |  |
|  | Y |  | U |  | V | Y |  | U |  | V | Y |  | U | V |  |
| Class A | -5.6% |  | 1.4% |  | 1.8% | -8.4% |  | 7.8% |  | 7.3% |  |  |  |  |  |
| Class B | -5.8% |  | 0.7% |  | 1.6% | -7.6% |  | 3.4% |  | 4.7% | -7.5% |  | 5.9% | 10.2% |  |
| Class C | -4.6% |  | 0.3% |  | 0.3% | -5.7% |  | 1.8% |  | 2.4% | -5.7% |  | 4.2% | 5.0% |  |
| Class D | -3.9% |  | 0.1% |  | 0.0% | -5.0% |  | 3.2% |  | 3.3% | -5.3% |  | 8.8% | 9.5% |  |
| Class E | -7.4% |  | 6.4% |  | 6.1% |  |  |  |  |  | -7.7% |  | 7.0% | 9.3% |  |
| Overall | -5.4% |  | 1.5% |  | 1.7% | -6.7% |  | 4.0% |  | 4.5% | -6.5% |  | 6.4% | 8.6% |  |
| Enc Time[%] | 106% |  |  |  |  | 105% |  |  |  |  | 104% |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  | 100% |  |  |  |  | 96% |  |  |  |  |

Cross-check document number: JCTVC-G072 (I2R), JCTVC-G753 (HiSilicon) Not loaded

Test 7: BAC, 8-bit init, Alt. PMU, TBC

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.0% | -0.5% | -0.7% | -1.1% | 0.4% | 0.5% |  |  |  |
| Class B | -0.8% | -1.2% | -1.3% | -0.8% | -0.4% | -0.5% | -0.7% | 0.2% | 0.1% |
| Class C | -0.8% | -1.2% | -1.4% | -0.8% | -0.5% | -0.7% | -0.6% | 0.2% | -0.1% |
| Class D | -0.8% | -1.8% | -1.9% | -0.7% | -0.7% | -0.9% | -0.5% | -0.1% | 0.0% |
| Class E | -0.9% | -1.8% | -1.7% |  |  |  | -0.9% | -2.3% | 0.8% |
| Overall | -0.8% | -1.3% | -1.4% | -0.8% | -0.3% | -0.4% | -0.7% | -0.3% | 0.2% |
| Enc Time[%] | 109% |  |  | 107% |  |  | 106% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  | 102% |  |  |

Cross-check document number: JCTVC-G300 (Panasonic)

Test 8: V2F, 8-bit init

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3.2% | 4.0% | 4.4% | -5.6% | 11.1% | 10.0% |  |  |  |
| Class B | -3.1% | 3.6% | 4.6% | -4.6% | 6.5% | 7.8% | -4.5% | 9.1% | 13.3% |
| Class C | -2.2% | 3.0% | 3.1% | -3.0% | 4.5% | 5.2% | -2.9% | 6.9% | 8.0% |
| Class D | -1.6% | 3.5% | 3.6% | -1.7% | 6.7% | 7.2% | -1.8% | 11.7% | 11.9% |
| Class E | -4.6% | 9.6% | 9.3% |  |  |  | -3.2% | 10.3% | 11.5% |
| Overall | -2.9% | 4.5% | 4.7% | -3.8% | 7.2% | 7.6% | -3.2% | 9.4% | 11.3% |
| Enc Time[%] | 106% |  |  | 105% |  |  | 104% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  | 100% |  |  |

Cross-check document number: JCTVC-G435 (Toshiba)

Test 9: V2F, LowDelay, 8-bit init

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3.2% | 4.0% | 4.4% | -5.6% | 11.1% | 10.0% |  |  |  |
| Class B | -3.1% | 3.6% | 4.6% | -4.6% | 6.5% | 7.8% | -4.5% | 9.1% | 13.3% |
| Class C | -2.2% | 3.0% | 3.1% | -3.0% | 4.5% | 5.2% | -2.9% | 6.9% | 8.0% |
| Class D | -1.6% | 3.5% | 3.6% | -1.7% | 6.7% | 7.2% | -1.8% | 11.7% | 11.9% |
| Class E | -4.6% | 9.6% | 9.3% |  |  |  | -3.2% | 10.3% | 11.5% |
| Overall | -2.9% | 4.5% | 4.7% | -3.8% | 7.2% | 7.6% | -3.2% | 9.4% | 11.3% |
| Enc Time[%] | 106% |  |  | 105% |  |  | 104% |  |  |
| Dec Time[%] | 105% |  |  | 101% |  |  | 99% |  |  |

Cross-check document number: JCTVC-G612 (HHI)

Test 10: V2F, MC mod., LowDelay, 8-bit init

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3,2% | 4,4% | 4,8% | -5,1% | 11,1% | 10,5% |  |  |  |
| Class B | -3,1% | 4,8% | 5,8% | -4,2% | 8,4% | 10,2% | -4,0% | 12,2% | 17,9% |
| Class C | -2,3% | 4,1% | 4,3% | -2,7% | 5,9% | 6,7% | -2,6% | 8,9% | 10,4% |
| Class D | -1,5% | 4,1% | 4,1% | -1,5% | 8,5% | 8,7% | -1,9% | 16,9% | 16,7% |
| Class E | -4,5% | 10,4% | 10,4% |  |  |  | -2,8% | 14,6% | 15,9% |
| Overall | -2,8% | 5,3% | 5,7% | -3,4% | 8,5% | 9,1% | -2,9% | 13,0% | 15,4% |
| Enc Time[%] | 107% |  |  | 105% |  |  | 104% |  |  |
| Dec Time[%] | 103% |  |  | 99% |  |  | 98% |  |  |

Cross-check document number: JCTVC-G435 (Toshiba)

Test 11: V2V, 8-bit init, Alt. PMU, TBC

Cross-check document number: JCTVC-G641 (RIM) Not uploaded

Test 12: BAC, RDOQ off, RDO PMU off

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -4,8% | -2,9% | -4,2% | -7,2% | 3,2% | 2,0% |  |  |  |
| Class B | -5,8% | -3,8% | -3,1% | -6,4% | -0,9% | -0,3% | -6,0% | 1,5% | 3,3% |
| Class C | -4,2% | -2,8% | -2,5% | -4,4% | -1,3% | -0,6% | -4,4% | 1,2% | 2,2% |
| Class D | -3,4% | -2,7% | -2,4% | -3,7% | -1,0% | -0,1% | -3,6% | 3,1% | 4,0% |
| Class E | -7,1% | -1,2% | -1,0% |  |  |  | -6,7% | -0,2% | 2,1% |
| Overall (-F) | -5,0% | -2,8% | -2,7% | -5,5% | -0,1% | 0,2% | -5,1% | 1,5% | 3,0% |
| Enc Time[%] | 127% |  |  | 108% |  |  | 108% |  |  |
| Dec Time[%] | 114% |  |  | 107% |  |  | 105% |  |  |

Cross-check document number: JCTVC-G765 (Qualcomm), JCTVC-G932 (Samsung) Not uploaded

Test 13: BAC, RDOQ off, PMU off

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 3,9% | -0,3% | -0,3% | 7,4% | 3,6% | 3,2% |  |  |  |
| Class B | 5,1% | 2,2% | 1,5% | 7,6% | 6,2% | 4,9% | 8,3% | 12,5% | 10,5% |
| Class C | 3,6% | 2,2% | 2,0% | 5,8% | 6,8% | 6,8% | 6,0% | 10,2% | 10,1% |
| Class D | 2,3% | 1,6% | 1,6% | 5,5% | 5,4% | 5,2% | 6,6% | 7,9% | 8,1% |
| Class E | 4,9% | -4,2% | -3,8% |  |  |  | 8,4% | 0,7% | -1,1% |
| Overall | 4,0% | 0,6% | 0,4% | 6,6% | 5,5% | 5,0% | 7,3% | 8,6% | 7,6% |
| Enc Time[%] | 98% |  |  | 92% |  |  | 94% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 101% |  |  |

Cross-check document number: JCTVC-G765 (Qualcomm)

Test 14: BAC, LCmod, 8-bit init, RDOQ off

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 1.3 | 1.8% | -0.1% | -1.1% | 5.7% | 3.8% |  |  |  |
| Class B | -0.6 | -0.1% | -0.7% | -0.5% | 2.5% | 1.3% | -0.5% | 5.6% | 4.8% |
| Class C | -0.2 | 1.4% | 1.6% | -0.4% | 2.7% | 2.8% | -1.0% | 4.7% | 4.8% |
| Class D | 0.1% | 1.4% | 1.6% | 0.4% | 2.7% | 3.5% | 0.3% | 5.1% | 5.7% |
| Class E | -1.9% | -1.2% | -0.6% |  |  |  | -0.1% | -0.9% | 1.9% |
| Overall | -0.2% | 0.7% | 0.4% | -0.4% | 3.4% | 2.8% | -0.4% | 4.1% | 4.5% |
| Enc Time[%] | 115% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  | 100% |  |  |

Test 15: V2V, LCmod, 8-bit init, Multi-bin, RDOQ off, TBC

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 1.1 | 1.0% | -0.6% | -1.3% | 4.9% | 3.2% |  |  |  |
| Class B | -0.4 | -0.3% | -0.8% | -0.6% | 1.6% | 0.9% | -0.3% | 3.9% | 3.9% |
| Class C | -0.1 | 1.2% | 1.4% | -0.3% | 2.3% | 2.4% | -0.8% | 3.6% | 3.9% |
| Class D | 0.1% | 1.2% | 1.5% | 0.5% | 2.3% | 2.9% | 0.5% | 4.1% | 4.7% |
| Class E | -1.7% | -1.3% | -0.7% |  |  |  | -0.3% | -1.4% | 0.0% |
| Overall | -0.2% | 0.4% | 0.2% | -0.4% | 2.7% | 2.3% | -0.2% | 2.9% | 3.4% |
| Enc Time[%] | 105% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  | 99% |  |  |

Test 17: BAC, TBC

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.8% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 96% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  | 99% |  |  |

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -5.6% | 1.5% | 1.8% | -8.3% | 7.8% | 6.9% |  |  |  |
| Class B | -5.8% | 0.8% | 1.7% | -7.4% | 3.3% | 4.8% | -7.3% | 5.9% | 10.1% |
| Class C | -4.6% | 0.6% | 0.8% | -5.4% | 1.8% | 2.6% | -5.4% | 4.3% | 5.0% |
| Class D | -3.9% | 1.1% | 1.1% | -4.6% | 3.6% | 3.9% | -4.7% | 8.7% | 8.9% |
| Class E | -7.4% | 6.5% | 6.3% |  |  |  | -7.1% | 5.9% | 9.3% |
| Overall | -5.4% | 1.8% | 2.1% | -6.5% | 4.1% | 4.6% | -6.2% | 6.2% | 8.4% |
| Enc Time[%] | 100% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  | 97% |  |  |

Cross-check document number: JCTVC-G072 (I2R)