| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G032 |
| --- | --- |

| Title: | CE2: Summary report of core experiment on Motion Partitioning and OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng In Suk Chong Il-Koo Kim | Tel: Email: | +86-755-28422009 zhengxiaozhen@huawei.com ichong@qualcomm.com ilkoo.kim@samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of CE2 related to Motion Partitioning and OBMC [1]. The description of the experiment can be found in JCTVC-F902. In this CE, six subtests have been studied and evaluated. Among those subtests, proponent of subtest B.1 and C.1 have released software before 13th September, and other subtests have been withdrawn.

At this meeting cycle, 16 companies have been involved as proponent or cross-checker.

# Objective

The purpose of Core Experiment 2 (CE2) is to investigate inter block partition issue at current HEVC coding structure, which contains three categories: block partitioning, overlapped block motion compensation (OBMC) and non-square TU.

Participants List

| Participant | Person, Email address |
| --- | --- |
| Qualcomm | Liwei Guo, liweig@qualcomm.com Xianglin Wang, xianglin@qualcomm.com I. S. Chong, ichong@qualcomm.com |
| Huawei & HiSilicon | Xiaozhen Zheng, zhengxiaozhen@huawei.com Haoping Yu, haopingyu@huawei.com |
| Samsung | Il-Koo Kim, ilkoo.kim@samsung.com |
| Tsinghua | Yuan Yuan, y-yuan08@mails.tsinghua.edu.cn |
| Intel | Y.-J Chiu, Yi-jen.chiu@intel.com W. Zhang, Wenhao.zhang@intel.com |
| NCTU/ITRI | Wen-Hsiao Peng, pawn@mail.si2lab.org Chun-Chi Chen, cheerchen.cs98g@g2.nctu.edu.tw Chung-Lin Lee, yoyoli.cs97g@nctu.edu.tw |
| Motorola Mobility | Krit Panusopone, krit@motorola.com Limin Wang, limin.wang@motorola.com Xue Fang, X.Fang@motorola.com Jae Hoon Kim, jaekim@motorola.com Wei-Ying Kung, Vivian.Kung@motorola.com |
| MediaTek | Yu-Wen Huang, yuwen.huang@mediatek.com Chih-Wei Hsu, cw.hsu@mediatek.com Chih-Ming Fu, chihming.fu@mediatek.com Yi-Wen Chen, yiwen.chen@mediatek.com Zhi Zhou, zhi.zhou@mediatek.com |
| SONY | Kazushi SATO, Kazushi.Sato@jp.sony.com |
| BBC | Marta Mrak, Marta.Mrak@bbc.co.uk |
| Mitsubishi | Kazuo Sugimoto, Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp Akira Minezawa, Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Nokia | Oguz Bici, ext-oguz.bici@nokia.com |
| Broadcom | Peisong Chen, peisongc@broadcom.com Wade Wan, wwan@broadcom.com |
| Fraunhofer HHI | Benjamin Bross, benjamin.bross@hhi.fraunhofer.de Philipp Helle, philipp.helle@hhi.fraunhofer.de Simon Oudin, simon.oudin@hhi.fraunhofer.de |
| Panasonic | Toshiyasu Sugio, sugio.toshiyasu@jp.panasonic.com Viktor Wahadaniah, viktor.wahadaniah@sg.panasonic.com |
| LG | Seungwook Park, seungwook.park@lge.com Yongjoon Jeon, yongjoon.jeon@lge.com Joonyoung Park, jy.park@lge.com |
| ETRI | S. Lee, shlee99@etri.re.kr |
| ZTE | Wen Zhang, zhang.wen4@zte.com.cn |

# CE2 Documents

The contributions submitted for CE2 along with the corresponding cross-check documents and related contributions are summarized as follow:.

- Summary document (this document)

  - JCTVC-G032[CE coordinators] CE2: Summary report on Motion Partitioning and OBMC

- Proposals and crosscheck documents

Subtest A.1: Non-Rectangular Motion Partitioning (NRMP)

  - Status: withdraw

Subtest B.1: OBMC for 2NxN and Nx2N Motion Partitions

  - Status: Software package was released on 13th September 2011.

Test results have been verified.

  - Proposal: JCTVC-G749 [Qualcomm] CE2: Overlapped Block Motion Compensation

  - Description: In OBMC, predictions in the overlapped regions are derived by weighted averaging prediction pixels from two different motion vectors. The overlapped region in the luma component is defined as 2 rows (columns) of pixels on both sides of the horizontal (vertical) partition boundary. For chroma components, the overlapped region is reduced to 1 row (column) of pixels. The weighting factors are (3/4, 1/4) for the pixels which are 1 row (column) apart from the boundary and (7/8, 1/8) for pixels which are 2 row (column) apart from the boundary. To limit the worst case memory bandwidth, for bi-prediction PU in 8x8 CU, the fetching of extra pixels (2 lines for luma, 1 line for chroma) is disabled, and thus there is no weighted weighted averaging for the other side of PU. By doing this the worst case memory bandwidth is not increasing for OBMC.

  - Cross-check: JCTVC-G432 [ETRI] CE2: Cross verification of Qualcomm's Overlapped Block Motion Compensation

  - Cross-check: JCTVC-G939(MediaTek) Non-CE2: Crosscheck for Qualcomm's overlapped block motion compensation in JCTVC-G749

  - Note: The results cross-checked by G432 and G939 are “Further improvement” results mentioned at section 3.

Subtest B.2: OBMC with Motion Merging

  - Status: withdraw

Subtest C.1: Non-Square Quadtree Transform (NSQT)

  - Status: Software package was released on 13th September 2011.

Test results have been verified.

  - Proposal1: JCTVC-G517 [HiSilicon, Tsinghua] CE2 subtest C.1: Harmonization of unified scan and NSQT

  - Description: A harmonized solution of unified scan and NSQT is provided at this contribution. 2Nx0.5N / 0.5Nx2N coefficient data to NxN coefficient data reordering process is proposed to be removed. 2Nx0.5N and 0.5Nx2N diagonal scans are also proposed to use at non-square block's coefficient coding.

  - Proposal2: JCTVC-G518 [Tsinghua, HiSilicon] CE2 subtest C.1: Non-square quadtree (NSQT) with 2x8 and 8x2 transform

  - Description: This contribution evaluates the coding performance and complexity of 2x8 and 8x2 transform under NSQT structure. A 2x2 hadamard-like transform is added to HEVC framework. Entropy coding and quantization process is same as non-square block's process at HM4.

  - Cross-check: JCTVC-G571 [HHI] CE2: Cross-check of Non-Square Quadtree Transform (NSQT) JCTVC-G517 & JCTVC-G518

Subtest C.2: Non-Square Transform for 2NxN and Nx2N Motion Partitions

  - Status: withdraw

Subtest C.3: RQT with rectangular transform unit

  - Status: withdraw

# Summary of Results

Test conditions as specified in JCTVC-F900 were used. The results for each tools is summarized as follow:

Subtest B.1 (JCTVC-G749 CE2: Overlapped Block Motion Compensation)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -1.2% | -1.1% | -0.5% | -0.5% | -0.5% |
| Class B | -0.5% | -0.8% | -0.8% | -0.5% | -0.6% | -0.6% |
| Class C | -0.9% | -1.2% | -1.0% | -0.7% | -1.0% | -1.2% |
| Class D | -0.8% | -0.9% | -1.0% | -0.6% | -0.9% | -0.7% |
| Class E |  |  |  |  |  |  |
| Overall | -0.6% | -1.0% | -1.0% | -0.6% | -0.7% | -0.7% |
|  | -0.6% | -1.0% | -1.0% | -0.6% | -0.7% | -0.7% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7% | -1.0% | -1.0% | -0.6% | -0.6% | -0.4% |
| Class C | -1.0% | -1.1% | -1.1% | -0.8% | -0.8% | -1.1% |
| Class D | -1.1% | -0.9% | -1.1% | -0.8% | -0.7% | -0.7% |
| Class E | -1.2% | -0.9% | 0.0% | -1.0% | -0.8% | -1.1% |
| Overall | -1.0% | -1.0% | -0.9% | -0.8% | -0.7% | -0.8% |
|  | -1.0% | -1.0% | -0.9% | -0.8% | -0.8% | -0.8% |
| Enc Time[%] | 96% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.4% | -1.6% | -1.4% | -1.7% | -1.2% | -1.3% |
| Class C | -2.0% | -2.0% | -2.2% | -1.9% | -1.5% | -1.9% |
| Class D | -2.7% | -2.3% | -2.2% | -2.5% | -1.7% | -2.2% |
| Class E | -2.2% | -1.7% | -1.5% | -2.3% | -1.7% | -1.6% |
| Overall | -2.0% | -1.9% | -1.8% | -2.0% | -1.5% | -1.7% |
|  | -2.1% | -1.9% | -2.0% | -2.1% | -1.5% | -1.7% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Further improvement (results and software are released after 13th September)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -1.5% | -1.0% | -0.5% | -0.6% | -0.7% |
| Class B | -0.5% | -0.9% | -0.8% | -0.5% | -0.8% | -0.7% |
| Class C | -0.9% | -1.3% | -1.2% | -0.8% | -1.1% | -1.3% |
| Class D | -0.7% | -1.0% | -1.0% | -0.6% | -0.9% | -0.8% |
| Class E |  |  |  |  |  |  |
| Overall | -0.6% | -1.2% | -1.0% | -0.6% | -0.8% | -0.8% |
|  | -0.6% | -1.2% | -1.0% | -0.6% | -0.8% | -0.8% |
| Enc Time[%] | 98% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7% | -1.1% | -0.9% | -0.6% | -0.6% | -0.5% |
| Class C | -1.0% | -1.4% | -1.4% | -0.9% | -0.8% | -1.3% |
| Class D | -1.1% | -1.1% | -0.7% | -0.8% | -0.6% | -0.9% |
| Class E | -1.1% | -0.7% | -0.8% | -1.0% | -0.7% | -0.7% |
| Overall | -0.9% | -1.1% | -0.9% | -0.8% | -0.7% | -0.8% |
|  | -0.9% | -1.1% | -1.0% | -0.8% | -0.7% | -0.8% |
| Enc Time[%] | 96% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.4% | -1.6% | -1.4% | -1.7% | -1.2% | -1.3% |
| Class C | -2.0% | -2.0% | -2.2% | -1.9% | -1.5% | -1.9% |
| Class D | -2.7% | -2.3% | -2.2% | -2.5% | -1.7% | -2.2% |
| Class E | -2.2% | -1.7% | -1.5% | -2.3% | -1.7% | -1.6% |
| Overall | -2.0% | -1.9% | -1.8% | -2.0% | -1.5% | -1.7% |
|  | -2.1% | -1.9% | -2.0% | -2.1% | -1.5% | -1.7% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Subtest C.1 (CTVC-G517 CE2 subtest C.1: Harmonization of unified scan and NSQT)

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% |
| Overall (without classF) | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] (without classF) | 101% |  |  |
| Dec Time[%] (without classF) | 99% |  |  |
| Overall (with classF) | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] (with classF) | 100% |  |  |
| Dec Time[%] (with classF) | 99% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.1% | -0.3% |
| Class E | 0.1% | -0.3% | 0.6% |
| Class F | -0.2% | -0.1% | 0.0% |
| Overall (without classF) | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% |
| Enc Time[%] (without classF) | 101% |  |  |
| Dec Time[%] (without classF) | 98% |  |  |
| Overall (with classF) | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% |
| Enc Time[%] (with classF) | 101% |  |  |
| Dec Time[%] (with classF) | 98% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.2% | 0.4% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | -0.1% | -0.6% |
| Class E | 0.0% | -0.5% | -0.2% |
| Class F | -0.3% | -0.2% | -0.2% |
| Overall (without classF) | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.0% | -0.1% |
| Enc Time[%] (without classF) | 101% |  |  |
| Dec Time[%] (without classF) | 98% |  |  |
| Overall (with classF) | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.2% |
| Enc Time[%] (with classF) | 101% |  |  |
| Dec Time[%] (with classF) | 99% |  |  |

Subtest C.1 (JCTVC-G518 CE2 subtest C.1: Non-square quadtree (NSQT) with 2x8 and 8x2 transform)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.6% | -0.5% | 0.1% | -0.2% | -0.3% |
| Class B | 0.1% | -0.8% | -0.7% | 0.0% | -0.6% | -0.4% |
| Class C | 0.1% | -0.6% | -0.5% | 0.0% | -0.6% | -0.5% |
| Class D | 0.0% | -0.7% | -1.0% | 0.0% | -0.6% | -0.8% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | -0.5% | -0.5% | -0.2% | -0.5% | -0.5% |
| Overall (without classF) | 0.0% | -0.7% | -0.7% | 0.0% | -0.5% | -0.5% |
|  | 0.0% | -0.7% | -0.7% | 0.0% | -0.5% | -0.5% |
| Enc Time[%] (without classF) | 101% |  |  | 100% |  |  |
| Dec Time[%] (without classF) | 99% |  |  | 100% |  |  |
| Overall (with classF) | 0.0% | -0.6% | -0.7% | 0.0% | -0.5% | -0.5% |
|  | 0.0% | -0.6% | -0.7% | 0.0% | -0.5% | -0.5% |
| Enc Time[%] (with classF) | 100% |  |  | 101% |  |  |
| Dec Time[%] (with classF) | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -1.6% | -1.7% | 0.0% | -0.8% | -0.8% |
| Class C | -0.1% | -1.2% | -1.2% | -0.2% | -0.9% | -1.2% |
| Class D | -0.2% | -1.5% | -2.0% | -0.3% | -1.3% | -1.3% |
| Class E | 0.0% | -0.4% | 0.3% | -0.1% | 0.6% | 0.2% |
| Class F | -0.9% | -1.5% | -1.4% | -0.7% | -1.4% | -1.6% |
| Overall (without classF) | -0.1% | -1.2% | -1.3% | -0.1% | -0.7% | -0.8% |
|  | -0.1% | -1.2% | -1.2% | -0.1% | -0.7% | -0.8% |
| Enc Time[%] (without classF) | 100% |  |  | 100% |  |  |
| Dec Time[%] (without classF) | 98% |  |  | 99% |  |  |
| Overall (with classF) | -0.2% | -1.3% | -1.3% | -0.3% | -0.8% | -1.0% |
|  | -0.2% | -1.3% | -1.3% | -0.2% | -0.9% | -1.0% |
| Enc Time[%] (with classF) | 101% |  |  | 100% |  |  |
| Dec Time[%] (with classF) | 99% |  |  | 99% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -1.8% | -1.9% | 0.0% | -0.9% | -1.1% |
| Class C | -0.2% | -1.5% | -1.5% | -0.3% | -1.2% | -1.1% |
| Class D | -0.3% | -1.8% | -1.8% | -0.4% | -1.0% | -1.5% |
| Class E | -0.1% | -0.6% | -0.8% | -0.1% | -0.5% | -0.3% |
| Class F | -1.0% | -1.3% | -1.9% | -0.8% | -1.4% | -1.5% |
| Overall (without classF) | -0.1% | -1.5% | -1.6% | -0.2% | -0.9% | -1.1% |
|  | -0.1% | -1.5% | -1.6% | -0.2% | -0.9% | -1.1% |
| Enc Time[%] (without classF) | 100% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 99% |  |  | 100% |  |  |
| Overall (with classF) | -0.3% | -1.5% | -1.6% | -0.3% | -1.0% | -1.1% |
|  | -0.3% | -1.5% | -1.6% | -0.3% | -1.0% | -1.2% |
| Enc Time[%] (with classF) | 100% |  |  | 101% |  |  |
| Dec Time[%] (with classF) | 100% |  |  | 100% |  |  |

# Analysis of CE2 tools

The following lists comments and analysis on CE2 tools came from CE2 participants.

JCTVC-G517  	CE2 subtest C.1: Harmonization of unified scan and NSQT

Comments:

- Since the description is not precise it is hard to confirm that the software does what was proposed

- Compiling the code with one or both of the two new macros disabled is not possible.

- A couple of bugs discovered when compiling the source code using gcc 4.4.5 and the provided make files.

  - Variables created but not used.

  - Variables created outside of the macro.

- A lot of confusing, undocumented changes and violations of the coding guidelines.

  - A macro NO_SMP_AMP_ME was added which is disabled by default.

  - NSQT checks whether vertical scan order is used whereas vertical scan is only used in intra (xRateDistOptQuant, l.3837).

  - Redundant code in the two branches of an if clause (getSigCtxInc, l.4219).

  - DTRACE_CABAC macros commented out.

  - Lines of code added without being in a macro (TEncCavlc::codeCoeffNxN, l.2224; TEncSbac::codeCoeffNxN, l.1639)

  - Lines of code added being under the macro CAVLC_COEF_LRG_BLK (TEncCavlc::codeCoeffNxN, l.2209)

  - Lines of code added being under the macro NSQT (TEncSbac::codeCoeffNxN, l.1765)

  - Two lines of memcpy commented out without being in a macro (TEncSearch::xEstimateResidualQT, l.4458)

  - Newly added lines of code are commented out.

  - A Lot of redundant code due to programming by copy and paste.

JCTVC-G518  	CE2 subtest C.1: Non-square quadtree (NSQT) with 2x8 and 8x2 transform

Comments:

- Small coding style violations like no line break and parentheses after if clause and redundant code due to programming by copy and paste.

- The shift used in the transforms of size 2 is always 1 which corresponds to log2(height). Transforms of all other size use an offset of 6 to be added to the shift (log2(height)+6)

- From our understanding of the code, some introduced changes to functions dealing with getting the x, y coordinates and sample array buffers of non-square transform blocks are not related to 2x8 and 8x2 blocks and could be applied independently.

The functions

TComYuv::getCbAddrPix(UInt iBlkX, UInt iBlkY)

TComYuv::getCrAddrPix(UInt iBlkX, UInt iBlkY)

are using the input parameters iBlkX and iBlkY shifted by one because these are obtained by

TComDataCU::getPixOffset(UInt uiTrMode,  UInt ui, UInt uiAbsPartIdx, UInt uiDepth, UInt& uiPix_X, UInt& uiPix_Y, TextType eTxt)

with eTxt set to TEXT_LUMA. Thus the default of eTxt is set to TEXT_LUMA and no longer passed to the function when called.

# Information - Non-CE2 Documents and non-CE tests

Besides CE2 documents, some contributions have strong relationship with CE2 activities. In CE6, A proposal related to the entropy coding of non-square blocks has been studied:

- JCTVC-G322 [Qualcomm] CE6.c: Harmonization of HE residual coding with non-square block transform

- Description: The mapping between non-square blocks to square blocks is not used. Instead, several modifications in the current ‘square-based’ entropy coding are described in order to deal with the coding of non-square coefficient blocks. Modifications are in the following parts: scans, coding of the last significant coefficient, significance map coding and coefficient level coding.

- Cross-check: JCTVC-G797 [HiSilicon] CE6.c: Cross-check report for JCTVC-G322 on Harmonization of HE residual coding with non-square block transforms

- Cross-check: JCTVC-G890[BBC] CE 6: Cross-check of G0322 on harmonization of non-square residual coding

Also some non-CE documents show strong relationship with CE2 activities. To better study and evaluate CE2 tools, those documents and corresponding cross-check proposals are listed as follow:

2. JCTVC-G202 [MediaTek] Non-CE2: Modified NSQT coefficient scan for CAVLC

  - Description: In this contribution, based on CE2.C.1, non-square transform blocks are directly coded using vertical scan. Extra non-square zigzag scan patterns can be removed in CAVLC.

  - Cross-check: JCTVC-G747 [HiSilicon] Non-CE2: Crosscheck of MediaTek's proposal on NSQT coefficient scan for CAVLC (JCTVC-G202)

3. JCTVC-G519 [HiSilicon, Tsinghua] Non-CE2: Harmonization of implicit TU, AMP and NSQT

  - Description: Harmonized solution of implicit TU, AMP and NSQT is provided at this contribution. When "QuadtreeTUMaxDepthInter" is set to 1, non-square TU is used at the case of AMP patitions.

  - Cross-check: JCTVC-G852 [Samsung] Cross-check report on Harmonization of implicit TU, AMP and NSQT (JCTVC-G519)

4. JCTVC-G521 [HiSilicon, Tsinghua] Non-CE2: Non-square hadamard transform for motion estimation and merge estimation

  - Description: This contribution provides a non-square hadamard transform solution which is used at motion estimation and merge estimation process. Different configurations of non-square hadamard transform are also discussed at the contribution.

  - Cross-check: JCTVC-G599 [HHI] Non-CE2: Cross-Check of Non-square hadamard transform for motion estimation and merge estimation

5. JCTVC-G750 [Qualcomm, LGE] Non-CE2: Harmonization of HE residual coding and NSQT

  - Description: Harmonization of CABAC with residual data coding of non-square block transforms (NSQT) is proposed. The method avoids the current mapping between square and non-square blocks. Scans and context derivation for non-square blocks are provided.

  - Cross-check: JCTVC-G596 [LGE] Cross-check report of Harmonization of HE residual coding and NSQT by Qualcomm and LGE (JCTVC-G750)

Results of non-CE2 tests are summarize as follows:

HM4.0 vs. HM4.0 – NSQT with 2x8 & 8x2 transform (G519) + encoder optimization

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.6% | -0.6% | 0.0% | -0.3% | -0.3% |
| Class B | 0.0% | -0.8% | -0.8% | -0.1% | -0.6% | -0.5% |
| Class C | -0.2% | -0.8% | -0.6% | -0.2% | -0.6% | -0.6% |
| Class D | -0.2% | -0.8% | -1.0% | -0.4% | -0.6% | -0.8% |
| Class E |  |  |  |  |  |  |
| Class F | -0.3% | -0.5% | -0.5% | -0.4% | -0.5% | -0.6% |
| Overall (without classF) | -0.1% | -0.7% | -0.8% | -0.2% | -0.5% | -0.5% |
|  | -0.1% | -0.7% | -0.7% | -0.2% | -0.5% | -0.5% |
| Enc Time[%] (without classF) | 102% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 100% |  |  | 100% |  |  |
| Overall (with classF) | -0.1% | -0.7% | -0.7% | -0.2% | -0.5% | -0.5% |
|  | -0.1% | -0.7% | -0.7% | -0.2% | -0.5% | -0.5% |
| Enc Time[%] (with classF) | 102% |  |  | 102% |  |  |
| Dec Time[%] (with classF) | 101% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -1.7% | -1.7% | -0.2% | -0.7% | -0.9% |
| Class C | -0.4% | -1.4% | -1.5% | -0.4% | -1.0% | -1.4% |
| Class D | -0.5% | -1.9% | -2.3% | -0.7% | -0.8% | -1.5% |
| Class E | -0.2% | -0.7% | 0.0% | -0.3% | 0.4% | 0.0% |
| Class F | -1.2% | -1.8% | -1.8% | -1.1% | -1.7% | -2.0% |
| Overall (without classF) | -0.3% | -1.5% | -1.5% | -0.4% | -0.6% | -1.0% |
|  | -0.3% | -1.5% | -1.5% | -0.4% | -0.7% | -1.0% |
| Enc Time[%] (without classF) | 101% |  |  | 102% |  |  |
| Dec Time[%] (without classF) | 99% |  |  | 100% |  |  |
| Overall (with classF) | -0.5% | -1.5% | -1.6% | -0.5% | -0.8% | -1.2% |
|  | -0.5% | -1.6% | -1.5% | -0.5% | -0.9% | -1.2% |
| Enc Time[%] (with classF) | 102% |  |  | 102% |  |  |
| Dec Time[%] (with classF) | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -2.0% | -2.2% | -0.2% | -0.9% | -0.9% |
| Class C | -0.4% | -1.6% | -1.8% | -0.5% | -1.3% | -1.4% |
| Class D | -0.6% | -2.4% | -2.2% | -0.8% | -1.5% | -1.5% |
| Class E | -0.2% | -0.6% | -1.0% | -0.3% | -0.1% | -0.3% |
| Class F | -1.3% | -1.5% | -1.7% | -1.3% | -1.7% | -1.6% |
| Overall (without classF) | -0.3% | -1.7% | -1.9% | -0.4% | -1.0% | -1.0% |
|  | -0.3% | -1.7% | -1.9% | -0.4% | -1.0% | -1.0% |
| Enc Time[%] (without classF) | 101% |  |  | 102% |  |  |
| Dec Time[%] (without classF) | 100% |  |  | 101% |  |  |
| Overall (with classF) | -0.5% | -1.7% | -1.8% | -0.6% | -1.1% | -1.1% |
|  | -0.5% | -1.7% | -1.9% | -0.6% | -1.1% | -1.2% |
| Enc Time[%] (with classF) | 102% |  |  | 102% |  |  |
| Dec Time[%] (with classF) | 102% |  |  | 101% |  |  |

JCTVC-G202 Non-CE2: Modified NSQT coefficient scan for CAVLC

| Random Access LC |  |  |
| --- | --- | --- |
| Y | U | V |
| 0.1% | 0.2% | 0.2% |
| -0.1% | -0.5% | -0.3% |
| 0.0% | -0.2% | -0.1% |
| 0.0% | -0.2% | -0.1% |
| 0.0% | -0.2% | -0.1% |
| 0.0% | -0.2% | -0.1% |
| 101% |  |  |
| 99% |  |  |
| Low Delay B LC |  |  |
| Y | U | V |
| -0.1% | -0.7% | -0.5% |
| 0.0% | -0.3% | -0.5% |
| -0.1% | -0.2% | -0.3% |
| -0.3% | -0.1% | -0.2% |
| -0.1% | -0.4% | -0.4% |
| -0.1% | -0.4% | -0.4% |
| 100% |  |  |
| 100% |  |  |
| Low Delay P LC |  |  |
| Y | U | V |
| -0.1% | -0.7% | -0.5% |
| 0.0% | -0.3% | -0.3% |
| -0.1% | -0.3% | -0.2% |
| -0.2% | -0.6% | -0.5% |
| -0.1% | -0.5% | -0.4% |
| -0.1% | -0.5% | -0.4% |
| 101% |  |  |
| 100% |  |  |

JCTVC-G519 Non-CE2: Harmonization of implicit TU, AMP and NSQT

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | -0.5% | -0.4% |
| Class B | -0.1% | -0.5% | -0.3% | -0.1% | -0.3% | -0.2% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% | 0.0% | 0.0% |
| Class D | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Overall (without classF) | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
|  | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
| Overall (with classF) | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
|  | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
| Enc Time[%] (without classF) | 101% |  |  | 100% |  |  |
| Enc Time[%] (with classF) | 99% |  |  | 100% |  |  |
| Dec Time[%] (without classF) | 101% |  |  | 100% |  |  |
| Dec Time[%] (with classF) | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.8% | -0.8% | -0.1% | -0.5% | -0.6% |
| Class C | -0.1% | -0.4% | -0.4% | -0.1% | -0.2% | 0.0% |
| Class D | -0.2% | -0.3% | -0.2% | -0.3% | 0.1% | 0.1% |
| Class E | -0.3% | -0.8% | 0.3% | -0.2% | -0.6% | -0.5% |
| Class F | -0.2% | -0.3% | 0.0% | -0.1% | -0.4% | -0.1% |
| Overall (without classF) | -0.2% | -0.6% | -0.3% | -0.2% | -0.3% | -0.3% |
|  | -0.2% | -0.6% | -0.3% | -0.2% | -0.3% | 0.0% |
| Overall (with classF) | -0.2% | -0.5% | -0.3% | -0.1% | -0.3% | -0.2% |
|  | -0.2% | -0.5% | -0.3% | -0.1% | -0.3% | -0.2% |
| Enc Time[%] (without classF) | 100% |  |  | 100% |  |  |
| Enc Time[%] (with classF) | 100% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 100% |  |  | 100% |  |  |
| Dec Time[%] (with classF) | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.8% | -1.0% | -0.1% | -0.4% | -0.2% |
| Class C | -0.1% | -0.5% | -0.2% | -0.1% | 0.0% | 0.0% |
| Class D | -0.2% | -0.1% | -0.2% | -0.2% | 0.3% | 0.1% |
| Class E | -0.2% | -0.8% | -0.1% | -0.2% | 0.5% | -0.2% |
| Class F | -0.2% | -0.1% | -0.1% | -0.1% | 0.1% | 0.2% |
| Overall (without classF) | -0.1% | -0.6% | -0.4% | -0.2% | 0.0% | -0.1% |
|  | -0.1% | -0.5% | -0.3% | -0.2% | 0.0% | -0.1% |
| Overall (with classF) | -0.1% | -0.5% | -0.4% | -0.1% | 0.0% | 0.0% |
|  | -0.2% | -0.4% | -0.3% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] (without classF) | 100% |  |  | 100% |  |  |
| Enc Time[%] (with classF) | 100% |  |  | 100% |  |  |
| Dec Time[%] (without classF) | 99% |  |  | 99% |  |  |
| Dec Time[%] (with classF) | 99% |  |  | 99% |  |  |

JCTVC-G521 Non-CE2: Non-square hadamard transform for motion estimation and merge estimation

HM4.0 vs. HM4.0 + NS-HAD

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
| Class B | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | -0.3% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Overall (without classF) | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] (without classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 99% |  |  | 100% |  |  |
| Overall (with classF) | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] (with classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (with classF) | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | -0.2% |
| Class C | -0.1% | -0.3% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class D | -0.1% | 0.0% | -0.1% | -0.1% | 0.2% | -0.1% |
| Class E | -0.1% | 0.2% | 0.3% | -0.2% | -0.2% | 0.4% |
| Class F | -0.1% | -0.2% | -0.2% | 0.1% | 0.0% | -0.1% |
| Overall (without classF) | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] (without classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 97% |  |  | 98% |  |  |
| Overall (with classF) | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] (with classF) | 102% |  |  | 101% |  |  |
| Dec Time[%] (with classF) | 98% |  |  | 99% |  |  |

HM4.0 + 2x8 & 8x2 transform vs. HM4.0 + 2x8 & 8x2 transform + NS-HAD

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class C | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% |
| Class D | -0.3% | -0.1% | 0.0% | -0.4% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Overall (without classF) | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |
| Enc Time[%] (without classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 101% |  |  | 100% |  |  |
| Overall (with classF) | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |
|  | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% |
| Enc Time[%] (with classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (with classF) | 102% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | 0.0% | -0.2% | 0.0% | -0.1% |
| Class C | -0.2% | -0.2% | -0.4% | -0.3% | -0.1% | -0.2% |
| Class D | -0.4% | -0.4% | -0.4% | -0.4% | 0.5% | -0.2% |
| Class E | -0.2% | -0.3% | -0.4% | -0.2% | -0.3% | -0.2% |
| Class F | -0.3% | -0.3% | -0.4% | -0.4% | -0.3% | -0.4% |
| Overall (without classF) | -0.2% | -0.3% | -0.2% | -0.3% | 0.1% | -0.2% |
|  | -0.2% | -0.3% | -0.2% | -0.3% | 0.0% | -0.2% |
| Enc Time[%] (without classF) | 101% |  |  | 102% |  |  |
| Dec Time[%] (without classF) | 101% |  |  | 102% |  |  |
| Overall (with classF) | -0.2% | -0.3% | -0.3% | -0.3% | 0.0% | -0.2% |
|  | -0.2% | -0.3% | -0.3% | -0.3% | 0.0% | -0.2% |
| Enc Time[%] (with classF) | 101% |  |  | 102% |  |  |
| Dec Time[%] (with classF) | 101% |  |  | 102% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.3% | -0.2% | 0.0% | 0.2% |
| Class C | -0.3% | -0.1% | -0.3% | -0.3% | -0.1% | -0.2% |
| Class D | -0.3% | -0.7% | -0.4% | -0.5% | -0.4% | 0.1% |
| Class E | -0.1% | -0.1% | -0.2% | -0.1% | 0.4% | 0.0% |
| Class F | -0.2% | -0.2% | 0.2% | -0.4% | -0.3% | -0.1% |
| Overall (without classF) | -0.2% | -0.2% | -0.3% | -0.3% | -0.1% | 0.0% |
|  | -0.2% | -0.2% | -0.3% | -0.3% | 0.0% | 0.1% |
| Enc Time[%] (without classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 101% |  |  | 101% |  |  |
| Overall (with classF) | -0.2% | -0.2% | -0.2% | -0.3% | -0.1% | 0.0% |
|  | -0.2% | -0.2% | -0.2% | -0.3% | -0.1% | 0.0% |
| Enc Time[%] (with classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (with classF) | 103% |  |  | 102% |  |  |

JCTVC-G750 Non-CE2: Harmonization of HE residual coding and NSQT

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | -0.21% | -0.22% |
| Class B | -0.05% | -0.10% | -0.07% |
| Class C | -0.03% | -0.11% | 0.01% |
| Class D | -0.02% | -0.14% | -0.13% |
| Class E |  |  |  |
| Overall | -0.03% | -0.14% | -0.10% |
|  | -0.02% | -0.13% | -0.10% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.06% | -0.18% | -0.29% |
| Class C | -0.06% | -0.22% | 0.00% |
| Class D | -0.03% | -0.10% | -0.21% |
| Class E | -0.03% | -0.37% | 0.12% |
| Overall | -0.05% | -0.20% | -0.12% |
|  | -0.04% | -0.21% | -0.13% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Reference

[1] X. Zheng, I. S. Chong, I-K Kim, “Core Experiment 2: Motion Partitioning and OBMC,” Doc. JCTVC-F902, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, IT, July 2011.

[2] Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.