| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G037_r3 WG11 Number: m21611 |
| --- | --- |

| Title: | CE7: Summary Report of Core Experiment on Additional Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories Chuohao Yeo Institute for Infocomm Research Rajan Joshi Qualcomm Felix Fernandes Samsung Electronics Co., Ltd. | Email: | cohen@merl.com chyeo@i2r.a-star.edu.sg rajanj@qualcomm.com felix.f@sta.samsung.com |
| Source: | Core Experiment 7 |  |  |

_____________________________

# Abstract

The purpose of Core Experiment 7 (CE7) is to characterize the performance, in terms of both compression efficiency and complexity, of several transforms other than those currently defined in HM 4.0. Three tools that operate on Intra blocks have been evaluated: A mode-dependent secondary transform applied for luma after the HM core transform (a unified Tool 1 and 2 from the CE description JCTVC-F907), a secondary rotational transform applied for luma after the HM core transform (Tool 3), and a mode-dependent DCT/DST for chroma (Tool 4), which replaces the HM core transform for chroma. This document provides a summary of activities and results for this core experiment.

# Summary of proposed tools and activities

Three types of additional transforms for Intra blocks have been evaluated:

- For luma, a mode-dependent secondary transform, applied after the HM core transform. This tool is a unified design for Tools 1 and 2 from the CE description JCTVC-F907 [1].

- For luma, a secondary rotational transform applied after the HM core transform.

- For chroma, a mode-dependent DCT/DST

Brief descriptions for each tool and associated activities are shown in Table 1. Bold text indicates the primary contact for the corresponding entry. Generalized block diagrams of the secondary transforms are shown in Figures 1 and 2. For more details, please refer to the document number associated with each tool or activity. For a list of documents and cross-checks related to CE7 but not performed as part of the CE7 activities defined in JCTVC-F907, please refer to Section 4.

Table 1. Description of tools and activities

| Tool # | Participants | Document | Syntax provided | Tool Description and cross-checkers |
| --- | --- | --- | --- | --- |
| Unified1&2 | A. Saxena, Y. Shibahara, F. Fernandes, T. Nishi | JCTVC-G108 | Yes | Depending on the intra prediction mode, a secondary transform may operate on the low-frequency coefficients of the 8x8 and larger HM core transform. Secondary transform sizes from 2x2 through 8x8 have been evaluated. This transform is derived from an analytical model. This tool unifies JCTVC-F224 and JCTVC-F554 from the previous meeting. |
|  | Cross-checks: 6x6, 8x8 3x3, 4x4 5x5 2x2, 7x7, 8x8 8x8 4x4 (HM transforms) | JCTVC-G814 JCTVC-G581 JCTVC-G425 JCTVC-G075 JCTVC-G930 JCTVC-G888 |  | 1. R. Cohen 2. R. Joshi 3. H. Yang 4. C. Yeo 5. A. Ichigaya 6. M. Mrak |
| 3 | Z. Ma, F. Fernandes, E. Alshina, A. Alshin | JCTVC-G304 | Yes | After the 8x8, 16x16, or 32x32 core transform is performed in the encoder, one of four 8x8 secondary Rotational Transforms (ROT) may be applied to the low-frequency transform coefficients output by the core transform. The decision as to which ROT to use is signaled by the encoder. Simplifications for improving complexity-performance tradeoffs will be investigated. This tool is from JCTVC-F294 from the previous meeting. |
|  | Cross-checks: | JCTVC-G375 JCTVC-G734 JCTVC-G580 |  | 1. Y. Shibahara, T. Nishi 2. P. Topiwala 3. R. Joshi |
| 4. | A. Saxena, F. Fernandes, E. Alshina, J. Chen | JCTVC-G107 | Yes | The 4x4 mode-dependent DCT/DST adopted in Working Draft 3 for luma blocks is extended to operate on chroma blocks. This tool is from JCTVC-F553 from the previous meeting. |
|  | Cross-checks: Tests 1,2,3,4,5,6,7 Tests 1,3,4 | JCTVC-G376 JCTVC-G076 |  | 1. Y. Shibahara, T. Nishi 2. C. Yeo |

Conditions

Decoder Operations

HM 4.0 NxN Inverse DCT

HM 4.0 Inverse Quant

K-Point Inv. Row Transforms

K-Point Inv. Col. Transforms

8x8 and larger TUs when IntraPredMode = Planar; VER-1, VER-2, …, VER-8; HOR-1, HOR-2, …, HOR-7

HM 4.0 NxN Inverse Transform

HM 4.0 Inverse Quant

4x4 TUs; 8x8 and larger TUs when IntraPredMode = DC

HM 4.0 NxN Inverse DCT

HM 4.0 Inverse Quant

K-Point Inv. Row Transforms

8x8 and larger TUs when IntraPredMode = HOR, HOR+1, HOR+2, …, HOR+8

HM 4.0 NxN Inverse DCT

HM 4.0 Inverse Quant

K-Point Inv. Col. Transforms

8x8 and larger TUs when IntraPredMode = VER, VER+1, VER+2, …, VER+8

Figure 1. Decoder operations for mode-dependent secondary transforms (Unified Tool 1&2).

HM 4.0 NxN Inverse DCT

HM 4.0 Inverse Quant

Vert. Inverse 8x8 ROTi

Horz. Inverse 8x8 ROTi

ROT Index i

0

1,2,3,4

HM 4.0 NxN Inverse Transform

HM 4.0 Inverse Quant

Conditions

4x4 TUs; 8x8 and larger TUs for which RD search signals i = 0

8x8 and larger TUs for which RD search signals i = 1,2,3,4

Decoder Operations

Figure 2. Example decoder operations for secondary ROT (Tool 3)

# Summary of Experimental Results

## Secondary transforms

Tables 2-3 summarize the BD-Rate, encoding time, and decoding time summaries for Unified Tool 1&2 and Tool 3, the secondary luma transforms for Intra.

Table 2. Performance summaries of secondary transforms tools for All-Intra configurations

| Tool | AI-HE |  |  | AI-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-Rate (%) | Enc Time (%) | Dec Time (%) | Y BD-Rate (%) | Enc Time (%) | Dec Time (%) |
| Unified 1&2: Mode-dependent secondary |  |  |  |  |  |  |
| 2x2 | -0.1 | 101 | 101 | -0.1 | 102 | 101 |
| 3x3 | -0.3 | 101 | 102 | -0.3 | 101 | 101 |
| 4x4 | -0.5 | 100 | 101 | -0.4 | 102 | 101 |
| 5x5 | -0.6 | 101 | 103 | -0.6 | 103 | 103 |
| 6x6 | -0.7 | 102 | 103 | -0.6 | 103 | 102 |
| 7x7 | -0.7 | 102 | 102 | -0.6 | 104 | 102 |
| 8x8 | -0.7 | 102 | 102 | -0.7 | 103 | 102 |
| 3: Secondary Rotational | -0.9 | 129 | 100 | -0.9 | 158 | 100 |

Table 3. Performance summaries of secondary transforms tools for Random Access configurations

| Tool | RA-HE |  |  | RA-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-Rate (%) | Enc Time (%) | Dec Time (%) | Y BD-Rate (%) | Enc Time (%) | Dec Time (%) |
| Unified 1&2: Mode-dependent secondary |  |  |  |  |  |  |
| 2x2 | -0.1 | 101 | 101 | -0.1 | 100 | 101 |
| 3x3 | -0.2 | 100 | 102 | -0.2 | 100 | 101 |
| 4x4 | -0.2 | 101 | 102 | -0.2 | 100 | 101 |
| 5x5 | -0.3 | 100 | 100 | -0.3 | 100 | 100 |
| 6x6 | -0.3 | 100 | 100 | -0.3 | 100 | 100 |
| 7x7 | -0.3 | 100 | 100 | -0.3 | 100 | 100 |
| 8x8 | -0.3 | 100 | 100 | -0.3 | 100 | 100 |
| 3: Secondary Rotational | -0.5 | 103 | 98 | -0.5 | 104 | 99 |

## Chroma transforms

Tables 5-6 summarizes the BD-Rate, encoding time, and decoding time summaries for the Tool 4 of Table 1, the chroma transform. The anchor used in all cases is HM 4.0.

Table 4 shows the summary of the test conditions and when the DCT/DST is applied for various chroma angular intra prediction modes (excluding LM). An ‘E’ in the column means that DCT/DST is applied if that mode is explicitly signaled, and a ‘D’ in the column means that DCT/DST is applied if that mode is signaled as a derived mode (DM). For example, Test 1 applies the DCT/DST to chroma components only when the intra prediction mode is horizontal or vertical and is explicitly signaled. Test 5 applies the DCT/DST only when the intra prediction mode is Planar. For all other modes and for luma, the system operates identically to HM 4.0. Additional tests conducted after formal CE7 activities were concluded can be found in the documents listed for Tool 4 in Table 1. The proponents of this tool recommend that Test 5 shown here or a later Test 8 (“E” on HOR, VER, Planar, and VER+8; results almost the same as Tests 7 and 2) from JCTVC-G108 be the focus for discussion.

Table 4. Tests for chroma transforms

| Test | HOR | VER | Planar | VER+8 | Other oblique modes | DC |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | E | E | - | - | - | - |
| Test 2 | E | E | E | - | - | - |
| Test 3 | E,D | E,D | E,D | - | - | - |
| Test 4 | E,D | E,D | E,D | E,D | D | - |
| Test 5 | - | - | E,D | - | - | - |
| Test 6 | E,D | E,D | - | - | - | - |
| Test 7 | E | E | E,D | E | - | - |

Table 5. Performance summaries of chroma transform tools for All-Intra configurations

| Test | AI-HE |  |  |  |  | AI-LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-Rate (%) | U BD-Rate (%) | V BD-Rate (%) | Enc Time (%) | Dec Time (%) | Y BD-Rate (%) | U BD-Rate (%) | V BD-Rate (%) | Enc Time (%) | Dec Time (%) |
| 1 | 0.0 | -0.4 | -0.4 | 101 | 100 | 0.0 | -0.6 | -0.7 | 101 | 101 |
| 2 | 0.0 | -0.7 | -0.8 | 101 | 100 | 0.0 | -1.0 | -1.2 | 101 | 101 |
| 3 | 0.0 | -0.4 | -0.5 | 101 | 101 | 0.0 | -0.8 | -0.9 | 102 | 102 |
| 4 | -0.1 | 0.3 | 0.3 | 101 | 102 | -0.1 | -0.3 | -0.5 | 102 | 103 |
| 5 | -0.1 | -0.5 | -0.6 | 101 | 99 | -0.1 | -0.9 | -1.0 | 101 | 99 |
| 6 | 0.0 | -0.1 | -0.2 | 101 | 99 | 0.0 | -0.3 | -0.5 | 101 | 100 |
| 7 | 0.0 | -0.6 | -0.7 | 101 | 100 | 0.0 | -1.1 | -1.2 | 102 | 101 |

Table 6. Performance summaries of chroma transform tools for Random Access configurations

| Test | RA-HE |  |  |  |  | RA-LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-Rate (%) | U BD-Rate (%) | V BD-Rate (%) | Enc Time (%) | Dec Time (%) | Y BD-Rate (%) | U BD-Rate (%) | V BD-Rate (%) | Enc Time (%) | Dec Time (%) |
| 1 | 0.0 | -0.5 | -0.7 | 100 | 99 | 0.0 | -0.7 | -0.5 | 100 | 100 |
| 2 | 0.0 | -0.9 | -1.1 | 100 | 99 | 0.0 | -0.9 | -0.9 | 100 | 100 |
| 3 | 0.0 | -0.8 | -0.9 | 101 | 102 | 0.0 | -0.9 | -0.8 | 101 | 103 |
| 4 | 0.0 | -0.4 | -0.6 | 101 | 102 | 0.0 | -0.5 | -0.4 | 101 | 102 |
| 5 | 0.0 | -0.7 | -0.8 | 100 | 99 | 0.0 | -0.7 | -0.7 | 100 | 100 |
| 6 | 0.1 | -0.5 | -0.6 | 100 | 99 | 0.0 | -0.5 | -0.6 | 100 | 99 |
| 7 | 0.0 | -0.9 | -1.1 | 100 | 100 | 0.0 | -1.0 | -1.0 | 100 | 101 |

# Summary of complexity

## Secondary transforms

As part of CE7 reporting, proponents were required to give details on the worst-case increase in operations count when performing the inverse transform due to the use of the secondary transforms tool, as well as the average-case increase. These results are summarized in Tables 7 and 8.

Table 7. Summary of worst-case operations count for secondary luma transforms

| Transform size | Operations | HM4 (DCT) | Unified Tools 1&2: Mode-dependent secondary transform |  |  |  |  |  |  | Tool 3: Secondary ROT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2x2 | 3x3 | 4x4 | 5x5 | 6x6 | 7x7 | 8x8 |  |
| 4x4 | Mults | 48 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Adds | 96 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Shifts | 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Clips | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8x8 | Mults | 352 | +16 | +54 | +128 | +250 | +432 | +686 | +1024 | +240 |
|  | Adds | 576 | +16 | +54 | +128 | +250 | +432 | +686 | +1024 | +288 |
|  | Shifts | 128 | +8 | +18 | +32 | +50 | +72 | +98 | +128 | +336 |
|  | Clips | 0 | +8 | +18 | +32 | +50 | +72 | +98 | +128 | 0 |
| 16x16 | Mults | 2752 | +16 | +54 | +128 | +250 | +432 | +686 | +1024 | +240 |
|  | Adds | 3712 | +16 | +54 | +128 | +250 | +432 | +686 | +1024 | +288 |
|  | Shifts | 512 | +8 | +18 | +32 | +50 | +72 | +98 | +128 | +336 |
|  | Clips | 0 | +8 | +18 | +32 | +50 | +72 | +98 | +128 | 0 |
| 32x32 | Mults | 21888 | +16 | +54 | +128 | +250 | +432 | +686 | +1024 | +240 |
|  | Adds | 25856 | +16 | +54 | +128 | +250 | +432 | +686 | +1024 | +288 |
|  | Shifts | 2048 | +8 | +18 | +32 | +50 | +72 | +98 | +128 | +336 |
|  | Clips | 0 | +8 | +18 | +32 | +50 | +72 | +98 | +128 | 0 |

Table 8. Summary of average-case operations count for secondary luma transforms

| Transform size | Operations | HM4 (DCT) | Unified Tools 1&2: Mode-dependent secondary transform |  |  |  |  |  |  | Tool 3: Secondary ROT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2x2 | 3x3 | 4x4 | 5x5 | 6x6 | 7x7 | 8x8 |  |
| 4x4 | Mults | 48 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Adds | 96 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Shifts | 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Clips | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8x8 | Mults | 352 | +11 | +39 | +91 | +179 | +309 | +490 | +731 | +163 |
|  | Adds | 576 | +11 | +39 | +91 | +179 | +309 | +490 | +731 | +230 |
|  | Shifts | 128 | +6 | +13 | +23 | +36 | +51 | +70 | +91 | +298 |
|  | Clips | 0 | +6 | +13 | +23 | +36 | +51 | +70 | +91 | 0 |
| 16x16 | Mults | 2752 | +11 | +39 | +91 | +179 | +309 | +490 | +731 | +163 |
|  | Adds | 3712 | +11 | +39 | +91 | +179 | +309 | +490 | +731 | +230 |
|  | Shifts | 512 | +6 | +13 | +23 | +36 | +51 | +70 | +91 | +298 |
|  | Clips | 0 | +6 | +13 | +23 | +36 | +51 | +70 | +91 | 0 |
| 32x32 | Mults | 21888 | +11 | +39 | +91 | +179 | +309 | +490 | +731 | +163 |
|  | Adds | 25856 | +11 | +39 | +91 | +179 | +309 | +490 | +731 | +230 |
|  | Shifts | 2048 | +6 | +13 | +23 | +36 | +51 | +70 | +91 | +298 |
|  | Clips | 0 | +6 | +13 | +23 | +36 | +51 | +70 | +91 | 0 |

## Chroma transforms

When a 2D DST-VII or a combination of DCT/DST-VII is used as the 2D transform for coding of chroma intra prediction residual for certain chroma intra prediction types, it is expected that the number of operations required will increase. To have some insights into the average case, an analysis of the average number of operations required for performing the inverse transform of 4x4 chroma intra prediction residual was carried out. The analysis assumes a simple model, whereby each combination of one of 35 Luma intra prediction mode and one of 6 Chroma intra prediction mode is equi-probable. Therefore, the probability of a particular chroma intra prediction type (either explicitly signaled or as a derived mode) being used can be computed and used to compute the average number of operations. More details about this computation can be found in JCTVC-G107 and JCTVC-G076.

Table 9 summarizes the number of operations for each test configuration in Tool 4, as well as the percentage increase over HM 4.0. Note that in the worst case, corresponding to applying the 2D DST-VII, 64 multiplications, 120 additions and 32 bitshifts are required.

Table 9. Average operations count for each test configuration Tool 4 (Chroma transforms)

| Case | HM4 | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 | Test 6 | Test 7 | Test 8 | Test 9 | Test 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mults | 48 | 50.59 | 53.18 | 53.33 | 55.24 | 50.67 | 50.67 | 54.41 | 53.33 | 50.59 | 53.52 |
| Adds | 96 | 99.89 | 103.77 | 104.00 | 106.86 | 100.00 | 100.00 | 104.11 | 104.00 | 99.89 | 104.29 |
| Shifts | 32 | 32.00 | 32.00 | 32.00 | 32.00 | 32.00 | 32.00 | 32.00 | 32.00 | 32.00 | 32.00 |
| ΔMults (%) | - | 5.40 | 10.79 | 11.11 | 15.08 | 5.56 | 5.56 | 11.26 | 11.11 | 5.40 | 11.51 |
| ΔAdds ( %) | - | 4.05 | 8.10 | 8.33 | 11.31 | 4.17 | 4.17 | 8.45 | 8.33 | 4.05 | 8.64 |
| ΔShifts (%) | - | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

# Non-CE7  (CE7-related) documents

The table below lists documents that are related to CE7 but were not part of the formal CE or were not conducted on tools scheduled for CE7.

| Directly-Related CE7 tool | Document No. | Title | Authors |
| --- | --- | --- | --- |
| Tool 1&2 Mode Dep Secondary | JCTVC-G629 | Non CE 7: Harmonization of SDIP and Mode-Dependent Secondary Transforms | A. Saxena (Samsung), Y. Shibahara (Panasonic), F. Fernandes (Samsung), T Nishi (Panasonic) |
|  | JCTVC-G937 | Cross-check of JCTVC-G629 : Harmonization of SDIP and Mode-Dependent Secondary Transforms | H. Yang (Huawei), C. Lai (Hisilicon) |
| Tool 3 Secondary ROT | JCTVC-G591 | Non-CE 7: Supplementary Results for the Rotational Transform | Z. Ma, F. Fernandes, E. Alshina, A. Alshin (Samsung) |
| Tool 4 Chroma DCT/DST | JCTVC-G282 | Non-CE7: Mode Dependent DCT/DST for Chroma | X. Zhao, X. Guo, M. Guo, S. Lei (MediaTek), S. Ma, W. Gao (PKU) |
|  | JCTVC-G879 | Non-CE 7: Cross Check Report for proposal on mode dependent DCT/DST for Chroma (JCTVC-G282) | A. Saxena, F. Fernandes (Samsung) |
| New tools |  |  |  |
| Inter transform | JCTVC-G281 | Non-CE7: Boundary Dependent Transform for Inter Predicted Residue | J. An, X. Zhao, X. Guo, S. Lei (MediaTek) |
| Secondary inter transform | JCTVC-G632 | Non CE 7: On secondary transforms for inter prediction residual | A. Saxena, F. Fernandes (Samsung) |
| 8x8 mode-dependent DCT/DST | JCTVC-G345 | Non-CE7: Restricted mode-dependent 8x8 DST for Intra prediction | A. Ichigaya, Y. Sugito, S. Sakaida (NHK) |
|  | JCTVC-G630 | Cross Check of G-345: Restricted Mode-Dependent 8x8 DST for Intra prediction | A. Saxena, F. Fernandes (Samsung) |
|  | JCTVC-G817 | Non-CE7: Cross verification of JCTVC-G345, Restricted mode-dependent 8x8 DST for Intra prediction | R. Cohen (MERL) |

# Summary

Two kinds of secondary luma transforms for intra TUs were tested: A mode-dependent secondary transorm, and a secondary rotational transform. Tools 1 and 2 from the original CE 7 description were unified while this CE was being conducted, resulting in a “Unified Tools 1&2” mode-dependent secondary transform.  From an encoder perspective, after the 8x8 or larger HM transform is applied, a secondary transform may be applied on the low-frequency coefficients, depending upon the intra prediction mode. Secondary transform sizes ranging from 2x2 through 8x8 were tested, resulting in luma BD-Rate gains ranging from 0.1% to 0.7% for both AI-HE and AI-LC configurations.  Encoder and decoder run time ratios ranged from 100% to 104%.

For Tool 3, the secondary rotational transform, after the 8x8 or larger HM transform is applied, one of four 8x8 secondary transforms may be applied as indicated by an explicit signal in the bit-stream. BD-Rate gains were 0.9% for both AI-HE and AI-LC configurations. Encoder run time ratios were 129% for AI-HE and 158% for AI-LC. Decoder run time ratios were 100% for both AI-HE and AI-LC.

Tool 4 was a primary transform, which replaces the existing HM transform for chroma components. The 4x4 mode-dependent DCT/DST already being used for 4x4 luma blocks in HM was extended to operate on chroma blocks, using the same mode-mapping table. Because the chroma intra prediction mode may be explicitly signaled or derived, several tests were performed, applying the mode-dependent DCT/DST on chroma components depending upon the value of the intra prediction mode and whether that mode was explicitly signaled or derived. For the tests that yielded the most gain, BD-Rate improvements for the chroma components ranged from 0.5% to 0.8% for AI-HE and 1.1% to 1.2% for AI-LC. Encoder and decoder run time ratios ranged from 99% to 103%.

For all tools, summaries of operation counts were provided. For secondary transforms applied after the HM transform (or before the HM inverse transform in the decoder), operation counts indicate the additional operations incurred with the proposed tool. For the primary chroma transforms, the operation counts are compared to the existing HM transform which they replace.

In addition to the tests conducted as a formal part CE7, later tests can be found in the documents corresponding to the three proposed tools. A table of related documents is also provided, listing cross-checks of those tests and additional transforms proposed outside CE7.

# Participant list

| Name | Affiliation | Email |
| --- | --- | --- |
| Robert Cohen (coordinator) | Mitsubishi Electric Research Labs | cohen@merl.com |
| Chuohao Yeo (coordinator) | Institute for Infocomm Research | chyeo@i2r.a-star.edu.sg |
| Rajan Joshi (coordinator) | Qualcomm | rajanj@qualcomm.com |
| Felix Fernandes (coordinator) | Samsung | felix.f@sta.samsung.com |
| Elena Alshina | Samsung | elena_a.alshina@samsung.com |
| Ankur Saxena | Samsung | asaxena@sta.samsung.com |
| Il-Koo Kim | Samsung | ilkoo.kim@samsung.com |
| Youji Shibahara | Panasonic | shibahara.youji@jp.panasonic.com |
| Takahiro Nishi | Panasonic | nishi.takahiro@jp.panasonic.com |
| Marta Mrak | BBC | Marta.Mrak@bbc.co.uk |
| Pankaj Topiwala | FastVDO | pankaj@fastvdo.com |
| Haitao Yang | Huawei | haitao.yang@huawei.com |
| Atsuro Ichigaya | NHK | ichigaya.a-go@nhk.or.jp |
| Akiyuki Tanizawa | Toshiba | akiyuki.tanizawa@toshiba.co.jp |
| Jun Yamaguchi | Toshiba | jun2.yamaguchi@toshiba.co.jp |
| Shinichi Sakaida | NHK | sakaida.s-gq@nhk.or.jp |
| Tung Nguyen | HHI | tung.nguyen@hhi.fraunhofer.de |
| Madhukar Budagavi | TI | madhukar@ti.com |
| TK Tan | NTT DOCOMO | ttk@pacific.net.sg |
| Kiran Misra | Sharp Labs of America | misrak@sharplabs.com |
| Jewon Kang | Nokia | ext-jewon.kang@nokia.com |
| Amir Naghdinezhad | eBrisk Video | amir@ebriskvideo.com |

# References

- R. Cohen, C. Yeo, R. Joshi, F. Fernandes, “Description of Core Experiment 7: Additional Transforms”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F907, Torino, IT, July 2011.

- HM 4.0 software, available at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware