| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G039 |
| --- | --- |

| Title: | CE9: Summary Report of Core Experiment on MV Coding and Skip/Merge Operations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | Benjamin Bross Einsteinufer 37 10587 Berlin Joel Jung 38-40 rue du Général Leclerc 92130 ISSY LES MOULINEAUX France Wei-Jung Chien Il-Koo Kim Minhua Zhou | Tel: Email: Tel: Email: | benjamin.bross@hhi.fraunhofer.de +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

In this CE report test results for the five different categories Simplification, Modification of the AMVP candidate selection, Parallel merge/skip, Additional merge mode and Merge/Skip operations for identical motion are reported.

# Introduction

The description of core experiment (CE) 9 on MV coding and Skip/Merge operations [1] was made in previous Torino meeting. There are six categories in CE9 description: Simplification, Modification of the AMVP/merge list pruning, Modification of the AMVP candidate selection, Parallel merge/skip, Additional merge mode and Merge/Skip operations for identical motion. Experiments were conducted aligned with CE9 descriptions in order to evaluate the coding efficiency and complexity for methods in each category. Specifically, all experiments were conducted using HM-4.0 reference software under the common test condition [2]. Since all tests for modification of the AMVP/merge list pruning are withdrawn by proponent, descriptions and results for remaining 5 categories are presented in details in the following sections.

# Document lists

## Simplification (SP)

Tester1

JCTVC-G084 CE9: Test results on SP01, SP02, SP03 and SP04

Tester2

JCTVC-G261 CE9: cross-check of experiment SP01 – Remove TMVP center position (JCTVC-G084)

JCTVC-G706 CE9: Cross-check report for TI's JCTVC-F083 by Motorola Mobility

JCTVC-G689 CE09: Crosscheck report of JCTVC-G084 test SP04

## Modification of the AMVP candidate selection (AMVP_SEL)

Tester1

JCTVC-G053 CE9: Results of AMVP_SEL01 and AMVP _SEL02

JCTVC-G702 CE9: Simplification of MVP Design for HEVC

Tester2

JCTVC-G513 CE9: Cross-check result of Sel01&02

JCTVC-G101 CE9 subset 2.3: cross-verification of MMI proposal on AMVP simplification

## Parallel merge/skip (MRG_PAR)

Tester1

JCTVC-G085 CE9: Test results on parallelized merge/skip mode

Tester2

JCTVC-G404 CE9: Cross-check of parallelized merge/skip mode (JCTVC-G085)

JCTVC-G179 CE9: Cross-check report of MRG_PAR03 and 04 on Parallel ME

## Additional merge mode (MRG_MVD)

Tester1

JCTVC-G052 CE9: Results of MRG_MVD series

Tester2

JCTVC-G190 CE9: Cross-verification result of MRG_MVD01

JCTVC-G536 CE9: Cross-check report of MRG_MVD02 by Panasonic

JCTVC-G688 CE09: Crosscheck report of JCTVC-G052 test MRG_MVD03

## Merge/Skip operations for identical motion (MRG_ID)

Tester1

JCTVC-G439 CE9 subtest 2.6: Results of MRG_ID01 in Merge/Skip operations for identical motion

JCTVC-G421 CE9: Results of Experiments MRG_ID02, MRG_ID03, and MRG_ID04

Tester2

JCTVC-G422 CE9: Cross-check for MRG_ID01 (JCTVC-G439)

JCTVC-G745 CE9: CE9: Cross-check report on MRG_ID02 A and B

JCTVC-G161 CE9: cross-verification of ETRI proposal on merge/skip operations for identical motion (MRG_ID03 A, MRG_ID03 B)

JCTVC-G437 CE9 subtest 2.6: Cross-checking reports of MRG_ID04 (ETRI/KHU’s proposal)

# Description of experiments and summary results

There are six categories in CE9. Since all tests for modification of the AMVP/merge list pruning are withdrawn by proponent, descriptions and results for remaining 5 categories are presented shortly in the following sections. Detailed descriptions can be found in JCTVC-F909 [1] and detailed results in the CE9 documents listed in section 2.

## Simplification

### Description of experiments

In this category, two aspects of temporal motion vector prediction (TMVP) are tested. The first one is the coding efficiency of TMVP center position, the other is coding efficiency of TMVP in the AMVP and Merge lists. SP01 is designed to quantify the coding efficiency of TMVP center position by using top-left corner instead of center position. SP02, SP03 and SP04 are designed to evaluate the benefits of TMVP in AMVP and Merge list construction.

Table 1 Description of experiments for simplification

| Test | Description | Related Doc. | Tester1 | Tester2 | S/W release |
| --- | --- | --- | --- | --- | --- |
| SP01 | Remove TMVP center position | JCTVC-F081 | TI | Orange Labs | 12 Sep. |
| SP02 | Disable AMVP TMVP use it in ME | JCTVC-F083 | TI | Motorola Mobility | 12 Sep. |
| SP03 | Disable AMVP TMVP | JCTVC-F083 | TI | Motorola Mobility | 12 Sep. |
| SP04 | Disable AMVP and merge/skip TMVP | JCTVC-F083 | TI | Qualcomm | 12 Sep. |

### Summary of experimental results

Table 2 Summary of experimental results for simplification

| Exp. | RA-HE |  |  | RA-LC |  |  | LD-HE |  |  | LD-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time |
| SP01 | 0.1 | 102 | 105 | 0.1 | 108 | 120 | 0.1 | 104 | 104 | 0.1 | 104 | 114 |
| xCheck | 0.1 | - |  | 0.1 |  |  | 0.1 |  |  | 0.1 |  |  |
| SP02 | 0.3 | 100 | 100 | 0.3 | 102 | 107 | 0.6 | 103 | 108 | 0.7 | 101 | 101 |
| xCheck | 0.3 | 99 | 98 | 0.3 | 100 | 101 | 0.6 | 100 | 99 | 0.7 | 101 | 100 |
| SP03 | 0.3 | 97 | 96 | 0.4 | 99 | 100 | 0.6 | 99 | 98 | 0.7 | 99 | 101 |
| xCheck | 0.3 | 99 | 98 | 0.4 | 100 | 100 | 0.6 | 100 | 100 | 0.7 | 101 | 102 |
| SP04 | 2.4 | 95 | 95 | 2.6 | 99 | 100 | 3.4 | 101 | 95 | 4.0 | 98 | 94 |
| xCheck | 2.4 | 99 | 99 | 2.6 | 99 | 97 | 3.4 | 99 | 97 | 4.0 | 100 | 96 |

## Modification of the AMVP/merge list pruning

All tests are withdrawn.

## Modification of the AMVP candidate selection

### Description of experiments

This category tries to improve the AMVP candidate selection process. AMVP_SEL01 and AMVP_SEL02 are designed to evaluate the benefit of grouping the position of candidates as 1 group instead of 2 groups in HM-4.0. Two spatial candidates S0 and S1 are derived by scanning the same spatial neighboring positions but in one group. AMVP_SEL03 and AMVP_SEL04 are designed to quantify the effects when list1 MVP is derived from list0 MVP by scaling.

Table 3 Description of experiments for AMVP candidate selection

| Test | Description | Related Doc. | Tester1 | Tester2 | SW release |
| --- | --- | --- | --- | --- | --- |
| AMVP_SEL01 | one-pass scanning | JCTVC-F419 | JVC Kenwood | Sony | 13 Sep. |
| AMVP_SEL02 | two-pass scanning | JCTVC-F419 | JVC Kenwood | Sony | 13 Sep. |
| AMVP_SEL03 | Scaling factor based on temporal distance | JCTVC-F575 | Motorola Mobility | TI | 13 Sep. 26. Sep. (fix) 27. Sep (fix) |
| AMVP_SEL04 | Scaling factor equal to 1 | JCTVC-F575 | Motorola Mobility | TI | 13 Sep. 26. Sep. (fix) 27. Sep (fix) |

### Summary of experimental results

Table 4 Summary of experimental results for AMVP candidate selection

| Exp. | RA-HE |  |  | RA-LC |  |  | LD-HE |  |  | LD-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time |
| SEL01 | 0.0 | 99 | 99 | 0.0 | 99 | 100 | 0.0 | 99 | 100 | 0.0 | 99 | 100 |
| xCheck | 0.0 | 100 | 100 | 0.0 | 100 | 100 | 0.0 | 100 | 100 | 0.0 | 100 | 100 |
| SEL02 | 0.0 | 100 | 100 | 0.0 | 100 | 100 | 0.0 | 100 | 100 | 0.0 | 100 | 100 |
| xCheck | 0.0 | 101 | 100 | 0.0 | 100 | 100 | 0.0 | 99 | 99 | 0.0 | 100 | 100 |
| SEL03 | 0.2 | 110 | 176* | 0.2(0.3)** | 101 | 229* | 0.0 | 108 | 186* | 0.0 | 99 | 231* |
| xCheck | 0.2 | 97 | 96 | 0.2(0.3)** | 99 | 100 | 0.0 | 99 | 97 | 0.0 | 99 | 102 |
| SEL04 | 0.4 | 99 | 98 | 0.4 | 100 | 101 | 0.5 | 100 | 100 | 0.3 | 100 | 101 |
| xCheck | 0.4 | 100 | 101 | 0.4 | 98 | 100 | 0.5 | 100 | 101 | 0.3 | 98 | 100 |

*According to the proponent, highlighted runtimes may not be accurate because the anchor and the test have been simulated using different platforms.

** Piece-wise cubic interpolation differs from cubic interpolation (in parentheses) for Nebuta. The RD plot is a straight line.

## Parallel merge/skip

### Description of experiments

In this category, methods to enable the parallel processing of merge and skip are evaluated. In these methods, a LCU is divided into a number of non-overlapped motion estimation regions (MER) of square shape based. Within a MER, all motion vectors are estimated in parallel, while from MER to MER motion estimation is performed sequentially. By marking spatial neighboring PUs as unavailable for the merge/skip candidate list derivation process, parallel merge/skip mode is possible for all PUs and CUs inside a MER if they belong to a same MER as the current PU.

In the current HM4.0 software the regular motion estimation is disabled for CUs larger than 8x8 if prediction mode is not 2Nx2N. This leads to problem for parallel merge/skip with the current HM4.0 method, that is, if the merge ME is also disabled for certain CUs due to parallel ME, the encoder will crash. In the new anchor, ME is enabled for CUs larger than 8x8 if prediction mode is not 2Nx2N. As shown in Table 5 below, the modified anchor shows average 0.3% gain relative to the HM4.0 anchor under a penalty of average 15% encoder runtime increase.

Table 5 BD-rate difference (anchor: HM4.0, tested: modified anchor)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.7% | -0.8% | -0.2% | -0.3% | -0.3% |
| Class B | -0.3% | -0.4% | -0.3% | -0.2% | -0.3% | -0.1% |
| Class C | -0.2% | -0.3% | -0.2% | -0.2% | -0.1% | -0.2% |
| Class D | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -0.2% | -0.4% | -0.4% | -0.2% | -0.2% | -0.2% |
|  | -0.2% | -0.4% | -0.4% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 111% |  |  | 116% |  |  |
| Dec Time[%] | 99% |  |  | 109% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.4% | -0.6% | -0.3% | -0.3% | -0.4% |
| Class C | -0.3% | -0.2% | -0.5% | -0.2% | 0.0% | -0.3% |
| Class D | -0.3% | -0.3% | 0.2% | -0.2% | -0.4% | -0.1% |
| Class E | -0.6% | -0.4% | 0.2% | -0.4% | -0.5% | -0.7% |
| Class F |  |  |  |  |  |  |
| Overall | -0.4% | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% |
|  | -0.4% | -0.4% | -0.3% | -0.3% | -0.3% | -0.3% |
| Enc Time[%] | 119% |  |  | 112% |  |  |
| Dec Time[%] | 101% |  |  | 96% |  |  |

Table 6 Description of experiments for parallel merge/skip

| Test | log2_parallel_merge_level_minus2 | Motion estimation region size | Merge/skip mode handling in parallel ME environment | Tester1 | Tester2 | SW release |
| --- | --- | --- | --- | --- | --- | --- |
| HM4.0 reference | 0 | 4 x 4 |  | - | - |  |
| MRG_PAR01a | 1 | 8 x 8 | HM4.0 | TI | Microsoft | 12 Sep. |
| MRG_PAR01b | 1 | 8 x 8 | Proposed | TI | Microsoft | 12 Sep. |
| MRG_PAR02a | 2 | 16 x 16 | HM4.0 | TI | Microsoft | 12 Sep. |
| MRG_PAR02b | 2 | 16 x 16 | Proposed | TI | Microsoft | 12 Sep. |
| MRG_PAR03a | 3 | 32 x 32 | HM4.0 | TI | LGE | 12 Sep. |
| MRG_PAR03b | 3 | 32 x 32 | Proposed | TI | LGE | 12 Sep. |
| MRG_PAR04a | 4 | 64 x 64 | HM4.0 | TI | LGE | 12 Sep. |
| MRG_PAR04b | 4 | 64 x 64 | Proposed | TI | LGE | 12 Sep. |

### Summary of experimental results

Table 7 Summary of experimental results for parallel merge/skip

| Test Anchor: a Tested: b | RA-HE |  |  | RA-LC |  |  | LD-HE |  |  | LD-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time |
| MRG_PAR01 | -0.2 | 98 | 97 | -0.2 | 101 | 105 | -0.3 | 101 | 97 | -0.3 | 102 | 103 |
| xCheck | -0.2 | 100 | 100 | -0.2 | 100 | 100 | -0.3 | 100 | 101 | -0.3 | 100 | 101 |
| MRG_PAR02 | -1.9 | 104 | 102 | -1.8 | 101 | 94 | -2.6 | 103 | 104 | -2.5 | 105 | 106 |
| xCheck | -1.9 | 103 | 101 | -1.8 | 103 | 101 | -2.6 | 102 | 102 | -2.5 | 102 | 103 |
| MRG_PAR03 | -3.4 | 104 | 120 | -3.5 | 97 | 99 | -4.4 | 99 | 111 | -5.0 | 99 | 123 |
| xCheck | -3.4 | 99 | 102 | -3.5 | 98 | 102 | -4.4 | 97 | 103 | -5.0 | 96 | 104 |
| MRG_PAR04 | -4.0 | 89 | 87 | -4.5 | 96 | 114 | -4.3 | 90 | 88 | -5.9 | 88 | 101 |
| xCheck | -4.0 | 96 | 102 | -4.5 | 94 | 102 | -4.3 | 94 | 104 | -5.9 | 92 | 104 |

## Additional merge mode

### Description of experiments

This category evaluates the coding efficiency and complexity impact of Merge-based MVD transmission. The motion vector derivation process for merge MVD mode is shared with existing merge mode and one MVD is transmitted for this additional merge MVD mode.

Table 8 Description of experiments for additional merge mode

| Test | Description | Related Doc. | Tester1 | Tester2 | SW release |
| --- | --- | --- | --- | --- | --- |
| MRG_MVD01 | five candidates No additional ME | JCTVC-F373 | JVC Kenwood | Fujitsu | 15 Sep. |
| MRG_MVD02 | five candidates Additional ME | JCTVC-F373 | JVC Kenwood | Panasonic | 15 Sep. |
| MRG_MVD03 | one candidate (merge_idx = 0) Additional ME | JCTVC-F373 | JVC Kenwood | Qualcomm | 15 Sep. |

### Summary of experimental results

Table 9 Summary of experimental results for additional merge mode

| Exp. | RA-HE |  |  | RA-LC |  |  | LD-HE |  |  | LD-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time |
| MRG_MVD01 | -0.1 | 105 | 100 | -0.1 | 106 | 100 | -0.2 | 104 | 100 | -0.3 | 104 | 100 |
| xCheck | -0.1 | 105 | 100 | -0.1 | 107 | 99 | -0.2 | 106 | 99 | -0.3 | 106 | 99 |
| MRG_MVD02 | -0.5 | 114 | 100 | -0.5 | 115 | 100 | -0.8 | 111 | 100 | -0.8 | 111 | 100 |
| xCheck | -0.5 | 112 | 100 | -0.5 | 114 | 100 | -0.8 | 109 | 100 | -0.8 | 111 | 101 |
| MRG_MVD03 | -0.4 | 109 | 101 | -0.4 | 110 | 99 | -0.7 | 107 | 100 | -0.7 | 108 | 100 |
| xCheck | -0.4 | 110 | 100 | -0.4 | 112 | 99 | -0.7 | 106 | 101 | -0.7 | 108 | 100 |

## Merge/Skip operations for identical motion

### Description of experiments

This category evaluates the coding efficiency and complexity impact of various methods utilized when motion information are identical. In A of MRG_ID01, collocated picture for L1 motion vector is changed from RefPicList1[0] to RefPicList1[1] if the two reference frames for the collocated MV candidates are identical. In B of MRG_ID01, the same derivation process as in A is performed if both reference frames and MVs of the collocated merge candidates are identical. In MRG_ID02 and MRG_ID03, Non-zero MV from spatial neighbor and different MV from spatial neighbor are utilized respectively when the collocated motion information of a bi-predicted Merge/SKIP PU is identical.

Table 10 Description of experiments for Merge/Skip operations for identical motion

| Test | Description | Related Doc. | Variation | Tester1 | Tester2 | SW release |
| --- | --- | --- | --- | --- | --- | --- |
| MRG_ID01 | MV from different collocated picture | JCTVC-F325 | A: When refPOCs are identical | Toshiba | ETRI/KHU | 13 Sep. |
|  |  |  | B: When both MVs and refPOCs are identical | Toshiba | ETRI/KHU | 13 Sep. |
| MRG_ID02 | Non-zero MV from spatial neighbor | JCTVC-F712 | A: Keep the original refIdxL1Col | ETRI/KHU | Samsung | 15 Sep |
|  |  |  | B: Derive refIdxL1Col from the one related to the found MV | ETRI/KHU | Samsung | 15 Sep |
| MRG_ID03 | Different MV from spatial neighbor | JCTVC-F712 | A: Keep the original refIdxL1Col | ETRI/KHU | LGE | 15 Sep |
|  |  |  | B: Derive refIdxL1Col from the one related to the found MV | ETRI/KHU | LGE | 15 Sep |
| MRG_ID04 | Combination of MRG_ID02 and MRG_ID03 (Non-zero and different MV from spatial neighbor) |  | A: Keep the original refIdxL1Col | ETRI/KHU | Toshiba | 15 Sep |
|  |  |  | B: Derive refIdxL1Col from the one related to the found MV | ETRI/KHU | Toshiba | 15 Sep |
| MRG_ID05 | Combination of the best result from MRG_ID01 and the best result from {MRG_ID02, MRG_ID03, MRG_ID04} |  |  | Toshiba | Canon | withdrawn |

### Summary of experimental results

Table 11 Summary of experimental results for Merge/Skip operations for identical motion

| Exp. | RA-HE |  |  | RA-LC |  |  | LD-HE |  |  | LD-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time |
| MRG_ID01A | 0.0 | 100 | 100 | 0.0 | 100 | 100 | 0.3 | 100 | 99 | 0.3 | 100 | 99 |
| xCheck | 0.0 | 110 | 100 | 0.0 | 107 | 100 | 0.3 | 110 | 99 | 0.3 | 108 | 100 |
| MRG_ID01B | 0.0 | 100 | 99 | 0.0 | 100 | 100 | 0.2 | 100 | 99 | 0.1 | 100 | 100 |
| xCheck | 0.0 | 110 | 100 | 0.0 | 107 | 100 | 0.2 | 110 | 99 | 0.1 | 108 | 99 |
| MRG_ID02A | 0.0 | 100 | 102 | 0.0 | 100 | 102 | 0.2 | 101 | 101 | 0.1 | 101 | 98 |
| xCheck | 0.0 | 104 | 99 | 0.0 | 104 | 101 | 0.2 | 105 | 98 | 0.1 | 106 | 99 |
| MRG_ID02B | 0.0 | 100 | 102 | 0.0 | 100 | 102 | 0.1 | 101 | 99 | 0.1 | 100 | 99 |
| xCheck | 0.0 | 100 | 102 | 0.0 | 100 | 102 | 0.1 | 101 | 99 | 0.1 | 100 | 99 |
| MRG_ID03A | 0.0 | 100 | 103 | 0.0 | 100 | 102 | 0.2 | 99 | 100 | 0.1 | 100 | 100 |
| xCheck | 0.0 | 103 | 101 | 0.0 | 105 | 100 | 0.2 | 105 | 100 | 0.1 | 105 | 99 |
| MRG_ID03B | 0.0 | 100 | 103 | 0.0 | 100 | 102 | 0.2 | 100 | 99 | 0.1 | 100 | 100 |
| xCheck | 0.0 | 104 | 100 | 0.0 | 105 | 100 | 0.2 | 105 | 100 | 0.1 | 106 | 99 |
| MRG_ID04A | 0.0 | 100 | 100 | 0.0 | 100 | 100 | 0.2 | 100 | 98 | 0.1 | 100 | 96 |
| xCheck | 0.0 | 105 | 100 | 0.0 | 105 | 100 | 0.2 | 106 | 98 | 0.1 | 106 | 98 |
| MRG_ID04B | 0.0 | 100 | 100 | 0.0 | 100 | 100 | 0.2 | 101 | 98 | 0.1 | 101 | 97 |
| xCheck | 0.0 | 104 | 99 | 0.0 | 104 | 100 | 0.2 | 105 | 98 | 0.1 | 105 | 98 |

# Conclusions

## General comments

The performed tests can be summarized regarding

- Parallelism

- Simplifications

- Coding efficiency improvements

For simplifications, tests AMVP_SEL01/02 modify AMVP candidates derivation without coding loss while the simplification in AMVP_SEL03 can reduce logic with an average loss of 0.2% for RA, 0.0% for LB and a maximum loss of 0.8% (RA LC, Class C RaceHorses ,BQTerrace) and 2.8% (LB HE BQSquare).

The parallel merge estimation tests MRG_PAR report improvements for parallel merge estimation environments by introducing a syntax element log2_parallel_merge_level_minus2 that defines the level (prediction size) on which parallel merge estimation can be performed.

Coding efficiency improvements between -0.1% and -0.8% BD rate are shown in MRG_MVD tests. This comes along with an increased encoder runtime between 105%-115%,

## Tentative recommendations

- Discuss in BoG what is simplified by AMVP_SEL01/02 compared to HM4.0 and what causes the high losses in AMVP_SEL03/04 for some sequences.

- Recommend MRG_PAR pending on a clarification. Clarification was not confirmed at the time, this report was uploaded.

- Recommended to further study the proposed additional merge mode technique in MRG_MVD tests..

## Tentative recommendation for process during the meeting

- A joint BoG is created for CE9 and CE13 to establish a set of tools generally agreed to be adopted in the CE. CE9 and CE13 recommend adopting this set of tools.

- A first pass consist of evaluating this set of tools together (interaction CE9 / CE13) and produce a first software version.

- A second pass (depending on available time) of the BoG is intended to evaluate on top of this software a selection of non-ce contributions.

# References

- B. Bross, J. Jung, W.-J. Chien, I.-K. Kim and M. Zhou, “Core Experiment 9: MV Coding and Skip/Merge Operations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F909, Jul. 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, Jul. 2011.