| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G040-r3 WG11 Document: m21829 |
| --- | --- |

| Title: | CE10: Summary report on Core transform design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | CE Report |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala FastVDO LLC Madhukar Budagavi Texas Instruments Arild Fuldseth Cisco Rajan Joshi Qualcomm Elena Alshina Samsung Electronics | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-443-455-0394 pankajtva@gmail.com madhukar@ti.com arilfuld@cisco.com rajanj@qualcomm.com elena.alshina@samsung.com |
| Source: | FastVDO LLC, Texas Instruments, Cisco, Qualcomm, Samsung Electronics |  |  |

_____________________________

# 1  Objective

This CE investigates the design of core spatial transforms, to study the tradeoff between coding efficiency and complexity, return evidence and a recommendation for design selection. The anchor for comparison are the transforms in the HM.

# Participant list

| Name | Affiliation | Email |
| --- | --- | --- |
| Pankaj Topiwala Wei Dai Madhu Krishnan Jay Topiwala | FastVDO LLC | pankajtva@yahoo.com daisy@fastvdo.com madhu@fastvdo.com jay@fastvdo.com |
| Madhukar Budagavi Minhua Zhou Vivienne Sze Mangesh Sadafale | Texas Instruments | madhukar@ti.com minhua@ti.com sze@ti.com mangesh@ti.com |
| Arild Fuldseth Gisla Bjontegaard | Cisco | arilfuld@cisco.com gbjeontag@cisco.com |
| Rajan Joshi | Qualcomm | rajanj@qualcomm.com |
| Il-Koo Kim Elena Alshina Felix Fernandes Wonjae Lee Kyo-Hyuk Lee Jihwan Kim | Samsung Electronics | ilkoo.kim@samsung.com elena.alshina@samsung.com felix.f@sta.samsung.com ellio.lee@samsung.com kyohyuk.lee@samsung.com jihwan08.kim@samsung.com |
| Dzung Hoang | Zenverge | Dzung.Hoang@Zenverge.com |
| David Flynn | BBC | davidf@rd.bbc.co.uk |
| Louie Kerofsky Andrew Segall Kiran Misra | Sharp | lkerofsky@sharplabs.com asegall@sharplabs.com misrak@sharplabs.com |
| Jian Lou | Motorola | lou@motorola.com |
| Robert Cohen | Mitsubishi Electric Research Laboratories | cohen@merl.com |
| Tung Nguyen | HHI | tung.nguyen@hhi.fraunhofer.de |
| Ankur Saxena | Samsung Telecommunications America | asaxena@sta.samsung.com |
| TK Tan | NTT DOCOMO, Inc. | tktan.p@rd.nttdocomo.co.jp |
| Daniele Alphonso Sumit Johar | ST Micro | Daniele.alphonso@st.com sumit.johar@st.com |
| Rajan Joshi | Qualcomm Inc. | rajanj@qualcomm.com |
| Xiang Tian | Altera | xtian@altera.com |
| Shevach Riabtsev | CSR (formerly Zoran) | Shevach.riabtsev@csr.com |
| Yeo Chuo Hao | Institute for Infocomm Research | chyeo@i2r.a-star.edu.sg |
| Xiaozhen Zheng | Hisilicon Technologies | zhengxiaozhen@huawei.com |
| Youji Shibahara | Panasonic | shibahara.youji@jp.panasonic.com |
| Michael Horowitz Amir | EBriskVideo | michael@ebriskvideo.com amir@ebriskvideo.com |

# 2  Description of tools

JCTVC-F251, E. Alshina, A. Alshin, I.-K. Kim (Samsung), P. Topiwala (FastVDO), Full factorized core transform proposal by Samsung/FastVDO.

This proposal develops a set of transforms that, it is claimed, support all three forms of implementation: matrix multiplication, partial butterfly and full-factorization, and identical in terms of performance.  This reported equivalence allows choosing the implementation according to platform: i.e. matrix multiplication form may be preferred for SIMD implementations while full-factorization may be preferred for platforms with strict limitations for number of multiplications. By utilizing Loeffler’s factorization in the 32pt transform, it is reported that a 4X reduction in the number of multiplications compared to partial butterfly, anda 12X reduction compared to matrix multiplication,  is achieved.

JCTVC-F352, R. Joshi, Y. Reznik, J. Sole, M. Karczewicz (Qualcomm), Scaled orthogonal integer transforms supporting recursive factorization structure

In this contribution, 8, 16, and 32 point transforms are proposed. These transforms are scaled orthogonal integer transforms supporting a recursive factorization structure. The 4-point transform is unchanged from HM 3.0. The 8, 16 and 32-point transforms use 192, 1152, and 5888 multiplications, respectively, compared to 352, 2752, and 21888 multiplications for the corresponding transforms in HM 3.0. A multiplication-free implementation for the proposed transforms is also provided.

JCTVC-F363, Wei Dai, Madhu Krishnan, Pankaj Topiwala (FastVDO), (FastVDO), Ilkoo Kim (Samsung), FastVDO-Samsung Core Transform Proposal

This submission is an update of E253, a joint proposal from FastVDO and Samsung. This version employs the 4-pt and 8-pt transforms from the Cisco/TI design, currently incorporated in the HM 3.0 software, and 16-pt and 32-pt transforms of FastVDO and Samsung design, which are variants of the designs from E253. It is reported that the compression efficiency of this design is comparable to that of the HM 3.0 transforms, while evidence is provided to indicate that the computational complexity of the larger transforms of size 16 and 32, which dominate the complexity burden, is significantly lower than those of the HM 3.0 design. Thus, it is claimed that this transform architecture provides a favorable tradeoff between complexity and performance.

JCTVC-F446, A. Fuldseth, G. Bjøntegaard (Cisco), M. Sadafale, M. Budagavi (TI), Transform design for HEVC with 16 bit intermediate data representation.

This contribution proposes a set of transform matrices for HEVC, covering all transform sizes from 4x4 to 32x32. The proposed transform design is reported to have the following properties: 16 bit data representation before and after each transform stage (applies to both IBDI off and IBDI on), 16 bit multipliers for all internal multiplications, no need for correction of different norms of basis vectors during quantization/dequantization, all transform sizes above 4x4 can reuse arithmetic operations for smaller transform sizes, and implementations using either pure matrix multiplication or a combination of matrix multiplication and butterfly structures are possible. This experiment will evaluate the coding performance and complexity of the transform.

Proposals to the Geneva meeting based on these are: G737, G579, G266, G495, respectively.

# 3  Core Experiment Tasks

3.1. Coding efficiency

Note: At the Turin meeting, it was agreed that all proposals had effectively identical objective performance, including for low and high qp values. So proposals unchanged from Turin did not need further coding efficiency check, while others can use HM3.0 for comparison.

3.1.1. For proposals that are updated (tentative date: 8/5/11, unnecessary otherwise), measure the RD performance of candidate transform designs using common conditions E700 and HM 3.0 software. If a transform design uses clipping etc. on intermediate variables, this clipping should be included in coding efficiency tests. (For convenience, we are planning to keep the HM3.0 for RD testing purposes.)

3.1.2. Also test at low and high QP values.   Provide BD-Rate results for the following QP sets: 1, 5, 9, 13 and 36, 42, 47, 51 to see if there is unusual behavior at these uncommon test points.  Run this test on 2 sec of video to speed up this test. Number of frames to encode is provided in  table 1 below.

B.1.3 (Optional). Also, report BD-rate performance using “12 bit input” (InternalBitDepth=12) with high efficiency settings. Provide BD-rate results for the low-, normal-, and high QP ranges described in B1.1 and B1.2.

Table 1. Number of test frames for 3.1.2.

| Sequence name | No. frames to encode |
| --- | --- |
| Traffic | 65 |
| PeopleOnStreet | 65 |
| Kimono | 49 |
| Nebuta | 129 |
| SteamLocomotive | 129 |
| ParkScene | 49 |
| Cactus | 97 |
| BasketballDrive | 97 |
| BQTerrace | 129 |
| BasketballDrill | 97 |
| BQMall | 129 |
| PartyScene | 97 |
| RaceHorses (C) | 65 |
| BasketballPass | 97 |
| BQSquare | 129 |
| BlowingBubbles | 97 |
| RaceHorses (D) | 65 |
| Vidyo1 | 129 |
| Vidyo3 | 129 |
| Vidyo4 | 129 |

Cross-check Schedule

| Proponent | Cross-Check Normal Qp | Cross-Check Low/High Qp |
| --- | --- | --- |
| FastVDO/Samsung Lifting (G266) | Motorola (G694) | Qualcomm (G863) |
| Cisco/TI (G495) | X | X |
| Qualcomm (G579) | Samsung (G819) | Samsung (G819) |
| Samsung/FastVDO Versatile (G737) | X | X |

Note: Proposals G495 and G737 had no design updates since Turin meeting, and do not require coding efficiency tests or cross-checks.  Proposals G266 and G579 did have updates.

3.1. Coding Efficiency Results

At the last meeting in Turin, coding efficiency tests strongly indicated that all proposals displayed almost identical coding efficiency performance, so that further coding efficiency tests were valuable mainly if there were any design updates.  That is the reason that HM3.0 was maintained for this test, and “as necessary”.  Of the four designs under test, two of the designs were unchanged (F251 and F446 (HM)), while two designs were updated (F352 and F363).  So it is mainly necessary to check that these updated designs remain very competitive in coding efficiency relative to the others.

3.1.1. Coding Efficiency Results (Normal Qp) for G266

|  | Intra |  |  | Intra LC |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |  |  |  |
| Class A | 0.1 | 0.0 | -0.1 | 0.2 | -0.2 | -0.3 |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | 0.0 |  |  |  |  |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |  |  |  |  |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.1 | -0.1 | -0.1 |  |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |  |
|  |  | Random access |  |  | Random access LC |  |  |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |  |  |  |
| Class A | 0.0 | -0.3 | -0.6 | 0.1 | -0.2 | -0.4 |  |  |  |  |
| Class B | 0.0 | -0.1 | 0.1 | 0.0 | 0.0 | -0.1 |  |  |  |  |
| Class C | 0.0 | 0.2 | -0.2 | 0.0 | 0.0 | 0.0 |  |  |  |  |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | -0.2 | 0.0 |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | 0.0 | -0.1 | -0.2 | 0.0 | -0.1 | -0.1 |  |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |  |
|  | Low delay |  |  | Low delay LC |  |  |  |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |  |  |  |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | -0.2 | 0.0 | -0.1 | -0.6 |  |  |  |  |
| Class C | 0.0 | -0.1 | 0.1 | 0.0 | 0.0 | 0.0 |  |  |  |  |
| Class D | 0.0 | 0.1 | 0.7 | 0.0 | 0.1 | -0.2 |  |  |  |  |
| Class E | 0.0 | -0.5 | -1.0 | 0.0 | 0.6 | -0.4 |  |  |  |  |
| All | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | -0.3 |  |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |  |

3.1.2. Coding Efficiency Results (Normal Qp) for G579

|  | All Intra HE |  |  | All Intra LC |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |  |
| Class A | 0.1% | 0.0% | -0.2% | 0.0% | 0.0% | 0.1% |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |
| Class E | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 106% |  |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |  |
| Class A | -0.1% | -0.1% | 0.3% | 0.1% | 0.0% | 0.1% |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.2% | 0.0% | 0.1% | 0.2% |  |  |  |  |
| Class C | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |  |
| Class D | 0.0% | 0.1% | -0.2% | 0.0% | -0.3% | -0.1% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |  |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 107% |  |  |  |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |  |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.4% | 0.5% | 0.1% | 0.0% | 0.0% |  |  |  |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |  |
| Class D | -0.1% | 0.1% | -0.1% | 0.0% | 0.2% | 0.0% |  |  |  |  |
| Class E | 0.1% | 0.8% | -0.4% | 0.0% | 1.3% | -0.1% |  |  |  |  |
| Overall | 0.0% | 0.3% | 0.1% | 0.1% | 0.3% | 0.0% |  |  |  |  |
|  | 0.0% | 0.3% | 0.0% | 0.1% | 0.3% | -0.1% |  |  |  |  |
| Enc Time[%] | 100% |  |  | 98% |  |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 108% |  |  |  |  |  |  |

## 3.2. Complexity Preliminaries

Analyze the complexity of candidate transform designs using the following parameters:

- Arithmetic operations

- Software execution speed

- Dynamic range

- Parallelizability on SIMD

- Number of sequential operations

- Quantization/dequantization matrices (optional)

- HW

Proponents are encouraged to provide additional complexity analyses voluntarily.

3.2.1. Arithmetic Operations

Report the number of arithmetic operations needed for implementing the 2D inverse transform, in terms of multiplications, additions, shifts, compares. Optionally, multiplications using 16-bit and 32-bit multipliers can be counted separately. If the design allows flexibility in the way it is implemented, report the range of arithmetic operations available (for example, methods with and without multiplications).  Include in the count TPE and final rounding if used.

3.2.1. Arithmetic Operations Results

With direct work in testing in hardware conducted this meeting cycle, arithmetic operations counts, while initially indicative, are somewhat less informative for further comparison. They are provided here for completeness.

a. FastVDO/Samsung (G266)

Table 2. Operations for full 2D transforms.

| Size | Proposed scheme (Full factorization) |  |  | Partial butterfly |  |
| --- | --- | --- | --- | --- | --- |
|  | “*” | “+” | “<<” | “*” | “+” |
| 4 | 40 | 72 | 40 |  |  |
| 8 | 208 | 464 | 208 |  |  |
| 16 | 1184 | 2592 | 1184 |  |  |
| 32 | 5568 | 14656 | 512 | 22016 | 23808 |

b. Cisco/TI (G495)

These transforms can be implemented using matrix multiplication, or using a partial butterfly.

Table 7. Number of arithmetic operations for the 2D inverse transform

|  | Matrix Multiplication |  |  |  | Partial Butterfly |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transform size | Mult, 16 bit | Mult, 32 bit | Add | Shift | Mult, 16 bit | Mult, 32 bit | Add | Shift |
| 4x4 | 128 | 0 | 128 | 32 | 48 | 0 | 64 | 32 |
| 8x8 | 1024 | 0 | 1024 | 128 | 352 | 0 | 448 | 128 |
| 16x16 | 8192 | 0 | 8192 | 512 | 2752 | 0 | 3200 | 512 |
| 32x32 | 65536 | 0 | 65536 | 2048 | 21888 | 0 | 23808 | 2048 |

Table 8. Number of arithmetic operations for 2D inverse transforms of size NxN where only the KxK low frequency sub-block is non-zero (partial butterfly structure)

| NxN | KxK | Multiplications (16-bit) | Add | Shift |
| --- | --- | --- | --- | --- |
| 8x8 | 8x8 | 352 | 448 | 128 |
| 8x8 | 4x4 | 132 | 228 | 96 |
| 16x16 | 16x16 | 2752 | 3200 | 512 |
| 16x16 | 8x8 | 1032 | 1512 | 384 |
| 16x16 | 4x4 | 420 | 820 | 320 |
| 32x32 | 32x32 | 21888 | 23808 | 2048 |
| 32x32 | 16x16 | 8208 | 10320 | 1536 |
| 32x32 | 8x8 | 3400 | 5320 | 1280 |
| 32x32 | 4x4 | 1476 | 3060 | 1152 |

c. Qualcomm (G579)

|  |  | HM 3.0 | Proposed transform |
| --- | --- | --- | --- |
| 8-point | Additions | 448 | 416 |
|  | Multiplications | 352 | 192 |
| 16-point | Additions | 3200 | 2304 |
|  | Multiplications | 2752 | 1152 |
| 32-point | Additions | 23808 | 11904 |
|  | Multiplications | 21888 | 5888 |

d. Samsung/FastVDO (G737)

|  |  | HM |  | G737 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Matrix multiplication | Partial-butterfly | Matrix multiplication | Partial-butterfly | Full-factorization |
| 8-point | Additions | 896 | 448 | 896 | 448 | 464 |
|  | Multiplications | 1024 | 384 | 1024 | 384 | 176 |
| 16-point | Additions | 7680 | 3200 | 7680 | 3200 | 2592 |
|  | Multiplications | 8192 | 2816 | 8192 | 2816 | 992 |
| 32-point | Additions | 63488 | 23808 | 63488 | 23808 | 14656 |
|  | Multiplications | 65536 | 22016 | 65536 | 22016 | 5568 |

## 3.3. Hardware and Software Complexity Tests

## 3.3.1. Preliminaries.

1. Any updates to proposals - by 8/5/11.

2. Test all proposals simultaneously.

3. Quantization to be included in test (decoder side).  Details to be finalized.

4. Test forward and inverse 32-pt.  (Emphasis was on inverse 32-pt transform).

## 3.3.2. HW Test.

a. Test using a Software Tool (Catapult C).

b. fixed set of test frequencies. 150, 250, 500 MHz.  45 nm. (Details tbd).

c. measure gate counts, latencies, throughputs.

For test purposes, we will need cooperation in test configurations and code.

Volunteer. Samsung. (Elena Alshina and team).  Will prepare Samsung code, accept other codes. Run all tests. Will create and distribute cleaned up C code for all proposals.       (9/2/11)

Volunteer:  STMicro (Daniele Alfonso).  Assist in preparing codes, run tests as necessary. Will use Synopsis Synphony (instead of Catapult C).

Volunteer:  TI. (Mangesh Sadafale) Cross-check settings, configurations, input code (review RTL).

3.3.2. HW Test Results.

Our HW tests were the most extensive of all work in this meeting cycle, and some valuable evidence has been obtained.  First, there were two classes of testers:

- independent testers (Altera, and ST Micro).  In particular, ST Micro deserves enormous credit for running extensive tests, as well as in releasing RTL data for further analysis and scrutiny.

- proponent testers (Samsung, and TI).  In addition to providing their own results, Samsung also cross-verified the ST Micro results.

We first present all the results obtained, and then offer some elementary observations.

Remarks:

1. There are effectively three classes of architectures: Matrix Multiplication (MM), Partial Butterfly (PB), and Full Factorization (FF).  The proposed designs then fall into the following categories.

In particular, design F251 offers all three representations.  In referring to proposed designs, we will use both the F and G proposal numbers as convenient, with this table providing the correspondence.

2. Note that for the hw tests regarding the 32-pt transforms, proposals G266 and G737 are identical (same 32-pt transforms), and henceforth in these tests, G266 has identical hw complexity to G737.

3. Most of these hardware tests were conducted with automated RTL generators, while one test was conducted using hand-written RTL. We remark that no testing methodology can be guaranteed to produce  fully fair tests for all proposals, which is always challenging.  One advantage of automated tools is that by nature they are unbiased. While fair tests can also be done in hand-written code, it appears more challenging to systematically enforce it. In the automated tools case, the fairness issue limits to having comparable input codes, which is still an issue, but more manageable. We have every reason to believe our testers, both independent and proponent, did their best to deal with these challenges as best possible.

3.3.2.1.  Altera Results (G263)

Altera compared two proposals for implementation on a specific Altera FPGA, Stratix V SGXEA7 : F251 and F446 (HM), and investigated all transform sizes, and tested both forward and inverse transforms.  For our purposes, the main interest is in the highly complex 32-pt transform, the focus of our hardware tests.  Here the performance metrics of interest are those corresponding to gate count (in this case, Altera is using Logic element count, labeled “logic utilization”), frequency (“Fmax”), and latency.

3.3.2.2.  ST Micro Results Using Synopsis Synphony Automated RTL Generator (G887)

ST Micro has thus completed tests on all design proposals (all 3 different 32-pt transforms).  ST Micro has also released all RTL Codes. (Available to the CE10 group at ftp site at www.fastvdo.com/ce10).  ST Micro's results have been cross-checked in two ways:

- first, the C code and RTLs produced have been shown to represent the same transform.

- the RTLs produced have been resynthesized by Samsung in key cases of L=2, 4, to reevaluate the hardware performance parameters.

3.3.2.3.  Samsung Results

3.3.2.3.1. Independent Resynthesis of ST Micro RTL Code (G737)

3.3.2.3.1. Samsung's Own RTL Results Using Catapult C Automated RTL Generator.

      - Texas Instruments Results Using Hand-Written RTL Code (G495)

TI has also compiled from all available hw test data an indication of the hw preference by ratios of gate counts between different proposals, independent of the latencies involved.  The results were distributed by email, and are reproduced below.

| Minimum area comparison: F251 v/s HM4 (ignoring latency effects) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency (MHz) | Catapult | SynphonyCC | SynphonyCC crosscheck | Hand RTL (TI) | Hand RTL (Samsung) |
| 150 | 75% | 46% | 56% | 61% | 71% |
| 200 | 71% | 52% | 63% | 65% | 78% |
| 250 | 71% | 58% | 72% | 65% |  |
| 300 | 63% | 61% | 62% | 65% |  |
| 350 |  | 67% | 54% | 64% |  |
| 400 |  | 55% | 53% | 64% |  |
| 450 |  |  | 50% | 68% |  |
| 500 |  |  | 52% | 70% |  |
| Minimum area comparison: F352 v/s HM4 (ignoring latency effects) |  |  |  |  |  |
| Frequency (MHz) | Catapult | SynphonyCC | SynphonyCC crosscheck | Hand RTL (TI) | Hand RTL (Samsung) |
| 150 | 62% | 70% |  |  |  |
| 200 | 61% | 85% |  |  |  |
| 250 | 62% | 83% |  |  |  |
| 300 | 55% | 86% |  |  |  |
| 350 |  | 90% |  |  |  |
| 400 |  | 77% |  |  |  |
| 450 |  |  |  |  |  |
| 500 |  |  |  |  |  |

3.3.2.5.  MIT Analysis (G265)

A group at MIT under Prof. Chandrakasan investigated one aspect of the hardware complexity of the G495 proposal – re-usability of transform coefficients.  In considering a straight matrix multiplication implementation (a 4x4 case is diagrammed), and looking at the actual number of unique matrix coefficients (which is less than the total), they are able to achieve up to a 25% reduction in logic as compared to a standard partial butterfly.

| N | HM-4.0 [3] |
| --- | --- |
| 4 | 3 |
| 8 | 7 |
| 16 | 15 |
| 32 | 29 |

This analysis is interesting, but at the moment limited to G495, and limited to the matrix multiplication part of a partial butterfly.  No implications for other proposals, or comparisons, are offered.  Proposal G737 claims to be able to realize similar simplifications based on the number of distinct multipliers.

3.3.2.6. HW Tests: Results Analysis

1. Altera Results

Altera's results indicate that for the important 32-pt transform case, F251 has a lower hw complexity than F446, by a margin of 2X or more, in each direction of transform.

2. ST Micro Results

ST Micro's independent hw analysis has been extensive, and cross-checked for 2 out of 3 proposals, and for two key latencies: L=2 and 4.  ST Micro tested all proposals, as requested by the JCTVC.  ST Micro offers this conclusion (from G887):

“The authors asserts that F251 proposal gives better area for the same performance when compared to HM and F352, as demonstrated by the reported simulation results, obtained without doing any hardware optimizations using SynphonyCC automatic RTL generator tool.”

Since ST Micro's RTLs were generated by an automated tool (Synopsis Synphony), which moreover generates RTLs that are sufficiently optimized for use in products – according to ST Micro's own testimony – these results are potentially also our most reliable.  ST Micro's RTLs have been released for cross-check, which has been carried out by Samsung.

ST Micro's results appear consistent with the results of Altera, that F251 appears to be about 2X less complex for hardware than F446.  The proposal F352 appears to be partially competitive against F446 (except at L=2, where F446 is favored), but less so against F251.  But as high frequencies are achieved only for high latencies for F352, that may make this design somewhat less favored relative to the other two.  It is thought that high frequencies are needed for high-quality, high resolution applications such as for 2K, 4K (based on remarks of hardware engineer Magesh Sadafale, then of TI, at Turin meeting).

3. Samsung's Results

Samsung generated its own RTLs based on its Catapult C automated generator, and tested the two other proposals (F251 and F352) relative to the HM in gate count.

| F-251, by SEC (corrected version) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 | 3 | 4 | 5 |
| 150 | 76% | 70% | 64% | 63% |
| 200 |  | 71% | 65% | 66% |
| 250 |  | 78% | 65% | 65% |
| 300 |  |  |  | 64% |
| 350 |  |  |  |  |
| 400 |  |  |  |  |
| 450 |  |  |  |  |
| 500 |  |  |  |  |
|  | 63% | 78% |  |  |
| F352, by SEC |  |  |  |  |
|  | 2 | 3 | 4 | 5 |
| 150 | 0% | 61% | 54% | 51% |
| 200 | #DIV/0! | 62% | 57% | 56% |
| 250 | #DIV/0! | 0% | 57% | 56% |
| 300 | #DIV/0! | #DIV/0! | #DIV/0! | 57% |
| 350 | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| 400 | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| 450 | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| 500 | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

3.1. Samsung Resynthesis of ST Micro RTLs

For the important latencies of L=2 and L=4, Samsung's resynthesis of ST Micro's RTLs appears to confirm the results of ST Micro on hardware complexity: that F251 is approximately 2X less complex than F446. Samsung is aiming to also run the L=7 case, which is the only remaining instance where there is comparative data between F251 and F446.

3.2. Samsung's Own Results (based on its RTLs)

4. TI's Results.

TI's RTLs were hand-written, providing a different perspective on the hw complexity issue.  While that part is valuable, one issue (besides the known greater challenge of managing fairness) is that the RTLs produced in this case are not released, so that no cross-checks are possible.

Nevertheless, the results for the inverse 32-pt transform are to a large extent consistent with all the other hw results, lending credibility to these results, although the L=2 results appear to be anomalous with respect to other tests.  In this test, F446 appears to be favored over F251 for the case L=2, and the reverse for all other latencies, with the proviso that in this test, for L=4, F446 also appears to allow one more frequency than F251.  One further observation is that for latencies L=4 or more, when considering both forward and inverse transforms together, this test indicates that at least for F446 there are significant advantages.  No results were generated for F251 in this regard.

Note that, as the table shows, since blanks in the table indicate unachievable performance points, one cannot achieve high frequencies for L=2.  Since it is anticipated that high frequencies may be needed for HD+ applications, the case L=2 may not be appropriate for such applications.  In this test, L=6 and L=8 appear to allow higher frequencies. For those cases, it appears that F251 is favorable in terms of gate count.

---------------------------------------------------------

## 3.3.3. SW (SIMD) - Intel Nehalem Platform, Linux OS.

Test

1.  Common Case.  Starting Assumption: complexity in SIMD is a matter of the architecture of the transform - whether it is Matrix Multiply, Partial Butterfly, or Full Factorization.

2.  Test. One of each architecture type: Matrix Mult, Partial Butterfly, and Full Factorization.

3.  Justify the Assumption with evidence.

Example: Test all full factorization 8-pt transforms, show equivalence.

4.  (Optional) Test fast matrix multiplication algorithms (e.g., Strassen) for SIMD implementation.

Volunteer: David Flynn (BBC).

9/2/11.  Test Results:

a. Clock cycles to do each size transform (2D).

b. Code size (bytes) for each transform.

3.3.3. SW (SIMD) Test Results

3.3.3.1.  BBC.

These tests were begun by David as planned, and initially discussed on the conference calls, but not completed.  In CE10 meetings, David expressed the opinion that based on his efforts, matrix multiplication appeared to be disfavored for SIMD implementation.  No definitive results on comparisons have been produced to date.

3.3.3.2.  CSR (formerly Zoran) (G857).

Independent SW SIMD optimization analysis was performed by CSR (formerly Zoran Corp.), by Shevach Riabtsev.  This information has very recently emerged.  He analyzed F251 and F446 in detail.  Quick summary of his results:

- both designs can be made fast enough for applications (can be run 30fps on 300 MHz processor).

Table 2.

| Block Size | # Luma Blocks | # instructions to execute | Time (ms) per frame (300Mhz) |
| --- | --- | --- | --- |
| 4x4 | 129600 | 6220800 | ~20 ms |
| 8x8 | 32400 | 6739200 | ~22 ms |
| 16x16 | 8100 | 9072000 | ~30 ms |
| 32x32 | 2025 | 7192800 | ~23 ms |

- Prefers F251 for hw implementations.

Quoting from G857:

“Upon analyzing of both proposals the author concludes that the proposal JCTVC-F251 is crucially beneficial for VLSI design than the alternative JCTVC-F446. The reason is full-factorization of data flow in both forward and inverse transforms.

This enables low-cost implementation of dedicated Transform Processor and IC-unit.

For illustration let’s consider the signal flowgraph in Fig 1. of JCTVC-F251, the flowgraph reveals the following architecture solution for 32x1 row or column transform:

Notice that Both Top and Bottom Processing units can be divided into individual serially dependent butterfly stages, which leads to an efficient, pipelined hardware implementation.  This is not true of F446, which leads to considerable inefficiencies, and is the main reason that F446 is not preferred.”

3.3.3.3. Cisco and Samsung

3.3.3.3.1. Cisco (G497).  A Cisco-led team investigated the SIMD feasibility of the G495 transforms, obtaining cycle counts on both Intel Clarkdale (Core i5, modest) and Sandy Bridge (Core i7, advanced, 3.46 GHz) processors.  Optimized software was produced. It is found that row-column ordering is superior to column-row ordering.  We provide an example result from this study, estimating the number of cycles needed on a Clarksdale processor to compute a (partial or full) transform of various sizes.  These results appear to indicate that real-time processing is feasible.

Note: no other transform designs were studied. Some work  towards comparison is provided below.

Table 1. Estimated cycle count for partial 2D inverse transforms of various size, Intel Clarkdale

| NxN | KxK | Estimated cycle count - Column-row order | Estimated cycle count - Row-column order |
| --- | --- | --- | --- |
| 8x8 | 8x8 | 183 | 173 |
| 8x8 | 4x4 | 102 | 83 |
| 16x16 | 16x16 | 1036 | 950 |
| 16x16 | 8x8 | 631 | 539 |
| 16x16 | 4x4 | 367 | 275 |
| 32x32 | 32x32 | 7164 | 6781 |
| 32x32 | 16x16 | 3177 | 2799 |
| 32x32 | 8x8 | 1920 | 1512 |
| 32x32 | 4x4 | 1381 | 985 |

3.3.3.3.2. Samsung (G737).  Samsung has undertaken a comparative study of SIMD optimization, for the G737 and  G495 transforms.  Results are obtained for cycle counts on a modest Core i5 processor, 2.66 GHz.  Cycle counts indicate a preference for Full Butterfly (FB, G737) over Partial Butterfly (PB, G495). The biggest difference is in the 32-pt transform, where FB appears to compute about 2X faster (in both cycles and speed).

Table 2. S/w test results.

|  | Time Taken(Secs) |  | Clock cycles/2D tr |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8pt |  |  |  |  |  |  |  |  |  |
| C Mat | 14.4874 |  |  | 3854 |  |  |  |  |  |
| C PB | 3.9302 |  |  | 1045 |  |  |  |  |  |
| C FB | 3.0351 |  |  | 807 |  |  |  |  |  |
| SSE Mat Normal | 2.5762 |  |  | 685 |  |  |  |  |  |
| SSE Mat Fast | 1.0952 |  |  | 291 |  |  |  |  |  |
| SSE PB | 0.8688 |  |  | 231 |  |  |  |  |  |
| SSE FB | 0.8675 |  |  | 231 |  |  |  |  |  |
| 16pt |  |  |  |  |  |  |  |  |  |
| C Mat | 104.9058 |  |  | 27905 |  |  |  |  |  |
| C PB | 23.9085 |  |  | 6360 |  |  |  |  |  |
| C FB | 16.5632 |  |  | 4406 |  |  |  |  |  |
| SSE Mat Normal | 12.6481 |  |  | 3364 |  |  |  |  |  |
| SSE Mat Fast | 6.7924 |  |  | 1807 |  |  |  |  |  |
| SSE PB | 8.2895 |  |  | 2205 |  |  |  |  |  |
| SSE FB | 5.3295 |  |  | 1418 |  |  |  |  |  |
| 32pt |  |  |  |  |  |  |  |  |  |
| C Mat | 793.3536 |  |  | 211032 |  |  |  |  |  |
| C PB | 161.5157 |  |  | 42963 |  |  |  |  |  |
| C FB | 93.7578 |  |  | 24940 |  |  |  |  |  |
| SSE Mat Normal | 66.8819 |  |  | 17791 |  |  |  |  |  |
| SSE Mat Fast | 56.0049 |  |  | 14897 |  |  |  |  |  |
| SSE PB | 54.6074 |  |  | 14526 |  |  |  |  |  |
| SSE FB | 27.2221 |  |  | 7241 |  |  |  |  |  |

Combined Results on SIMD tests so far, as provided by Cisco and Samsung.

| JCTVC-G |  | 737 | 737 | 497 | 497 | 737 | 757 | 990 | 990 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code Proponent |  | Samsung | Samsung | Cisco | Cisco | Cisco | Docomo | Docomo | Docomo |
| Tester |  | Samsung | Cisco | Cisco | Cisco | Samsung | Docomo | Cisco | Cisco |
| Intel Processor Type |  | i7(Sandybridge) | i7(Sandybridge) | i7(Sandybridge) | i7(Sandybridge) | i7(Sandybridge) | i5(Nehalem ) | i7(Sandybridge) | i7(Sandybridge) |
| System |  | 3.4GHz, 4GB | ?? GHz, ?GB | 3.53GHz,16GB | 3.53GHz,16GB | 3.4GHz, 4GB | 2.53 GHz,?GB | 3.53GHz,16GB | 3.53GHz,16GB |
| OS |  | Ubuntu 11.10 | Linux | Linux | Linux | Ubuntu 11.10 | MAC | Linux | Linux |
|  |  | 64 bit | 64 bit | 64 bit | 64 bit | 64 bit | 64 bit | 64 bit | 64 bit |
| SIMD |  | SSE4 | SSE4 | SSE4 | AVX | SSE4 | SSE4 | SSE4 | AVX |
| turbo boost |  | disabled | disabled | disabled | disabled | disabled | enabled | disabled | disabled |
|  |  |  |  | Intel compiler | Intel compiler | Intel compiler | GCC compiler | Intel compiler | Intel compiler |
| 8pt | 8pt |  | 8pt |  |  | 8pt |  |  |  |
| SSE PB |  | 144 | 146 | 138 | 123 | 142 | 158 |  |  |
| SSE FB |  | 167 | 168 |  |  |  |  |  |  |
| 16pt | 16pt |  | 16pt |  |  | 16pt |  |  |  |
| SSE PB |  | 848 | 859 | 836 | 708 | 918 | 861 |  |  |
| SSE FB |  | 890 | 901 |  |  |  |  |  |  |
| 32pt | 32pt |  | 32pt |  |  | 32pt |  |  |  |
| SSE PB |  | 5900 | 5990 | 5329 | 4556 | 5643 | 4669 | 4732 | 3974 |
| SSE FB |  | 4902 | 4937 |  |  |  |  |  |  |

# 4  Timeline  Teleconferences / Milestones

8/5/11.    Discuss code drop, issues. Confirm test plans.

8/19/11.  Discuss partial progress, share intermediate results, and feedback from proponents.

8/26/11.  Any issues in progressing SIMD test.

9/2/11.   Test conclusion and distribution of results. Confirm cleaned up C code dist. for HW test.

9/20/11.  Any feedback from proponents on SIMD.  Completion of HW test and feedback from cross-checkers.

9/20/11 – 11/5/11.  Any discussion.

11/5/11.  Upload of CE Report and Test Results.

4. Actual Teleconferences

Teleconferences, which were sponsored by FastVDO, were held on seven occasions: August 8, 19, 23, 30; September 2, 16; and October 7.

Meeting notes from these conference call were circulated shortly after each call, and will be included in the package.