| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 |  |  | Document: JCTVC-G041 |  |
| --- | --- | --- | --- | --- |
| Title: | CE11: Summary report of Core Experiment on coefficient scanning and coding |  |  |  |
| Status: | Input Document |  |  |  |
| Purpose: | Core Experiment Summary |  |  |  |
| Author(s) or Contact(s): | Vivienne Sze (primary) Tung Nguyen Jianle Chen Joel Sole Krit Panusopone | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: |  | +1-214-567-4005 sze@ti.com +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de +82-31-279-8826 jianle.chen@samsung.com +1-858-845-2429 joels@qualcomm.com +1-858-404-3618 krit@motorola.com |
| Source: | CE coordinators |  |  |  |

_____________________________

# Abstract

This contribution is a summary of core experiment 11, Coefficient scanning and coding.  Fourteen companies have been registered in CE11, and eight tools from seven proposals have been evaluated on the condition defined in document by the Software Ad Hoc Group.  An eighth proposal was withdrawn; a ninth proposal (G301) released software for cross-check more than 6 weeks after deadline and it was agreed that it would be tested outside of CE.

# Introduction

Various methods have been proposed for coding the transform coefficients of the residual signal to reduce complexity and/or improve the coding efficiency. This core experiment evaluates the coding efficiency and complexity impact of:

- Context modeling/selection for syntax elements related to transform coefficients in HE configuration

- Transform coefficient scanning order methods for CAVLC and/or CABAC.

The objectives of this CE include:

- Investigation of the impact of context selection schemes for transform coefficient coding on the coding efficiency and complexity

- Coding efficiency and complexity measurement of transform coefficient scanning order methods

Complexity measures are described in Section 6.2.

# Participants List

| Company | Participants | Proponent | Cross-Checker |
| --- | --- | --- | --- |
| TI | Vivienne Sze, sze@ti.com | X | X |
| Qualcomm | Marta Karczewicz, martak@qualcomm.com Liwei Guo, liweig@qualcomm.com Yunfei Zheng, zhengy@qualcomm.com Xianglin (Shawn) Wang, xianglin@qualcomm.com Joel Sole, joels@qualcomm.com | X | X |
| Bejing Univ. Tech | Wenpeng Ding, wpding@gmail.com |  |  |
| Motorola Mobility | Yue Yu, yyu@motorola.com | X |  |
| Huawei | Jin Song, andy.songjin@huawei.com Haitao Yang, haitao.yang@huawei.com Haoping Yu, haoping.yu@huawei.com | X |  |
| Canon | Chris Rosewarne, Chris.Rosewarne@cisra.canon.com.au |  | X |
| MediaTek | Xun Guo, xun.guo@mediatek.com Shih-Ta Hsiang, shih-ta.hsiang@mediatek.com | X |  |
| Samsung | Ankur Saxena, asaxena@sta.samsung.com Jianle Chen, jianle.chen@samsung.com v.seregin@samsung.com, tammy.lee@samsung.com, yj1003.piao@samsung.com, chanelwind@gmail.com |  | X |
| Mitsubishi | Kazuo Sugimoto, Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp Akira Minezawa, Minezawa.Akira@ds.MitsubishiElectric.co.jp |  |  |
| I2R | Yeo Chuo Hao, chyeo@i2r.a-star.edu.sg |  | X |
| Nokia | Jewon Kang, ext-jewon.kang@nokia.com |  |  |
| Panasonic | Hisao Sasai, sasai.hisao@jp.panasonic.com |  | X |
| Sony | Cheung Ayeung, Cheung.Auyeung@am.sony.com |  | X |
| HHI | Tung Nguyen, tung.nguyen@hhi.fraunhofer.de |  | X |
| LGE | Seungwook.park@lge.com yongjoon.jeon@lge.com jy.park@lge.com |  |  |

# Tools under Test

The following tools will be considered:

| Topic | Proponent | Document | Tool description | Cross-checker |
| --- | --- | --- | --- | --- |
| Context Reduction | TI | JCTVC-F132 | Reduction in context used for significant_coeff_flag and coefficient level | Qualcomm |
| CAVLC Scan | Qualcomm | JCTVC-F612 | Modifications to intra blocks coefficient coding with VLC | I2R |
| CAVLC Scan | Huawei | JCTVC-F501 | Mode dependent coefficient scan for inter blocks | Samsung, Motorola Mobility |
| CABAC Scan | Qualcomm | JCTVC-F550 | Removal of the parsing dependency of residual coding on intra mode | TI, Panasonic |
| CABAC Scan | Qualcomm | JCTVC-F552 | Parallel Processing of Residual Data in HE | Canon |
| CAVLC & CABAC Scan | Motorola Mobility | JCTVC-F569 | Adaptive Scan for Large Blocks for HEVC | HHI |
| CAVLC & CABAC Scan | MediaTek | JCTVC-F124 | Extended Mode-Dependent Coefficient Scanning | Sony |

# List of Contributions

| Proposal Document # | Cross-Verification Document # | Author(s) | Title |
| --- | --- | --- | --- |
| G121/F132 | G327 | V. Sze (TI) | CE11: Reduction in contexts used for coefficient level |
| G679/F612 | G007 | M. Karczewicz, Y. Zheng, L. Guo, X. Wang (Qualcomm) | CE11: Extending horizontal and vertical scan to big block for CAVLC |
| G269/F501 | G708 | J. Song, X. Zheng, H. Yang, H. Yu (Huawei) | CE11 Report on Prediction Unit Dependent Coefficient Scanning For Inter Frame |
| G321/F550 | G124, G302 | J. Sole, Y. Zheng, W.-J. Chien, R. Joshi, X. Wang, M. Karczewicz (Qualcomm) | CE11: Removal of the parsing dependency of residual coding on intra mode |
| G320/F552 | G412 | J. Sole, R. Joshi, M. Karczewicz (Qualcomm) | CE11: Scanning of Residual Data in HE |
| G703/F569 | G975 | Y. Yu, K. Panusopone, J. Lou, L. Wang (Motorola Mobility) | CE11: Adaptive Scan for Large Blocks for HEVC |
| G284/F124 | G364 | X. Zhao, X. Guo, S. Lei (MediaTek), S. Ma, W. Gao (PKU) | CE11: Extended Mode Dependent Coefficient Scanning |

# Summary of Modifications in Proposals

| Doc# | Entropy Coder | Summary of Changes |
| --- | --- | --- |
| G121 | CABAC | Change c1 and c2 thresholds in context selection of coeff_abs_level_greater1_flag & coeff_abs_level_greater2_flag |
| G679 | CAVLC | Use horizontal and vertical scans on intra coded 16x16 and 32x32 TU |
| G269 | CAVLC | Add horizontal and vertical scans to inter coded TU for NSQT |
| G321 | CABAC | Change context selection of significant_coeff_flag to depend on scan position rather than block position Change last_significant_coeff_x and last_significant_coeff_y to always assume diagonal scan |
| G320 | CABAC | Change ordering/grouping of significant_coeff_flag and coefficient level information; Group by 16 or group by TU |
| G703 | CABAC/CAVLC | Additional scans for intra coded 16x16 and 32x32 TU |
| G284 | CABAC/CAVLC | Additional scans for intra coded 16x16 and 32x32 TU |

# Definition of Performance Measurement Criteria

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment.

## Coding Performance Measurements

The objective rate-distortion measurements using 4-point BD-PSNR and BD-Rate [1] for all tools in section 3 are used. Proposals in sub experiment CE. B relating to coefficient scanning methods should be tested with both high efficiency and low complexity configurations.

## Complexity Measurements

For the complexity measurement, the anchor, the reference software and the reference software with the proposed method implemented will be executed on the same machine and the computational time will be measured. A time factor will then be calculated between the reference software and the reference software with the proposed method implemented.

However, the complexity analysis does not solely rely on the encoding and decoding time measurements of a particular implementation (reference software) of the working draft.

Core experiment participants who investigate parallelization aspects are requested to provide a description showing how the parallelization is improved and which architecture(s) would see the benefit of the parallelization. Average and worst case throughput gain from parallelized methods should be considered.

In addition, any tools proposing a different method of scanning are requested to report

- Scheme of selecting the scan pattern during encoding needs to be described.

- Scheme of updating the scan pattern during parsing needs to be described.

# Summary of Results

## Sub Experiment CE.A: Context modeling/selection for transform coefficient related syntax elements

High Efficiency Configurations

| Proposal Document # | Intra | Random Access | Low Delay |
| --- | --- | --- | --- |
| G121/F132 | 0.1 | 0.0 | 0.0 |
| G121/F132 (low QP) | 0.1 | 0.0 | 0.0 |
| G121/F132 (RDOQoff) | 0.0 | 0.0 | 0.0 |

## Sub Experiment CE.B: Coefficient Scanning Methods

|  | High Efficiency |  |  | Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Intra | Random Access | Low Delay | Intra | Random Access | Low Delay |
| G679/F612 |  |  |  | -0.3 | -0.1 | -0.1 |
| G269/F501 |  |  |  |  | 0.0 | -0.1 |
| G321/F550 | 0.2 | 0.1 | 0.1 |  |  |  |
| G320/F552 (Size16) | 0.0 | 0.0 | 0.0 |  |  |  |
| G320/F552 (TUSize) | 0.0 | 0.0 | 0.0 |  |  |  |
| G703/F569 | -0.1 | 0.0 | 0.0 | -0.3 | -0.2 | -0.1 |
| G284/F124 (M0) | -0.2 |  |  | -0.6 |  |  |
| G284/F124 (M1)* | -0.1 |  |  | -0.5 |  |  |

*Not in original document F124 from Torino Meeting.

# Sub Experiment CE.A: Context Modeling/Selection and Summary of CE11 Adoptions and Progress

Since CE11 began at the 3rd meeting, there have been several adoptions to simplify CABAC.  In particular,

- E253 reduced the number of context coded bins in coefficient level (from 14 to 2)

- E227/ D260 reduced the number of contexts in significant_coeff_flag and the complexity in selecting the contexts

- E338 separated information about the last significant coefficient position from significant_coeff_flag to enable easier parallel processing of contexts.

- D239 remove adaptive scan

- F129 diagonal scan

- F288 unified scan for significance maps and coefficient level

All these adoptions enable CABAC in HEVC to achieve higher throughput than in AVC. CE11 has also made progress in improving coding efficiency via adoption of D393, mode dependent coefficient scanning for intra predicted TU.

# References

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.